3GPP TSG-SA WG6 Meeting #69	S6-254529

Wuhan, China 13th – 17th Oct 2025	(revision of S6-25xxxx)

Source:	CATT

Title:	New solution for supporting periodical and event-triggered AIoT service requests

Spec:	3GPP TR 23.700-26 v0.1.0

Agenda item:	9.8

Document for:	Approval

Contact:	Liping Wu <wuliping@cictmobile.com>

1. Introduction

The contribution proposes a new solution to support the periodical and event-triggered AIoT service requests.

2. Reason for Change

Just as the example described in the KI#2, a 3rd-party retail App-server (i.e., VAL server) may issue periodic or event-triggered inventory service requests to check the changed volume of some specific types of AIoT devices in a certain area. When the number of the available AIoT devices of a type drops below a certain threshold, the retail store might run into service interruption of not having sufficient goods for customers. In this scenario, the corresponding retailer is better off replenishing the stock in advance by adding more devices of the type. If the retail store asks for periodic inventory service, it’s expected to be informed when and which type of devices will be increased or decreased.

This solution is proposed to fulfil the above scenario and mainly support the following service requests:

- Periodic inventory service request;

- Report changed volume of some specific types of AIoT devices in a certain area;

- Report when and which type of devices will be increased or decreased.

3. Proposal

It is proposed to agree the following changes for 3GPP TR 23.700-26 v0.1.0.

* * * First Change * * * *

7.x	Solution #x: Support of periodical and event-triggered AIoT service operations

7.x.1	Solution description

7.x.1.1	General

This solution addresses the KI#2.

Just as the example described in the KI#2, a 3rd-party retail App-server (i.e., VAL server) may issue periodic or event-triggered inventory service requests to check the changed volume of some specific types of AIoT devices in a certain area. When the number of the available AIoT devices of a type drops below a certain threshold, the retail store might run into service interruption of not having sufficient goods for customers. In this scenario, the corresponding retailer is better off replenishing the stock in advance by adding more devices of the type. If the retail store asks for periodic inventory service, it’s expected to be informed when and which type of devices will be increased or decreased.

This solution is proposed to fulfil the above scenario and mainly support the following service requests:

- 	Periodic inventory service request;

- 	Event-triggered inventory service request;

- 	Report changed volume of some specific types of AIoT devices in a certain area;

- 	Report when and which type of devices will be increased or decreased.

Clause 7.x.1.2 illustrates the high-level procedure of periodic AIoT inventory service operations.

Clause 7.x.1.3 illustrates the high-level procedure of event-triggered inventory service operations.

7.x.1.2	Procedure of periodic AIoT inventory service operations

Figure 7.x.1.2-1 illustrates the high-level procedure of periodic inventory service request triggered by the VAL server(s).

Pre-condition:

-	The AIoT enabler is the new application enabler in the application enablement layer which supports the AIoT related operations and services. For the functionalities of new AIoT enabler, pls check the related solution for KI#1 for more details.

Figure 7.x.1.2-1: Procedure of periodic AIoT inventory service operations

1.	The VAL server sends an AIoT Inventory subscription request to the AIoT enabler, including the service type indication (e.g., inventory), service ID, target area information, target AIoT device information , triggering conditions (e.g., periodic reporting), time interval for periodic reporting, only reporting the changed volume indicator for AIoT devices, etc.

The information about the target AIoT device(s) may include the Filtering Information as described in clause 5.8 of 3GPP TS 23.369[3].

The indication of only reporting the changed volume for AIoT devices means only the changed volume compared to last report for the specific AIoT device will be reported instead of all of AIoT device IDs.

2.	The AIoT enabler checks whether the VAL server is authorized to request the Inventory subscription request, may be based on e.g., the pre-configurations or the operator policies.

3.	If the request is authorized, the AIoT enabler sends an AIoT Inventory subscription response to the VAL server.

4.	The AIoT enabler invokes Nnef_AIoT_Inventory request periodically to the 3GPP CN to obtain the AIoT device data as specified in clause 6.2.2 of 3GPP TS 23.369 [3]. And the AIoT enabler will set the time interval for the periodical Nnef_AIoT_Inventory request less than or equals to the time interval received in Step 1.

5.	The AIoT enabler stores the received AIoT device information and analyse them based on the AIoT Inventory subscription service request. The AIoT enabler may classify the AIoT devices based on the requested AIoT Device(s) filtered information (e.g., EPC ID), count the number for each type of AIoT devices (including maximum, minimum and average granularity), compare the number for two sequential times for the same target AIoT device in the same target area, and then calculate the corresponding changed information including e.g., the change is increase/decrease, the changed number/volume, the specific AIoT device type corresponding to the changes, etc.

6.	The AIoT enabler will periodically report the AIoT device information to the VAL server based on the time interval received in Step 1. The report may include the AIoT device ID, the number of AIoT devices which may be all devices or the devices filtered based on different criteria (e.g., PLMN, third-party ID, EPC ID, etc.), the changed information compared to the previous report (e.g., the change is increase/decrease, the number/volume of change, etc.), the time point of the report, the last report indication, etc.

NOTE 1:	Whether the AIoT enabler reports the changed information depends on the indication of only reporting the changed volume for AIoT devices in Step 1 is included or not.

NOTE 2:	Reporting of changed information is only applicable to the second and subsequent notification messages.

7.x.1.3	Procedure of event triggered AIoT inventory operations

Figure 7.x.1.3-1 illustrates the high-level procedure of event triggered AIoT inventory operations which is based on the periodic AIoT inventory operations described in clause 7.x.1.2.

Pre-condition:

-	The AIoT enabler is the new application enabler in the application enablement layer which supports the AIoT related operations and services. For the functionalities of new AIoT enabler, pls check the related solution for KI#1 for more details.

Figure 7.x.1.3-1: Procedure of event triggered AIoT inventory operations

1.	Similar with Step 1 in clause 7.x.1.2. The difference is the triggering condition is not the periodic reporting but the event-triggered reporting, and the event is e.g. when the number of one type of AIoT devices is going to reach the pre-defined threshold.

2.	Similar with Step 2 in clause 7.x.1.2.

3.	Similar with Step 3 in clause 7.x.1.2.

4.	The AIoT enabler may set the time interval to trigger the periodic AIoT inventory operations described in clause 7.x.1.2 to obtain the target AIoT devices information.

5.	The AIoT enabler stores the obtained AIoT devices information and classifies the AIoT devices based on the requested filtered information (e.g., EPC ID), count the number for requested type of AIoT devices, and then compare the number with pre-defined threshold received in Step 1. If the number of certain AIoT devices are going to reach the pre-defined threshold, the AIoT enabler may immediately notify the VAL server to indicate the number of specific AIoT devices will reach the pre-defined threshold, with the AIoT device ID, the total number, the trigger events, the suggested actions (e.g., increase/decrease the number of specific AIoT devices), etc.

If the event trigger condition is met, the AIoT enabler will no longer trigger the periodic AIoT inventory request to the 3GPP CN.

6.	Similar with Step 7 in clause 7.x.1.2. The AIoT enabler notifies the VAL server of the AIoT device information. The notification may include the target AIoT device ID, the number of target AIoT devices, the trigger events, and the suggested actions (e.g., increase/decrease the number of specific AIoT devices), the time point of the report, etc.

7.x.2	Architecture Impacts

This solution proposes a new architecture (i.e., new AIoT enabler) to support AIoT services.

For the new architecture, pls check the related solution for KI#1 for more details.

7.x.3	Corresponding APIs

This clause provides the corresponding APIs for supporting the solution.

7.x.4	Solution evaluation

This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

* * * End of Change * * * *