3GPP TSG-SA WG6 Meeting #69	S6-254530

Wuhan, China 13th – 17th Oct 2025	(revision of S6-25xxxx)

Source:	CATT

Title:	New solution on support of querying history data for AIoT devices

Spec:	3GPP TR 23.700-26 v0.1.0

Agenda item:	9.8

Document for:	Approval

Contact:	Liping Wu <wuliping@cictmobile.com>

1. Introduction

The contribution proposes a new solution to support the querying history data requests for AIoT devices.

2. Reason for Change

Some application servers (e.g., a super market) may need to check the history data for some specific type of AIoT devices (e.g., check the sales volume of umbrellas from last year) to predict the trends for this year and beyond. So it’s necessary for application servers to query the history data for the AIoT devices in the application enabled layer.

This solution is proposed to fulfil the above scenario and mainly support the following service requests:

- 	Query the history data (e.g., device number, device location info, etc.) for AIoT devices;

- 	Report the history data to the application server.

3. Proposal

It is proposed to agree the following changes for 3GPP TR 23.700-26 v0.1.0.

* * * First Change * * * *

7.x	Solution #x: Support of querying history data for AIoT devices

7.x.1	Solution description

7.x.1.1	General

This solution addresses the KI#2.

Some application servers (e.g., a super market) may need to check the history data for some specific type of AIoT devices (e.g., check the sales volume of umbrellas from last year) to predict the trends for this year and beyond. So it’s necessary for application servers to query the history data for the AIoT devices in the application enabled layer.

This solution is proposed to fulfil the above scenario and mainly support the following service requests:

- 	Query the history data (e.g., device number, device location info, etc.) for AIoT devices;

- 	Report the history data to the application server.

7.x.1.2	Procedure of querying history data for AIoT devices

Figure 7.x.1.2-1 illustrates the high-level procedure of querying history data for AIoT devices.

Pre-condition:

-	The AIoT enabler is the new application enabler in the application enablement layer which supports the AIoT related operations and services.

-	The AIoT enabler support to store the history data for requested AIoT devices.

-	The VAL server doesn’t have the requested AIoT history data.

Figure 7.x.1.2-1: Procedure of querying history data for AIoT devices

0.	The AIoT enabler may interact with 3GPP CN to query the AIoT devices data periodically as described in solution S6-254529 and store the obtained AIoT device data.

1. The VAL server sends an AIoT history data request to the AIoT enabler, including the query service type (e.g., device location, device number), service ID, target area information, target AIoT device information, and target time point/period, etc.

Information about the target AIoT Device(s) may include Filtering Information as described in clause 5.8 of 3GPP TS 23.369 [3].

2.	The AIoT enabler checks whether the VAL server is authorized to request the AIoT history data request may be based on e.g. the pre-configurations or operator policies.

3.	If the request is authorized, the AIoT enabler checks if the stored sensing data in the local storage could meet the service requirements received in step 1.  If yes, the AIoT enabler may report the requested history data and perform the statistical analysis for the retrieved information. E.g., if the query type is device number, the report may include the total number for the specific AIoT devices as well as the maximum/minimum/average number. If there is no local storage or the local storage cannot fulfil the service requirements, the AIoT enabler may response with the failure cause.

4.	The AIoT enabler reports the history data for the requested AIoT device to the VAL server via sending AIoT history data response message. The report may include the AIoT device ID, the retrieved history data for the AIoT device, and related statistical analysis for the retrieved data, etc.

7.x.2	Architecture Impacts

This solution proposes a new architecture (i.e., new AIoT enabler) to support AIoT services.

For the new architecture, pls check the related solution for KI#1 for more details.

7.x.3	Corresponding APIs

This clause provides the corresponding APIs for supporting the solution.

7.x.4	Solution evaluation

This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

* * * End of Change * * * *