3GPP TSG-SA WG6 Meeting #69	S6-254546

Wuhan, China, 13th – 17th October 2025                                                    was S6-254178

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.433 | CR | 0167 | rev | 1 | Current version: | 19.7.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification related to Multi-modal Service ID |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2_App |  |  |  |  |  | Date: |  |  | 2025-10-6 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In S6-245678, Multi-modal flows alignment and monitoring feature was introduced. As part of the CR, SEALDD policy configuration request information flow was updated to include Multi-modal flows alignment policy attribute, that contained Multi-modal Service ID. This procedure and the corresponding information flow implies that VAL Server provides the Multi-Modal Service ID to SEALDD Server. However, in S6-250570, where SEALDD enabled XR data transmission establishment procedure is updated which implies SEALDD server determines the Multi-modal Service ID based on VAL service ID and/or VAL server ID. Hence, there is misalignment between the procedure on who assigns the Multi-modal Service ID, which is addressed in this CR. The SEALDD server includes the Multi-modal Service ID in the AF request sent to 5GC in the SEALDD enabled multi-modal flow synchronization procedure as well, which is not indicated in the current specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - SEALDD enabled multi-modal data transmission establishment procedure is clarified that SEALDD server receives the Multi-modal Service ID from the VAL server and the corresponding Information flow (clause 9.12.3.1) is updated. - SEALDD enabled multi-modal flow synchronization procedure is updated to indicate that SEALDD server includes Multi-modal Service ID in the AF request sent to 5GC. - Additional typo corrections in the Figure 9.12.2.1.2-1 and clause 9.13.3.1 (model->modal). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment between SEALDD policy configuration procedure and SEALDD enabled multi-modal data procedures. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.12.2.1.2, 9.12.2.2.2, 9.12.3.1, 9.13.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 9.12.2.1.2	SEALDD enabled multi-modal data transmission establishment

Figure 9.12.2.1.2-1 illustrate the procedure for establishing multi-modal data transmission connection, and the SEALDD facilitates the multi-modal application to transmit its data between the VAL client and VAL server with RTP packetilization and PDU set inclusion.

Pre-condition:

-	The VAL server has discovered and selected the SEALDD server by CAPIF functions.

Figure 9.12.2.1.2-1: SEALDD enabled multi-modal data transmission connection establishment procedure

1.	The VAL server decides to use SEALDD service for multi-modal traffic transfer and allocates address/port as SEALDD-S Data transmission connection information for receiving the data packets from SEALDD server. The VAL server sends Sdd_multi-modalTransmission request to the SEALDD server discovered by CAPIF. The service request includes UE ID/address, VAL server ID, VAL service ID, list of requested flows, including SEALDD-S Data transmission connection information of the VAL server side, and optionally, the protocol description, the QoS information for the application traffic, e.g. QoS requirements. As one of key parameters for multi-modal application, Crossflow measurement can be measured as procedure in clause 9.7.2.2 step 1.

2.	Same as step 2 of clause 9.2.2.2. For application multi-modal service, the SEALDD server derives PDU Set related assistance information based on received VAL service ID and/or VAL server ID, and protocol description for interacting with NEF/PCF. The SEALDD server may send the AF request to provide the required QoS information to 5GC via N33/N5 for each requested flow provided in step 1. The AF request may also include the Multi-modal Service ID as received from the VAL server, or may be determined by the SEALDD server according to VAL service ID and/or VAL server ID to ensure uniqueness of Multi-modal Service ID.

3-5.	Same as step 3-5 of clause 9.2.2.2.

6.	The SEALDD client establishes multi-modal transmission connection with the SEALDD server. The request includes the SEALDD client ID, VAL user/UE ID, VAL server ID, VAL service ID, SEALDD-UU flow IDs, and traffic descriptors for the multiple flows from the SEALDD client side. The SEALDD server sends the SEALDD traffic descriptors for multiple flows from SEALDD server side (e.g. address/port for multiple SEALDD-UU flow) to the SEALDD client, and may send the protocol description (UL related info) received from the VAL server to the SEALDD client in the multi-modal transmission connection response. The response also includes a multi-modal SEALDD flow ID which the SEALDD server allocates. And the SEALDD client policy may also be included in its response. The SEALDD client shall then perform authorization checks and reply with a policy configuration response as specified in clause 9.10.2.4.

7-8.	Same as step 8-9 of clause 9.2.2.2.

NOTE 2:	If multiple requested flows are provided in step 1, the SEALDD client repeats steps 6-8 for each requested flow provided in step 1.

The multi-modal application traffic is exchanged between VAL client and VAL server via SEALDD layer as described in clause 9.2.2.2. If packetization indication indicates that SEALDD layer needs to perform packetization, the SEALDD server performs packetization and sends streaming data (e.g. RTP packet) via SEALDD-UU user plane (e.g. SEALDD/UDP/IP) to the SEALDD client for downlink application traffic. Similarly, the SEALDD client performs packetization and sends streaming data (e.g. RTP packet) via SEALDD-UU user plane (e.g. SEALDD/UDP/IP) to the SEALDD server for uplink application traffic. The SEALDD server and client also perform PDU Set inclusion (e.g. in RTP extension as defined in 3GPP TS 26.522 [TS26522]), and if needed, stream session and transport management (e.g. RTCP, RTSP).

* * * Next Change * * * *

##### 9.12.2.2.2	SEALDD enabled multi-modal flow synchronization

Figure 9.12.2.2.2-1 illustrate the procedure for SEALDD enabled multi-modal flow synchronization, the SEALDD server determines/updates the required QoS information for multi-modal flow(s), and further interacts with 5G network.

Pre-condition:

-	The VAL server has discovered and selected the SEALDD server by CAPIF functions.

-	The SEALDD server has been provisioned with a multi-modal SEALDD policy including the synchronization threshold, as specified in clause 9.10.3.1.

-	The SEALDD client has been provisioned with a multi-modal SEALDD policy, as specified in clause 9.10.2.4.

Figure 9.12.2.2.2-1: SEALDD enabled multi-flow synchronization procedure

1.	An on-going multi-modal data transmission connection is established according to the steps 1-8 of clause 9.12.2.1.2.

2.	Upon the multi-modal flows alignment policy triggered, the SEALDD server may help to provide the flows alignment assistance information (e.g. timestamp in the RTP header, RTCP) to the SEALDD client. If the maximum acceptable duration for traffic flow alignment is not provided, then the SEALDD server may determine the maximum acceptable duration for traffic flow alignment based on VAL service ID, flows transmission requirement, transmission quality, and the synchronization threshold. The server may translate the traffic descriptor into multi-modal SEALDD flow ID.

The SEALDD server may communicate with the 5GC to get the analytics and prediction for the RAN congestion as per clause 6.6, clause 6.7, and clause 6.8 3GPP TS 23.288 [19]. Based on the RAN congestion analytics and prediction information, synchronization threshold and/or multi-modal flows alignment policy, the SEALDD server derives the delay introduced by the RAN and adjusts the flow alignment assistance information like DL/UL transmission timing advance or delay period. In case flow alignment is required, the flow alignment assistance information may contain UL/DL transmission timing advance period, UL/DL transmission timing delay period and the associated flow IDs in the multi-modal flows. The UL flow alignment assistance information may be sent to the SEALDD client in the Transmission quality management request specified in clause 9.9.3.1.

NOTE 1:	The flow alignment assistance information can be obtained by SEALDD server based on SEALDD policy.

NOTE 2:	The SEALDD client performs the caching and transmission to align multi-modal flows based on the flow alignment assistance information and maximum acceptable time duration.

NOTE 3:	The NTP timestamp and RTP timestamp in RTCP sender report (SR) can be used to identify the associated packets among multi-modal flow, and further be used to perform alignment in SEALDD client.

NOTE 4:	The option to rely on UE congestion analytics and RAN congestion analytics, as exposed by the 5GC via NWDAF has a margin of error, as with any predictions. To handle the real-time/near real-time constraint of these predictions, these UE congestion analytics can be requested ahead of time from the 5GC via NWDAF, e.g., once the XR session is started by the UE.

3.	Upon the multi-modal flows alignment policy triggered, the SEALDD client initiates the multi-modal flows alignment based on the policy. The flows need to be aligned are identified by the VAL service ID, multi-modal SEALDD flow ID, and flow alignment assistance information.

If the flow alignment assistance information is provided, the SEALDD client identifies the associated packets (e.g., those with the same RTP timestamp) in the multi-modal flows. After all associated packets in the multi-modal flows have arrived, the SEALDD client sends the associated packets to the application client. If the maximum acceptable time duration is provided, once this maximum acceptable time is reached, the SEALDD client will no longer wait for the associated packets in multi-modal flows, even if they have not arrived yet.

For the UL synchronization, the SEALDD client sends the UL packets as per the flow alignment assistance information. The UL transmission timing advance period indicates SEALDD client to advance the UL packet transmission by advance period. The UL transmission timing delay period correction mode indicates SEALDD client to delay the transmission of the UL packet by UL delay period units. After all associated packets in the multi-modal flow have arrived at the SEALDD server, then the SEALDD server sends the associated UL packets to the VAL server.

4.	The SEALDD server performs data transmission quality measurement, as defined in clause 9.7.2.1 or clause 9.7.2.3, in SEALDD-UU interface based on the mapping information for multiple flow association information between SEALDD-S interface and SEALDD-UU interface. Upon receiving the packets from multiple associated flows in SEALDD-S interface, the SEALDD server performs the packet encapsulation with sending timestamp information in the corresponding SEALDD-UU interface, and calculates the transmission delay measurement result of multiple associated flows after obtaining the receiving timestamp from the SEALDD client. As one of key parameters for XR application, delay difference can be measured as procedure in clause 9.7.2.3 step 10.

5.	Based on the data transmission quality measurement results obtained for multiple associated flow over SEALDD-UU interface in step 4, and the synchronization threshold for multi-modal application as described in pre-condition, the SEALDD server determines the service flow(s) (i.e. address/port for SEALDD-UU flow) that needs to be adjusted among the multiple associated flows in SEALDD-UU interface, and the corresponding required QoS information (i.e. transmission delay). E.g., the SEALDD server evaluates whether the difference of the packet delay measurements for the flows that need to be in synchrony is above the synchronization threshold, and if it is so, the SEALDD server takes the corresponding corrective actions. The SEALDD server may advance or delay the transmission of the DL packets according to the flow assistance information to maintain the synchronization of DL flows. The DL transmission timing advance period correction mode indicates SEALDD server to advance the DL packet transmission by advance period. The DL transmission timing delay period correction mode indicates SEALDD server to delay the transmission of the DL packet by DL delay period units.

The SEALDD server may further update the flow alignment assistance information like DL/UL transmission timing advance or delay period to fine-tune as per the data transmission quality measurement results.

6.	The SEALDD server sends the AF request to 5GC via N33/N5 with the SEALDD traffic descriptor of the adjusted flow(s) (i.e. address/port for the adjusted SEALDD-UU flow, Multi-modal Service ID) and the corresponding required QoS information determined in step 5, by utilizing the AF session with required QoS procedure in clause 4.15.6.6 of TS 23.502 [6]. The SEALDD traffic descriptor of the adjusted flow(s) contains the address or port in SEALDD server side, and/or SEALDD client side.

NOTE 5:	This procedure is applicable for both downlink and uplink synchronization of multi-modal flow. For downlink and/or uplink, the step 5 is determined according to the measured downlink and/or uplink data transmission quality measurements in step 4. For uplink synchronization, the SEALDD server performs the caching and transmission to align multi-modal flows based on the flow alignment assistance information before sending the associated packets to the VAL server.

After requesting the transmission quality optimization on 5G network with the required QoS for the adjusted flow(s), the multi-flow synchronization of multi-modal application is satisfied.

* * * Next Change * * * *

#### 9.12.3.1	SEALDD enabled multi-modal transmission request

Table 9.12.3.1-1 describes the information flow from the VAL server to the SEALDD server for requesting multi-modal transmission service.

Table 9.12.3.1-1: SEALDD enabled multi-modal transmission request

| Information element | Status | Description |
| --- | --- | --- |
| VAL server ID | M | Identity of the VAL server |
| VAL service ID | O | Identity of the VAL service |
| Identity | O | Identifier of specific UE or VAL user |
| Multi-modal Service ID | O | Identifier of the Multi-modal Service |
| List of requested flows | M | List of the requested multi-modal flows |
| >SEALDD-S Data transmission connection information for multi-modal XR | M | Address(s)/port(s) and/or URL(s) of the VAL server to receive the multi-flow packets from the SEALDD server (e.g. audio flow, tactile flow) |
| >QoS information | O | QoS information provided by VAL server |
| >Protocol description | O | The protocol description of VAL traffic. It includes header extension information (e.g. RTP extension with PDU set), packetization indication, payload type and format (e.g. H.264/RTP, H.265/RTP, H.264, H.265). Header extension information is only applicable when payload indicates RTP. |

* * * Next Change * * * *

#### 9.13.3.1	SEALDD adaptive data transmission request

Table 9.13.3.1-1 describes the information flow from the consumer to the SEALDD server for requesting or updating request for an adaptive data transmission.

Table 9.13.3.1-1: SEALDD adaptive data transmission request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor ID | M (NOTE) | The identifier of the consumer (e.g. VAL server ID, SEALDD client ID, VAL client ID). |
| VAL service ID | O | Identity of the VAL service. |
| VAL UE ID(s) | O | Identifier(s) of the endpoints (i.e. VAL UE(s)). |
| Adaptive data assign Indicator | O | Indicate that the data can be assigned to different endpoints adaptively. |
| Data information | M | The information of the data needs to be delivered or distributed. |
| >Data type | O | The type of data, could be e.g. image, video, multi-modal data (e.g. audio, video, positioning, haptic data). |
| >Data size information | M | The maximum/minimum/average size of the data or a range of size for the data (a pair of minimum and maximum data volumes) to be delivered/distributed. |
| >Time duration | M | The time duration (start and end time) expected for sending the volume of data. |
| QoS requirement | O | The QoS requirement, e.g. latency for data transmission. |
| Assistance information | O | The assistance information for deliver/distribute the data, e.g. time point/time window for the data transmission to the split AI/ML endpoints. |
| NOTE: This information element shall not be updated. |  |  |

* * * End of Change * * * *