3GPP TSG-SA WG6 Meeting #69	S6-254622

Wuhan, China 13th – 17th Oct 2025	(revision of S6-25xxxx)

Source:	CATT

Title:	New Solution on new architecture for sensing services in application enablement layer

Spec:	3GPP TR 23.700-15 v0.1.0

Agenda item:	9.13

Document for:	Approval

Contact:	Liping Wu <wuliping@cictmobile.com>

1. Introduction

The contribution proposes a new solution to set up a new architecture for sensing services in application enablement layer.

2. Reason for Change

The use cases and scenarios for the sensing services are very rich (such as UAV detection, HD map, V2X), it’s necessary to set up a generic new architecture and functional model to describe how to obtain and store the sensing results and then generate the value-added sensing results for different sensing related use cases.

3. Proposal

It is proposed to agree the following changes for 3GPP TR 23.700-15 v0.1.0.

* * * First Change * * *

5.x	Solution #x: Support of exposure of sensing results

This solution addresses the KI#1.

5.x.1	Procedure of exposure of sensing results

This solution proposes a new enabler (i.e., Sensing Enabler Server) to support the exposure of sensing results to the consumers (e.g., VAL server). The Sensing Enabler server may interact with other SEAL enabler to get the value-added sensing results.

Figure 5.x.1.1-1 illustrates the high-level procedure of exposure of sensing results.

Figure 5.x.1.1-1: Procedure of exposure of sensing results

- The VAL server sends a sensing service request to the Sensing Enabler server, including the service type (e.g., UAV detection, dynamic HD map, Vehicle tracking, etc.), service ID, sensing target object information (e.g., UAV ID, UE ID, Vehicle ID), sensing service requirements for the target object (e.g., the target area, sensing accuracy, sensing resolution, max sensing service latency, etc.), the reporting conditions (e.g., periodic reporting or event-triggered reporting), the time duration for the sensing, etc.

2.	The Sensing Enabler server checks whether the VAL server is authorized to invoke such request, may be based on e.g. the pre-configurations or operator policies.

3.	If the request is authorized, the SM server may invoke the sensing related service request to the 3GPP CN and obtain the sensing results from the 3GPP CN.

Editor’s note:	The exposure of sensing results from 3GPP CN depends on SA2’s progress. What will be exposed to the consumers is FFS.

4.  The SM server may interact with other SEAL enablers (e.g., SEALDD, ADAES) to get more sensing results (e.g., the transmission latency, sensing accuracy analysis) utilizing the obtained sensing results from the 3GPP CN, in case the 3GPP core network will not provide.

-	The SM server may interact with ADAES to get the e.g., sensing accuracy analysis and the prediction analysis for the target object utilizing the obtained sensing results from the 3GPP CN;

-	The SM server may interact with SEALDD to get the e.g., the transmission latency for the target object utilizing the obtained sensing results from the 3GPP CN.

Editor’s note:	How the ADAES and SEALDD support the sensing related functionalities are FFS.

5.	The SM server may analyse and aggregate the sensing results obtained in step 3 and step 4 to generate the value-added sensing results (e.g. the statistic results in a location/time/direction/height granularity), and then expose to the VAL server if the results can fulfil the requested sensing service requirements.

6.	The SM server sends the value-added sensing results to the VAL server via sensing service response message, including all of target objects and the value-added sensing information.

5.x.2	Solution evaluation

This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

* * * End of Change * * * *