3GPP TSG-SA WG6 Meeting #69	S6-254686

Wuhan, P.R.China, 13th – 17th October 2025	revision of S6-254236

Source:	Ericsson, KPN N.V.

Title:	New Solution for KI#4 on Use Sensing Results to Enhance HD Map for V2X Services

Spec:	3GPP TR 23.700-15 v0.1.0

Agenda item:	9.13

Document for:	Approval

Contact:	Jing Yue <jing.yue@ericsson.com>

1. Introduction

As described in Key Issue #4 (TR 23.700-15 clause 4.4), the study aspects on utilizing sensing results for HD map in V2X include:

- How to enhance VAE layer to provide enhanced HD map including traditional vehicles information with considering sensing results.

- How to enhance VAE layer to provide enhanced HD map including environment information (e.g. pedestrian/animal objects, etc) with considering sensing results.

Sensing plays an important role in smart transportation. Most modern cars are equipped with onboard sensors – radar, lidar, and cameras just to mention a few – which help the car to get an overview of its immediate surroundings. However, onboard systems fail to provide information further away and cannot “see around the corner”. An sensing system can complement onboard sensory data with more “global” data that cannot be obtained by sensors mounted on the vehicle. Another automotive-related example is the detection of pedestrians or animals on highways.

Application enablement layer can aggregate the sensing demands of a large number of vehicles, enabling unified subscription to sensing data from different sensing entities. By tracking or predicting vehicle locations in real time, this layer transforms the geographically scoped sensing data into a vehicle-centric dynamic view, and simplifies the vehicle's data consumption process and significantly enhances processing efficiency and system responsiveness.

Thus, this pCR proposes to use sensing results from Sensing Application Enablement service (SAE, which is introduced in S6-254233) to enhance HD map for supporting V2X services.

2. Reason for Change

To address the study aspects in Key Issue #4, this pCR proposes a new solution on use sensing results to enhance HD map for supporting V2X services.

3. Conclusions

This paper proposes solution on use sensing results to enhance HD map for supporting V2X services.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-15 v0.1.0.

* * * First Change * * * *

## 6.1	Mapping of solutions to key issues

Table 6.1-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 |
| --- | --- | --- | --- | --- |
| Sol #1 |  |  |  |  |
| Sol #2 |  |  |  |  |
| Sol #X3 |  |  |  | X |

* * * Next Change * * * *

## 6.X3	Solution #X3: Use Sensing Results to Enhance HD Map for V2X Services

### 6.X3.1	Solution Description

#### 6.X2.1.0	General

Sensing plays an important role in smart transportation. Most modern cars are equipped with onboard sensors – radar, lidar, and cameras just to mention a few – which help the car to get an overview of its immediate surroundings. However, onboard systems fail to provide information further away and cannot “see around the corner”. An sensing system can complement onboard sensory data with more “global” data that cannot be obtained by sensors mounted on the vehicle. Another automotive-related example is the detection of pedestrians or animals on highways.

Application enablement layer can aggregate the sensing demands of a large number of vehicles, enabling unified subscription to sensing data from different sensing entities. By tracking or predicting vehicle locations in real time, this layer transforms the geographically scoped sensing data into a vehicle-centric dynamic view, and simplifies the vehicle's data consumption process and significantly enhances processing efficiency and system responsiveness.

The following clauses specify use sensing results to enhance HD map for supporting V2X services.

#### 6.X2.1.1	Procedure

Figure 6.X2.1.1-1: Sensing result subscription for enhance HD map

- The Consumer (e.g., VAL server/client, VAE server/client) sends sensing result subscription request to the SEAL Sensing Server for device-centric sensing result. The request contains the context information, e.g. planned route of the vehicle.

- The SEAL Sensing Server authenticates the request, checks the stored sensing data, and determines the next actions (e.g., to collect sensing data from sensing entities, to collect device location information/analytics, to generate device-centric sensing result).

- The SEAL Sensing Server responds to the VAL server for the sensing result subscription request.

- The SEAL Sensing Server interacts with the 5GC NF (e.g. NEF) for receiving notifications with the sensing data from the 3GPP network. The SEAL sensing server determines whether the sensing result fulfils the request from the consumer provided in step 1, and determines whether additional sensing data is needed to enhance the sensing result.

- The SAE Server interacts with the 3rd party sensing server for receiving the sensing data from the 3rd party sensing data.

Editor’s Note:	Whether the revision of the architecture is needed for the 3rd party sensing server is FFS.

- After receiving the sensing data from the sensing entities in steps 4-5, the SEAL Sensing Server interacts with the other SEAL servers for collection of location information/analytics (e.g., LMS for device location information, ADAES for device location analytics).

- The SEAL Sensing Server filters the sensing data and location information/analytics, to generate device-centric sensing results.

- The SEAL Sensing Server sends sensing result subscription notifications the Consumer with the generated device-centric sensing results.

- The Consumer may use the sensing result for e.g., create/update HD map.

#### 6.X3.1.2	Information Flows

Editor’s Note:	The information flows of the solution is FFS.

### 6.X3.2	Architecture impacts

Editor’s Note:	The architecture impacts of the solution is FFS.

### 6.X3.3	Solution evaluation

Editor’s Note:	The evaluation of the solution is FFS.

# *** End of Changes ***