3GPP TSG-CT Meeting #109	CP-252215

# Beijing, P.R. China; 15th – 16th September 2025							revision of C4-253558

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.510 | CR | 1226 | rev | 3 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Access token to support VFL |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Huawei, Nokia |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | SA3 has updated TS 33.501 with CR#2154 (S3-252432) and annex X.12.2 "Authorization of candidate VFL participants" has been added to its Annex X. VFL server may be an NWDAF, an Internal AF or an External AF: *** In case of an NWDAF or an Internal AF as the VFL server, when requesting the access token to access a VFL client, the VFL server includes the following additional IEs in the Nnrf_AccessToken_Get request: - Analytics ID, - AF ID (for external AF as client) - Optionally VFL interoperability indicator per analytics ID. In this case the client may be an NWDAF, an Internal AF or an External AF. (See step 3 in Annex X.12.2.1 of TS 33.501). *** In case of an external AF as the VFL server, when requesting the access token to access a VFL client (i.e., NWDAF), the VFL server includes the following additional IEs in the Nnrf_AccessToken_Get request: - Analytics ID, - AF ID (of the external AF) - Optionally VFL interoperability indicator per analytics ID. In this case the client shall be an NWDAF. (See step 4 in Annex X.12.2.2 of TS 33.501). *** Similarly the token claim provided by NRF may additionally include VFL interoperability indicator for that particular analytics ID, and Analytics ID. It also includes the AF ID of the external AF when applicable. Therefore, for authorization of a VFL server to access VFL client, additional IEs with description about applicability of each IE needs to be added to Access Token request and Access Token claims in stage-3 specification of NRF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add IEs with description about applicability of each IE to access token request and and access token claim to support authorization of a VFL server to access VFL client(s). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete stage-3 specification. Authorization of a VFL server to access VFL client(s) as defined by SA3 is not supported by NRF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.6.2.48, 6.1.6.2.138, 6.3.5.2.2, 6.3.5.2.4, 6.3.5.2.6, A.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible new feature to the following APIs: TS24283_Lms_LocationInformation.yaml TS24283_Lms_LocationReport.yaml TS24283_Lms_Registration.yaml TS26510_Maf_Provisioning_ConsumptionReporting.yaml TS26510_Maf_Provisioning_ContentHosting.yaml TS26510_Maf_Provisioning_ContentPreparationTemplates.yaml TS26510_Maf_Provisioning_ContentProtocols.yaml TS26510_Maf_Provisioning_ContentPublishing.yaml TS26510_Maf_Provisioning_EdgeResources.yaml TS26510_Maf_Provisioning_EventDataProcessing.yaml TS26510_Maf_Provisioning_MetricsReporting.yaml TS26510_Maf_Provisioning_PolicyTemplates.yaml TS26510_Maf_Provisioning_ProvisioningSessions.yaml TS26510_Maf_Provisioning_RealTimeCommunication.yaml TS26510_Maf_Provisioning_ServerCertificates.yaml TS26510_Maf_SessionHandling_ConsumptionReporting.yaml TS26510_Maf_SessionHandling_DynamicPolicy.yaml TS26510_Maf_SessionHandling_MetricsReporting.yaml TS26510_Maf_SessionHandling_NetworkAssistance.yaml TS26510_Maf_SessionHandling_ServiceAccessInformation.yaml TS26512_Mas_Configuration_ContentHosting.yaml TS26512_Mas_Configuration_ContentPreparationTemplates.yaml TS26512_Mas_Configuration_ContentPublishing.yaml TS26512_Mas_Configuration_ServerCertificates.yaml TS26517_MBSUserServiceAnnouncement.yaml TS26532_Ndcaf_DataReporting.yaml TS26532_Ndcaf_DataReportingProvisioning.yaml TS29175_Nimsas_ImsEE.yaml TS29175_Nimsas_ImsSessionManagement.yaml TS29175_Nimsas_MediaControl.yaml TS29175_Nimsas_SessionEventControl.yaml TS29176_Nmf_MRM.yaml TS29222_CAPIF_Security_API.yaml TS29256_Nnef_Authentication.yaml TS29309_Nbsp_GBA.yaml TS29369_Nadm_DM.yaml TS29486_VAE_ApplicationRequirement.yaml TS29486_VAE_DynamicGroup.yaml TS29486_VAE_FileDistribution.yaml TS29486_VAE_MessageDelivery.yaml TS29486_VAE_ServiceContinuity.yaml TS29502_Nsmf_PDUSession.yaml TS29503_Nudm_EE.yaml TS29503_Nudm_MT.yaml TS29503_Nudm_NIDDAU.yaml TS29503_Nudm_PP.yaml TS29503_Nudm_RSDS.yaml TS29503_Nudm_SDM.yaml TS29503_Nudm_SSAU.yaml TS29503_Nudm_UEAU.yaml TS29503_Nudm_UECM.yaml TS29503_Nudm_UEID.yaml TS29504_Nudr_DR.yaml TS29504_Nudr_GroupIDmap.yaml TS29507_Npcf_AMPolicyControl.yaml TS29508_Nsmf_EventExposure.yaml TS29509_Nausf_SoRProtection.yaml TS29509_Nausf_UEAuthentication.yaml TS29509_Nausf_UPUProtection.yaml TS29510_Nnrf_AccessToken.yaml TS29510_Nnrf_Bootstrapping.yaml TS29510_Nnrf_NFDiscovery.yaml TS29510_Nnrf_NFManagement.yaml TS29511_N5g-eir_EquipmentIdentityCheck.yaml TS29512_Npcf_SMPolicyControl.yaml TS29514_Npcf_PolicyAuthorization.yaml TS29515_Ngmlc_Location.yaml TS29517_Naf_EventExposure.yaml TS29518_Namf_AIoT.yaml TS29518_Namf_Communication.yaml TS29518_Namf_EventExposure.yaml TS29518_Namf_Location.yaml TS29518_Namf_MBSBroadcast.yaml TS29518_Namf_MBSCommunication.yaml TS29518_Namf_MT.yaml TS29520_Nnwdaf_AnalyticsInfo.yaml TS29520_Nnwdaf_DataManagement.yaml TS29520_Nnwdaf_EventsSubscription.yaml TS29520_Nnwdaf_MLModelMonitor.yaml TS29520_Nnwdaf_MLModelProvision.yaml TS29520_Nnwdaf_MLModelTraining.yaml TS29520_Nnwdaf_RoamingAnalytics.yaml TS29520_Nnwdaf_RoamingData.yaml TS29520_Nnwdaf_VFLInference.yaml TS29520_Nnwdaf_VFLTraining.yaml TS29521_Nbsf_Management.yaml TS29522_AnalyticsExposure.yaml TS29523_Npcf_EventExposure.yaml TS29525_Npcf_UEPolicyControl.yaml TS29526_Nnssaaf_AIW.yaml TS29526_Nnssaaf_NSSAA.yaml TS29531_Nnssf_NSSAIAvailability.yaml TS29531_Nnssf_NSSelection.yaml TS29532_Nmbsmf_MBSSession.yaml TS29532_Nmbsmf_TMGI.yaml TS29534_Npcf_AMPolicyAuthorization.yaml TS29535_Naanf_AKMA.yaml TS29536_Nnsacf_NSAC.yaml TS29536_Nnsacf_SliceEventExposure.yaml TS29537_Npcf_MBSPolicyAuthorization.yaml TS29537_Npcf_MBSPolicyControl.yaml TS29540_Nsmsf_SMService.yaml TS29541_Nnef_SMContext.yaml TS29541_Nnef_SMService.yaml TS29542_Nsmf_NIDD.yaml TS29543_Npcf_PDTQPolicyControl.yaml TS29544_Nspaf_SecuredPacket.yaml TS29550_Nsoraf_SOR.yaml TS29551_Nnef_PFDmanagement.yaml TS29553_Npanf_ProseKey.yaml TS29553_Npanf_ResolveRemoteUserId.yaml TS29554_Npcf_BDTPolicyControl.yaml TS29555_N5g-ddnmf_Discovery.yaml TS29556_Neasdf_BaselineDNSPattern.yaml TS29556_Neasdf_DNSContext.yaml TS29557_Naf_ProSe.yaml TS29559_Npkmf_Discovery.yaml TS29559_Npkmf_PKMFKeyRequest.yaml TS29559_Npkmf_UserId.yaml TS29562_Nhss_gbaSDM.yaml TS29562_Nhss_gbaUEAU.yaml TS29562_Nhss_imsSDM.yaml TS29562_Nhss_imsUEAU.yaml TS29562_Nhss_imsUECM.yaml TS29563_Nhss_EE.yaml TS29563_Nhss_PP.yaml TS29563_Nhss_SDM.yaml TS29563_Nhss_UEAU.yaml TS29563_Nhss_UECM.yaml TS29564_Nupf_EventExposure.yaml TS29564_Nupf_GetUEPrivateIPaddrAndIdentifiers.yaml TS29565_Ntsctsf_ASTI.yaml TS29565_Ntsctsf_QoSandTSCAssistance.yaml TS29565_Ntsctsf_TimeSynchronization.yaml TS29569_Naiotf_AIoT.yaml TS29570_Nscp_EventExposure.yaml TS29571_CommonData.yaml TS29572_Nlmf_Broadcast.yaml TS29572_Nlmf_DataExposure.yaml TS29572_Nlmf_Location.yaml TS29573_JOSEProtectedMessageForwarding.yaml TS29573_N32_Handshake.yaml TS29573_SeppTelescopicFqdnMapping.yaml TS29574_Ndccf_ContextManagement.yaml TS29574_Ndccf_DataManagement.yaml TS29575_Nadrf_DataManagement.yaml TS29575_Nadrf_MLModelManagement.yaml TS29576_Nmfaf_3caDataManagement.yaml TS29576_Nmfaf_3daDataManagement.yaml TS29576_Nmfaf_ContextManagement.yaml TS29577_Nipsmgw_SMService.yaml TS29577_Nrouter_SMService.yaml TS29578_Nmnpf_NPStatus.yaml TS29579_Niwmsc_SMService.yaml TS29580_Nmbsf_MBSUserDataIngestSession.yaml TS29580_Nmbsf_MBSUserService.yaml TS29581_Nmbstf_DistSession.yaml TS29586_Nslpkmf_Discovery.yaml TS29586_Nslpkmf_SLPKMFKeyRequest.yaml TS29591_Nnef_DNAIMapping.yaml TS29591_Nnef_EASDeployment.yaml TS29591_Nnef_ECSAddress.yaml TS29591_Nnef_EventExposure.yaml TS29591_Nnef_TrafficInfluenceData.yaml TS29591_Nnef_UEId.yaml TS29594_Nchf_SpendingLimitControl.yaml TS29598_Nudsf_DataRepository.yaml TS29598_Nudsf_Timer.yaml TS29673_Nucmf_UERCM.yaml TS29675_Nucmf_Provisioning.yaml TS32291_Nchf_ConvergedCharging.yaml TS32291_Nchf_OfflineOnlyCharging.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: - Nokia added as co-source. - Change in clauses 6.1.6.2.48 and 6.1.6.2.138 added. - The attribute descrption texts are updated and aligned with SA3. - A NOTE is added to Table 6.3.5.2.6-1. Rev2: change “clause X.12” to “Annex X.12” and correct hard space. Rev3: The only change is in the other comments on the cover sheet, to change: “This CR introduces backward compatible correction to the following APIs:” to: “This CR introduces backward compatible new feature to the following APIs:” |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 6.1.6.2.48	Type: NefInfo

Table 6.1.6.2.48-1: Definition of type NefInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| nefId | NefId | C | 0..1 | This IE shall be present and contain the NEF ID of the NEF if NIDD service is supported. |
| pfdData | PfdData | O | 0..1 | PFD data, containing the list of internal application identifiers and/or the list of application function identifiers for which the PFDs can be provided. Absence of this attribute indicates that the PFDs for any internal application identifier and for any application function identifier can be provided. |
| afEeData | AfEventExposureData | O | 0..1 | The AF provided event exposure data. The NEF registers such information in the NRF on behalf of the AF. |
| gpsiRanges | array(IdentityRange) | O | 1..N | Range(s) of External Identifiers |
| externalGroupIdentifiersRanges | array(IdentityRange) | O | 1..N | Range(s) of External Group Identifiers |
| servedFqdnList | array(string) | O | 1..N | Pattern (regular expression according to the ECMA-262 dialect [8]) representing the Domain names served by the NEF |
| taiList | array(Tai) | O | 1..N | The list of TAIs the NEF can serve. It may contain one or more non-3GPP access TAIs. The absence of this attribute and the taiRangeList attribute indicates that the NEF can be selected for any TAI in the serving network. |
| taiRangeList | array(TaiRange) | O | 1..N | The range of TAIs the NEF can serve. It may contain non-3GPP access TAIs. The absence of this attribute and the taiList attribute indicates that the NEF can be selected for any TAI in the serving network. |
| dnaiList | array(Dnai) | O | 1..N | List of Data network access identifiers supported by the NEF. The absence of this attribute indicates that the NEF can be selected for any DNAI. |
| unTrustAfInfoList | array(UnTrustAfInfo) | O | 1..N | List of information corresponding to the AFs. This IE may include AF’s VFL capability information when the AF supports Vertical Federated Learning capabilities. |
| uasNfFunctionalityInd | boolean | O | 0..1 | When present, this IE shall indicate whether the NEF supports UAS NF functionality: - true: UAS NF functionality is supported by the NEF - false (default): UAS NF functionality is not supported by the NEF. |
| multiMemAfSessQosInd | boolean | O | 0..1 | When present, this IE shall indicate whether the NEF supports Multi-member AF session with required QoS functionality: - true: Multi-member AF session with required QoS functionality is supported by the NEF - false (default): Multi-member AF session with required QoS functionality is not supported by the NEF. |
| memberUESelAssistInd | boolean | O | 0..1 | When present, this IE shall indicate whether the NEF supports member UE selection assistance functionality: - true: member UE selection assistance functionality is supported by the NEF - false (default): member UE selection assistance functionality is not supported by the NEF. |

* * * Next Change * * *

##### 6.1.6.2.138	Type: VflInfo

Table 6.1.6.2.138-1: Definition of type VflInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| vflAnalyticsIds | array(NwdafEvent) | M | 1..N | Analytics Id(s) for which VFL is supported. The included Analytics Id(s) shall have the same attribute values, e.g. they shall share the same vflCapabilityType. |
| vflCapabilityType | VflCapabilityType | M | 1 | Type of the supported Vertical Federated Learning capability as specified in clause 5.2 of 3GPP TS 23.288 [34]. |
| vflClientAggrCap | boolean | C | 0..1 | When present, this IE indicates whether a VFL client supporting aggregating the intermediate results of other VFL clients. true: an VFL client supporting aggregating the intermediate results of other VFL clients; false (default): an VFL client not supporting aggregating the intermediate results of other VFL clients. This IE shall be present if aggregating the intermediate results of other VFL clients is supported and the vflCapabilityType is set to "VFL_CLIENT" or "VFL_SERVER_AND_CLIENT". |
| vflTimeInterval | TimeWindow | O | 0..1 | When present, this IE shall indicate the Time Period of Interest. See clause 5.2 of 3GPP TS 23.288 [34]. |
| vflInterInfo | MlModelInterInfo | C | 0..1 | VFL interoperability indicator, shall be present if the NWDAF or AF supports the VFL interoperability for the provided Analytics Id(s). If none are provided the NWDAF or AF is not allowed to perform the VFL operation. If mlAnalyticsIds is not provided, this IE, if present, shall contain the VFL interoperability indicator information that is valid for any Analytics Id. |
| featureId | array(string) | C | 1..N | The different feature information supported by the NWDAF for the provided Analytics Id(s). Only the VFL clients and the VFL server sharing the same VFL interoperability indicator can understand the content of feature ID(s). |

* * * Next Change * * *

##### 6.3.5.2.2	Type: AccessTokenReq

Table 6.3.5.2.2-1: Definition of type AccessTokenReq

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| grant_type | string | M | 1 | This IE shall contain the grant type as "client_credentials". Enum: "client_credentials" |
| nfInstanceId | NfInstanceId | M | 1 | This IE shall contain the NF instance id of the NF service consumer. |
| nfType | NFType | C | 0..1 | This IE shall be included when the access token request is for an NF type and not for a specific NF / NF service instance. When present, this IE shall contain the NF type of the NF service consumer. (NOTE 3) |
| targetNfType | NFType | C | 0..1 | This IE shall be included when the access token request is for an NF type and not for a specific NF / NF service instance. When present, this IE shall contain the NF type of the NF service producer. |
| scope | string | M | 1 | This IE shall contain the scopes requested by the NF service consumer. The scopes shall consist of a list of NF service name(s) of the NF service producer(s) or resource/operation-level scopes defined by each service API, separated by whitespaces, as described in IETF RFC 6749 [16], clause 3.3. The service name(s) included in this attribute shall be any of the services defined in the ServiceName enumerated type (see clause 6.1.6.3.11). The resource/operation-level scopes shall be any of those defined in the "securitySchemes" clause of each service API. pattern: '^([a-zA-Z0-9_:-]+)( [a-zA-Z0-9_:-]+)*$' See NOTE 2. |
| targetNfInstanceId | NfInstanceId | C | 0..1 | This IE shall be included, if available and if it is an access token request for a specific NF Service Producer. When present this IE shall contain the NF Instance ID of the specific NF Service Producer for which the access token is requested. |
| requesterPlmn | PlmnId | C | 0..1 | This IE shall be included when the NF service consumer in one PLMN requests a service access authorization for an NF service producer from a different PLMN. It may be present when a service access authorization in the same PLMN need to be requested. When present, this IE shall contain the PLMN ID of the requester NF service consumer. (NOTE 3) (NOTE 4) |
| requesterPlmnList | array(PlmnId) | C | 2..N | This IE shall be included when the NF service consumer serving a PLMN, with more than one PLMN ID, requests a service access authorization for an NF service producer from a different PLMN. It may be present when a service access authorization in the same PLMN, with more than one PLMN ID, need to be requested. When present, this IE shall contain the PLMN IDs of the requester NF service consumer. (NOTE 4) |
| requesterSnssaiList | array(Snssai) | O | 1..N | When present, this IE shall contain the list of S-NSSAIs of the requester NF service consumer. If this IE is included in an Access Token Request sent towards a different PLMN, the requester NF shall provide S-NSSAI values of the target PLMN, that correspond to the S-NSSAI values of the requester NF. This may be used by the NRF to validate that the requester NF service consumer is allowed to access the target NF Service Producer. (NOTE 3) |
| requesterFqdn | Fqdn | O | 0..1 | When present, this IE shall contain the FQDN of the requester NF Service Consumer. This may be used by the NRF to validate that the requester NF service consumer is allowed to access the target NF Service Producer. (NOTE 3) |
| requesterSnpnList | array(PlmnIdNid) | O | 1..N | When present, this IE shall contain the list of SNPNs the requester NF service consumer belongs to. This may be used by the NRF to validate that the requester NF service consumer is allowed to access the target NF Service Producer. (NOTE 3) |
| targetPlmn | PlmnId | C | 0..1 | This IE shall be included when the NF service consumer in one PLMN or SNPN requests a service access authorization for an NF service producer from a different PLMN. When present, this IE shall contain the PLMN ID of the target PLMN (i.e., PLMN ID of the NF service producer). (NOTE 5) |
| targetSnpn | PlmnIdNid | C | 0..1 | This IE shall be included when the NF service consumer in one PLMN or SNPN requests a service access authorization for an NF service producer from a different SNPN. When present, this IE shall contain the SNPN ID of the target SNPN (i.e., SNPN ID of the NF service producer). |
| targetSnssaiList | array(Snssai) | O | 1..N | This IE may be included during an access token request for an NF type and not for a specific NF / NF service instance. When present, this IE shall contain the list of S-NSSAIs of the NF Service Producer. |
| targetNsiList | array(string) | O | 1..N | This IE may be included during an access token request for an NF type and not for a specific NF / NF service instance. When present, this IE shall contain the list of NSIs of the NF Service Producer. |
| targetNfSetId | NfSetId | O | 0..1 | This IE may be included during an access token request for an NF type and not for a specific NF / NF service instance. When present, this IE shall contain the NF Set ID of the NF Service Producer. |
| targetNfServiceSetId | NfServiceSetId | O | 0..1 | This IE may be included during an access token request for a specific NF / NF service instance. When present, this IE shall contain the NF Service Set ID of the NF Service Producer. This may be used by the NRF to validate that the requester NF service consumer is allowed to access the target NF service instance. (NOTE 3) |
| hnrfAccessTokenUri | Uri | C | 0..1 | If included, this IE shall contain the API URI of the Access Token Service (see clause 6.3.2) of the NRF in home PLMN. It shall be included during an access token request for an hSMF in the home routed roaming scenario, if it is returned from the NSSF in the home PLMN (see clause 6.1.6.2.11 of 3GPP TS 29.531 [42]). |
| sourceNfInstanceId | NfInstanceId | C | 0..1 | This IE shall be included, if available and if it is an access token request from the DCCF as NF Service Consumer request data from NF Service Producers on behalf of the source NF. When present this IE shall contain the NF Instance ID of the source NF which intend to collect data from NF Service Producer. This IE shall be included, if available and if the requester is an NWDAF containing MTLF service that requests ML model(s) on behalf of another ML model consumer. When present, this IE shall contain the NF Instance ID of the ML model consumer (e.g. NWDAF containing AnLF). See Annex X.10 of 3GPP TS 33.501 [15]. |
| vendorId | VendorId | C | 0..1 | Vendor Id of requester, shall be included if the requester is an NWDAF containing MTLF and/or AnLF requesting access token to be used for MTLF and/or AnLF service(s) (see Annex X.10 of 3GPP TS 33.501 [15]). This IE may be present when the NWDAF acting as VFL Server or Internal AF acting as VFL Server or NEF on behalf of an external AF as VFL Server requests an access token from the NRF as specified in clause 13.4.1 of 3GPP TS 33.501 [15] to perform VFL service towards the VFL clients. (See Annex X12.2 of 3GPP TS 33.501 [15].) |
| analyticsIds | array(NwdafEvent) | C | 1..N | Analytics Id(s) of requester, shall be included if the requester is an NWDAF containing MTLF and/or AnLF requesting access token to be used for MTLF and/or AnLF service(s) (see Annex X.10 of 3GPP TS 33.501 [15]). This IE shall be present when the NWDAF acting as VFL Server or Internal AF acting as VFL Server or NEF on behalf of an external AF as VFL Server requests an access token from the NRF as specified in clause 13.4.1 of 3GPP TS 33.501 [15] to perform VFL service towards the VFL clients. (See Annex X12.2 of 3GPP TS 33.501 [15].) |
| requesterInterIndList | array(MlModelInterInd) | C | 1..N | This IE shall be included if the NF Service consumer is NWDAF containing MTLF requesting access token to be used for MTLF service(s) and the requester supports interoperability, i.e. the IE is available. The IE contains the Interoperability indicator of the requester per Analytics ID (see Annex X.10 of 3GPP TS 33.501 [15]). This IE shall be present when the NWDAF acting as VFL Server or Internal AF acting as VFL Server or NEF on behalf of an external AF as VFL Server requests an access token from the NRF as specified in clause 13.4.1 of 3GPP TS 33.501 [15] to perform VFL service towards the VFL clients. (See Annex X12.2 of 3GPP TS 33.501 [15].) |
| sourceVendorId | VendorId | C | 0..1 | This IE shall be included, if available and if the requester is an NWDAF containing MTLF service that requests ML model(s) on behalf of another ML model consumer. When present, this IE shall contain the Vendor ID of the ML model consumer (e.g. NWDAF containing AnLF). See Annex X.10 of 3GPP TS 33.501 [15]. |
| afId | string | C | 0..1 | Indicates the application function identifier, i.e., AF ID. This IE shall be included when the NWDAF acting as VFL Server requests an access token from the NRF for accessing external AF with the indicated AF ID as VFL client as specified in Annex X12.2.1 of 3GPP TS 33.501 [15]. This IE shall be included when the NEF requests an access token for an external AF with the indicated AF ID to perform VFL service towards the VFL clients, i.e., NWDAF, as specified in Annex X12.2.2 of 3GPP TS 33.501 [15]. |
| NOTE 1: This data structure shall not be treated as a JSON object. It shall be treated as a key, value pair data structure to be encoded using x-www-form-urlencoded format as specified in clause 17.13.4 of W3C HTML 4.01 Specification [26]. NOTE 2: Though scope attribute is optional as per IETF RFC 6749 [16], it is mandatory for 3GPP as per 3GPP TS 33.501 [15]. NOTE 3: An access token request should be rejected if the requester NF is not allowed to access the target NF based on the authorization parameters in the NF profile of the target NF. The authorization parameters in NF Profile are those used by NRF to determine whether a given NF Instance / NF Service Instance can be discovered by an NF Service Consumer in order to consume its offered services (e.g. "allowedNfTypes", "allowedNfDomains", etc.). Based on operator's policies, an access token request not including the requester's information necessary to validate the authorization parameters in the target NF Profile may be rejected. NOTE 4: When the NF service consumer is serving a PLMN consisting of one PLMN ID, the attribute "requesterPlmn" shall be used; otherwise, if the NF service consumer is serving a PLMN consisting of more than one PLMN ID, the attribute "requesterPlmnList" shall be used. NOTE 5: In the case of Subscription-based routing to a target core network, the NF service consumer shall set the targetPlmn attribute to the PLMN ID of the NF Service Producer, as learned e.g. from the NF profile received during the NF discovery procedure (e.g. for an AMF requesting an access token to access UDM or AUSF services) or from the Operator Identifier within the DNN (e.g. for a V-SMF requesting an access token to access the H-SMF PDUSession service). Accordingly, for an NF service producer pertaining to a target PLMN, the access token request shall be routed directly to the target PLMN, e.g. from the VPLMN to the target PLMN without transiting through the HPLMN. |  |  |  |  |

EXAMPLE:

The following is an example of an Access Token Request message, with a request body encoded as x-www-form-urlencoded, with following input parameters:

-	NF Instance Id of the NF Service Consumer: 4e0b2760-0356-42c4-b739-8d6aaa491b63

-	NF Type of the NF Service Consumer: AMF

-	NF Type of the NF Service Producer: UDM

-	Requested scopes: "nudm-sdm", "nudm-uecm" and "nudm-ueau"

-	PLMN ID of the NF Service Consumer: MCC=123, MNC=456

-	PLMN ID of the NF Service Producer: MCC=321, MNC=654

-	S-NSSAIs of the NF Service Producer: (SST=1, SD=A08923) and (SST=2)

-	NSIs of the NF Service Producer: "Slice A, instance 1" and "Slice B, instance 2"

Note that the URL-encoding of the request body requires to percent-encode the reserved characters ([ ] { } " : ,) that appear in JSON-encoded structured input parameters (such as "requesterPlmn"), and in string input parameters (such as "scope", or "targetNsiList" array elements). Spaces are percent-encoded as '+'.

The request body, before URL-encoding, and displayed in multiples lines only for illustration purposes, would be:

grant_type=client_credentials

&nfInstanceId=4e0b2760-0356-42c4-b739-8d6aaa491b63

&nfType=AMF

&targetNfType=UDM

&scope=nudm-sdm nudm-uecm nudm-ueau

&requesterPlmn={"mcc":"123","mnc":"456"}

&targetPlmn={"mcc":"321","mnc":"654"}

&targetSnssaiList=[{"sst":1,"sd":"A08923"},{"sst":2}]

&targetNsiList=Slice A, instance 1

&targetNsiList=Slice B, instance 2

The actual request message, after URL-encoding, and where all input parameters are contained into one single line in the request body, would be:

POST /oauth2/token

Content-Type: application/x-www-form-urlencoded

Accept: application/json

grant_type=client_credentials&nfInstanceId=4e0b2760-0356-42c4-b739-8d6aaa491b63&nfType=AMF&targetNfType=UDM&scope=nudm-sdm+nudm-uecm+nudm-ueau&requesterPlmn=%7B%22mcc%22%3A%22123%22%2C%22mnc%22%3A%22456%22%7D&targetPlmn=%7B%22mcc%22%3A%22321%22%2C%22mnc%22%3A%22654%22%7D&targetSnssaiList=%5B%7B%22sst%22%3A1%2C%22sd%22%3A%22A08923%22%7D%2C%7B%22sst%22%3A2%7D%5D&targetNsiList=Slice+A%2C+instance+1&targetNsiList=Slice+B%2C+instance+2

* * * Next Change * * * *

##### 6.3.5.2.4	Type: AccessTokenClaims

Table 6.3.5.2.4-1: Definition of type AccessTokenClaims

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| iss | NfInstanceId | M | 1 | This IE shall contain NF instance id of the NRF. , corresponding to the standard "Issuer" claim described in IETF RFC 7519 [25], clause 4.1.1 |
| sub | NfInstanceId | M | 1 | This IE shall contain the NF instance ID of the NF service consumer, corresponding to the standard "Subject" claim described in IETF RFC 7519 [25], clause 4.1.2. |
| aud | Audience | M | 1 | This IE shall contain the NF service producer's NF instance ID(s) (if the exact NF instance(s) of the NF service producer is known) or the NF type of NF service producers for which the claim is applicable, corresponding to the standard "Audience" claim described in IETF RFC 7519 [25], clause 4.1.3. |
| scope | string | M | 1 | This IE shall contain the name of the NF services and the resource/operation-level scopes for which the access_token is authorized for use; this claim corresponds to a private claim, as described in IETF RFC 7519 [25], clause 4.3. pattern: '^([a-zA-Z0-9_:-]+)( [a-zA-Z0-9_:-]+)*$' |
| exp | integer | M | 1 | This IE shall contain the expiration time after which the access_token is considered to be expired, corresponding to the standard "Expiration Time" claim described in IETF RFC 7519 [25], clause 4.1.4. |
| consumerPlmnId | PlmnId | C | 0..1 | This IE shall be included if the NRF supports providing PLMN ID of the NF service consumer in the access token claims, to be interpreted for subject (sub IE), as specified in clause 13.4.1.2 of 3GPP TS 33.501 [15]. If an NF service producer that receives this IE in the token included in the authorization header does not understand this IE, it shall be ignored. |
| consumerSnpnId | PlmnIdNid | C | 0..1 | This IE shall be included if the NRF supports providing SNPN ID of the NF service consumer in the access token claims, to be interpreted for subject (sub IE), as specified in clause 13.4.1.2 of 3GPP TS 33.501[15]. If an NF service producer that receives this IE in the token included in the authorization header does not understand this IE, it shall be ignored. |
| producerPlmnId | PlmnId | C | 0..1 | This IE shall be included if the access token is granted for an NF service producer belonging to an PLMN and the NRF supports providing PLMN ID of the NF service producer in the access token claims, to be interpretted for audience (aud IE), as specified in clause 13.4.1.2 of 3GPP TS 33.501 [15]. If an NF service producer that receives this IE in the token included in the authorization header does not understand this IE, it shall be ignored. |
| producerSnpnId | PlmnIdNid | C | 0..1 | This IE shall be included if the access token is granted for an NF service producer belonging to an SNPN and the NRF supports providing SNPN ID of the NF service producer in the access token claims, to be interpreted for audience (aud IE), as specified in clause 13.4.1.2 of 3GPP TS 33.501 [15]. When present, it shall contain the SNPN ID of the SNPN the target NF service producer belongs to. If an NF service producer that receives this IE in the token included in the authorization header does not understand this IE, it shall be ignored. |
| producerSnssaiList | array(Snssai) | O | 1..N | This IE may be included if the NRF supports providing list of S-NSSAIs of the NF service producer in the access token claims. If an NF service producer that receives this IE in the token included in the authorization header does not understand this IE, it shall be ignored. |
| producerNsiList | array(string) | O | 1..N | This IE may be included if the NRF supports providing list of NSIs of the NF service producer in the access token claims. If an NF service producer that receives this IE in the token included in the authorization header does not understand this IE, it shall be ignored. |
| producerNfSetId | NfSetId | O | 0..1 | This IE may be included if the NRF supports providing NF Set ID of the NF service producer in the access token claims and if the audience contains an NF type. When present, it shall indicate the NF Set ID of the NF service producer instances for which the claim is applicable. If an NF service producer that receives this IE in the token included in the authorization header does not understand this IE, it shall be ignored. |
| producerNfServiceSetId | NfServiceSetId | O | 0..1 | This IE may be included during an access token request for a specific NF / NF service instance, if the targetNfServiceSetId IE is present in the Access Token Request. When present, this IE shall contain the NF Service Set ID of the NF Service Producer for which the access token is applicable. If an NF service producer that receives this IE in the token included in the authorization header does not understand this IE, it shall be ignored. |
| sourceNfInstanceId | NfInstanceId | C | 0..1 | This IE shall be included if the NRF supports providing the NF Instance ID of the source NF in the access token claims. This applies in the following cases: - When the access token request is requested by the DCCF to request data from an NF Service Producers on behalf of a source NF (e.g. NWDAF). In this case, this IE shall contain the NF Instance ID of the source NF (e.g. NWDAF). See Annex X.2 of 3GPP TS 33.501 [15]. - When the access token request is requested by an NWDAF containing MTLF service, to request ML models on behalf of another ML model consumer. In this case, this IE shall contain the NF Instance ID of the ML model consumer (e.g. NWDAF containing AnLF). See Annex X.10 of 3GPP TS 33.501 [15]. |
| analyticsIdList | array(NwdafEvent) | C | 1..N | This IE shall be provided when the producer is an NWDAF containg MTLF and/or AnLF and requested access token to be used for MTLF and/or AnLF service(s). The IE contains Analytics Id(s) that the consumer is authorized to access as specified in Annex X.10 of 3GPP TS 33.501 [15]. This IE shall be provided when an NWDAF acting as VFL Server or an Internal AF acting as VFL Server or an NEF on behalf of an external AF as VFL server requested an access token to be used to perform VFL service towards the VFL clients. (See Annex X12.2 of 3GPP TS 33.501 [15].) |
| applicableResourceContentFilters | array(RcfId) | C | 1..N | This IE may be included if the NRF supports providing applicable resource content filters in the access token claims for use by the producer. (NOTE) |
| consumerRcfInfo | map(array(string)) | O | 1..N(1..M) | A map of tag name/values pairs, where the tag name is a unique string name that is the primary key of the map and is paired with an array of string values. (NOTE) |
| iat | integer | O | 0..1 | The attribute shall contain the NumericDate value of "issued at". When present, this IE shall indicate the time when the access token was issued and may be used to determine the age of the JWT as indicated in IETF RFC 7519 [25]. |
| afId | string | C | 0..1 | Indicates the application function identifier, i.e., AF ID. This IE shall be provided in the access token claim to the NWDAF acting as VFL Server which has requested an access token from the NRF for accessing an external AF with the indicated AF ID as VFL client as specified in Annex X12.2.1 of 3GPP TS 33.501 [15]. This IE shall be provided in the access token claim to the NEF which reqested the access token for an external AF with the indicated AF ID to perform VFL service towards the VFL clients, i.e., NWDAF, as specified in Annex X12.2.2 of 3GPP TS 33.501 [15]. |
| interOpIndList | array(MlModelInterInd) | O | 1..N | Interoperability indicator. This IE may be provided when an NWDAF acting as VFL Server or an Internal AF acting as VFL Server or an NEF on behalf of an external AF as VFL server requested an access token to be used to perform VFL service towards the VFL clients. (See Annex X12.2 of 3GPP TS 33.501 [15].) |
| NOTE: May be present in AccessToken Claims within AccessTokenRsp corresponding to requests containing a targetNfInstanceId. This may be applicable to the particular token type, which is captured in the 3GPP TS 33.501 [15] clause 13.4.1.1.2, Access token request for accessing services of a specific NF Service Producer instance / NF Service Producer service instance. |  |  |  |  |

* * * Next Change * * * *

##### 6.3.5.2.6	Type: MlModelInterInd

Table 6.3.5.2.6-1: Definition of type MlModelInterInd

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| analyticsId | NwdafEvent | M | 1 | Analytics Id that support interoperability. |
| vendorList | array(VendorId) | M | 1..N | This IE shall include the list of NWDAF vendors or AF vendors that are allowed to retrieve ML models from the NWDAF containing MTLF for the provided analyticsId. If this data type is provided to and from NEF, in communicateions on behalf of an external AF, i.e. Untrusted AF, this IE referes to the vendors supported by the AF. |

* * * Next Change * * * *

# A.4	Nnrf_AccessToken API (NRF OAuth2 Authorization)

openapi: 3.0.0

info:

version: '1.4.0-alpha.2'

title: 'NRF OAuth2'

description: |

NRF OAuth2 Authorization.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.510 V19.3.0; 5G System; Network Function Repository Services; Stage 3

url: 'https://www.3gpp.org/ftp/Specs/archive/29_series/29.510/'

[non-changing parts skipped for clarity]

AccessTokenReq:

description: Contains information related to the access token request

type: object

required:

- grant_type

- nfInstanceId

- scope

properties:

grant_type:

type: string

enum:

- client_credentials

nfInstanceId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

nfType:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/NFType'

targetNfType:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/NFType'

scope:

type: string

pattern: '^([a-zA-Z0-9_:-]+)( [a-zA-Z0-9_:-]+)*$'

targetNfInstanceId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

requesterPlmn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

requesterPlmnList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

minItems: 2

requesterSnssaiList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

minItems: 1

requesterFqdn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Fqdn'

requesterSnpnList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnIdNid'

minItems: 1

targetPlmn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

targetSnpn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnIdNid'

targetSnssaiList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

minItems: 1

targetNsiList:

type: array

items:

type: string

minItems: 1

targetNfSetId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfSetId'

targetNfServiceSetId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfServiceSetId'

hnrfAccessTokenUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

sourceNfInstanceId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

vendorId:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/VendorId'

analyticsIds:

type: array

items:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/NwdafEvent'

minItems: 1

requesterInterIndList:

type: array

items:

$ref: '#/components/schemas/MlModelInterInd'

minItems: 1

sourceVendorId:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/VendorId'

afId:

type: string

AccessTokenRsp:

description: Contains information related to the access token response

type: object

required:

- access_token

- token_type

properties:

access_token:

type: string

description: >

JWS Compact Serialized representation of JWS signed JSON object (AccessTokenClaims)

token_type:

type: string

enum:

- Bearer

expires_in:

type: integer

scope:

type: string

pattern: '^([a-zA-Z0-9_:-]+)( [a-zA-Z0-9_:-]+)*$'

AccessTokenClaims:

description: The claims data structure for the access token

type: object

required:

- iss

- sub

- aud

- scope

- exp

properties:

iss:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

sub:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

aud:

anyOf:

- $ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/NFType'

- type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

minItems: 1

scope:

type: string

pattern: '^([a-zA-Z0-9_:-]+)( [a-zA-Z0-9_:-]+)*$'

exp:

type: integer

consumerPlmnId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

consumerSnpnId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnIdNid'

producerPlmnId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

producerSnpnId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnIdNid'

producerSnssaiList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

minItems: 1

producerNsiList:

type: array

items:

type: string

minItems: 1

producerNfSetId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfSetId'

producerNfServiceSetId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfServiceSetId'

sourceNfInstanceId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

analyticsIdList:

type: array

items:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/NwdafEvent'

minItems: 1

applicableResourceContentFilters:

type: array

items:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/RcfId'

minItems: 1

consumerRcfInfo:

description: A map (list of key-value pairs) where a (unique) valid JSON string

serves as key

type: object

additionalProperties:

type: array

items:

type: string

minItems: 1

minProperties: 1

iat:

type: integer

afId:

type: string

interOpIndList:

type: array

items:

$ref: '#/components/schemas/MlModelInterInd'

minItems: 1

[non-changing parts skipped for clarity]

* * * End of Changes * * * *