3GPP TSG CT WG1 Meeting #157 	C1-256052

Sophia Antipolis, France, 13-17 October 2025

# Source:	Huawei, HiSilicon

# Title:	New SID on Study on the 3GPP Network selection aspects of the 6G System

# Document for:	Aggrement

# Agenda Item:	20.2 (New WIDs/SIDs for Rel-20)

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	Study on Network selection aspects of the 6G System

Acronym:	FS_6G_NS-CT

Unique identifier:	TBD

Potential target Release:	Rel-20

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  | X |  |
| No |  |  | X |  |  |
| Don't know | X |  |  |  | X |

# 2	Classification of the Study Item and linked work items

## 2.1	Primary classification

### This work item is a:

| X | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| N/A |  |  |  |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 1050110 | Study on 6G Use Cases and Service Requirements | SA1 study on 6G Use Cases and Service Requirements. |
| 1080057 | Study on Architecture for 6G System | SA2 study on Architecture for 6G System. |

Dependency on non-3GPP (draft) specification:

# 3	Justification

As per the CT WG1 terms of reference (ToR), the group holds stage-2 responsibility for "non-access stratum (NAS) functions related to the UE in idle mode", as defined in 3GPP TS 23.122. This specification describes among others how a UE discovers, selects and registers on a public land mobile network (PLMN) when idle.

In 5GS, network-selection procedures were enhanced to, e.g.:

-	support new (5G) steering of roaming (SoR) mechanism;

-	enable closed access group (CAG) selection;

-	facilitate standalone non-public network (SNPN) selection (including triggered by ProSe);

-	introduce PLMN selection triggered by V2X, A2X communication over PC5, and ProSe; and

-	integrate satellite access into the 5GS.

Considering 6G, network selection (including PLMN selection) might need to evolve to ensure ultra-reliable, intelligent and seamless connectivity across terrestrial, aerial and local environments, so that UEs always attach to the optimal network.

3GPP TR 22.870 on "Study on 6G use cases and potential service requirements" identifies new scenarios (use cases) that might go beyond today’s macro-PLMN model, e.g.:

-	local area networks and localized vertical networks;

-	in-vehicle local communication;

-	autonomous AI agents with intent understanding, action planning and multi-agent collaboration.

These emerging scenarios may expose gaps in existing 3GPP TS 23.122 procedures, indicating a need to analyze and potentially enhance idle-mode network selection procedures, particularly around, e.g., multi-access domain selection, dynamic reselection or service-aware triggers.

# 4	Objective

The objective of this study item is to analyze existing network-selection procedures, building on the SA1 work, to determine whether enhancements or new procedures are required.

The study will address CT1-relevant stage-2 aspects of network selection (non-exhaustive list):

WT#1: evaluate whether current PLMN-selection procedures are sufficient for multi-access domain selection in 6G systems (covering legacy 3GPP RATs, non-3GPP access and emergent 6G air interfaces);

WT#2: assess whether 6G use cases and requirements demand capabilities analogous to (5G) non-public networks (NPNs);

WT#3: assess whether 6G use cases and requirements demand capabilities analogous to (5G) closed access groups (CAGs);

WT#4: evaluate whether 6G use cases and requirements require enhancements to scan of higher prioritized PLMN search, no suitable cell (limited service state), or both;

WT#5: evaluate whether 6G AI-agent use cases impose new or enhanced network-selection capabilities; and

WT#6: analyse the adequacy of existing steering of roaming (SoR) mechanisms for 6G systems.

The work of this study is going to be documented in a TR. The complete or partial conclusions of the study item could form the basis for the normative work together with requirements from stage 1.

This study will consider the progress of the stage-1 work on 6G use cases and potential service requirements captured in 3GPP TR 22.870.

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| TR | 24.8ab | Study on Network selection aspects of the 6G System | CT#115 (Mar. 2027) | CT#116 (June 2027) | TBD |

NOTE: 	The final timeline for the study will be decided at CT#110 (Dec 2025).

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| N/A |  |  |  |

# 6	Work item Rapporteur(s)

TBD

# 7	Work item leadership

CT1

# 8	Aspects that involve other WGs

SA1 for stage 1.

# 9	Supporting Individual Members

Supporting IM name

Huawei

HiSilicon