3GPP TSG-CT WG1 Meeting #157	C1-256101

Sophia Antipolis, France, 13-17 October 2025

Source:	OPPO

Title:	Discussion paper on LMF relocation procedure

Spec:	3GPP TS 24.272 v19.4.0

Agenda item:	19.4 (TEI19, 5G_eLCS_Ph3)

Document for:	Discussion

1. Introduction

CT1#156 has discussed the implications of the relatively new 3GPP Rel-19 feature, which allows UE to have a single UP-connection to more than one LMF. As the consensus was not reached, all elated CRs were postponed. This paper offers a solution that could be used as a possible compromise for reaching an agreement.

2. Discussion

Below is the outline of the solution and also how this can be used in various scenarios.

- Requirement. During LMF relocation procedure, the target/new LMF needs to tell the UE which of the UP-connections shall be relocated from the source/old LMF. As long as the UE may have only a handful of UP-connections, it is required that this ID shall be unique only within the UE. Below presented solution meets this requirement.

- During the user plane connection establishment procedure:

a.	UE allocates an ID, e.g. ID-1 which shall be unique within the UE.

b.	UE sends the new ID-1 with the LCS UP-connection binding request, i.e. via UP-connection. This requires the new IE, the ID-1 shall be added to the message as a mandatory IE (in the message definition this will be marked as optional, but the description shall state it is mandatory). Impact on LCS-UPP.

- During the LMF relocation, if the target/new LMF does not have UP-connection with the given UE:

a.	Source/old LMF sends the UE allocated ID-1 to the target/new LMF. This requires the new attribute, the ID-1 shall be added to the Nlmf service operations. Impact on TS 29.572 under CT4 remit.

b.	Target/new LMF forwards ID-1 to the UE with the UP-connection establishment command. This requires the new IE, the ID-1 shall be added to the message as a conditional IE (in the message definition this will be marked as optional, but the description shall specify the conditions). Impact on UPP-CM.

NOTE: 	In the previous meeting, several CT1 delegates indicated a preference to specify a brand-new NAS relocation procedure. The problem with this is twofold, (a) stage 2 does not specify such new NAS procedure, but refers to UP establishment one, see Figure 6.18.3-1 in TS 23.273 and (b) new procedure should not be specified if the existing one can serve the purpose. This DISC assumes that NAS-wise, UP-connection establishment procedure can be reused.

c.	The UE compares the received ID-1 with the IDs that are in use and finds the match. The associated routing identifier, e.g. RoutingID-1 points the source/old LMF, while the received routing identifier, e.g. RoutingID-2 points to the target/new LMF, but this one does not associate to any of the UP-connection IDs in use. These clearly indicate LMF relocation is underway to the LMF that does not have UP-connection to the UE. ID-1 is unique per UE and therefore the UE and the target/new LMF can safely continue using this as the UP-connection identifier.

d.	Same as (1b), which modifies LCS-UPP. Sending ID-1 confirms to the target/new LMF that ID-1 continue identifying the UP-connection within the UE.

- During the LMF relocation, if the target/new LMF does have UP-connection with the given UE:

a.	Same as 2a.

b.	Same as 2b.

c.	Similar to 2c with the following differences. As the UE already has an association between ID-1 and RoutingID-1 vs. ID-2 and RoutingID-2, these clearly indicate LMF relocation is underway to the new LMF, which already has UP-connection to the UE. As the UE cannot have more than one UP-connection to the given LMF, these two UP-connections shall be merged into a single one. Therefore, the UE shall select a respective ID. In principle, UE could select ID-1, ID-2 or any new unused ID, but for simplicity it is preferred UE adopts ID-2.

d.	Similar to 2d and (1b), which modifies LCS-UPP with the following difference. ID-2 will identify to the target/new LMF that the merged UP-connection within the UE is now ID-2.

3. Conclusions

When discussing the matter, CT1 should agree on the following key points of divergence:

- During the LMF relocation, which of the below alternatives shall be used:

- UP-connection establishment command, which contains the new ID-1 that was received from the source/old LMF. This triggers Binding UE request containing the new ID.

- Alternatively, specify a brand-new relocation procedure with NAS message pair, which however is not specified in stage 2, at least not explicitly.

- Which entity shall generate the UP-connection ID and how that shall be encoded? The below choice will have a direct impact on when and how the ID is sent over NAS.

- UE shall generate the ID. It shall be a simple data type, e.g. 8 bits long integer.

- Routing id shall be used. AMF shall derive the ID from the LMF NfInstanceId, see Table 5.3.2-1 in 3GPP TS 29.571. Therefore, both the AMF and the LMF will have this readily available, i.e. no need to exchange. For further details, see point E.

4. Proposal

It is proposed to agree CR124-TS24.572 in C1-256102, which implements the above A1 and B1 protocol choice.