3GPP TSG-CT WG1 Meeting #157	C1-256102

Sophia Antipolis, France, 13-17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.572 | CR | 0124 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | LMF relocation procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT1 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19, 5G_eLCS_Ph3 |  |  |  |  |  | Date: |  |  | 2025-10-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 3GPP Rel-19 feature, which allows UE to establish a UP connection to one or more LMFs, requires amendments to CT1 and CT4 specs. Discussion paper in C1-256101 offers few alternative avenues to be explored. This CR implements one of these options. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New clause 6.2.1.x is added, which describes stage 3 solution, i.e. implications on UPP-CM and LCS-UPP procedures. The solution requires the definition of a new IE, the LCS-UP connection tag IE (new clause 11.2.x), which shall be added to the LCS-UP CONNECTION BINDING REQUEST and USER PLANE CONNECTION ESTABLISHMENT COMMAND messages (clauses 10.2.3.1 and 10.3.1.1). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing stage 3 support for the stage 2 feature, which allows UE to establish a UP connection to one or more LMFs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1.x (new), 10.2.3.1, 10.3.1.1, 11.2.x (new). |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 6.2.1.x	Network initiated user plane modification procedure with the LMF relocation

##### 6.2.1.x.1	General

The network may decide to relocate the user plane connection from the given LMF to a new LMF. The target/new LMF indicates to the UE, which of the user plane connections in the UE is being relocated (see clause 6.18.3 in 3GPP TS 23.273 [2]). This clause specifies the implications of the LMF relocation procedure on the UPP-CM and LCS-UPP procedures for scenarios when the UE establishes a user plane connection to one or more LMFs.

Within the scope of this specification, LMF relocation procedure is based on the Network initiated user plane connection establishment procedure (see clause 6.2.1.1) with the following differences.

Before the LMF relocation:

- UE receives the USER PLANE CONNECTION ESTABLISHMENT COMMAND message with DL NAS TRANSPORT message. The message contains a routing identifier, which identifies the LMF.

- Before UE initiates the LCS-UP connection binding procedure, the UE shall allocate an LCS-UP connection tag (i.e. UE-internal identifier of the user plane connection, see clause 11.2.x) and associate this with the received routing identifier.

- UE shall send the LCS-UP connection tag IE to the LMF with the LCS-UP CONNECTION BINDING REQUEST message.

During the LMF relocation (see step 4a in clause 6.18.3 of the 3GPP TS 23.273 [2]):

-	New LMF receives the LCS-UP connection tag from the old LMF.

-	UE receives USER PLANE CONNECTION ESTABLISHMENT COMMAND message, which shall include the LCS-UP connection tag that the new LMF has received from the old LMF. The DL NAS TRANSPORT message contains the routing identifier, which identifies the new LMF. The UE shall select the LCS-UP connection tag as follows:

-	If the received LCS-UP connection tag is associated with the old LMF and the routing identifier is new, i.e. it identifies the new LMF, the UE shall move the user plan connection from the old LMF to the new LMF. Therefore, UE shall associate the received LCS-UP connection tag and associate it with the new LMF's routing identifier, i.e. the UE selects this LCS-UP connection tag for the follow up messaging.

-	If the received LCS-UP connection tag is associated with the old LMF and the routing identifier is associated with the new LMF, the UE shall move the user plan connection from the old LMF to the new LMF and merge these two into one. Therefore, UE shall mark obsolete both old LMF identifiers (LCS-UP connection tag and the routing identifier) and select the new LMF's LCS-UP connection tag for the follow up messaging.

-	UE shall send back the selected LCS-UP connection tag IE to the new LMF with the LCS-UP CONNECTION BINDING REQUEST message.

* * * 2nd Change * * * *

#### 10.2.3.1	Message definition

The LCS-UP CONNECTION BINDING REQUEST message is sent by the UE to the LMF to associate the TLS connection with the UE. See table 10.2.3.1.1.

Message type:	LCS-UP CONNECTION BINDING REQUEST

Significance:	dual

Direction:	UE to network

Table 10.2.3.1.1: LCS-UP CONNECTION BINDING REQUEST message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | LCS-UP CONNECTION BINDING REQUEST message identity | Message type 11.1.3 | M | V | 1 |
|  | LCS-UP binding ID | LCS-UP binding ID 11.3.4 | M | LV | 5-256 |
| x- | LCS-UP connection tag | LCS-UP connection tag 11.2.x | O | TV | 1 |
| NOTE: If one or more LMFs are allowed to establish a user plane connection to the same UE, then the UE shall include the LCS-UP connection tag IE in this message (see clause 6.2.1.x). |  |  |  |  |  |

* * * 3rd Change * * * *

#### 10.3.1.1	Message definition

The USER PLANE CONNECTION ESTABLISHMENT COMMAND message is sent by the LMF to the UE to transport the user plane connection information and trigger the UE to establish the LCS secured user plane connection towards the LMF. See table 10.3.1.1.1.

The USER PLANE CONNECTION ESTABLISHMENT COMMAND is also used Network initiated user plane modification procedure with the LMF relocation (see clause 6.2.1.x and also clause 6.18.3 in 3GPP TS 23.273 [2]). In this case, the new LMF shall also send to the UE the LCS-UP connection tag (see clause 11.2.x).

Message type:	USER PLANE CONNECTION ESTABLISHMENT COMMAND

Significance:	dual

Direction:	network to UE

Table 10.3.1.1.1: USER PLANE CONNECTION ESTABLISHMENT COMMAND message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | USER PLANE CONNECTION ESTABLISHMENT COMMAND message identity | Message type 11.1.3 | M | V | 1 |
|  | LCS-UP binding ID | LCS-UP binding ID 11.3.4 | M | LV | 5-256 |
|  | LMF LCS-UP address | LMF LCS-UP address 11.3.1 | M | LV | 3-256 |
| x- | LCS-UP connection tag | LCS-UP connection tag 11.2.x (NOTE) | O | TV | 1 |
| NOTE: If one or more LMFs are allowed to establish a user plane connection to the same UE, then during the network-initiated user plane modification procedure with the LMF relocation, the new LMF will receive the LCS-UP connection tag IE from the old LMF. The new LMF shall include the received LCS-UP connection tag IE in this message (see clause 6.2.1.x). |  |  |  |  |  |

* * * 4th Change * * * *

### 11.2.x	LCS-UP connection tag

The purpose of the LCS-UP connection tag information element is used during the LMF relocation procedure to help the UE identify the LCS secured user plane connection to the LMF (see clause 6.2.1.x, also clause 6.18.3 in 3GPP TS 23.273 [2]).

The LCS-UP connection tag information element is coded as shown in figure 11.2.x.1.

The LCS-UP connection tag information element is a type 1 information element of TV format.

| 8 |  | 7 | 6 | 5 | 4 |  | 3 | 2 | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IEI |  |  |  |  | LCS-UP connection tag value |  |  |  |  | octet 1 |  |

Figure 11.2.x.1: LCS-UP connection tag information element

* * * End of Changes * * * *