3GPP TSG-CT WG1 Meeting #157	C1-256107

Sophia Antipolis, France, 13-17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.501 | CR | 7014 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NAS signalling support for the UE level LP-WUS enable/disable |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT1 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19, NR_LPWUS-Core |  |  |  |  |  | Date: |  |  | 2025-10-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | RAN2 identified a problem (see R2-2506245) and has sent an LS to CT1 (see R2-2506261). The LS asks CT1 to add a new LP-WUSPS assistance information type to the Registration Request / Accept messages. Currently, the LP-WUSPS assistance information IE encoding has an unjustified limitation – the LP-WUSPS assistance information types are not extendable. Table 9.11.3.111.1 reads "all other values are reserved" for no apparent reason. This CR proposes to lift the limitation and also permit sending more than one type of information simultaneously. The latter would be a future proof solution, which may be used already in 3GPP Rel-19. For example, if the LP-WUS is disabled, and the UE decides to send e.g. LP-WUSPS paging subgroup ID, then the same NAS message needs to carry both LP-WUS enable request and also LP-WUSPS paging subgroup ID. See discussion paper in C1-256106. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following changes apply – (a) the LP-WUSPS assistance information types are made extendable by removing the limitation, (b) new LP-WUSPS assistance information type is added and (c) as the LP-WUSPS assistance information IE may contain more than one LP-WUSPS assistance information types, the encoding is fixed to permit that. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RAN2 requirements cannot be met. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.11.3.111. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 9.11.3.111	LP-WUSPS assistance information

The purpose of the LP-WUSPS assistance information IE is to transfer the required assistance information for LP-WUSPS.

The LP-WUSPS assistance information, information element is coded as shown in figure 9.11.3.111.1, figure 9.11.3.111.2, figure 9.11.3.111.3, figure 9.11.3.111.4 and table 9.11.3.111.1.

The LP-WUSPS assistance information IE may contain more than one LP-WUSPS assistance information types, each encoded within one octet. In this case, the order in which each of the LP-WUSPS assistance information types appear is irrelevant, i.e. this is left up to the implementation.

NOTE:	"Octet 3" in figure 9.11.3.111.2, figure 9.11.3.111.3 and figure 9.11.3.111.4 is illustrative. If multiple types are present, obviously octet 4 or higher are used.

The LP-WUSPS assistance information is a type 4 information element, with a minimum length of 2 octets. The value in octet 2 (Length of LP-WUSPS assistance information contents) determines the number of the LP-WUSPS assistance information types, which are encoded within the LP-WUSPS assistance information IE.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LP-WUSPS assistance information IEI |  |  |  |  |  |  |  | octet 1 |
| Length of LP-WUSPS assistance information contents |  |  |  |  |  |  |  | octet 2 |
| LP-WUSPS assistance information type |  |  |  |  |  |  |  | octet 3* |

Figure 9.11.3.111.1: LP-WUSPS assistance information information element

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type of information |  |  | LP-WUSPS paging subgroup ID value |  |  |  |  | octet 3 |

Figure 9.11.3.111.2: LP-WUSPS assistance information type – type of information= "000"

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type of information |  |  | UE paging probability information value |  |  |  |  | octet 3 |

Figure 9.11.3.111.3: LP-WUSPS assistance information type – type of information= "001"

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type of information |  |  | LP-WUS requested/enabled/disabled information value |  |  |  |  | octet 3 |

Figure 9.11.3.111.4: LP-WUSPS assistance information type – type of information= "010"

Table 9.11.3.111.1: LP-WUSPS assistance information information element

| Value part of the LP-WUSPS assistance information information element (octet 3 or higher) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| The value part of the LP-WUSPS assistance information information element consists of one or more LP-WUSPS assistance information type parameter(s). |  |  |  |  |  |  |
| LP-WUSPS assistance information type: |  |  |  |  |  |  |
| Type of information (octet 3 or higher, bits 6 to 8) |  |  |  |  |  |  |
| Bits |  |  |  |  |  |  |
| 8 | 7 | 6 |  |  |  |  |
| 0 | 0 | 0 | LP-WUSPS paging subgroup ID |  |  |  |
| 0 | 0 | 1 | UE paging probability information |  |  |  |
| 0 | 1 | 0 | LP-WUS requested/enabled/disabled information |  |  |  |
| All other values arespare, for future use and shall be ignored by this version of the protocol. |  |  |  |  |  |  |
| LP-WUSPS paging subgroup ID value (octet 3 or higher, bits 1 to 5) |  |  |  |  |  |  |
| This field contains one out of 31 (decimal) LP-WUSPS paging subgroup IDs that is assigned by the AMF for paging the UE. |  |  |  |  |  |  |
| Bits |  |  |  |  |  |  |  |
| 5 |  | 4 | 3 | 2 | 1 |  | LP-WUSPS paging subgroup ID value |
| 0 |  | 0 | 0 | 0 | 0 |  | LP-WUSPS paging subgroup ID 0 (decimal) |
|  |  |  | to |  |  |  |  |
| 1 |  | 1 | 1 | 1 | 0 |  | LP-WUSPS paging subgroup ID 30 (decimal) |
| 1 |  | 1 | 1 | 1 | 1 |  | reserved |
| UE paging probability information value (octet 3 or higher, bits 1 to 5) |  |  |  |  |  |  |
| This field contains the value of UE paging probability information provided by the UE to the AMF. It represents the probability of the UE receiving the paging. |  |  |  |  |  |  |
| Bit |  |  |  |  |  |  |
| 5 | 4 | 3 | 2 | 1 |  | UE paging probability information value |
| 0 | 0 | 0 | 0 | 0 |  | p00 (UE calculated paging probability is 0%) |
| 0 | 0 | 0 | 0 | 1 |  | p05 (UE calculated paging probability > 0% and <= 5%) |
| 0 | 0 | 0 | 1 | 0 |  | p10 (UE calculated paging probability > 5% and <= 10%) |
| 0 | 0 | 0 | 1 | 1 |  | p15 (UE calculated paging probability > 10% and <= 15%) |
| 0 | 0 | 1 | 0 | 0 |  | p20 (UE calculated paging probability > 15% and <= 20%) |
| 0 | 0 | 1 | 0 | 1 |  | p25 (UE calculated paging probability > 20% and <= 25%) |
| 0 | 0 | 1 | 1 | 0 |  | p30 (UE calculated paging probability > 25% and <= 30%) |
| 0 | 0 | 1 | 1 | 1 |  | p35 (UE calculated paging probability > 30% and <= 35%) |
| 0 | 1 | 0 | 0 | 0 |  | p40 (UE calculated paging probability > 35% and <= 40%) |
| 0 | 1 | 0 | 0 | 1 |  | p45 (UE calculated paging probability > 40% and <= 45%) |
| 0 | 1 | 0 | 1 | 0 |  | p50 (UE calculated paging probability > 45% and <= 50%) |
| 0 | 1 | 0 | 1 | 1 |  | p55 (UE calculated paging probability > 50% and <= 55%) |
| 0 | 1 | 1 | 0 | 0 |  | p60 (UE calculated paging probability > 55% and <= 60%) |
| 0 | 1 | 1 | 0 | 1 |  | p65 (UE calculated paging probability > 60% and <= 65%) |
| 0 | 1 | 1 | 1 | 0 |  | p70 (UE calculated paging probability > 65% and <= 70%) |
| 0 | 1 | 1 | 1 | 1 |  | p75 (UE calculated paging probability > 70% and <= 75%) |
| 1 | 0 | 0 | 0 | 0 |  | p80 (UE calculated paging probability > 75% and <= 80%) |
| 1 | 0 | 0 | 0 | 1 |  | p85 (UE calculated paging probability > 80% and <= 85%) |
| 1 | 0 | 0 | 1 | 0 |  | p90 (UE calculated paging probability > 85% and <= 90%) |
| 1 | 0 | 0 | 1 | 1 |  | p95 (UE calculated paging probability > 90% and <= 95%) |
| 1 | 0 | 1 | 0 | 0 |  | p100 (UE calculated paging probability > 95% and <= 100%) |
| All other values shall be interpreted as 10100 by this version of the protocol. |  |  |  |  |  |  |
| LP-WUS requested/enabled/disabled information value (octet 3 or higher, bits 1 to 5) |  |  |  |  |  |  |
| Bits |  |  |  |  |  |  |
| 5 | 4 | 3 | 2 | 1 |  | LP-WUS requested/enabled/disabled information value |
| 0 | 0 | 0 | 0 | 0 |  | Reserved |
| 0 | 0 | 0 | 0 | 1 |  | LP-WUS requested (UE to network) |
| 0 | 0 | 0 | 1 | 0 |  | LP-WUS enabled (network to UE) |
| 0 | 0 | 0 | 1 | 1 |  | LP-WUS disabled (network to UE) |
| All other values are spare, for future use and shall be ignored by this version of the protocol. |  |  |  |  |  |  |

* * * End of Changes * * * *