3GPP TSG-CT WG1 Meeting #157	C1-256108

Sophia Antipolis, France, 13-17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.193 | CR | 0239 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Link-specific multipath IP address/prefix and proxy information for MA PDU session of type IPv4v6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | MASSS |  |  |  |  |  | Date: |  |  | 2025-10-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | SA2 has agreed stage-2 CR S2-2507895 (SA2#170, Gothenburg, August 2025) that clarifies that when PDU Session is of type IPv4v6 and MPQUIC-IP or MPQUIC-UDP functionality is applied, then 2 IP address/prefixes per access can be allocated to the UE (i.e. total of four address/prefixes). Same clarification is needed in CT1 stage-3 spec, since current encoding of the "network steering functionalities information" already supports indicating two IP versions for the addresses of the "link-specific multipath" and the proxy information, but it is not clear in the procedure description how would the network uses this encoding. Specially the same was clarified for Ethernet MA PDU session when MPQUIC-E is applied (see the agreed CR C1-253597 in CT1#155 (May 2025)) and hence it may be wrongly understood that MA PDU session of type IPv4V6 is not considered in this requirement. Moreover, it is not clear how would the UE use those allocated IPv4 and IPv6 addresses when both types are allocated. This needs to be clarified as well. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specifies that, for MA PDU session of type IPv4v6 when MPQUIC-IP or MPQUIC-UDP is enabled, the network indicates the two versions of the IP addresses (IPv4 and IPv6) to the UE for both the "link-specific multipath" IP addresses and the proxy information. Also clarifying that, in that case, the UE can select one version of the IP addresses (either IPv4 or IPv6) depending on whether IPv4 or IPv6 traffic is to be sent on the MPQUIC connection. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No requirements for the network related to which IP version to indicate to the UE for MA PDU session of type IPv4v6 when MPQUIC-IP or MPQUIC-UDP is enabled, which would possibly lead the network to limit the indicated IP version to only one of them and hence limiting the possible traffic types, and misalignment with stage-2 requirements |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.4.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** First change *****

##### 6.1.4.1.1	Steering information for different steering functionalities

When the network accepts to enable only the ATSSS-LL functionality with only the active-standby steering mode for an MA PDU session in the UPF as specified in the clause 5.32.2 of 3GPP TS 23.501 [2], then the network shall provide to the UE one or more ATSSS rules including an "ATSSS-LL functionality" steering functionality and an "active-standby" steering mode.

When the UE indicates support for ATSSS-LL functionality with any steering mode allowed for ATSSS-LL and the network accepts to enable this functionality for an MA PDU session of any supported type, then the network shall enable ATSSS-LL functionality with any steering mode allowed for ATSSS-LL in the UPF as specified in the clause 5.32.2 of 3GPP TS 23.501 [2] and provide one or more ATSSS rules to the UE.

When the UE indicates support for one or more of MPTCP functionality with any steering mode, MPQUIC-UDP functionality with any steering mode, MPQUIC-IP functionality with any steering mode, and MPQUIC-E functionality with any steering mode, as well as the ATSSS-LL functionality with only the active-standby steering mode and the network accepts to enable one or more of the indicated functionalities for an MA PDU session in the UPF as specified in the clause 5.32.2 of 3GPP TS 23.501 [2], then the network shall provide the following information to the UE:

NOTE 0:	MPQUIC-E functionality and ATSSS-LL functionality cannot be enabled for the same Ethernet type MA PDU Session (see clause 5.32.6.1 of 3GPP TS 23.501 [2]).

a)	two "link-specific multipath" IP addresses/prefixes used by the indicated steering functionality in the UE, one associated with the 3GPP access and another associated with the non-3GPP access;

b)	the IP address, port number and the type of one or more proxies in the UPF; and

NOTE 1:	It is possible that the network provides the "link-specific multipath" IP addresses/prefixes and the proxy information that are not routable via N6 (e.g. IPv6 link local address).

c)	one or more ATSSS rules including an ATSSS rule which is composed of a precedence with value "255", a "match-all type" traffic descriptor, an "ATSSS-LL functionality" steering functionality and an "active-standby" steering mode.

The UE shall use the "link-specific multipath" addresses/prefixes to establish subflows over non-3GPP access and over 3GPP access.

When the MA PDU session is IP type (i.e. IPv4, IPv6 or IPv4v6), the network shall not enable the MPQUIC-E functionality.

When the MA PDU session is Ethernet type, the network shall not enable the MPTCP functionality, the MPQUIC-UDP functionality nor the MPQUIC-IP functionality.

When the MA PDU session is Ethernet type, the MPQUIC-E functionality is enabled and the network supports indicating IPv4v6 in the IP address type, or when the MA PDU session is of type IPv4v6 and the MPQUIC-UDP functionality or the MPQUIC-IP functionality is enabled, the network shall:

a)	set the IP address type of the "link-specific multipath" IP address/prefix associated with the 3GPP access to "IPv4v6" and indicate two IP addresses in the corresponding IP address field (an IPv4 address and an IPv6 address);

b)	set the IP address type of the "link-specific multipath" IP address/prefix associated with the non-3GPP access to "IPv4v6" and indicate two IP addresses in the corresponding IP address field (an IPv4 address and an IPv6 address); and

c)	set the proxy IP address type to "IPv4v6" and indicate two IP addresses in the corresponding IP address field (an IPv4 address and an IPv6 address).

NOTE 2:	When the MPQUIC-E functionality is enabled, if the network provides the "link-specific multipath" IP addresses/prefixes and the proxy information for both IP versions (IPv4 and IPv6), the UE can select one version of the IP addresses (either IPv4 or IPv6) based on its capabilities.

NOTE 3:	When the MA PDU session is of type IPv4v6 and the MPQUIC-UDP functionality or the MPQUIC-IP functionality is enabled, the UE can select one version of the IP addresses (either IPv4 or IPv6) of the "link-specific multipath" IP addresses/prefixes and the proxy information per MPQUIC connection depending on whether IPv4 or IPv6 traffic is to be sent on that MPQUIC connection.

***** End of changes *****