3GPP TSG-CT WG1 Meeting #157	C1-256124

Sophia Antipolis, France, 13-17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.282 | CR | 0469 | rev | - | Current version: | 18.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Affiliation check upon receiving a "SIP MESSAGE request for absolute URI discovery request" message |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCImp-MCData-CT |  |  |  |  |  | Date: |  |  | 2025-10-02 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Ever since Section 10.2.1.3.4 was introduced (C1-173498), the reference for the affiliation check has not been updated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Reference update Backward compatibility analysis The proposed change is backward compatible. According to the current version of the specification, a controlling MCData function will not indicate "120 user is not affiliated to this group" in a response to a "SIP MESSAGE request for absolute URI discovery request" message. However, an MCData client compliant to the current version of the specification can handle the warning text. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Upon receiving a "SIP MESSAGE request for absolute URI discovery request" message, a controlling MCData function cannot evaluate the condition specified in 4)-f)-ii) of Section 10.2.1.3.4. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.2.1.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * *

##### 10.2.1.3.4	Controlling MCData function procedures

Upon receiving a "SIP MESSAGE request for absolute URI discovery request" message, the controlling MCData function:

1)	if unable to process the request due to a lack of resources or a risk of congestion exists, may reject the SIP MESSAGE request with a SIP 500 (Server Internal Error) response. The controlling MCData function may include a Retry-After header field to the SIP 500 (Server Internal Error) response as specified in IETF RFC 3261 [4]. Otherwise, continue with the rest of the steps;

2)	if the SIP MESSAGE does not contain	an application/vnd.3gpp.mcdata-info+xml MIME body, shall reject the SIP MESSAGE request with a SIP 403 (Forbidden) response, with warning text set to "199 expected MIME bodies not in the request" in a Warning header field as specified in clause 4.9, and shall not continue with the rest of the steps in this clause;

3)	shall decode the contents of the application/vnd.3gpp.mcdata-info+xml MIME body contained in the SIP MESSAGE;

4)	if the <mcdata-calling-group-id> element in the application/vnd.3gpp.mcdata-info+xml MIME body of the SIP MESSAGE request is present:

a)	shall retrieve the group document associated with the group identity in the SIP MESSAGE request by following the procedures in clause 6.3.3, and shall continue with the remaining steps if the procedures in clause 6.3.3 were successful;

b)	if the <on-network-disabled> element is present in the group document, shall send a SIP 403 (Forbidden) response with the warning text set to "115 group is disabled" in a Warning header field as specified in clause 4.9 and shall not continue with the rest of the steps;

b1)	if the group document contains a <list-service> element that contains a <preconfigured-group-use-only> element that is set to the value "true", shall reject the SIP INVITE request with a SIP 403 (Forbidden) response with the warning text set to "167 call is not allowed on the preconfigured group" as specified in clause 4.9 "Warning header field" and shall skip the rest of this procedure;

c)	if the <list> element of the <list-service> element in the group document does not contain an <entry> element with a "uri" attribute matching the MCData ID of the originating user contained in the <mcdata-calling-user-id> element of the application/vnd.3gpp.mcdata-info+xml MIME body in the SIP MESSAGE request, shall send a SIP 403 (Forbidden) response with the warning text set to "116 user is not part of the MCData group" in a Warning header field as specified in clause 4.9 and shall not continue with the rest of the steps;

d)	if the <list-service> element contains a<mcdata-allow-file-distribution> element in the group document set to a value of "false", shall send a SIP 403 (Forbidden) response with the warning text set to "213 file distribution not allowed for this group" in a Warning header field as specified in clause 4.9 and shall not continue with the rest of the steps;

e)	if the <supported-services> element is not present in the group document or is present and contains a <service> element containing an "enabler" attribute which is not set to the value "urn:urn-7:3gpp-service.ims.icsi.mcdata.fd", shall send a SIP 488 (Not Acceptable) response with the warning text set to "214 FD services not supported for this group" in a Warning header field as specified in clause 4.9 and shall not continue with the rest of the steps;

f)	if the MCData server group FD procedures in clause 11.1 indicate that the user identified by the MCData ID:

i)	is not allowed to send group MCData communications on this group identity as determined by step 1) of clause 11.1, shall reject the SIP MESSAGE request with a SIP 403 (Forbidden) response, with warning text set to "201 user not authorised to transmit data on this group identity" in a Warning header field as specified in clause 4.9, and shall not continue with the rest of the steps in this clause; and

ii)	the originating user identified by the MCData ID is not affiliated to the group identity contained in the SIP MESSAGE request, as specified in clause 6.3.5, shall return a SIP 403 (Forbidden) response with the warning text set to "120 user is not affiliated to this group" in a Warning header field as specified in clause 4.9, and skip the rest of the steps below;

5)	shall generate a SIP 200 (OK) response in response to the "SIP MESSAGE request for absolute URI discovery request for controlling MCData function";

6)	shall send the SIP 200 (OK) response towards the originating participating MCData function according to 3GPP TS 24.229 [5]; and

7)	shall generate a SIP MESSAGE request in accordance with 3GPP TS 24.229 [5] and IETF RFC 3428 [6]. In the generation of the SIP MESSAGE request, the controlling MCData function:

a)	shall include an Accept-Contact header field with the media feature tag g.3gpp.icsi-ref with the value of "urn:urn-7:3gpp-service.ims.icsi.mcdata.fd" along with parameters "require" and "explicit" according to IETF RFC 3841 [8] in the outgoing SIP MESSAGE request;

b)	shall identify the absolute URI of the media storage function associated with the controlling function:

c)	shall include a P-Asserted-Service header field with the value "urn:urn-7:3gpp-service.ims.icsi.mcdata.fd";

d)	shall include an application/vnd.3gpp.mcdata-info+xml MIME body in the SIP MESSAGE request, following the rules specified in clause 6.4 for the handling of MIME bodies in a SIP message, with:

i)	a <request-type> element containing the value "msf-disc-res"; and

ii)	an <mcdata-controller-psi> element set to the absolute URI of the media storage function if in step b) above;

e)	shall set the Request-URI of the outgoing SIP MESSAGE request to the public service identity of the participating MCData function associated to the MCData ID of the originating user mentioned in the <mcdata-calling-user-id> element of the application/vnd.3gpp.mcdata-info+xml MIME body of the outgoing SIP MESSAGE request; and

NOTE 1:	The public service identity can identify the participating MCData function in the local MCData system or in an interconnected MCData system.

NOTE 2:	If the participating MCData function is in an interconnected MCData system in a different trust domain, then the public service identity can identify the MCData gateway server that acts as an entry point in the interconnected MCData system from the local MCData system.

NOTE 3:	If the participating MCData function is in an interconnected MCData system in a different trust domain, then the local MCData system can route the SIP request through an MCData gateway server that acts as an exit point from the local MCData system to the interconnected MCData system.

NOTE 4:	How the controlling MCData function determines the public service identity of the participating MCData function serving the target MCData ID or of the MCData gateway server in the interconnected MCData system is out of the scope of the present document.

NOTE 5:	How the local MCData system routes the SIP request through an exit MCData gateway server is out of the scope of the present document.

f)	shall include a P-Asserted-Identity header field in the outgoing SIP MESSAGE request set to the public service identity of the controlling MCData function; and

8)	shall send the SIP MESSAGE request towards the participating MCData function as specified in 3GPP TS 24.229 [5].

* * * End of Change(s) * * *