3GPP TSG-CT WG1 Meeting #157	C1-256143

# Sophia-Antipolis, France, 13-17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.301 | CR | 4533 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of UE initiated transport of user data via the control plane with overhead reduction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NORDAT_CP |  |  |  |  |  | Date: |  |  | 2025-10-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | UE initiated transport of user data via the control plane with overhead reduction procedure is not consistent on referring to EMM sublayer, as: - in 6.6.5.2 and 6.6.5.3, user data are solely provided to EMM sublayer, without mentioning a particular EMM procedure; and - in 6.6.5.4, EMM procedures are mentioned. |  |  |  |  |  |  |  |  |
| Summary of change: |  | EMM procedures are mentioned consistently. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.5.2, 6.6.5.3, 6.6.5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** change *****

#### 6.6.5.2	UE initiated transport of user data via the control plane with overhead reduction

Upon receipt of a request to transfer user data via the control plane, the UE shall initiate a service request procedure or an EMM data transport procedure for transport of the user data, the EPS bearer identity, and the DDX, if received from upper layers.

The length of the user data should not exceed the link MTU size for the respective type of user data (IPv4, IPv6 or Non-IP). If the user data is an Ethernet frame, then the length of the Ethernet frame payload should not exceed the Ethernet frame payload MTU size.

NOTE:	The recommended maximum size for link MTU is 1358 octets to prevent fragmentation in the backbone network (see 3GPP TS 23.060 [74]). Depending on the network configuration, setting link MTU size to a value larger than 1358 octets could lead to inefficient core network implementation due to fragmentation.

Upon reception of the user data and the EPS bearer identity from EMM sublayer, the MME shall identify the PDN connection to the SCEF or to the PDN GW, based on the EPS bearer identity, and forward the user data accordingly.

***** change *****

#### 6.6.5.3	Network initiated transport of user data via the control plane with overhead reduction

Upon receipt of a request to transfer user data via the control plane, the network shall initiate an EMM data transport procedure for transport of the user data and the EPS bearer identity.

Upon reception of the user data and the EPS bearer identity from EMM sublayer, the UE shall:

-	forward the user data to the upper layers;

-	stop timer T3396 if it is running for the APN provided by the UE, if the UE provided an APN for the establishment of the PDN connection; and

-	stop the timer T3396 associated with no APN if it is running, if the UE did not provide an APN for the establishment of the PDN connection and the request type was different from "emergency" and from "handover of emergency bearer services".

***** change *****

#### 6.6.5.4	Abnormal cases in the UE

The following abnormal case can be identified:

a)	T3396 is running

The UE shall not initiate a service request procedure or an EMM data transport procedure for transport of user data, unless:

-	the UE is a UE configured to use AC11 – 15 in selected PLMN; or

-	the user data is used for an exception data reporting, and the UE is allowed to use exception data reporting (see the ExceptionDataReportingAllowed leaf of the NAS configuration MO in 3GPP TS 24.368 [15A] or the USIM file EFNASCONFIG in 3GPP TS 31.102 [17]).

The UE can initiate a service request procedure or a EMM data transport procedure for transport of user data, if still necessary, when timer T3396 expires or is stopped.