3GPP TSG-CT WG1 Meeting #157	C1-256148

# Sophia-Antipolis, France, 13-17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.301 | CR | 4535 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for ciphering of EMM TRANSPORT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NORDAT_CP |  |  |  |  |  | Date: |  |  | 2025-10-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 1) 24.301 states: ------------ The UE shall partially cipher the EMM TRANSPORT message by ciphering the part of the message consisting of octet 7 and all subsequent octets, using the ciphering algorithm of the current EPS security context. ------------ However, the EMM TRANSPORT message without the Data container IE is only 6 octets long. Thus, the statement above should apply only for the EMM TRANSPORT message with the Data container IE. 2) The below text expects that there is a special handling for EMM TRANSPORT message, regardless of whether the Data container IE is included or not: ------------ Except for a) the CONTROL PLANE SERVICE REQUEST message including an ESM message container information element or a NAS message container information element and b) the EMM TRANSPORT message, the UE shall start the ciphering and deciphering of NAS messages when the secure exchange of NAS messages has been established for a NAS signalling connection. ------------ For the EMM TRANSPORT message without Data container IE, the above is not correct - the EMM TRANSPORT message without Data container IE is not ciphered and thus "the UE shall start the ciphering and deciphering of NAS messages when the secure exchange of NAS messages has been established for a NAS signalling connection" applies for the EMM TRANSPORT without Data container IE. Thus, the exception in the above should apply only for the EMM TRANSPORT message with the Data container IE. 3) When e.g. the UPLINK GENERIC NAS TRANSPORT message (included the SECURITY PROTECTED NAS MESSAGE message) is sent, then the entire UPLINK GENERIC NAS TRANSPORT message is ciphered, including the Protocol discrimator IE and the Security header type IE of the UPLINK GENERIC NAS TRANSPORT message. However, when the EMM TRANSPORT message is sent (NOT included in the SECURITY PROTECTED NAS MESSAGE message), the Protocol discriminator IE and the Security header type IE of the EMM TRANSPORT message are NOT ciphered as 24.301 states: ------------ The UE shall partially cipher the EMM TRANSPORT message by ciphering the part of the message consisting of octet 7 and all subsequent octets, using the ciphering algorithm of the current EPS security context. ------------ Thus, the EMM TRANSPORT message should be "partially ciphered" (rather than "ciphered"). In some places, "partially ciphered" is used: ------------ The UE shall partially cipher the EMM TRANSPORT message by ciphering the part of the message consisting of octet 7 and all subsequent octets, using the ciphering algorithm of the current EPS security context. ------------ and ------------ The MME shall discard any: ... - EMM TRANSPORT message including Data container information element, which has not been partially ciphered according to the rules described above. ------------ However, in other places, solely "ciphered" is used: ------------ a1) ciphered, if it is an EMM TRANSPORT message including Data container information element; or ------------ |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) The EMM TRANSPORT message without the Data container IE is not ciphered. 2) The following statement is updated so that the EMM TRANSPORT message with the Data container IE is the exception. ------------ Except for a) the CONTROL PLANE SERVICE REQUEST message including an ESM message container information element or a NAS message container information element and b) the EMM TRANSPORT message, the UE shall start the ciphering and deciphering of NAS messages when the secure exchange of NAS messages has been established for a NAS signalling connection. ------------ 3) the EMM TRANSPORT message is "partially ciphered". |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 1) misleading specification, leading to longer implementation times. 2) confusing specification, leading to longer implementation times. 3) possible interoperability issues, if the sender cipheres entire EMM TRANSPORT message. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** change *****

### 4.4.5	Ciphering of NAS signalling messages

The use of ciphering in a network is an operator option subject to MME configuration. When operation of the network without ciphering is configured, the MME shall indicate the use of "null ciphering algorithm" EEA0 (see clause 9.9.3.23) in the current security context for all UEs. For setting the security header type in outbound NAS messages, the UE and the MME shall apply the same rules irrespective of whether the "null ciphering algorithm" or any other ciphering algorithm is indicated in the security context.

When the UE establishes a new NAS signalling connection, it shall send the initial NAS message

a)	partially ciphered, if it is a CONTROL PLANE SERVICE REQUEST message including an ESM message container information element or a NAS message container information element;

a1)	partially ciphered, if it is an EMM TRANSPORT message including Data container information element; or

b)	unciphered, if it is any other initial NAS message.

The UE shall partially cipher the CONTROL PLANE SERVICE REQUEST message by ciphering the value part of the ESM message container IE or the value part of the NAS message container, using the ciphering algorithm of the current EPS security context.

The UE shall partially cipher the EMM TRANSPORT message including the Data container information element by ciphering the part of the message consisting of octet 7 and all subsequent octets, using the ciphering algorithm of the current EPS security context.

The UE shall send the ATTACH REQUEST message always unciphered.

The UE shall send the TRACKING AREA UPDATE REQUEST message always unciphered.

Except for a) the CONTROL PLANE SERVICE REQUEST message including an ESM message container information element or a NAS message container information element and b) the EMM TRANSPORT message including the Data container information element, the UE shall start the ciphering and deciphering of NAS messages when the secure exchange of NAS messages has been established for a NAS signalling connection. From this time onward, unless explicitly defined, the UE shall send all NAS messages ciphered until the NAS signalling connection is released, or the UE performs intersystem handover to A/Gb mode or Iu mode.

The MME shall start ciphering and deciphering of NAS messages as described in clause 4.4.2.3. From this time onward, except for the SECURITY MODE COMMAND message, the MME shall send all NAS messages ciphered until the NAS signalling connection is released, or the UE performs intersystem handover to A/Gb mode or Iu mode.

Once the encryption of NAS messages has been started between the MME and the UE, the receiver shall discard the unciphered NAS messages which shall have been ciphered according to the rules described in this specification. The MME shall discard any:

-	CONTROL PLANE SERVICE REQUEST message including an ESM message container information element or a NAS message container information element; or

-	EMM TRANSPORT message including Data container information element,

which has not been partially ciphered according to the rules described above.

If the "null ciphering algorithm" EEA0 has been selected as a ciphering algorithm, the NAS messages with the security header indicating ciphering are regarded as ciphered.

Details of ciphering and deciphering of NAS signalling messages are specified in 3GPP TS 33.401 [19].