3GPP TSG-CT WG1 Meeting #157	C1-256204

# Sophia-Antipolis, France, 13th – 17th October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 43.022 | CR | draftCR | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Restriction on RAT utilization |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple, OPPO, InterDigital, Huawei, HiSilicon, Nokia, Samsung, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN |  |  |  |  |  |  |  |  |  |
| Work item code: | ECRATU |  |  |  |  |  | Date: |  |  | 2025-09-29 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | With the LS "UE usage of the RAT restrictions" from CT1 (C1-247193), R2 is informed that network may indicate the access technology utilization restriction information to UE with NAS EMM and 5GMM procedures. And UE will maintain a list of "PLMNs with associated access technology restrictions" which is applicable for the PLMN and optionally for EPLMN: - not consider any PLMN/access technology combination for which an access technology restriction is configured in the list of "PLMNs with associated access technology restrictions" as a candidate PLMN/access technology for PLMN selection purpose. - If an access technology is restricted in a PLMN, not select any cell of the PLMN deployed in the restricted access technology during cell selection and re-selection for normal service. The restrictions are not applicable if the UE is selecting a cell for limited service. In summary, this feature requires UE to avoid selecting or re-selecting a cell which is deployed in an access technology for which an access technology restriction is applicable for the PLMN or, if applicable EPLMN. In RAN2#130 meeting, RAN2 confirms that AS layer support of ECRATU feature (i.e., cell selection and reselection procedure) by GERAN/UTRAN is optional. And according to CT1 Reply LS (C1-255319), CT1 sees no need to introduce separate NAS capabilities for 2G/3G RAT. Hence, from AS layer perspective, we need consider the case that the MS may not support this feature requirement when specifying the corresponding UE behavior when dealing with RAT restriction. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce a description associated with access technology utilization restriction information including the following aspects: - The usage of the list of "PLMNs with associated access technology restrictions" during cell selection and cell reselection. - The impact on the suitable cell definition. (NOTE: The restrictions are not applicable if the UE is selecting a cell for limited service.) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Cell selection and reselection taking RAT utilization restriction information as received over NAS is not possible. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

### 3.2.1	Normal camping

For normal service, the MS has to camp on a suitable cell, tune to that cell's control channel(s), and possibly register within the PLMN so that the MS can:

a)	Receive system information from the PLMN, e.g., the cell options;

b)	Receive paging messages from the PLMN, e.g., when there is an incoming call for the MS;

c)	Initiate call setup for outgoing calls or other actions from the MS (where possible, see subclauses 3.5.3 and 3.5.4).

The choice of such a suitable cell for the purpose of receiving normal service is referred to as "normal camping". There are various requirements that a cell must satisfy before an MS can perform normal camping on it:

i)	It should be a cell of the selected PLMN or, if the selected PLMN is equal to the last registered PLMN, an equivalent PLMN;

ii)	It should not be "barred" (see subclause 3.5.1);

iii)	It should not be in an LA which is in the list of "forbidden LAs for roaming";

iv)	The radio path loss between MS and BTS must be below a threshold set by the PLMN operator. This is estimated as shown in subclause 3.6;

v)	It should not be a SoLSA exclusive cell to which MS does not subscribe.  This requirement is only valid for MSs supporting SoLSA;

vi)	The access technology of the cell is not marked as restricted for the PLMN that fulfils the case i) above in the list of "PLMNs with associated access technology restrictions" (TS 23.122 [31]). This requirement is only valid for MSs supporting "access technology restriction".

Initially, the MS looks for a cell which satisfies these above constraints ("suitable cell") by checking cells in descending order of received signal strength. If a suitable cell is found, the MS camps on it and performs any registration necessary. Cells can have two levels of priority, suitable cells which are of low priority are only camped on if there are no other suitable cells of normal priority. (This is called "cell selection").

When camped on a cell the MS regularly looks to see if there is a better cell in terms of a cell re-selection criterion, and if there is, the better cell is selected. Also if one of the other criteria changes, (e.g., the current serving cell becomes barred), or there is a downlink signalling failure (see subclause 3.6), a new cell is selected. (This is called "cell reselection"). A MS that has enabled PEO or EC operation has relaxed requirements for how often it verifies the suitability of its serving cell and the suitability of neighbour cells for re-selection (see 3GPP TS 45.008 [25] and 3GPP TS 44.018 [23]).

In order to speed up these processes, a list of the RF channels containing BCCH or CPBCCH carriers of the same PLMN is broadcast in the system information messages, see subclause 4.8. Also, the MS does not need to search all possible RF channels to find a suitable cell. If, after searching the number of RF channels, given for each frequency band below, with the strongest received signal level, a BCCH or CPBCCH carrier has been found but no suitable cell of the selected PLMN has been found, the MS can stop the attempt to find a suitable cell of the selected PLMN.

### The number of channels to be searched are 15 for T-GSM 380, 15 for T-GSM 410, 15 for GSM 450, 15 for GSM 480, 25 for GSM 710, 25 for GSM 750, 25 for T-GSM 810, 30 for GSM 850 Band, 30 for GSM 900, 15 for T-GSM 900 and 40 for DCS 1800 and PCS 1900.

End of change