3GPP TSG-CT WG1 Meeting #157	C1-256205

# Sophia Antipolis, France; 13th – 17th October 2025

Source:	Lenovo

Title:	DP on unsupported AIoT Devices

Spec:	-

Agenda item:	19.70

Document for:	Discussion

## 1. Introduction

In 3GPP Release 19 Ambient IoT, the SA2 working group has specified the command procedure in TS 23.369 clause 6.2.3, as illustrated in the figure 1 below. The procedure consists of two phases: the Inventory procedure (Phase 1), shown in Step 7 of the figure 1, and the Command Request/Response procedure (Phase 2), depicted in Steps 8 to 11.

Figure 1: Command Procedure (see Figure 6.2.3-1 of TS 23.369)

Furthermore, 3GPP TS 23.389 clause 5.2.2.2 “Read and Write Command” specifies, that if an AIoT Device does not support Read Request or Write Request command, or the parameters in the commands are invalid the AIoT Device shall reject the command.

If an AIoT Device does not support Read Request or Write Request commands, or the parameters in the commands are invalid the AIoT Device shall reject the command.

Additionally, the 3GPP CT1 working group has agreed in 3GPP TS 24.389 on the new AIoT NAS message types for the AIoT device to reject the Read and Write Command using Read command reject AIoT NAS message type and Write command reject AIoT NAS message type. The complete list of AIoT NAS message types are shown in Figure 2:

| Bits |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| 0 | 1 | 0 | 0 | 0 | 0 | 0 | 1 |  | Inventory report |
| 0 | 1 | 0 | 0 | 0 | 0 | 1 | 0 |  | Read command |
| 0 | 1 | 0 | 0 | 0 | 0 | 1 | 1 |  | Read complete |
| 0 | 1 | 0 | 0 | 0 | 1 | 0 | 0 |  | Read command reject |
| 0 | 1 | 0 | 0 | 0 | 1 | 0 | 1 |  | Write command |
| 0 | 0 | 0 | 0 | 0 | 1 | 1 | 0 |  | Write complete |
| 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 |  | Write command reject |
| 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 |  | Permanent disable command |
| 0 | 0 | 0 | 0 | 1 | 0 | 0 | 1 |  | Permanent disable complete |

Figure 2: Message types for AIoT NAS (see Table 7.2.2-1 of TS 24.369)

However, if a specific AIoT device does not support the Read or Write command, it is still required to complete all phases of the procedure in Figure 1. This situation is expected to occur frequently, for example in a group command using filter information, where the group consists of heterogeneous devices, some of which support Read and Write commands while others do not. On the device side this includes receiving the command message, performing security validation, generating a response message (e.g., Read Command Reject or Write Command Reject AIoT NAS message), applying message protection, and transmitting the response over the air interface. Likewise, the AIoT Reader must generate a downlink RF signal and allocate uplink resources to enable the AIoT device to respond to the command request.

Observation 1: If an AIoT device does not support Read or Write commands, it must still complete all phases of the command procedure, causing unnecessary radio resource allocation, signaling and energy drain. This scenario is common in group commands involving heterogeneous devices.

Furthermore, it is anticipated that future 3GPP releases will introduce additional AIoT command types and message formats. As a result, the likelihood of command rejection will increase, particularly when legacy Release 19 AIoT devices, limited in functionality, must continue to be supported. It remains unclear how a Rel-19 AIoT device can correctly reject a command from a later release that it cannot interpret or validate.

Observation 2: In future 3GPP releases additional AIoT command types and message formats might be introduced. As a result, the likelihood of command rejection will increase, particularly when legacy Release 19 AIoT devices, limited in functionality, must continue to be supported. It remains unclear how a Rel-19 AIoT device can correctly reject a command from a later release that it cannot interpret or validate.

Finally, in topology-2 deployments starting from 3GPP Rel-20, where a UE acts as an intermediate reader, a UE may experience significant processing load, RF activity and energy consumption due to unnecessary signaling for incompatible devices. This may impact spectrum efficiency and UE battery life.

Observation 3: In topology-2 deployments starting from 3GPP Rel-20, where a UE acts as an intermediate reader, a UE may experience significant processing load, RF activity and energy consumption due to unnecessary signaling for incompatible devices. This may impact spectrum efficiency and UE battery life.

## 2. Possible solution

To reduce the number of device and network interactions when an AIoT device does not support a specific command request from the network the following solutions have been identified.

### 2.1	Solution 1: Command message type in Inventory Request

The AIOTF includes the command type of the follow-on command already in the Inventory Request message in step 7 of the procedure illustrated in Figure 1. Upon receiving the Inventory Request, an AIoT device evaluates the command type of anticipated follow-on command against its supported capabilities.

- If compatible, the device responds with an Inventory Report message as per normal procedure.

- If incompatible, the device does not send an Inventory Report message.

At the AIoTF, the absence of an Inventory Report message for a given AIoT device is interpreted as incompatibility with the anticipated command. Consequently, the AIoTF does not initiate the Command Request phase for that AIoT device.

### 2.2	Solution 2: Device capability information in Inventory Report

The AIOT device includes its capability information in Inventory Report message in step 7 of the procedure illustrated in Figure 1. Upon receiving the Inventory Report message, the AIOTF sends the Command request in step 8 only if it determines that the received device capability information supports the Command request.

### 2.3	Solution 3: New Device Capability Command

A new Device Capability Command is used by the AIOTF to explicitly query an AIoT device for its supported capability. The AIOTF stores the received device capability in the profile data of the AIoT device. When a subsequent command procedure is initiated (as shown in Figure 1), the AIoTF retrieves the device capability from the profile data of the AIoT device and sends the Command Request in step 8 only if it determines that the sored device capability supports the Command request.

### 2.3	Solution 4: Device capability in new generic error message.

A new generic error message is introduced for the AIoT device to reject unknown or unsupported command requests in step 11 of Figure 1. The AIoT device includes its capability in the generic error message. The AIOTF stores the received device capability in the profile data of the AIoT device. When a subsequent command procedure is initiated (as shown in Figure 1), the AIoTF retrieves the device capability from the profile data of the AIoT device and sends the Command Request in step 8 only if it determines that the device capability supports the Command request.

## 3. Evaluation

Based on the previous observations, this section evaluates the two proposed solutions for reducing unnecessary signaling when an AIoT device does not support a specific command request. The evaluation considers signaling efficiency, backward compatibility, implementation complexity, and impact on device and network behavior.

Observation 1: If an AIoT device does not support Read or Write commands, it must still complete all phases of the command procedure, causing unnecessary radio resource allocation, signaling and energy drain. This scenario is common in group commands involving heterogeneous devices..

- Solution 1: Addresses the issue by allowing a device to pre-filter commands based on known and supported types. Avoids inventory report and command request/response for unsupported devices.

- Solution 2: Addresses the issue by allowing a device to provide its capability information in the Inventory report. Avoids command request/response for unsupported devices.

- Solution 3: Addresses the issue by allowing the AIOTF to learn the capability of the device using a separate capability command procedure. Adds extra capability command procedure initially but avoids subsequent command request/response for unsupported devices.

- Solution 4: Addresses the issue by allowing the AIOTF to learn the capability of the device during an initial failure attempt. Requires initial failure attempt but avoids subsequent command request/response for unsupported devices.

Observation 2: In future 3GPP releases, additional AIoT command types and message formats might be introduced. This increases the likelihood of command rejection, especially for legacy Rel-19 devices. It also remains unclear how such devices will correctly reject commands they cannot interpret or validate.

- Solution 1: Addresses the issue by allowing devices to pre-filter commands based on a known set of supported types. Avoids inventory report and command request/response for future commands on Rel-19 devices.

- Solution 2: Addresses the issue by allowing devices to provide their capability information in the Inventory report. AIOTF determines device support and avoids command request/response for future commands on Rel-19 devices.

- Solution 3: Addresses the issue by allowing the AIOTF to learn the capability of the device using a separate capability command procedure. Adds extra capability command procedure initially but avoids subsequent command request/response for future commands on Rel-19 devices.

Solution 4: Addresses the issue by allowing the AIOTF to learn the capability of the device during an initial failure attempt. Requires initial failure attempt but avoids subsequent command request/response for future commands on Rel-19 devices.Observation 3: In topology-2 deployments starting from 3GPP Rel-20, where a UE acts as an intermediate reader, the UE may experience significant processing load, RF activity, and energy consumption due to unnecessary signaling for incompatible devices.

- Solution 1: Addresses the issue by allowing the procedure to skip the Inventory report, Command request, and Command response messages, thereby reducing RF activity and energy consumption of the UE acting as a reader.

- Solution 2: Addresses the issue by allowing the procedure to skip the Command request, and Command response messages, thereby reducing RF activity and energy consumption of the UE acting as a reader.

- Solution 3: Addresses the issue by allowing the procedure to skip the Command request, and Command response messages (based on the learned capability) and thereby reducing RF activity and energy consumption of the UE acting as a reader.

- Solution 4: Addresses the issue by allowing the procedure to skip the Command request, and Command response messages (based on the learned capability) and thereby reducing RF activity and energy consumption of the UE acting as a reader.

### 3.1 Evaluation of the complexity and efficiency of the solutions

| Aspect | Solution 1 (Command message type in Inventory Request) | Solution 2 (Device capability information in Inventory Report) | Solution 3 (New Device Capability Command) | Solution 4 (Device capability in new generic error message) |
| --- | --- | --- | --- | --- |
| Signaling Efficiency | High – avoids inventory report and command request/response for unsupported devices | Moderate – avoids command request/response for unsupported devices. Inventory report is still required. | Moderate – adds one extra procedure initially, avoids command request/response for unsupported devices later. | Moderate – first failure still occurs, avoids command request/response for unsupported devices later. |
| Heterogeneous groups support | High – skips Inventory Report for unsupported device | Low – all devices still send Inventory Report | Low – all devices still send Inventory Report | Low – all devices still send Inventory Report |
| Deterministic in identifying the device capability | Moderate – relies on implicit absence of Inventory Report | High – explicit capability transfer | High – explicit capability transfer | High – explicit capability transfer in error message |
| Security | Command type is unprotected, Risk for false detection (same for filter information). | Capability is unprotected, Risk of bidding down attack. | Capability exchange can reuse NAS security; no new risks if integrity protected | Same as Solution 3; error message is integrity protected |
| Implementation Complexity | Low - AIOTF treats incompatible devices the same as if they are not present | Moderate – AIOTF must interpret the device capability information | High – requires new Command type. | Moderate – adds new NAS message and parsing logic |
| Device Processing Load | Significantly reduced – avoids Inventory report and command procedure | Reduced – avoids command procedure | Slightly increased initially, then reduced for future commands | Slightly increased initially, then reduced for future commands |
| Network Resource Utilization | Significantly reduced - avoids unnecessary RF signaling, uplink and downlink allocation | Reduced – partially avoids unnecessary RF signaling, uplink and downlink allocation | Moderate – extra signaling for capability query, then optimized | Moderate – first failure costs resources, then optimized |

## 4. Proposal

CT1 is invited to agree on sending a LS to SA2, requesting SA2 to address the signaling inefficiencies identified in this paper. The LS should highlight the need for architectural support to avoid unnecessary command procedures for unsupported AIoT devices. The draft LS is in C1-256206.