3GPP TSG-CT1 Meeting #157	C1-256213

# Sophia-Antipolis, France, 13th Oct 2025 - 17th Oct 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.229 | CR | 6750 | rev | - | Current version: | 19.4.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | S-CSCF procedures for signing RCD info |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2025-10-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | S-CSCF procedures supporting the RCD verification using assertion of RCD info functionality for signing the RCD info has to be completed |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change 1: Clause 5.4.3.2 specifies that if the Call-Info header field with RCD info is received within the initial INVITE or MESSAGE request and if an "rcd-np" parameter and a "verified" parameter are not included in the Call-Info header field, the S-CSCF shall include the RCD info in the signingRequest. Change 2: The conditional status c81 in table A.204 is updated to include S-CSCF for the Identity header field. Change 3: The conditional status c75 in table A.218A is updated to include S-CSCF for the Identity header field. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | S-CSCF procedures supporting the RCD verification using assertion of RCD info functionality for signing the RCD info will not be complete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.3.2, A.2.2.4.7, A.2.2.4.7A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** First Change ***

#### 5.4.3.2	Requests initiated by the served user

For all SIP transactions identified:

-	if priority is supported, as containing an authorised Resource-Priority header field or a temporarily authorised Resource-Priority header field, or, if such an option is supported, relating to a dialog which previously contained an authorised Resource-Priority header field;

the S-CSCF shall give priority over other transactions or dialogs. This allows special treatment of such transactions or dialogs. If priority is supported, the S-CSCF shall adjust the priority treatment of transactions or dialogs according to the most recently received authorized Resource-Priority header field or backwards indication value.

NOTE 1:	The special treatment can include filtering, higher priority processing, routeing, call gapping. The exact meaning of priority is not defined further in this document, but is left to national regulation and network configuration.

When the S-CSCF receives from the UE an initial request for a dialog, which contains a GRUU and an "ob" SIP URI parameter in the Contact header field, and multiple contact addresses have been registered for the specific GRUU, then for all subsequent in-dialog requests sent toward the UE's, the S-CSCF shall populate the Request-URI with the registered contact address from which the UE sent the initial request for the dialog.

NOTE 2:	When a given contact address is registered, the S-CSCF can use a dedicated value in its Service-Route header field entry to identify the given contact address. When the S-CSCF receives an initial request for a dialog, the S-CSCF can find out from which contact address the initial request was sent by looking at the preloaded Route header field (constructed from the Service-Route header field returned in the response for the REGISTER request) which contains the entry of the S-CSCF.

When performing SIP digest without TLS, when the S-CSCF receives from the served user an initial request for a dialog or a request for a standalone transaction, the S-CSCF may perform the steps in subclause 5.4.3.6 to challenge the request based on local policy.

NOTE 3:	If the user registration is associated with the state "tls-protected", then the execution of Proxy-Authorization as described in subclause 5.4.3.6 is still possible, although it is unlikely this would add additional security provided the P-CSCF is trusted. Thus, in most cases the state "tls-protected" will be reason for the S-CSCF to not desire Proxy-Authentication for this user.

NOTE 4:	The option for the S-CSCF to challenge the request does not apply to a request from an AS acting as an originating UA.

When performing GPRS-IMS-Bundled authentication, when the S-CSCF receives from the served user an initial request for a dialog or a request for a standalone transaction, the S-CSCF shall check whether a "received" header field parameter exists in the Via header field provided by the UE. If a "received" header field parameter exists, S-CSCF shall compare the (prefix of the) IP address received in the "received" header field parameter against the UE's IP address (or prefix) stored during registration. If no "received" header field parameter exists in the Via header field provided by the UE, then S-CSCF shall compare the (prefix of the) IP address received in the "sent-by" parameter against the IP address (or prefix) stored during registration. If the stored IP address (or prefix) and the (prefix of the) IP address in the "received" Via header field parameter provided by the UE do not match, the S-CSCF shall reject the request with a 403 (Forbidden) response. In case the stored IP address (or prefix) and the (prefix of the) IP address in the "received" Via header field parameter provided by the UE do match, the S-CSCF shall proceed as described in the remainder of this subclause.

If the S-CSCF supports HSS based P-CSCF restoration, and receives a request from a P-CSCF that the S-CSCF considers is not reachable, the S-CSCF shall consider this P-CSCF as being reachable.

If the S-CSCF supports PCRF based P-CSCF restoration, and receives a request from a P-CSCF that the S-CSCF considers is in a not reachable, the S-CSCF shall consider this P-CSCF as being reachable.

When the S-CSCF receives from the served user or from a PSI an initial request for a dialog or a request for a standalone transaction, and the request is received either from a functional entity within the same trust domain or contains a valid original dialog identifier (see step 3) or the dialog identifier (From, To and Call-ID header fields) relates to an existing request processed by the S-CSCF, then prior to forwarding the request, the S-CSCF shall:

0)	if the request is received from a P-CSCF that does not support the trust domain handling of the P-Served-User header field then remove any P-Served-User header fields;

1)	determine the served user as follows:

a)	if the request contains a P-Served-User header field then

i)	determine the served user by taking the identity contained in a P-Served-User header field as defined in RFC 5502 [133]. Then check whether the determined served user is a barred public user identity. In case the said header field contains the served user identity is a barred public user identity for the user, then the S-CSCF shall reject the request by generating a 403 (Forbidden) response. Otherwise, the S-CSCF shall save the public user identity of the served user and continue with the rest of the steps;

NOTE 5:	If the P-Served-User header field contains a barred public user identity, then the message has been received, either directly or indirectly, from a non-compliant entity which should have had generated the content with a non-barred public user identity.

b)	if the request does not contain a P-Served-User header field then

i)	determine the served user by taking the identity contained in one of the URI(s) of the P-Asserted-Identity header field. In case the determined served user is a barred public user identity, then the S-CSCF shall reject the request by generating a 403 (Forbidden) response. Otherwise, the S-CSCF shall save the public user identity of the served user and continue with the rest of the steps; and

ii)	if the P-Asserted-Identity header field contains two URIs and the URI other than the determined served user is not an alias of the determined served user or is barred then act based on local policy, e.g. reject the request by generating a 403 (Forbidden) response or remove the URI not identifying the determined served user from the P-Asserted-Identity header field;

NOTE 6:	If the P-Asserted-Identity header field contains a barred public user identity, then the message has been received, either directly or indirectly, from a non-compliant entity which should have had generated the content with a non-barred public user identity.

1A)	if the Contact is a GRUU, but is not valid as defined in subclause 5.4.7A.4, then return a 4xx response as specified in RFC 5627 [93];

2)	store the value of the "orig-ioi" header field parameter received in the P-Charging-Vector header field if present, and remove it from any forwarded request;

NOTE 7:	Any received "orig-ioi" header field parameter will be either a type 1 IOI or a type 3 IOI. The type 1 IOI identifies the network from which the request was sent and the type 3 IOI identifies the service provider from which the request was sent (AS initiating a session on behalf of a user or a PSI);

3)	check if an original dialog identifier that the S-CSCF previously placed in a Route header field is present in the topmost Route header field of the incoming request.

-	If not present, the S-CSCF shall build an ordered list of initial filter criteria based on the public user identity of the served user (as determined in step 1) of the received request as described in 3GPP TS 23.218 [5].

-	If present, the request has been sent from an AS in response to a previously sent request, an ordered list of initial filter criteria already exists and the S-CSCF shall not change the ordered list of initial filter criteria even if the AS has changed the P-Served-User header field or the P-Asserted-Identity header field;

NOTE 8:	An original dialog identifier is sent to each AS invoked due to iFC evaluation such that the S-CSCF can associate requests as part of the same sequence that trigger iFC evaluation in priority order (and not rely on SIP dialog information that can change due to B2BUA AS). If the same original dialog identifier is included in more than one request from a particular AS (based on service logic in the AS), then the S-CSCF will continue the iFC evaluation sequence rather than build a new ordered list of iFC;

4)	remove its own SIP URI from the topmost Route header field;

4A)	if a reference location was received from the HSS at registration as part of the user profile and the request does not contain a message body with the content type application/pidf+xml in accordance with RFC 6442 [89] and does not contain a P-Access-Network-Info header field containing the "network-provided" parameter, the S-CSCF shall insert a P-Access-Network-Info header field constructed according to the reference location received from the HSS and containing the "network-provided" parameter. The access type information received from the HSS shall be mapped into the corresponding access-type parameter of the P-Access-Network-Info header field and the location information shall be mapped into the location parameter corresponding to the access-type parameter, i.e. into "dsl-location" parameter, "fiber-location" parameter or "eth-location" parameter;

4B)	if there was an original dialog identifier present in the topmost Route header field of the incoming request and the request is received from a functional entity within the same trust domain and contains a P-Asserted-Service header field, continue the procedure with step 5;

4C) if the request contains a P-Preferred-Service header field, check whether the ICSI value contained in the P-Preferred-Service header field is part of the set of the subscribed services for the served user and determine, using operator-configured data, whether the contents of the request match the ICSI for the subscribed service. The operator-configured data used to determine if there is a matching between the request and the ICSI value may be based on any information in the request (e.g. SDP media capabilities, Content-Type header field, request method). Then:

a)	if there is no match between the request and the ICSI value, as an operator option, the S-CSCF may reject the request by generating a 403 (Forbidden) response. Otherwise remove the P-Preferred-Service header field and continue with the rest of the steps; and

b)	if there is a match between the request and the ICSI value, then include a P-Asserted-Service header field in the request containing the ICSI value contained in the P-Preferred-Service header field, remove the P-Preferred-Service header field, and continue the procedure with step 5;

4D)	if the request does not contain a P-Preferred-Service header field, check, using operator-configured data, whether the contents of the request match a subscribed service for each and any of the subscribed services for the served user:

a)	if not, as an operator option, the S-CSCF may reject the request by generating a 403 (Forbidden) response; and

b)	if so, and if the request is related to an IMS communication service and the IMS communication service requires the use of an ICSI value then select an ICSI value for the related IMS communication service and include a P-Asserted-Service header field in the request containing the selected ICSI value; and

NOTE 9:	If more than one ICSI values match the contents of the request, the S-CSCF selects an ICSI value based on local policy.

c)	if so, and if the request is related to an IMS communication service and the IMS communication service does not require the use of an ICSI value then continue without including an ICSI value; and

d)	if so, and if the request does not relate to an IMS communication service (or if the S-CSCF is unable to unambiguously determine the service being requested but decides to allow the session to continue) then continue without including an ICSI value;

5)	check whether the initial request matches any unexecuted initial filter criteria. If there is a match, then the S-CSCF shall select the first matching unexecuted initial filter criteria from the ordered list of initial filter criteria and the S-CSCF shall:

a)	insert the AS URI to be contacted into the Route header field as the topmost entry followed by its own URI populated as specified in the subclause 5.4.3.4;

NOTE 10:	If the AS is accessed via an ISC gateway function, then the URI will be the address of the ISC gateway function.

b)	if the S-CSCF supports the P-Served-User extension as specified in RFC 5502 [133] and RFC 8498 [239] insert P-Served-User header field populated with the served user identity as determined in step 1. If required by operator policy, the S-CSCF shall:

-	if the associated session case is "Originating" as specified in 3GPP TS 29.228 [14], include the sescase header field parameter set to "orig" and the regstate header field parameter set to "reg";

-	if the associated session case is "Originating_Unregistered" as specified in 3GPP TS 29.228 [14], include the sescase header field parameter set to "orig" and the regstate header field parameter set to "unreg";

-	if the associated session case is "Originating_CDIV" as specified in 3GPP TS 29.228 [14], include the "orig-cdiv" header field parameter, defined in RFC 8498 [239]; and

c)	if the AS is located outside the trust domain then the S-CSCF shall remove the access-network-charging-info parameter in the P-Charging-Vector header field from the request that is forwarded to the AS; if the AS is located within the trust domain, then the S-CSCF shall retain the access-network-charging-info parameter in the P-Charging-Vector header field in the request that is forwarded to the AS;

d)	insert a type 3 "orig-ioi" header field parameter in place of any received "orig-ioi" header field parameters in the P-Charging-Vector header field. The S-CSCF shall set the type 3 "orig-ioi" header field parameter to a value that identifies the sending network of the request. The S-CSCF shall not include the type 3 "term-ioi" header field parameter;

e)	remove the "transit-ioi" header field parameter, if received;

f)	based on operator policy insert in a Relayed-Charge header field the value of the received "transit-ioi" header field parameter in the P-Charging-Vector header field;

g)	based on local policy, the S-CSCF shall add an "fe-addr" element of the "fe-identifier" header field parameter to the P-Charging-Vector header field with its own address or identifier;

h)	if the S-CSCF supports using a token to identify the registration and if a registration exists, insert a "+g.3gpp.registration-token" Feature-Caps header field parameter, as defined in subclause 7.9A.8, set to the same value as included in the "+g.3gpp.registration-token" Contact header field parameter of the third party REGISTER request sent to the AS when the UE registered; and

i)	if an IP address associated with the served user and the AS SIP URI is stored as described in subclause 5.4.0 exists, then the S-CSCF forwards the SIP message to the IP address associated with the served user and the AS SIP URI;

NOTE 11:	Depending on the result of processing the filter criteria the S-CSCF might contact one or more AS(s) before processing the outgoing Request-URI.

NOTE 12:	An AS can activate or deactivate its own filter criteria via the Sh interface. As the S-CSCF checks initial filter criteria only on receipt of an initial request for a dialog, or a standalone transaction, a modified service profile will have no impact on transactions or dialogs already in progress and the modified profile will be effective only for new transactions and dialogs. If the S-CSCF receives a modification of the iFC during their execution, then it should not update the stored initial Filter Criteria until the iFC related to the initial request have been completely executed.

6)	if there was no original dialog identifier present in the topmost Route header field of the incoming request store the value of the "icid-value" header field parameter received in the P-Charging-Vector header field and retain the "icid-value" header field parameter in the P-Charging-Vector header field. Optionally, the S-CSCF may generate a new, globally unique ICID and insert the new value in the "icid-value" header field parameter of the P-Charging-Vector header field when forwarding the message. If the S-CSCF creates a new ICID, then it is responsible for maintaining the two ICID values in the subsequent messaging. Based on local policy, the S-CSCF shall add an "fe-addr" element of the "fe-identifier" header field parameter to the P-Charging-Vector header field with its own address or identifier if not already available;

7)	in step 5, if the initial request did not match any unexecuted initial filter criteria (i.e. the request is not forwarded to an AS):

a)	remove the received "transit-ioi" from the P-Charging-Vector header field, if present;

b)	insert a type 2 "orig-ioi" header field parameter into the P-Charging-Vector header field. The S-CSCF shall set the type 2 "orig-ioi" header field parameter to a value that identifies the sending network. The S-CSCF shall not include the type 2 "term-ioi" header field parameter; and

c)	remove the Relayed-Charge header field, if present;

8)	insert a P-Charging-Function-Addresses header field populated with values received from the HSS if the request does not contain a P-Charging-Function-Addresses header field and the message is forwarded within the S-CSCF home network, including towards AS;

9)	if there was no original dialog identifier present in the topmost Route header field of the incoming request and if the served user is not considered a privileged sender then:

a)	if the P-Asserted-Identity header field contains only a SIP URI and if the S-CSCF has knowledge that the SIP URI contained in the received P-Asserted-Identity header field is an alias SIP URI for a tel URI, add a second P-Asserted-Identity header field containing this tel-URI, including the display name associated with the tel URI, if available; and

b)	if the P-Asserted-Identity header field contains only a tel URI, the S-CSCF shall add a second P-Asserted-Identity header field containing a SIP URI. The added SIP URI shall contain in the user part a "+" followed by the international public telecommunication number contained in tel URI, and user's home network domain name in the hostport part. The added SIP URI shall contain the same value in the display name as contained in the tel URI. The S-CSCF shall also add a "user" SIP URI parameter equals "phone" to the SIP URI;

NOTE 13:	If tel URI is shared URI so is the alias SIP URI.

10)	if the request is not forwarded to an AS and if the outgoing Request-URI is:

-	a SIP URI with the user part starting with a + and the "user" SIP URI parameter equals "phone", and if configured per local operator policy, the S-CSCF shall perform the procedure described here. Local policy can dictate whether this procedure is performed for all domains of the SIP URI, only if the domain belongs to the home network, or not at all. If local policy indicates that the procedure is to be performed, then the S-CSCF shall translate the international public telecommunications number contained in the user part of the SIP URI (see RFC 3966 [22]) to a globally routeable SIP URI using either an ENUM/DNS translation mechanism with the format specified in RFC 6116 [24], or any other available database. Database aspects of ENUM are outside the scope of the present document. An S-CSCF that implements the additional routeing functionality described in annex I may forward the request without attempting translation. If an agreement exists between the home network and the visited network to support roaming architecture for voice over IMS with local breakout, the S-CSCF does not enable NP capabilities, and the S-CSCF decides to loopback the call to the visited network, the S-CSCF may forward the request without attempting translation. If a translation is in fact performed and it succeeds, the S-CSCF shall update the Request-URI with the globally routeable SIP URI either returned by ENUM/DNS or obtained from any other available database. If this translation fails, the request may be forwarded to a BGCF or any other appropriate entity (e.g. a MRFC to play an announcement) in the originator's home network or the S-CSCF may send an appropriate SIP response to the originator. When forwarding the request to a BGCF or any other appropriate entity, the S-CSCF shall leave the original Request-URI containing the SIP URI with "user" SIP URI parameter equals phone unmodified. If the request is forwarded, the S-CSCF shall remove the access-network-charging-info parameter from the P-Charging-Vector header field prior to forwarding the message;

-	a SIP URI with a "user" SIP URI parameter equals "dialstring" and the domain name of the SIP URI belongs to the home network (i.e. the local number analysis and handling is either failed in the appropriate AS or the request has not been forwarded to AS for local number analysis and handling at all), either forward the request to any appropriate entity (e.g a MRFC to play an announcement) in the originator's home network or send an appropriate SIP response to the originator;

-	a SIP URI with a local number (see RFC 3966 [22]) in the user part and a "user" SIP URI parameter equals "phone" and the domain name of the SIP URI belongs to the home network (i.e. the local number analysis and handling is either failed in the appropriate AS or the request has not been forwarded to AS for local number analysis and handling at all), either forward the request to to a BGCFor any appropriate entity (e.g a MRFC to play an announcement) in the originator's home network or send an appropriate SIP response to the originator;

-	a tel URI containing a global number (see RFC 3966 [22]) in the international format, the S-CSCF shall translate the E.164 address to a globally routeable SIP URI using either an ENUM/DNS translation mechanism with the format specified in RFC 6116 [24], or any other available database. Database aspects of ENUM are outside the scope of the present document. An S-CSCF that implements the additional routeing functionality described in annex I may forward the request without attempting translation. If an agreement exists between the home network and the visited network to support roaming architecture for voice over IMS with local breakout, the S-CSCF does not enable NP capabilities, and the S-CSCF decides to loopback the call to the visited network, the S-CSCF may forward the request without attempting translation. If this translation is in fact performed and it succeeds, the S-CSCF shall update the Request-URI with the globally routeable SIP URI returned by ENUM/DNS or any other available database. If this translation fails, the request may be forwarded to a BGCF or any other appropriate entity (e.g a MRFC to play an announcement) in the originator's home network or the S-CSCF may send an appropriate SIP response to the originator. When forwarding the request to a BGCF or any other appropriate entity, the S-CSCF shall leave the original Request-URI containing the tel URI unmodified. If the request is forwarded, the S-CSCF shall remove the access-network-charging-info parameter from the P-Charging-Vector header field prior to forwarding the message;

-	a tel URI containing a local number (see RFC 3966 [22]) (i.e. the local number analysis and handling is either failed in the appropriate AS or the request has not been forwarded to AS for local number analysis and handling at all), either forward the request to a BGCF or any other appropriate entity (e.g. a MRFC to play an announcement) in the originator's home network or send an appropriate SIP response to the originator;

-	a pres URI or an im URI, the S-CSCF shall forward the request as specified in RFC 3861 [63]. In this case, the S-CSCF shall not modify the received Request-URI: and

-	a service URN, e.g. a service URN with a top-level service type of "sos" as specified in RFC 5031 [69]. In this case the S-CSCF shall not modify the received Request-URI.

NOTE 14:	If there is no SIP-based transport found after applying the procedure specified in RFC 3861 [63], the S-CSCF can forward the request to a translating gateway.

Additional procedures apply if the S-CSCF supports NP capabilities and these capabilities are enabled by local policy, and the database used for translation from an international public telecommunications number to a SIP URI also provides NP data (for example, based on the PSTN Enumservice as defined by RFC 4769 [114] or other appropriate data bases). If the above translation from an international public telecommunications number to a SIP URI failed, but NP data was obtained from the database and there is no "npdi" parameter in the received request, then the S-CSCF shall, based on operator policy, replace the URI in the Request-URI with the obtained NP data, prior to forwarding the request to the BGCF or other appropriate entity. If the received request already contains a tel-URI "npdi" parameter, then the S-CSCF may update the URI with the obtained NP data. The URI is updated by the S-CSCF by adding NP parameters defined by RFC 4694 [112]. If the Request-URI is a tel-URI, then an "npdi" tel-URI parameter is added to indicate that NP data retrieval has been performed, and if the number is ported, an "rn" tel-URI parameter is added to identify the ported-to routeing number. If the Request-URI is in the form of a SIP URI user=phone, the "npdi" and "rn" tel-URI parameters are added as described above to the userinfo part of the SIP URI;

NOTE 15:	When the S-CSCF replaces the tel-URI in the Request-URI with the obtained NP data, all tel URI parameters in the received Request-URI will be replaced by the obtained NP data.

10A)	if the request is not forwarded to an AS and if local policy requires the application of additional routeing capabilities, as defined in annex I, the S-CSCF shall apply the additional routeing capabilities if they are locally available or forward the request to an entity that implements the additional routeing capabilities;

10B)	if an agreement exists between the home network and the visited network to support Roaming Architecture for Voice over IMS with Local Breakout then continue with the following steps. Otherwise continue with step 11. If:

-	the top most Route header contains an indication that this is the UE-originating case;

NOTE 16:	This indication can e.g. be in a URI parameter, a character string in the user part of the URI or can be a port number in the URI added by the S-CSCF during the registration in the Service-Route header field.

-	the UE is roaming (as identified by the P-Visited-Network-ID header field value in the original REGISTER request); and

-	the request is an INVITE request;

determine if loopback routeing is applicable for this request using local policy, and save this decision for subsequent processing along with the following information:

a)	any URI representing the TRF address preference received from the visited network; and

b)	the ICID received in the request.

In addition, the S-CSCF shall also include in the request a Feature-Caps header field with the "+g.3gpp.home-visited" header field parameter according to RFC 6809 [190] with the "+g.3gpp.home-visited" header field parameter set to the identifier of the visited network received in the P-Visited-Network-ID header field in the original registration request;

10C)	if the request is an INVITE request, then determine whether loopback is applied for this request. The information saved in step 10B, and the presence or absence of the Feature-Caps header field with the "+g.3gpp.home-visited" header field parameter in the received INVITE request are taken into account in making this decision:

a)	if loopback routeing is not to be performed for this request remove any "+g.3gpp.trf" header field parameter or "+g.3gpp.home-visited" header field parameter from the Feature-Caps header field of the outgoing request;

b)	if loopback routeing is applied for this request;

i)	remove all entries in the Route header field;

ii)	if a "+g.3gpp.trf" header field parameter with a parameter value containing a valid URI, is included in the Feature-Caps header field of the request, insert the URI in a Route header field;

iii)	if a "+g.3gpp.trf" header field parameter, with a parameter value containing a valid URI is not included in the Feature-Caps header field of the request, insert a locally configured TRF address, associated with the visited network for this call, in the Route header field;

iv)	remove any "+g.3gpp.home-visited" header field parameter from the Feature-Caps header field of the outgoing request;

v)	insert the "+g.3gpp.loopback" header field parameter as specified in subclause 7.9A.4 in the Feature-Caps header field of the request, in accordance with the RFC 6809 [190]. If providing the identifier of the home network is supported by the S-CSCF and the visited network, the S-CSCF may based on operator agreement insert the "+g.3gpp.loopback" header field parameter set to the identifier of the home network;

vi)	if included in the incoming request, remove the "+g.3gpp.trf" header field parameter from the Feature-Caps header field from the outgoing request;

vii)	remove a type 2 "orig-ioi" header field parameter that was added in step 7 from the P-Charging-Vector header field and insert a type 1 "orig-ioi" header field parameter into the P-Charging-Vector header field. The S-CSCF shall set the type 1 "orig-ioi" header field parameter to a value that identifies the network in which the S-CSCF resides. The S-CSCF shall not include the "term-ioi" header field parameter; and

viii)	if the S-CSCF supports indicating the traffic leg associated with a URI as specified in RFC 7549 [225] and if an "iotl" SIP URI parameter is not included in the TRF URI in the Route header field and if required by local policy, append an "iotl" URI parameter with a value set to "homeA-visitedA" to the URI in the Route header field; and

c)	if the final decision on loopback routeing is deferred to a subsequent entity in the home network, the BGCF, then the S-CSCF includes, if absent, in the request a Feature-Caps header field with the "+g.3gpp.home-visited" header field parameter, with the parameter value set to the identifier of the visited network received in the P-Visited-Network-ID header field in the original registration request. The S-CSCF is expected to know by means of network configuration that such a subsequent entity exists; and

NOTE 17:	The subsequent entity in the home network, the BGCF, will remove the "+g.3gpp.home-visited" header field parameter from the Feature-Caps header field when a final routeing decision is taken.

11)	determine the destination address (e.g. DNS access) using the URI placed in the topmost Route header field if present, otherwise based on the Request-URI. If the destination requires interconnect functionalities (e.g. the destination address is of an IP address type other than the IP address type used in the IM CN subsystem), the S-CSCF shall forward the request to the destination address via an IBCF in the same network;

12)	if network hiding is needed due to local policy, put the address of the IBCF to the topmost Route header field;

13)	in case of an initial request for a dialog:

a)	determine the need for GRUU processing. GRUU processing is required if:

-	an original dialog identifier that the S-CSCF previously placed in a Route header field is not present in the topmost Route header field of the incoming request (this means the request is not returning after having been sent to an AS), and

-	the contact address contains a GRUU that was either assigned by the S-CSCF that is valid as specified in subclause 5.4.7A.4 or a temporary GRUU self assigned by the UE based on the "temp-gruu-cookie" header parameter provided to the UE;

NOTE 18:	The procedures for determining that a URI is a temporary GRUU assigned by the UE are specified in subclause 7.1.2.3 of RFC 6140 [191].

b)	if GRUU processing is not required and the initial request originated from a served user, then determine the need to record-route for other reasons:

-	if the request is routed to an AS which is part of the trust domain, the S-CSCF shall decide, based on operator policy, whether to record-route or not. The decision is configured in the S-CSCF using any information in the received request that may otherwise be used for the initial filter criteria. If the request is record-routed the S-CSCF shall create a Record-Route header field containing its own SIP URI;

-	if the request is a SUBSCRIBE request and routed elsewhere, the S-CSCF shall decide, based on operator policy, whether to record-route or not. The decision is configured in the S-CSCF using any information in the received request (e.g. event package name). If the request is record-routed the S-CSCF shall create a Record-Route header field containing its own SIP URI; or

NOTE 19: Some subscriptions to event packages (e.g. presence) can result in virtually persistent subscriptions and if the S-CSCF Record-Routes this can prevent reassignment of the S-CSCF.

NOTE 20: If the S-CSCF does not Record-Route the initial SUBSCRIBE request, it will not be possible to perform SIP digest authentication of SIP requests sent inside the SIP dialog related to the associated subscription.

-	if the request not a SUBSCRIBE request and is routed elsewhere, create a Record-Route header field containing its own SIP URI;

NOTE 21:	For requests originated from a PSI the S-CSCF can decide whether to record-route or not based on operator policy.

c)	if GRUU processing is required, the S-CSCF shall create a Record-Route header field containing its own SIP URI;

d)	if GRUU processing is required, the S-CSCF shall save an indication that GRUU-routeing is to be performed for in-dialog requests that reach the S-CSCF because of the Record-route header field added in step c);

NOTE 22:	The manner of representing the GRUU-routeing indication is a private matter for the S-CSCF. The indication is used during termination processing of in-dialog requests to cause the S-CSCF to replace a Request-URI containing a GRUU with the corresponding registered contact address. It can be saved using values in the Record-Route header field, or in dialog state.

14)	based on the destination user (Request-URI), remove any P-Access-Network-Info header field and the access-network-charging-info parameter in the P-Charging-Vector header field prior to forwarding the message;

14A)	if the request is not routed to an AS, to a BGCF or to an entity that implements the additional routeing functionality, remove the P-Served-User header field prior to forwarding the request;

14B)	if the S-CSCF supports indicating the traffic leg as specified in RFC 7549 [225], the request is not routed to an AS, to a BGCF or to an entity that implements the additional routeing functionality, loopback routeing is not to be performed for this request, required by local policy and the Request-URI contains a SIP URI, append the "iotl" SIP URI parameter set to "homeA-homeB" to the Request-URI;

15)	route the request based on SIP routeing procedures;

16)	if the request is an INVITE request, save the Contact, CSeq and Record-Route header field values received in the request such that the S-CSCF is able to release the session if needed;

17)	if the request contains a "logme"parameter in the Session-ID header field, treat this dialog as one for which logging is in progress and log SIP signalling for this dialog according to its trace configuration;

18)	if the S-CSCF supports using a token to identify the registration and if the request is not forwarded to an AS, remove the "+g.3gpp.registration-token" Feature-Caps header field parameter, defined in subclause 7.9A.8, if received in the request; and

19)	if the received request is an INVITE request or a MESSAGE request and the S-CSCF supports calling number verification using signature verification and attestation information as specified in subclause 3.1, the S-CSCF shall based on local policy perform attestation of the user identity by inserting:

-	a "verstat" tel URI parameter, specified in subclause 7.2A.20, to the tel URI or SIP URI with a user=phone parameter in the From header field or the P-Asserted-Identity header field;

an Origination-Id header field, specified in subclause 7.2.19, set to a UUID identifying the S-CSCF which is configured based on local policy and requirements from national regulation: and

-	an Attestation-Info header field, specified in subclause 7.2.18, set to the value "A".

If the received request is an INVITE request or MESSAGE request containing the Call-Info header field with RCD info and the RCD verification using assertion of RCD info functionality is implemented as standalone feature, based on local policy and if:

1)	an "rcd-np" parameter is not included;

2)	a "verified" parameter is not included; and

3)	no Identity header field exists;

the S-CSCF sends a signingRequest to the AS for signing over the Ms reference point, as specified in annex V. Upon receiving the HTTP 200 (OK) response to the signing request, the S-CSCF shall include an Identity header field with the "identity" claim in the forwarded initial INVITE request or MESSAGE request.

When the S-CSCF receives, an initial request for a dialog or a request for a standalone transaction, from an AS acting on behalf of an unregistered user, the S-CSCF shall:

1)	execute the procedures described in the steps 1, 2, 3, 4, 4B, 4C, 4D, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15 and 16 in the above paragraph (when the S-CSCF receives, from a registered served user, an initial request for a dialog or a request for a standalone transaction).

NOTE 23:	When the S-CSCF does not have the user profile, before executing the actions as listed above, it initiates the S-CSCF Registration/deregistration notification procedure, as described in 3GPP TS 29.228 [14]; with the purpose of downloading the relevant user profile (i.e. for unregistered user) and informs the HSS that the user is unregistered. The S-CSCF will assess triggering of services for the unregistered user, as described in 3GPP TS 29.228 [14]. When requesting the user profile, and the request received by the S-CSCF contains a P-Profile-Key header field, the S-CSCF can include the header field value in S-CSCF Registration/deregistration notification. If the response from the HSS includes a Wildcarded Public Identity AVP, and if the request received by the S-CSCF did not include a P-Profile-Key header field, the S-CSCF uses the AVP value to set the P-Profile-Key header field before forwarding the request to an AS.

When the S-CSCF receives a request initiated by the served user for which the S-CSCF does not have the user profile or does not trust the data that it has (e.g. due to restart), the S-CSCF shall attempt to retrieve the user profile from the HSS. If the S-CSCF receives a Diameter result code of DIAMETER_UNABLE_TO_COMPLY as defined in 3GPP TS 29.228 [14], the S-CSCF supports S-CSCF restoration procedures, and the Request-URI of the request does not match an emergency service URN, i.e. a service URN with a top-level service type of "sos" as specified in RFC 5031 [69], then the S-CSCF shall:

I)	reject the request by returning a 504 (Server Time-out) response to the UE;

II)	assume that the UE supports version 1 of the XML Schema for the 3GPP IM CN subsystem XML body if support for the 3GPP IM CN subsystem XML body as described in subclause 7.6 in the Accept header field is not indicated; and

III)	include in the 504 (Server Time-out) response:

-	a Content-Type header field with the value set to associated MIME type of the 3GPP IM CN subsystem XML body as described in subclause 7.6.1;

-	a P-Asserted-Identity header field set to the value of the SIP URI of the S-CSCF included in the Service-Route header field (see subclause 5.4.1.2.2F) during the registration of the user whose UE sent the request causing this response; and

-	a 3GPP IM CN subsystem XML body:

a)	an <ims-3gpp> element with the "version" attribute set to "1" and with an <alternative-service> child element, set to the parameters of the alternative service;

i)	a <type> child element, set to "restoration" (see table 7.6.2) to indicate that S-CSCF restoration procedures are supported;

ii)	a <reason> child element, set to an operator configurable reason; and

iii)	an <action> child element, set to "initial-registration" (see table 7.6.3).

NOTE 24:	These procedures do not prevent the usage of unspecified reliability or recovery techniques above and beyond those specified in this subclause.

Depending on operator configuration (see subclause 5.4.1.8), when the S-CSCF receives a request with a Request-URI that does not match an emergency service URN, i.e. a service URN with a top-level service type of "sos" as specified in RFC 5031 [69], the request initiated by the served user for which the S-CSCF has modified but not synchronized the service profile for the served user and the S-CSCF supports S-CSCF restoration procedures, then the S-CSCF shall reject the request as described in items I), II) and III).

If the S-CSCF:

a)	fails to receive a SIP response within a configurable time; or

b)	receives a 408 (Request Timeout) response or a 5xx response from the AS without previously receiving a 1xx response to the original SIP request, and without previously receiving a SIP request from the AS that contained the same original dialog identifier as the original request;

the S-CSCF shall:

-	if the default handling defined in the filter criteria indicates the value "SESSION_CONTINUED" as specified in 3GPP TS 29.228 [14] or no default handling is indicated, execute the procedure from step 5; and

-	if the default handling defined in the filter criteria indicates the value "SESSION_TERMINATED" as specified in 3GPP TS 29.228 [14], either forward the received response or, if the request is an initial INVITE request, send a 408 (Request Timeout) response or a 5xx response towards the served UE as appropriate (without verifying the matching of filter criteria of lower priority and without proceeding for further steps).

If the S-CSCF receives any final response from the AS, the S-CSCF shall forward the response towards the served UE (without verifying the matching of filter criteria of lower priority and without proceeding for further steps).

When the S-CSCF receives any response to the above request containing a Relayed-Charge header field, and the next hop is not an AS, the S-CSCF shall remove the Relayed-Charge header field from the forwarded response.

When the S-CSCF receives any response to the above request, the S-CSCF may:

1)	apply any privacy required by RFC 3323 [33] and RFC 3325 [34] to the P-Asserted-Identity header field.

NOTE 25:	The P-Asserted-Identity header field would normally only be expected in 1xx or 2xx responses.

NOTE 26:	The optional procedure above is in addition to any procedure for the application of privacy at the edge of the trust domain specified by RFC 3325 [34].

When the S-CSCF receives any response to the above request, the S-CSCF shall:

1)	If logging is in progress for this dialog, check whether a trigger for stopping logging of SIP signalling has occurred, as described in RFC 8497 [140] and configured in the trace management object defined in 3GPP TS 24.323 [8K]. If a stop trigger event has occurred then stop treating this as a dialog for which logging is in progress, else the S-CSCF shall append a "logme" header field parameter to the SIP Session-ID header field if the parameter is missing and determine, by checking its trace configuration, whether to log the response.

When the S-CSCF receives any response to the above request containing a "term-ioi" header field parameter in the P-Charging-Vector header field, the S-CSCF shall:

1)	store the value of the received "term-ioi" header field parameter if present;

2)	remove all received "orig-ioi", "term-ioi" and "transit-ioi" header field parameters from the forwarded response;

3)	include the stored "orig-ioi" header field parameter if received in the request;

4)	include a type 1 "term-ioi" header field parameter if next hop is not an AS, or a type 3 "term-ioi" header field parameter. The "term-ioi" header field parameter is set to a value that identifies the sending network of the response

NOTE 27:	Any received "term-ioi" header field parameter will be a type 2 IOI, if received from an S-CSCF, or type 3 IOI, if received from an AS, or type 1 IOI if the S-CSCF performed loopback routeing for this request. A type 2 IOI identifies the sending network of the response, a type 3 IOI identifies the sending service provider of the response, and a type 1 IOI identifies the visited network of the served user.

5)	based on operator policy include any received "transit-ioi" header field parameter, from the P-Charging-Vector header field, in a Relayed-Charge header field, if the next hop is an AS.

When the S-CSCF receives any 1xx or 2xx response to the initial request for a dialog, if the response corresponds to an INVITE request, the S-CSCF shall save the Contact and Record-Route header field values in the response in order to be able to release the session if needed.

When the S-CSCF receives any 1xx or 2xx response to an initial request for a dialog or a request for a standalone transaction, if the response is forwarded within the S-CSCF home network and not to an AS, the S-CSCF shall insert a P-Charging-Function-Addresses header field populated with values received from the HSS.

When the S-CSCF, upon sending an initial INVITE request that includes an IP address in the SDP offer (in "c=" parameter), receives an error response indicating that the IP address type is not supported, (e.g., the S-CSCF receives the 488 (Not Acceptable Here) with 301 Warning header field indicating "incompatible network address format"), the S-CSCF shall either:

-	fork the initial INVITE request to the IBCF; or

-	process the error response and forward it using the Via header field.

NOTE 28:	If the S-CSCF knows that the originating UE supports both IPv6 and IPv4 addresses simultaneously, the S-CSCF will forward the error response to the UE using the Via header field. The present version of the specification does not specify how the S-CSCF determines whether the UE supports both IPv6 and IPv4 addressing simultaneously.

When the S-CSCF receives from the served user a target refresh request for a dialog, prior to forwarding the request the S-CSCF shall:

0A)	if the dialog is related to an IMS communication service determine whether the contents of the request (e.g. SDP media capabilities, Content-Type header field) match the IMS communication service as received as the ICSI value in the P-Asserted-Service header field in the initial request. As an operator option, if the contents of the request do not match the IMS communication service the S-CSCF may reject the request by generating a status code reflecting which added contents are not matching. Otherwise, continue with the rest of the steps;

1)	remove its own URI from the topmost Route header field;

2)	create a Record-Route header field containing its own SIP URI;

3)	for INVITE dialogs (i.e. dialogs initiated by an INVITE request), save the Contact and CSeq header field values received in the request such that the S-CSCF is able to release the session if needed;

4)	in case the request is routed towards the destination user (Request-URI) or in case the request is routed to an AS located outside the trust domain, remove the access-network-charging-info parameter in the P-Charging-Vector header field;

5)	route the request based on the topmost Route header field; and

6)	if the request was sent on a dialog for which logging of signalling is in progress, check whether a trigger for stopping logging of SIP signalling has occurred, as described in RFC 8497 [140] and configured in the trace management object defined in 3GPP TS 24.323 [8K]. If a stop trigger event has occurred then stop logging of signalling, else determine, by checking its trace configuration, whether to log the response.

When the S-CSCF receives any response to the above request, the S-CSCF shall:

1)	If logging is in progress for this dialog, check whether a trigger for stopping logging of SIP signalling has occurred, as described in RFC 8497 [140] and configured in the trace management object defined in 3GPP TS 24.323 [8K]. If a stop trigger event has occurred then stop logging of signalling, else determine, by checking its trace configuration, whether to log the response.

When the S-CSCF receives any 1xx or 2xx response to the target refresh request for an INVITE dialog, the S-CSCF shall replace the saved Contact header field values in the response such that the S-CSCF is able to release the session if needed.

If the S-CSCF inserted in the initial request for the dialog the header field parameters into the Feature-Caps header field then the S-CSCF shall include the header field parameters with the same parameter values into the Feature-Caps header field in any target refresh request for the dialog, and in each 1xx or 2xx response to target refresh request sent in the same direction.

When the S-CSCF receives from the served user a subsequent request other than a target refresh request for a dialog, prior to forwarding the request the S-CSCF shall:

1)	remove its own URI from the topmost Route header field;

2)	in case the request is routed towards the destination user (Request-URI) or in case the request is routed to an AS located outside the trust domain, remove the access-network-charging-info parameter in the P-Charging-Vector header field; and

3)	route the request based on the topmost Route header field; and

4)	if the request was sent on a dialog for which logging of signalling is in progress, check whether a trigger for stopping logging of SIP signalling has occurred, as described in RFC 8497 [140] and configured in the trace management object defined in 3GPP TS 24.323 [8K]. If a stop trigger event has occurred, stop logging of signalling, else determine, by checking its trace configuration, whether to log the request.

When the S-CSCF receives any response to the above request, the S-CSCF shall:

1)	If logging is in progress for this dialog, check whether a trigger for stopping logging of SIP signalling has occurred, as described in RFC 8497 [140] and configured in the trace management object defined in 3GPP TS 24.323 [8K]. If a stop trigger event has occurred then stop logging of signalling, else determine, by checking its trace configuration, whether to log the response.

With the exception of 305 (Use Proxy) responses, the S-CSCF shall not recurse on 3xx responses.

# *** Next Change ***

#### A.2.2.4.7	INVITE method

Prerequisite A.163/8 - - INVITE request

Table A.204: Supported header fields within the INVITE request

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 1 | Accept | [26] 20.1 | m | m | [26] 20.1 | i | i |
| 1A | Accept-Contact | [56B] 9.2 | c34 | c34 | [56B] 9.2 | c34 | c35 |
| 2 | Accept-Encoding | [26] 20.2 | m | m | [26] 20.2 | i | i |
| 3 | Accept-Language | [26] 20.3 | m | m | [26] 20.3 | i | i |
| 3A | Additional-Identity | 7.2.20 | n/a | c85 | 7.2.20 | n/a | c85 |
| 4 | Alert-Info | [26] 20.4 | c2 | c2 | [26] 20.4 | c3 | c3 |
| 5 | Allow | [26] 20.5 | m | m | [26] 20.5 | i | i |
| 6 | Allow-Events | [28] 8.2.2 | m | m | [28] 8.2.2 | c1 | c1 |
| 7 | Answer-Mode | [158] | c67 | c67 | [158] | c68 | c68 |
| 7A | Attestation-Info | 7.2.18 | n/a | c82 | 7.2.18 | n/a | c82 |
| 8 | Authorization | [26] 20.7 | m | m | [26] 20.7 | i | i |
| 9 | Call-ID | [26] 20.8 | m | m | [26] 20.8 | m | m |
| 10 | Call-Info | [26] 20.9, [303] | m | m | [26] 20.9 | c12 | c12 |
| 10A | Cellular-Network-Info | 7.2.15 | n/a | c78 | 7.2.15 | n/a | c79 |
| 11 | Contact | [26] 20.10 | m | m | [26] 20.10 | i | i |
| 12 | Content-Disposition | [26] 20.11 | m | m | [26] 20.11 | i | c6 |
| 13 | Content-Encoding | [26] 20.12 | m | m | [26] 20.12 | i | c6 |
| 14 | Content-Language | [26] 20.13 | m | m | [26] 20.13 | i | c6 |
| 15 | Content-Length | [26] 20.14 | m | m | [26] 20.14 | m | m |
| 16 | Content-Type | [26] 20.15 | m | m | [26] 20.15 | i | c6 |
| 17 | CSeq | [26] 20.16 | m | m | [26] 20.16 | m | m |
| 18 | Date | [26] 20.17 | m | m | [26] 20.17 | c4 | c4 |
| 18A |  | DC-Info | Subclause 7.2.x | n/a | cbb | Subclause 7.2.x | n/a | cbb |
| 19 | Expires | [26] 20.19 | m | m | [26] 20.19 | i | i |
| 19A | Feature-Caps | [190] | c73 | c73 | [190] | c73 | c73 |
| 20 | From | [26] 20.20 | m | m | [26] 20.20 | m | m |
| 20A | Geolocation | [89] 4.1 | c47 | c47 | [89] 4.1 | c48 | c48 |
| 20B | Geolocation-Routing | [89] 4.1 | c47 | c47 | [89] 4.1 | c48 | c48 |
| 20C | History-Info | [66] 4.1 | c43 | c43 | [66] 4.1 | c43 | c43 |
| 20D | Identity | [252] 4 | c81 | c81 | [252] 4 | c81 | c81 |
| 21 | In-Reply-To | [26] 20.21 | m | m | [26] 20.21 | i | i |
| 21A | Join | [61] 7.1 | c41 | c41 | [61] 7.1 | c42 | c42 |
| 21B | Max-Breadth | [117] 5.8 | c63 | c63 | [117] 5.8 | c64 | c64 |
| 22 | Max-Forwards | [26] 20.22 | m | m | [26] 20.22 | m | m |
| 23 | MIME-Version | [26] 20.24 | m | m | [26] 20.24 | i | c6 |
| 23A | Min-SE | [58] 5 | o | o | [58] 5 | o | o |
| 24 | Organization | [26] 20.25 | m | m | [26] 20.25 | c5 | c5 |
| 24AA | Origination-Id | 7.2.19 | n/a | c83 | 7.2.19 | n/a | c83 |
| 24A | P-Access-Network-Info | [52] 4.4, [234] 2 | c28 | c28 | [52] 4.4, [234] 2 | c29 | c30 |
| 24B | P-Asserted-Identity | [34] 9.1 | c15 | c15 | [34] 9.1 | c16 | c16 |
| 24C | P-Asserted-Service | [121] 4.1 | c53 | c53 | [121] 4.1 | c54 | c54 |
| 24D | P-Called-Party-ID | [52] 4.2 | c19 | c19 | [52] 4.2 | c20 | c21 |
| 24E | P-Charging-Function-Addresses | [52] 4.5 | c26 | c27 | [52] 4.5 | c26 | c27 |
| 24F | P-Charging-Vector | [52] 4.6 | c24 | c24 | [52] 4.6 | c25 | c25 |
| 24H | P-Early-Media | [109] 8 | o | c50 | [109] 8 | o | c50 |
| 25 | P-Media-Authorization | [31] 5.1 | c9 | x | [31] 5.1 | n/a | n/a |
| 25A | P-Preferred-Identity | [34] 9.2 | x | c69 | [34] 9.2 | c14 | c14 |
| 25B | P-Preferred-Service | [121] 4.2 | x | x | [121] 4.2 | c52 | c52 |
| 25C | P-Private-Network-Indication | [134] | c59 | c59 | [134] | c59 | c59 |
| 25D | P-Profile-Key | [97] 5 | c45 | c45 | [97] 5 | c46 | c46 |
| 25E | P-Served-User | [133] 6 | c60 | c60 | [133] 6 | c60 | c60 |
| 25F | P-User-Database | [82] 4 | c44 | c44 | [82] 4 | c44 | c44 |
| 25G | P-Visited-Network-ID | [52] 4.3 | c22 | o | [52] 4.3 | c23 | o |
| 26 | Priority | [26] 20.26 | m | m | [26] 20.26 | i | c74 |
| 26AA | Priority-Share | Subclause 7.2.16 | n/a | c80 | Subclause 7.2.16 | n/a | c80 |
| 26A | Privacy | [33] 4.2 | c17 | c17 | [33] 4.2 | c18 | c18 |
| 26AB | Priority-Verstat | Subclause 7.2.21 | n/a | c86 | Subclause 7.2.21 | n/a | c86 |
| 26B | Priv-Answer-Mode | [158] | c67 | c67 | [158] | c68 | c68 |
| 27 | Proxy-Authorization | [26] 20.28 | m | m | [26] 20.28 | c13 | c13 |
| 28 | Proxy-Require | [26] 20.29, [34] 4 | m | m | [26] 20.29, [34] 4 | m | m |
| 28A | Reason | [34A] 2 | c32 | c32 | [34A] 2 | c33 | c33 |
| 29 | Record-Route | [26] 20.30 | m | m | [26] 20.30 | c11 | c11 |
| 29A | Recv-Info | [25] 5.2.3 | c65 | c65 | [25] 5.2.3 | c66 | c66 |
| 30 | Referred-By | [59] 3 | c37 | c37 | [59] 3 | c38 | c38 |
| 31 | Reject-Contact | [56B] 9.2 | c34 | c34 | [56B] 9.2 | c34 | c35 |
| 31A | Relayed-Charge | 7.2.12 | n/a | c76 | 7.2.12 | n/a | c76 |
| 31B | Replaces | [60] 6.1 | c39 | c39 | [60] 6.1 | c40 | c40 |
| 31C | Reply-To | [26] 20.31 | m | m | [26] 20.31 | i | i |
| 31D | Request-Disposition | [56B] 9.1 | c34 | c34 | [56B] 9.1 | c34 | c34 |
| 32 | Require | [26] 20.32 | m | m | [26] 20.32 | c7 | c7 |
| 32A | Resource-Priority | [116] 3.1 | c49 | c49 | [116] 3.1 | c49 | c49 |
| 32B | Restoration-Info | Subclause 7.2.11 | n/a | c75 | Subclause 7.2.11 | n/a | c75 |
| 32C | Resource-Share | Subclause 4.15 | n/a | c77 | Subclause 4.15 | n/a | c77 |
| 33 | Route | [26] 20.34 | m | m | [26] 20.34 | m | m |
| 33A | Security-Client | [48] 2.3.1 | x | x | [48] 2.3.1 | c31 | c31 |
| 33B | Security-Verify | [48] 2.3.1 | x | x | [48] 2.3.1 | c31 | c31 |
| 33DA | Service-Interact-Info | Subclause 7.2.14 | n/a | c84 | Subclause 7.2.14 | n/a | c84 |
| 33D | Session-Expires | [58] 4 | c36 | c36 | [58] 4 | c36 | c36 |
| 33E | Session-ID | [162] | c70 | c70 | [162] | c70 | c70 |
| 34 | Subject | [26] 20.36 | m | m | [26] 20.36 | i | i |
| 35 | Supported | [26] 20.37 | m | m | [26] 20.37 | c8 | c8 |
| 35A | Target-Dialog | [184] 7 | c71 | c71 | [184] 7 | c72 | c72 |
| 36 | Timestamp | [26] 20.38 | m | m | [26] 20.38 | i | i |
| 37 | To | [26] 20.39 | m | m | [26] 20.39 | m | m |
| 37A | Trigger-Consent | [125] 5.11.2 | c55 | c55 | [125] 5.11.2 | c56 | c56 |
| 38 | User-Agent | [26] 20.41 | m | m | [26] 20.41 | i | i |
| 38A | User-to-User | [126] 7 | c57 | c57 | [126] 7 | c58 | c58 |
| 39 | Via | [26] 20.42 | m | m | [26] 20.42 | m | m |
| c1: IF A.4/20 THEN m ELSE i - - SIP specific event notification extension. c2: IF A.162/10 THEN n/a ELSE m - - suppression or modification of alerting information data. c3: IF A.162/10 THEN m ELSE i - - suppression or modification of alerting information data. c4: IF A.162/9 THEN m ELSE i - - insertion of date in requests and responses. c5: IF A.162/19A OR A.162/19B THEN m ELSE i - - reading, adding or concatenating the Organization header. c6: IF A.3/2 OR A.3/4 THEN m ELSE i - - P-CSCF or S-CSCF. c7: IF A.162/11 OR A.162/13 THEN m ELSE i - - reading the contents of the Require header before proxying the request or response or adding or modifying the contents of the Require header before proxying the request or response for methods other than REGISTER. c8: IF A.162/16 THEN m ELSE i - - reading the contents of the Supported header before proxying the response. c9: IF A.162/26 THEN m ELSE n/a - - SIP extensions for media authorization. c11: IF A.162/14 THEN m ELSE i - - the requirement to be able to insert itself in the subsequent transactions in a dialog. c12: IF A.162/19C OR A.162/19D THEN m ELSE IF A.162/n134 THEN o ELSE i - - reading, adding or concatenating the Call-Info header, SIP Call-Info Parameters for Rich Call Data. c13: IF A.162/8A THEN m ELSE i - - authentication between UA and proxy. c14: IF A.162/30A OR A.162/30C THEN m ELSE n/a - - act as first entity within the trust domain for asserted identity, act as entity passing on identity transparently independent of trust domain. c15: IF A.162/30 THEN m ELSE n/a - - extensions to the Session Initiation Protocol (SIP) for asserted identity within trusted networks. c16: IF A.162/30A or A.162/30B THEN m ELSE i - - extensions to the Session Initiation Protocol (SIP) for asserted identity within trusted networks or subsequent entity within trust network that can route outside the trust network. c17: IF A.162/31 THEN m ELSE n/a - - a privacy mechanism for the Session Initiation Protocol (SIP). c18: IF A.162/31D OR A.162/31G THEN m ELSE IF A.162/31C THEN i ELSE n/a - - application of the privacy option "header" or application of the privacy option "id" or passing on of the Privacy header transparently. c19: IF A.162/37 THEN m ELSE n/a - - the P-Called-Party-ID header extension. c20: IF A.162/37 THEN i ELSE n/a - - the P-Called-Party-ID header extension. c21: IF A.162/37 AND A.3/2 THEN m ELSE IF A.162/37 AND (A.3/3 OR A.3/9A) THEN i ELSE n/a - - the P-Called-Party-ID header extension and P-CSCF or (I-CSCF or IBCF (THIG)). c22: IF A.162/38 THEN m ELSE n/a - - the P-Visited-Network-ID header extension. c23: IF A.162/39 THEN m ELSE i - - reading, or deleting the P-Visited-Network-ID header before proxying the request or response. c24: IF A.162/45 THEN m ELSE n/a - - the P-Charging-Vector header extension. c25: IF A.162/46 THEN m ELSE IF A.162/45 THEN i ELSE n/a - - adding, deleting, reading or modifying the P-Charging-Vector header before proxying the request or response or the P-Charging-Vector header extension. c26: IF A.162/44 THEN m ELSE n/a - - the P-Charging-Function-Addresses header extension. c27: IF A.162/44A THEN m ELSE IF A.162/44 THEN i ELSE n/a - - adding, deleting or reading the P-Charging-Function-Addresses header before proxying the request or response, or the P-Charging-Function-Addresses header extension. c28: IF A.162/43 THEN x ELSE IF A.162/41 THEN m ELSE n/a - - act as subsequent entity within trust network for access network information that can route outside the trust network, the P-Access-Network-Info header extension. c29: IF A.162/43 THEN m ELSE IF A.162/41 THEN i ELSE n/a - - act as subsequent entity within trust network for access network information that can route outside the trust network, the P-Access-Network-Info header extension. c30: IF A.162/43 OR (A.162/41 AND A.3/2) THEN m ELSE IF A.162/41 THEN i ELSE n/a - - act as subsequent entity within trust network for access network information that can route outside the trust network, the P-Access-Network-Info header extension (with or without P-CSCF). c31: IF A.162/47 OR A.162/47A THEN m ELSE n/a - - security mechanism agreement for the session initiation protocol or mediasec header field parameter for marking security mechanisms related to media. c32: IF A.162/48 THEN m ELSE n/a - - the Reason header field for the session initiation protocol. c33: IF A.162/48 THEN i ELSE n/a - - the Reason header field for the session initiation protocol. c34: IF A.162/50 THEN m ELSE n/a - - caller preferences for the session initiation protocol. c35: IF A.162/50 AND A.4/3 THEN m ELSE IF A.162/50 AND NOT A.4/3 THEN i ELSE n/a - - caller preferences for the session initiation protocol, and S-CSCF. c36: IF A.162/52 THEN m ELSE n/a - - the SIP session timer. c37: IF A.162/53 THEN i ELSE n/a - - the SIP Referred-By mechanism. c38: IF A.162/53 THEN m ELSE n/a - - the SIP Referred-By mechanism. c39: IF A.162/54 THEN m ELSE n/a - - the Session Inititation Protocol (SIP) "Replaces" header. c40: IF A.162/54 THEN i ELSE n/a - - the Session Inititation Protocol (SIP) "Replaces" header. |  |  |  |  |  |  |  |
| c41: IF A.162/55 THEN m ELSE n/a - - the Session Inititation Protocol (SIP) "Join" header. c42: IF A.162/55 THEN i ELSE n/a - - the Session Inititation Protocol (SIP) "Join" header. c43: IF A.162/57 THEN m ELSE n/a - - an extension to the session initiation protocol for request history information. c44: IF A.162/60 THEN m ELSE n/a - - the P-User-Database private header extension. c45: IF A.162/66A THEN m ELSE n/a - - making the first query to the database in order to populate the P-Profile-Key header. c46: IF A.162/66B THEN m ELSE n/a - - using the information in the P-Profile-Key header. c47: IF A.162/70 THEN m ELSE n/a - - SIP location conveyance. c48: IF A.162/70A THEN m ELSE IF A.162/70B THEN i ELSE n/a - - addition or modification of location in a SIP method, passes on locations in SIP method without modification. c49: IF A.162/80 THEN m ELSE n/a - - communications resource priority for the session initiation protocol. c50: IF A.162/76 THEN m ELSE n/a - - the SIP P-Early-Media private header extension for authorization of early media. c52: IF A.162/84A THEN m ELSE n/a - - act as authentication entity within the trust domain for asserted service. c53: IF A.162/84 THEN m ELSE n/a - - SIP extension for the identification of services. c54: IF A.162/84 OR A.162/30B THEN m ELSE i - - SIP extension for the identification of services or subsequent entity within trust network that can route outside the trust network. c55: IF A.162/85 THEN m ELSE n/a - - a framework for consent-based communications in SIP. c56: IF A.162/85 THEN i ELSE n/a - - a framework for consent-based communications in SIP. c57: IF A.162/86 THEN m - - transporting user to user information for call centers using SIP. c58: IF A.162/86 THEN i - - transporting user to user information for call centers using SIP. c59: IF A.162/87 THEN m ELSE n/a - - the SIP P-Private-Network-Indication private-header (P-Header). c60: IF A.162/88 THEN m - - the SIP P-Served-User private header. c63: IF A.162/81 THEN m ELSE n/a - - addressing an amplification vulnerability in session initiation protocol forking proxies. c64: IF A.162/81 AND A.162/6 THEN m ELSE IF A.162/81 AND NOT A.162/6 THEN i ELSE n/a - - addressing an amplification vulnerability in session initiation protocol forking proxies, forking of initial requests. c65: IF A.162/20 THEN m ELSE n/a - - SIP INFO method and package framework. c66: IF A.162/20 THEN i ELSE n/a - - SIP INFO method and package framework. c67: IF A.162/97 THEN m ELSE n/a - - requesting answering modes for SIP. c68: IF NOT A.162/97 THEN n/a ELSE IF A.162/97A THEN m ELSE i - - requesting answering modes for SIP, adding, deleting or reading the Answer-Mode header or Priv-Answer-Mode header before proxying the request or response. c69: IF A.162/30C THEN m ELSE x - - act as entity passing on identity transparently independent of trust domain. c70: IF A.162/101 THEN m ELSE n/a - - the Session-ID header. c71: IF A.162/109 THEN m ELSE n/a - - request authorization through dialog Identification in the session initiation protocol. c72: IF A.162/109 THEN i ELSE n/a - - request authorization through dialog Identification in the session initiation protocol. c73: IF A.162/110 THEN m ELSE n/a - - indication of features supported by proxy. c74: IF A.162/115 THEN m ELSE i - - PSAP callback indicator. c75: IF A.162/119 THEN o ELSE n/a - - PCRF based P-CSCF restoration. c76: IF A.162/121 THEN m ELSE n/a - - the Relayed-Charge header field extension. c77: IF A.162/122 THEN o ELSE n/a - - resource sharing. c78: IF A.162/43 THEN x ELSE IF A.162/123 THEN m ELSE n/a - - act as subsequent entity within trust network for access network information that can route outside the trust network, the Cellular-Network-Info header extension. c79: IF A.162/43 OR (A.162/41A AND A.3/2) THEN m ELSE IF A.162/123 THEN i ELSE n/a - - act as subsequent entity within trust network for access network information that can route outside the trust network, the Cellular-Network-Info header extension (with or without P-CSCF). c80: IF A.162/124 THEN o ELSE n/a - - priority sharing. c81: IF A.162/126 OR (A.162/n134 AND A.3/4) THEN o ELSE n/a - - authenticated identity management in the Session Initiation Protocol, SIP Call-Info Parameters for Rich Call Data, S-CSCF. c82: IF A.162/128 THEN o ELSE n/a - - the Attestation-Info header field extension. c83: IF A.162/129 THEN o ELSE n/a - - the Origination-Id header field extension. c84: IF A.162/130 THEN m ELSE n/a - - Dynamic services interactions. c85: IF A.162/131 THEN o ELSE n/a - - the Additional-Identity header field extension. c86: IF A.162/80 AND A.162/133 THEN o ELSE n/a - - communications resource priority for the session initiation protocol, the Priority-Verstat header field extension. cbb: IF A.162/yyy THEN o ELSE n/a - - the DC-Info header field extension. |  |  |  |  |  |  |  |
| NOTE: c1 refers to the UA role major capability as this is the case of a proxy that also acts as a UA specifically for SUBSCRIBE and NOTIFY. |  |  |  |  |  |  |  |

Prerequisite A.163/8 - - INVITE request

Table A.205: Supported message bodies within the INVITE request

| Item | Header | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 1 | XML Schema for PSTN | [11B] | n/a | c1 | [11B] | n/a | i |
| 2 | application/vnd.3gpp.ussd | [8W] | n/a | m | [8W] | n/a | i |
| 3 | application/vnd.3gpp.mcptt-info+xml | [8ZE] | n/a | c1 | [8ZE] | n/a | i |
| c1: A.3/3 OR A.3/4 OR A.3/5 OR A.3/7C OR A.3/9A OR A.3/10 OR A.3/11 OR A.3/13A THEN m ELSE n/a - - I-CSCF, S-CSCF, BGCF, AS acting as proxy, IBCF (THIG), additional routeing functionality, E-CSCF, ISC gateway function (THIG). |  |  |  |  |  |  |  |

Prerequisite A.163/9 - - INVITE response

Prerequisite: A.164/1 - - Additional for 100 (Trying) response

Table A.206: Supported header fields within the INVITE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 1 | Call-ID | [26] 20.8 | m | m | [26] 20.8 | m | m |
| 2 | Content-Length | [26] 20.14 | m | m | [26] 20.14 | m | m |
| 3 | CSeq | [26] 20.16 | m | m | [26] 20.16 | m | m |
| 4 | Date | [26] 20.17 | c1 | c1 | [26] 20.17 | c2 | c2 |
| 5 | From | [26] 20.20 | m | m | [26] 20.20 | m | m |
| 6 | To | [26] 20.39 | m | m | [26] 20.39 | m | m |
| 7 | Via | [26] 20.42 | m | m | [26] 20.42 | m | m |
| c1: IF (A.162/9 AND A.162/5) OR A.162/4 THEN m ELSE n/a - - stateful proxy behaviour that inserts date, or stateless proxies. c2: IF A.162/4 THEN i ELSE m - - Stateless proxy passes on. |  |  |  |  |  |  |  |

Prerequisite A.163/9 - - INVITE response for all remaining status-codes

Table A.207: Supported header fields within the INVITE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 0A | Allow | [26] 20.5 | m | m | [26] 20.5 | i | i |
| 1 | Call-ID | [26] 20.8 | m | m | [26] 20.8 | m | m |
| 1A | Call-Info | [26] 20.9 | m | m | [26] 20.9 | c4 | c4 |
| 1B | Cellular-Network-Info | 7.2.15 | n/a | c27 | 7.2.15 | n/a | c28 |
| 2 | Content-Disposition | [26] 20.11 | m | m | [26] 20.11 | i | c3 |
| 3 | Content-Encoding | [26] 20.12 | m | m | [26] 20.12 | i | c3 |
| 4 | Content-Language | [26] 20.13 | m | m | [26] 20.13 | i | c3 |
| 5 | Content-Length | [26] 20.14 | m | m | [26] 20.14 | m | m |
| 6 | Content-Type | [26] 20.15 | m | m | [26] 20.15 | i | c3 |
| 7 | CSeq | [26] 20.16 | m | m | [26] 20.16 | m | m |
| 8 | Date | [26] 20.17 | m | m | [26] 20.17 | c1 | c1 |
| 8A | Expires | [26] 20.19 | m | m | [26] 20.19 | i | i |
| 9 | From | [26] 20.20 | m | m | [26] 20.20 | m | m |
| 9A | Geolocation-Error | [89] 4.3 | c24 | c24 | [89] 4.3 | c24 | c24 |
| 9B | History-Info | [66] 4.1 | c17 | c17 | [66] 4.1 | c17 | c17 |
| 10 | MIME-Version | [26] 20.24 | m | m | [26] 20.24 | i | c3 |
| 11 | Organization | [26] 20.25 | m | m | [26] 20.25 | c2 | c2 |
| 11A | P-Access-Network-Info | [52] 4.4, [52A] 4, [234] 2 | c14 | c14 | [52] 4.4, [52A] 4, [234] 2 | c15 | c15 |
| 11B | P-Asserted-Identity | [34] 9.1 | c6 | c6 | [34] 9.1 | c7 | c7 |
| 11C | P-Charging-Function-Addresses | [52] 4.5, [52A] 4 | c12 | c12 | [52] 4.5, [52A] 4 | c13 | c13 |
| 11D | P-Charging-Vector | [52] 4.6, [52A] 4 | c10 | c10 | [52] 4.6, [52A] 4 | c11 | c11 |
| 11F | P-Preferred-Identity | [34] 9.2 | x | x | [34] 9.2 | c5 | n/a |
| 11G | Privacy | [33] 4.2 | c8 | c8 | [33] 4.2 | c9 | c9 |
| 11H | Relayed-Charge | 7.2.12 | n/a | c26 | 7.2.12 | n/a | c26 |
| 11I | Reply-To | [26] 20.31 | m | m | [26] 20.31 | i | i |
| 11J | Require | [26] 20.32 | m | m | [26] 20.32 | c16 | c16 |
| 11K | Server | [26] 20.35 | m | m | [26] 20.35 | i | i |
| 11LA | Service-Interact-Info | Subclause 7.2.14 | n/a | c29 | Subclause 7.2.14 | n/a | c29 |
| 11L | Session-ID | [162] | c25 | c25 | [162] | c25 | c25 |
| 12 | Timestamp | [26] 20.38 | m | m | [26] 20.38 | i | i |
| 13 | To | [26] 20.39 | m | m | [26] 20.39 | m | m |
| 13A | User-Agent | [26] 20.41 | m | m | [26] 20.41 | i | i |
| 13B | User-to-User | [126] 7 | c20 | c20 | [126] 7 | c21 | c21 |
| 14 | Via | [26] 20.42 | m | m | [26] 20.42 | m | m |
| 15 | Warning | [26] 20.43 | m | m | [26] 20.43 | i | i |
| c1: IF A.162/9 THEN m ELSE i - - insertion of date in requests and responses. c2: IF A.162/19A OR A.162/19B THEN m ELSE i - - reading, adding or concatenating the Organization header. c3: IF A.3/2 OR A.3/4 THEN m ELSE i - - P-CSCF or S-CSCF. c4: IF A.162/19C OR A.162/19D THEN m ELSE i - - reading, adding or concatenating the Call-Info header. c5: IF A.162/30A THEN m ELSE n/a - - act as first entity within the trust domain for asserted identity. c6: IF A.162/30 THEN m ELSE n/a - - extensions to the Session Initiation Protocol (SIP) for asserted identity within trusted networks. c7: IF A.162/30A or A.162/30B THEN m ELSE i - - extensions to the Session Initiation Protocol (SIP) for asserted identity within trusted networks or subsequent entity within trust network that can route outside the trust network. c8: IF A.162/31 THEN m ELSE n/a - - a privacy mechanism for the Session Initiation Protocol (SIP). c9: IF A.162/31D OR A.162/31G THEN m ELSE IF A.162/31C THEN i ELSE n/a - - application of the privacy option "header" or application of the privacy option "id" or passing on of the Privacy header transparently. c10: IF A.162/45 THEN m ELSE n/a - - the P-Charging-Vector header extension. c11: IF A.162/46 THEN m ELSE IF A.162/45 THEN i ELSE n/a - - adding, deleting, reading or modifying the P-Charging-Vector header before proxying the request or response or the P-Charging-Vector header extension. c12: IF A.162/44 THEN m ELSE n/a - - the P-Charging-Function-Addresses header extension. c13: IF A.162/44A THEN m ELSE IF A.162/44 THEN i ELSE n/a - - adding, deleting or reading the P-Charging-Function-Addresses header before proxying the request or response, or the P-Charging-Function-Addresses header extension. c14: IF A.162/43 THEN x ELSE IF A.162/41 THEN m ELSE n/a - - act as subsequent entity within trust network for access network information that can route outside the trust network, the P-Access-Network-Info header extension. c15: IF A.162/43 THEN m ELSE IF A.162/41 THEN i ELSE n/a - - act as subsequent entity within trust network for access network information that can route outside the trust network, the P-Access-Network-Info header extension. c16: IF A.162/11 OR A.162/13 THEN m ELSE i - - reading the contents of the Require header before proxying the request or response or adding or modifying the contents of the Require header before proxying the request or response for methods other than REGISTER. c17: IF A.162/57 THEN m ELSE n/a - - an extension to the session initiation protocol for request history information. c18: IF A.162/70 THEN m ELSE n/a - - SIP location conveyance. c19: IF A.162/70A THEN m ELSE IF A.162/70B THEN i ELSE n/a - - addition or modification of location in a SIP method, passes on locations in SIP method without modification. c20: IF A.162/86 THEN m - - transporting user to user information for call centers using SIP. c21: IF A.162/86 THEN i - - transporting user to user information for call centers using SIP. c24: IF A.4/60 THEN m ELSE n/a - - SIP location conveyance. c25: IF A.162/101 THEN m ELSE n/a - - the Session-ID header. c26: IF A.162/121 THEN m ELSE n/a - - the Relayed-Charge header field extension. c27: IF A.162/43 THEN x ELSE IF A.162/123 THEN m ELSE n/a - - act as subsequent entity within trust network for access network information that can route outside the trust network, the Cellular-Network-Info header extension. c28: IF A.162/43 THEN m ELSE IF A.162/123 THEN i ELSE n/a - - act as subsequent entity within trust network for access network information that can route outside the trust network, the Cellular-Network-Info header extension. c29: IF A.162/130 THEN m ELSE n/a - - Dynamic services interactions. |  |  |  |  |  |  |  |

Prerequisite A.163/9 - - INVITE response

Prerequisite: A.164/101A - - Additional for 18x response

Table A.208: Supported header fields within the INVITE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 4 | Contact | [26] 20.10 | m | m | [26] 20.10 | i | i |
| 4A | Feature-Caps | [190] | c19 | c19 | [190] | c19 | c19 |
| 5 | P-Answer-State | [111] | c13 | c13 | [111] | c14 | c14 |
| 5A | P-Early-Media | [109] 8 | o | c11 | [109] 8 | o | c11 |
| 6 | P-Media-Authorization | [31] 5.1 | c9 | x | [31] 5.1 | n/a | n/a |
| 6AA | Priority-Share | Subclause 7.2.16 | n/a | c21 | Subclause 7.2.16 | n/a | c21 |
| 6A | Reason | [130], [294] | o | c18 | [130], [294] | o | c18 |
| 7 | Record-Route | [26] 20.30 | m | m | [26] 20.30 | c15 | c15 |
| 8 | Recv-Info | [25] 5.2.3 | c16 | c16 | [25] 5.2.3 | c17 | c17 |
| 8A | Resource-Share | Subclause 4.15 | n/a | c20 | Subclause 4.15 | n/a | c20 |
| 9 | RSeq | [27] 7.1 | m | m | [27] 7.1 | i | i |
| c9: IF A.162/26 THEN m ELSE n/a - - SIP extensions for media authorization. c11: IF A.162/76 THEN m ELSE n/a - - the SIP P-Early-Media private header extension for authorization of early media. c13: IF A.162/75 THEN m ELSE n/a - - the P-Answer-State header extension to the session initiation protocol for the open mobile alliance push to talk over cellular. c14: IF A.162/75 THEN i ELSE n/a - - the P-Answer-State header extension to the session initiation protocol for the open mobile alliance push to talk over cellular. c15: IF A.162/14 THEN m ELSE i - - the requirement to be able to insert itself in the subsequent transactions in a dialog. c16: IF A.162/20 THEN m ELSE n/a - - SIP INFO method and package framework. c17: IF A.162/20 THEN i ELSE n/a - - SIP INFO method and package framework. c18: IF A.162/48A OR A.162/48C THEN o ELSE n/a - - use of the Reason header field in Session Initiation Protocol (SIP) responses (carrying Q.850 or STIR codes). c19: IF A.162/110 THEN m ELSE n/a - - indication of features supported by proxy. c20: IF A.162/122 THEN o ELSE n/a - - resource sharing. c21: IF A.162/124 THEN o ELSE n/a - - priority sharing. |  |  |  |  |  |  |  |

Prerequisite A.163/9 - - INVITE response

Prerequisite: A.164/2 - - Additional for 180 (Ringing) response

Table A.208A: Supported header fields within the INVITE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 1 | Alert-Info | [26] 20.4 | m | m | [26] 20.4 | i | i |

Prerequisite A.163/9 - - INVITE response

Prerequisite: A.164/5A - - Additional for 199 (Early Dialog Terminated) response

Table A.208B: Supported header fields within the INVITE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 4 | Contact | [26] 20.10 | m | m | [26] 20.10 | i | i |
| 5 | Reason | [130], [294] | o | c18 | [130], [294] | o | c18 |
| 7 | Record-Route | [26] 20.30 | m | m | [26] 20.30 | c15 | c15 |
| 8 | Recv-Info | [25] 5.2.3 | c16 | c16 | [25] 5.2.3 | c17 | c17 |
| 9 | RSeq | [27] 7.1 | m | m | [27] 7.1 | i | i |
| c15: IF A.162/14 THEN m ELSE i - - the requirement to be able to insert itself in the subsequent transactions in a dialog. c16: IF A.162/20 THEN m ELSE n/a - - SIP INFO method and package framework. c17: IF A.162/20 THEN i ELSE n/a - - SIP INFO method and package framework. c18: IF A.162/48A OR A.162/48C THEN o ELSE n/a - - use of the Reason header field in Session Initiation Protocol (SIP) responses (carrying Q.850 or STIR codes). |  |  |  |  |  |  |  |

Prerequisite A.163/9 - - INVITE response

Prerequisite: A.164/102 - - Additional for 2xx response

Table A.209: Supported header fields within the INVITE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 1 | Accept | [26] 20.1 | m | m | [26] 20.1 | i | i |
| 1A | Accept-Encoding | [26] 20.2 | m | m | [26] 20.2 | i | i |
| 1B | Accept-Language | [26] 20.3 | m | m | [26] 20.3 | i | i |
| 1C | Accept-Resource-Priority | [116] 3.2 | c12 | c12 | [116] 3.2 | c12 | c12 |
| 2 | Allow-Events | [28] 8.2.2 | m | m | [28] 8.2.2 | c1 | c1 |
| 3 | Answer-Mode | [158] | c19 | c19 | [158] | c20 | c20 |
| 4 | Authentication-Info | [26] 20.6 | m | m | [26] 20.6 | i | i |
| 6 | Contact | [26] 20.10 | m | m | [26] 20.10 | i | i |
| 6A | Feature-Caps | [190] | c22 | c22 | [190] | c22 | c22 |
| 7 | P-Answer-State | [111] | c13 | c13 | [111] | c14 | c14 |
| 7A | P-Visited-Network-ID | [52B] 3 | o | o | [52B] 3 | o | o |
| 8 | P-Media-Authorization | [31] 5.1 | c9 | x | [31] 5.1 | n/a | n/a |
| 8AA | Priority-Share | Subclause 7.2.16 | n/a | c24 | Subclause 7.2.16 | n/a | c24 |
| 8A | Priv-Answer-Mode | [158] | c19 | c19 | [158] | c20 | c20 |
| 8B | Reason | [294] | o | c25 | [294] | o | c25 |
| 9 | Record-Route | [26] 20.30 | m | m | [26] 20.30 | c3 | c3 |
| 9A | Recv-Info | [25] 5.2.3 | c17 | c17 | [25] 5.2.3 | c18 | c18 |
| 9B | Resource-Share | Subclause 4.15 | n/a | c23 | Subclause 4.15 | n/a | c23 |
| 10 | Session-Expires | [58] 4 | c11 | c11 | [58] 4 | c11 | c11 |
| 13 | Supported | [26] 20.37 | m | m | [26] 20.37 | i | i |
| c1: IF A.4/20 THEN m ELSE i - - SIP specific event notification extension. c3: IF A.162/14 THEN m ELSE i - - the requirement to be able to insert itself in the subsequent transactions in a dialog. c9: IF A.162/26 THEN m ELSE n/a - - SIP extensions for media authorization. c11: IF A.162/52 THEN m ELSE n/a - - the SIP session timer. c12: IF A.162/80 THEN m ELSE n/a - - communications resource priority for the session initiation protocol. c13: IF A.162/75 THEN m ELSE n/a - - the P-Answer-State header extension to the session initiation protocol for the open mobile alliance push to talk over cellular. c14: IF A.162/75 THEN i ELSE n/a - - the P-Answer-State header extension to the session initiation protocol for the open mobile alliance push to talk over cellular. c17: IF A.162/20 THEN m ELSE n/a - - SIP INFO method and package framework. c18: IF A.162/20 THEN i ELSE n/a - - SIP INFO method and package framework. c19: IF A.162/97 THEN m ELSE n/a - - requesting answering modes for SIP. c20: IF NOT A.162/97 THEN n/a ELSE IF A.162/97A THEN m ELSE i - - requesting answering modes for SIP, adding, deleting or reading the Answer-Mode header or Priv-Answer-Mode header before proxying the request or response. c22: IF A.162/110 THEN m ELSE n/a - - indication of features supported by proxy. c23: IF A.162/122 THEN o ELSE n/a - - resource sharing. c24: IF A.162/124 THEN o ELSE n/a - - priority sharing. c25: IF A.162/48C THEN o ELSE n/a - - use of the Reason header field in Session Initiation Protocol (SIP) responses (carrying STIR codes). |  |  |  |  |  |  |  |

Prerequisite A.163/9 - - INVITE response

Prerequisite: A.164/103 OR A.164/104 OR A.164/105 OR A.164/106 - - Additional for 3xx – 6xx response

Table A.209A: Supported header fields within the INVITE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 1 | Error-Info | [26] 20.18 | m | m | [26] 20.18 | i | i |
| 2 | Reason | [130], [294] | o | c1 | [130], [294] | o | c1 |
| 3 | Response-Source | 7.2.17 | n/a | c2 | 7.2.17 | n/a | c2 |
| c1: IF A.162/48A OR A.162/48C THEN o ELSE n/a - - use of the Reason header field in Session Initiation Protocol (SIP) responses (carrying Q.850 or STIR codes). c2: IF A.162/125 THEN o ELSE n/a - - use of the Response-Source header field in SIP error responses? |  |  |  |  |  |  |  |

Prerequisite A.163/9 - - INVITE response

Prerequisite: A.164/103 OR A.164/35 - - Additional for 3xx or 485 (Ambiguous) response

Table A.210: Supported header fields within the INVITE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 4 | Contact | [26] 20.10 | m | m | [26] 20.10 | c1 | c1 |
| c1: IF A.162/19E THEN m ELSE i - - deleting Contact headers. |  |  |  |  |  |  |  |

Prerequisite A.163/9 - - INVITE response

Prerequisite: A.164/14 - - Additional for 401 (Unauthorized) response

Table A.211: Supported header fields within the INVITE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 6 | Proxy-Authenticate | [26] 20.27 | m | m | [26] 20.27 | m | m |
| 15 | WWW-Authenticate | [26] 20.44 | m | m | [26] 20.44 | i | i |

Prerequisite A.163/9 - - INVITE response

Prerequisite: A.6/16 - - Additional for 403 (Forbidden) response

Table A.211A: Supported header fields within the INVITE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 1 | P-Refused-URI-List | [183] | c1 | c1 | [183] | c1 | c1 |
| c1: IF A.162/108 THEN m ELSE n/a -- The SIP P-Refused-URI-List private-header. |  |  |  |  |  |  |  |

Prerequisite A.163/9 - - INVITE response

Prerequisite: A.164/17 OR A.164/23 OR A.164/30 OR A.164/36 OR A.164/50 OR A.164/51 - - Additional for 404 (Not Found), 413 (Request Entity Too Large), 480(Temporarily not available), 486 (Busy Here), 500 (Internal Server Error), 600 (Busy Everywhere), 603 (Decline) response

Table A.212: Supported header fields within the INVITE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 8 | Retry-After | [26] 20.33 | m | m | [26] 20.33 | i | i |

Table A.213: Void

Prerequisite A.163/9 - - INVITE response

Prerequisite: A.164/20 - - Additional for 407 (Proxy Authentication Required) response

Table A.214: Supported header fields within the INVITE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 6 | Proxy-Authenticate | [26] 20.27 | m | m | [26] 20.27 | m | m |
| 11 | WWW-Authenticate | [26] 20.44 | m | m | [26] 20.44 | i | i |

Prerequisite A.163/9 - - INVITE response

Prerequisite: A.164/21 - - Additional for 408 (Request timeout) response

Table A.214A: Supported header fields within the INVITE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 1 | Restoration-Info | subclause 7.2.11 | n/a | c1 | subclause 7.2.11 | n/a | c2 |
| c1: IF A.162/120 THEN o ELSE n/a - - HSS based P-CSCF restoration. c2: IF A.162/120 THEN m ELSE n/a - - HSS based P-CSCF restoration. |  |  |  |  |  |  |  |

Prerequisite A.163/9 - - INVITE response

Prerequisite: A.164/25 - - Additional for 415 (Unsupported Media Type) response

Table A.215: Supported header fields within the INVITE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 1 | Accept | [26] 20.1 | m | m | [26] 20.1 | i | i |
| 2 | Accept-Encoding | [26] 20.2 | m | m | [26] 20.2 | i | i |
| 3 | Accept-Language | [26] 20.3 | m | m | [26] 20.3 | i | i |

Prerequisite A.163/9 - - INVITE response

Prerequisite: A.164/26A - - Additional for 417 (Unknown Resource-Priority) response

Table A.215A: Supported header field	s within the INVITE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 1 | Accept-Resource-Priority | [116] 3.2 | c1 | c1 | [116] 3.2 | c1 | c1 |
| c1: IF A.162/80 THEN m ELSE n/a - - communications resource priority for the session initiation protocol. |  |  |  |  |  |  |  |

Prerequisite A.163/9 - - INVITE response

Prerequisite: A.164/27 - - Additional for 420 (Bad Extension) response

Table A.216: Supported header fields within the INVITE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 10 | Unsupported | [26] 20.40 | m | m | [26] 20.40 | c3 | c3 |
| c3: IF A.162/18 THEN m ELSE i - - reading the contents of the Unsupported header before proxying the 420 response to a method other than REGISTER. |  |  |  |  |  |  |  |

Prerequisite A.163/9 - - INVITE response

Prerequisite: A.164/28 OR A.164/41A - - Additional for 421 (Extension Required), 494 (Security Agreement Required) response

Table A.216A: Supported header fields within the INVITE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 3 | Security-Server | [48] 2 | c1 | c1 | [48] 2 | n/a | n/a |
| c1: IF A.162/47 THEN m ELSE n/a - - security mechanism agreement for the session initiation protocol. |  |  |  |  |  |  |  |

Prerequisite A.16/9 - - INVITE response

Prerequisite: A.164/28A - - Additional for 422 (Session Interval Too Small) response

Table A.216B: Supported header fields within the INVITE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 1 | Min-SE | [58] 5 | c1 | c1 | [58] 5 | c1 | c1 |
| c1: IF A.162/52 THEN m ELSE n/a - - the SIP session timer. |  |  |  |  |  |  |  |

Table A.217: Void

Table A.217A: Void

Prerequisite A.163/9 - - INVITE response

Prerequisite: A.164/29H - - Additional for 470 (Consent Needed) response

Table A.217AA: Supported header fields within the INVITE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 1 | Permission-Missing | [125] 5.9.3 | m | m | [125] 5.9.3 | m | m |

Prerequisite A.163/9 - - INVITE response

Prerequisite: A.164/45 - - 503 (Service Unavailable)

Table A.217B: Supported header fields within the INVITE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 8 | Retry-After | [26] 20.33 | m | m | [26] 20.33 | i | i |

Table A.217C: void

Prerequisite A.163/9 - - INVITE response

Table A.218: Supported message bodies within the INVITE response

| Item | Header | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 1 | XML Schema for PSTN | [11B] |  | c1 | [11B] |  | i |
| 2 | Recipient List | [183] | c2 | c2 | [183] | c3 | c3 |
| 3 | 3GPP IM CN subsystem XML body | subclause 7.6 | n/a | c4 | subclause 7.6 | n/a | c5 |
| 4 | application/vnd.3gpp.mcptt-info+xml | [8ZE] | n/a | c1 | [8ZE] | n/a | i |
| c1: A.3/3 OR A.3/4 OR A.3/5 OR A.3/7C OR A.3/9A OR A.3/10 OR A.3/11 OR A.3/13A THEN m ELSE n/a - - I-CSCF, S-CSCF, BGCF, AS acting as proxy, IBCF (THIG), additional routeing functionality, E-CSCF, ISC gateway function (THIG). c2 IF A.3/9B THEN m ELSE IF A.3/7A OR A.3/7B OR A.3/7D THEN m ELSE n/a - - IBCF (IMS-ALG), AS acting as terminating UA, AS acting as originating UA, AS performing 3rd party call control. c3 IF A.3/9B THEN m ELSE IF A.3/7A OR A.3/7B OR A.3/7D THEN i ELSE n/a - - IBCF (IMS-ALG), AS acting as terminating UA, AS acting as originating UA, AS performing 3rd party call control. c4: IF A.3/2 OR (A.3/9 AND NOT A.3/9B) OR A.3A/88 THEN m ELSE n/a - - P-CSCF, IBCF, IBCF (IMS-ALG), ATCF (proxy). c5: IF A.3/2 OR (A.3/9 AND NOT A.3/9B) OR A.3A/88 THEN i ELSE n/a - - P-CSCF, IBCF, IBCF (IMS-ALG), ATCF (proxy). |  |  |  |  |  |  |  |

# *** Next Change ***

#### A.2.2.4.7A	MESSAGE method

Prerequisite A.163/9A - - MESSAGE request

Table A.218A: Supported header fields within the MESSAGE request

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 1 | Accept-Contact | [56B] 9.2 | c28 | c28 | [56B] 9.2 | c28 | c29 |
| 1AA | Additional-Identity | 7.2.20 | n/a | c79 | 7.2.20 | n/a | c79 |
| 1A | Allow | [26] 20.5 | m | m | [50] 10 | i | i |
| 2 | Allow-Events | [28] 8.2.2 | m | m | [28] 8.2.2 | c1 | c1 |
| 2A | Attestation-Info | 7.2.18 | n/a | c76 | 7.2.18 | n/a | c76 |
| 3 | Authorization | [26] 20.7 | m | m | [26] 20.7 | i | i |
| 4 | Call-ID | [26] 20.8 | m | m | [26] 20.8 | m | m |
| 5 | Call-Info | [26] 20.9, [303] | m | m | [26] 20.9 | c4 | c4 |
| 5A | Cellular-Network-Info | 7.2.15 | n/a | c73 | 7.2.15 | n/a | c74 |
| 6 | Content-Disposition | [26] 20.11 | m | m | [26] 20.11 | i | i |
| 7 | Content-Encoding | [26] 20.12 | m | m | [26] 20.12 | i | i |
| 8 | Content-Language | [26] 20.13 | m | m | [26] 20.13 | i | i |
| 9 | Content-Length | [26] 20.14 | m | m | [26] 20.14 | m | m |
| 10 | Content-Type | [26] 20.15 | m | m | [26] 20.15 | i | i |
| 11 | CSeq | [26] 20.16 | m | m | [26] 20.16 | m | m |
| 12 | Date | [26] 20.17 | m | m | [26] 20.17 | c2 | c2 |
| 13 | Expires | [26] 20.19 | m | m | [26] 20.19 | I | i |
| 13A | Feature-Caps | [190] | c71 | c71 | [190] | c71 | c71 |
| 14 | From | [26] 20.20 | m | m | [26] 20.20 | m | m |
| 14A | Geolocation | [89] 4.1 | c36 | c36 | [89] 4.1 | c37 | c37 |
| 14B | History-Info | [66] 4.1 | c32 | c32 | [66] 4.1 | c32 | c32 |
| 14C | Geolocation-Routing | [89] 4.1 | c36 | c36 | [89] 4.1 | c37 | c37 |
| 14D | Identity | [252] 4 | c75 | c75 | [252] 4 | c75 | c75 |
| 15 | In-Reply-To | [26] 20.21 | m | m | [50] 10 | i | i |
| 15A | Max-Breadth | [117] 5.8 | c48 | c48 | [117] 5.8 | c49 | c49 |
| 16 | Max-Forwards | [26] 20.22 | m | m | [26] 20.22 | m | m |
| 17 | MIME-Version | [26] 20.24 | m | m | [26] 20.24 | i | i |
| 18 | Organization | [26] 20.25 | m | m | [26] 20.25 | c3 | c3 |
| 18AA | Origination-Id | 7.2.19 | n/a | c77 | 7.2.19 | n/a | c77 |
| 18A | P-Access-Network-Info | [52] 4.4, [234] 2 | c23 | c23 | [52] 4.4, [234] 2 | c24 | c24 |
| 18B | P-Asserted-Identity | [34] 9.1 | c10 | c10 | [34] 9.1 | c11 | c11 |
| 18C | P-Asserted-Service | [121] 4.1 | c40 | c40 | [121] 4.1 | c41 | c41 |
| 18D | P-Called-Party-ID | [52] 4.2 | c14 | c14 | [52] 4.2 | c15 | c16 |
| 18E | P-Charging-Function-Addresses | [52] 4.5 | c21 | c21 | [52] 4.5 | c22 | c22 |
| 18F | P-Charging-Vector | [52] 4.6 | c19 | c19 | [52] 4.6 | c20 | c20 |
| 18H | P-Preferred-Identity | [34] 9.2 | x | c69 | [34] 9.2 | c9 | c9 |
| 18I | P-Preferred-Service | [121] 4.2 | x | x | [121] 4.2 | c39 | c39 |
| 18J | P-Private-Network-Indication | [134] | c44 | c44 | [134] | c44 | c44 |
| 18K | P-Profile-Key | [97] 5 | c34 | c34 | [97] 5 | c35 | c35 |
| 18L | P-Served-User | [133] 6 | c45 | c45 | [133] 6 | c45 | c45 |
| 18M | P-User-Database | [82] 4 | c33 | c33 | [82] 4 | c33 | c33 |
| 18N | P-Visited-Network-ID | [52] 4.3 | c17 | o | [52] 4.3 | c18 | o |
| 19 | Priority | [26] 20.26 | m | m | [26] 20.26 | i | c50 |
| 19A | Privacy | [33] 4.2 | c12 | c12 | [33] 4.2 | c13 | c13 |
| 20 | Proxy-Authorization | [26] 20.28 | m | m | [26] 20.28 | c8 | c8 |
| 21 | Proxy-Require | [26] 20.29 | m | m | [26] 20.29 | m | m |
| 21A | Reason | [34A] 2 | c26 | c26 | [34A] 2 | c27 | c27 |
| 22A | Referred-By | [59] 3 | c30 | c30 | [59] 3 | c31 | c31 |
| 23 | Reject-Contact | [56B] 9.2 | c28 | c28 | [56B] 9.2 | c28 | c29 |
| 23A | Relayed-Charge | 7.2.12 | n/a | c72 | 7.2.12 | n/a | c72 |
| 23B | Reply-To | [26] 20.31 | m | m | [26] 20.31 | i | i |
| 23C | Request-Disposition | [56B] 9.1 | c28 | c28 | [56B] 9.1 | c28 | c28 |
| 24 | Require | [26] 20.32 | m | m | [26] 20.32 | c5 | c5 |
| 24A | Resource-Priority | [116] 3.1 | c38 | c38 | [116] 3.1 | c38 | c38 |
| 25 | Route | [26] 20.34 | m | m | [26] 20.34 | m | m |
| 25A | Security-Client | [48] 2.3.1 | x | x | [48] 2.3.1 | c25 | c25 |
| 25B | Security-Verify | [48] 2.3.1 | x | x | [48] 2.3.1 | c25 | c25 |
| 25D | Service-Interact-Info | Subclause 7.2.14 | n/a | c78 | Subclause 7.2.14 | n/a | c78 |
| 26 | Subject | [26] 20.36 | m | m | [26] 20.36 | i | i |
| 25C | Session-ID | [162] | c70 | c70 | [162] | c70 | c70 |
| 27 | Supported | [26] 20.37 | m | m | [26] 20.37 | c6 | c6 |
| 28 | Timestamp | [26] 20.38 | m | m | [26] 20.38 | i | i |
| 29 | To | [26] 20.39 | m | m | [26] 20.39 | m | m |
| 29A | Trigger-Consent | [125] 5.11.2 | c42 | c42 | [125] 5.11.2 | c43 | c43 |
| 30 | User-Agent | [26] 20.41 | m | m | [26] 20.41 | i | i |
| 31 | Via | [26] 20.42 | m | m | [26] 20.42 | m | m |
| c1: IF A.4/20 THEN m ELSE i - - SIP specific event notification extension. c2: IF A.162/9 THEN m ELSE i - - insertion of date in requests and responses. c3: IF A.162/19A OR A.162/19B THEN m ELSE i - - reading, adding or concatenating the Organization header. c4: IF A.162/19C OR A.162/19D THEN m ELSE IF A.162/n134 THEN o ELSE i - - reading, adding or concatenating the Call-Info header, SIP Call-Info Parameters for Rich Call Data. c5: IF A.162/11 OR A.162/13 THEN m ELSE i - - reading the contents of the Require header before proxying the request or response or adding or modifying the contents of the Require header before proxying the request or response for methods other than REGISTER. c6: IF A.162/16 THEN m ELSE i - - reading the contents of the Supported header before proxying the response. c8: IF A.162/8A THEN m ELSE i - - authentication between UA and proxy. c9: IF A.162/30A OR A.162/30C THEN m ELSE n/a - - act as first entity within the trust domain for asserted identity, act as entity passing on identity transparently independent of trust domain. c10: IF A.162/30 THEN m ELSE n/a - - extensions to the Session Initiation Protocol (SIP) for asserted identity within trusted networks. c11: IF A.162/30A or A.162/30B THEN m ELSE i - - extensions to the Session Initiation Protocol (SIP) for asserted identity within trusted networks or subsequent entity within trust network that can route outside the trust network. c12: IF A.162/31 THEN m ELSE n/a - - a privacy mechanism for the Session Initiation Protocol (SIP). c13: IF A.162/31D OR A.162/31G THEN m ELSE IF A.162/31C THEN i ELSE n/a - - application of the privacy option "header" or application of the privacy option "id" or passing on of the Privacy header transparently. c14: IF A.162/37 THEN m ELSE n/a - - the P-Called-Party-ID header extension. c15: IF A.162/37 THEN i ELSE n/a - - the P-Called-Party-ID header extension. c16: IF A.162/37 AND A.3/2 THEN m ELSE IF A.162/37 AND (A.3/3 OR A.3/9A) THEN i ELSE n/a - - the P-Called-Party-ID header extension and P-CSCF or (I-CSCF or IBCF (THIG)). c17: IF A.162/38 THEN m ELSE n/a - - the P-Visited-Network-ID header extension. c18: IF A.162/39 THEN m ELSE i - - reading, or deleting the P-Visited-Network-ID header before proxying the request or response. c19: IF A.162/45 THEN m ELSE n/a - - the P-Charging-Vector header extension. c20: IF A.162/46 THEN m ELSE IF A.162/45 THEN i ELSE n/a - - adding, deleting, reading or modifying the P-Charging-Vector header before proxying the request or response or the P-Charging-Vector header extension. c21: IF A.162/44 THEN m ELSE n/a - - the P-Charging-Function-Addresses header extension. c22: IF A.162/44A THEN m ELSE IF A.162/44 THEN i ELSE n/a - - adding, deleting or reading the P-Charging-Function-Addresses header before proxying the request or response, or the P-Charging-Function-Addresses header extension. c23: IF A.162/43 THEN x ELSE IF A.162/41 THEN m ELSE n/a - - act as subsequent entity within trust network for access network information that can route outside the trust network, the P-Access-Network-Info header extension. c24: IF A.162/43 THEN m ELSE IF A.162/41 THEN i ELSE n/a - - act as subsequent entity within trust network for access network information that can route outside the trust network, the P-Access-Network-Info header extension. c25: IF A.162/47 OR A.162/47A THEN m ELSE n/a - - security mechanism agreement for the session initiation protocol or mediasec header field parameter for marking security mechanisms related to media. c26: IF A.162/48 THEN m ELSE n/a - - the Reason header field for the session initiation protocol. c27: IF A.162/48 THEN i ELSE n/a - - the Reason header field for the session initiation protocol. c28: IF A.162/50 THEN m ELSE n/a - - caller preferences for the session initiation protocol. c29: IF A.162/50 AND A.4/3 THEN m ELSE IF A.162/50 AND NOT A.4/3 THEN i ELSE n/a - - caller preferences for the session initiation protocol, and S-CSCF. c30: IF A.162/53 THEN i ELSE n/a - - the SIP Referred-By mechanism. c31: IF A.162/53 THEN m ELSE n/a - - the SIP Referred-By mechanism. c32: IF A.162/57 THEN m ELSE n/a - - an extension to the session initiation protocol for request history information. c33: IF A.162/60 THEN m ELSE n/a - - the P-User-Database private header extension. c34: IF A.162/66A THEN m ELSE n/a - - making the first query to the database in order to populate the P-Profile-Key header. c35: IF A.162/66B THEN m ELSE n/a - - using the information in the P-Profile-Key header. c36: IF A.162/70 THEN m ELSE n/a - - SIP location conveyance. c37: IF A.162/70A THEN m ELSE IF A.162/70B THEN i ELSE n/a - - addition or modification of location in a SIP method, passes on locations in SIP method without modification. c38: IF A.162/80A THEN m ELSE n/a - - inclusion of MESSAGE, SUBSCRIBE, NOTIFY in communications resource priority for the session initiation protocol. c39: IF A.162/84A THEN m ELSE n/a - - act as authentication entity within the trust domain for asserted service. |  |  |  |  |  |  |  |
| c40: IF A.162/84 THEN m ELSE n/a - - SIP extension for the identification of services. c41: IF A.162/84 OR A.162/30B THEN m ELSE i - - SIP extension for the identification of services or subsequent entity within trust network that can route outside the trust network. c42: IF A.162/85 THEN m ELSE n/a - - a framework for consent-based communications in SIP. c43: IF A.162/85 THEN i ELSE n/a - - a framework for consent-based communications in SIP. c44: IF A.162/87 THEN m ELSE n/a - - the SIP P-Private-Network-Indication private-header (P-Header). c45: IF A.162/88 THEN m ELSE n/a - - the SIP P-Served-User private header. c48: IF A.162/81 THEN m ELSE n/a - - addressing an amplification vulnerability in session initiation protocol forking proxies. c49: IF A.162/81 AND A.162/6 THEN m ELSE IF A.162/81 AND NOT A.162/6 THEN i ELSE n/a - - addressing an amplification vulnerability in session initiation protocol forking proxies, forking of initial requests. c50: IF A.162/115 THEN m ELSE i - - PSAP callback indicator. c69: IF A.162/30C THEN m ELSE x - - act as entity passing on identity transparently independent of trust domain. c70: IF A.162/101 THEN m ELSE n/a - - the Session-ID header. c71: IF A.162/110 THEN m ELSE n/a - - indication of features supported by proxy. c72: IF A.162/121 THEN m ELSE n/a - - the Relayed-Charge header field extension. c73: IF A.162/43 THEN x ELSE IF A.162/123 THEN m ELSE n/a - - act as subsequent entity within trust network for access network information that can route outside the trust network, the Cellular-Network-Info header extension. c74: IF A.162/43 THEN m ELSE IF A.162/123 THEN i ELSE n/a - - act as subsequent entity within trust network for access network information that can route outside the trust network, the Cellular-Network-Info header extension. c75: IF A.162/126 OR (A.162/n134 AND A.3/4) THEN o ELSE n/a - - authenticated identity management in the Session Initiation Protocol, SIP Call-Info Parameters for Rich Call Data, S-CSCF. c76: IF A.162/128 THEN o ELSE n/a - - the Attestation-Info header field extension. c77: IF A.162/129 THEN o ELSE n/a - - the Origination-Id header field extension. c78: IF A.162/130 THEN m ELSE n/a - - Dynamic services interactions. c79: IF A.162/131 THEN o ELSE n/a - - the Additional-Identity header field extension. |  |  |  |  |  |  |  |
| NOTE: c1 refers to the UA role major capability as this is the case of a proxy that also acts as a UA specifically for SUBSCRIBE and NOTIFY. |  |  |  |  |  |  |  |

Prerequisite A.163/9A - - MESSAGE request

Table A.218B: Supported message bodies within the MESSAGE request

| Item | Header | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 1 | permission document | [125] 5.4 | c1 | c1 | [125] 5.4 | c2 | c2 |
| 2 | application/vnd.3gpp.sms | [4D] | m | m | [4D] | i | i |
| 2 | message/cpim | [151] | c3 | c3 | [151] | c4 | c4 |
| 3 | message/imdn+xml | [157] | c5 | c5 | [157] | c6 | c6 |
| 4 | application/vnd.3gpp.mcptt-info+xml | [8ZE] | n/a | c7 | [8ZE] | n/a | i |
| 5 | application/vnd.3gpp.mcptt-mbms-usage-info+xml | [8ZE] | n/a | c7 | [8ZE] | n/a | i |
| 6 | application/vnd.3gpp.mcptt-location-info+xml | [8ZE] | n/a | c7 | [8ZE] | n/a | i |
| 7 | application/vnd.3gpp.mcptt-floor-request+xml | [8ZE] | n/a | c7 | [8ZE] | n/a | i |
| 8 | application/vnd.3gpp.mcptt-affiliation-command+xml | [8ZE] | n/a | c7 | [8ZE] | n/a | i |
| c1: IF A.162/85 THEN m ELSE n/a - - a framework for consent-based communications in SIP. c2: IF A.162/85 THEN i ELSE n/a - - a framework for consent-based communications in SIP. c3: IF A.162/95 THEN m ELSE n/a - - common presence and instant messaging (CPIM): message format. c4: IF A.162/95 THEN i ELSE n/a - - common presence and instant messaging (CPIM): message format. c5: IF A.162/96 THEN m ELSE n/a - - instant message disposition notification. c6: IF A.162/96 THEN i ELSE n/a - - instant message disposition notification. c7: A.3/3 OR A.3/4 OR A.3/5 OR A.3/7C OR A.3/9A OR A.3/10 OR A.3/11 OR A.3/13A THEN m ELSE n/a - - I-CSCF, S-CSCF, BGCF, AS acting as proxy, IBCF (THIG), additional routeing functionality, E-CSCF, ISC gateway function (THIG). |  |  |  |  |  |  |  |

Prerequisite A.163/9B - - MESSAGE response

Prerequisite: A.164/1 - - Additional for 100 (Trying) response

Table A.218BA: Supported header fields within the MESSAGE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 1 | Call-ID | [26] 20.8 | m | m | [26] 20.8 | m | m |
| 2 | Content-Length | [26] 20.14 | m | m | [26] 20.14 | m | m |
| 3 | CSeq | [26] 20.16 | m | m | [26] 20.16 | m | m |
| 4 | Date | [26] 20.17 | c1 | c1 | [26] 20.17 | c2 | c2 |
| 5 | From | [26] 20.20 | m | m | [26] 20.20 | m | m |
| 6 | To | [26] 20.39 | m | m | [26] 20.39 | m | m |
| 7 | Via | [26] 20.42 | m | m | [26] 20.42 | m | m |
| c1: IF (A.162/9 AND A.162/5) OR A.162/4 THEN m ELSE n/a - - stateful proxy behaviour that inserts date, or stateless proxies. c2: IF A.162/4 THEN i ELSE m - - Stateless proxy passes on. |  |  |  |  |  |  |  |

Prerequisite A.163/9B - - MESSAGE response for all remaining status-codes

Table A.218C: Supported header fields within the MESSAGE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 0A | Allow | [26] 20.5 | m | m | [26] 20.5 | i | i |
| 1 | Call-ID | [26] 20.8 | m | m | [26] 20.8 | m | m |
| 2 | Call-Info | [26] 20.9 | m | m | [26] 20.9 | c3 | c3 |
| 2A | Cellular-Network-Info | 7.2.15 | n/a | c23 | 7.2.15 | n/a | c24 |
| 3 | Content-Disposition | [26] 20.11 | m | m | [26] 20.11 | i | i |
| 4 | Content-Encoding | [26] 20.12 | m | m | [26] 20.12 | i | i |
| 5 | Content-Language | [26] 20.13 | m | m | [26] 20.13 | i | i |
| 6 | Content-Length | [26] 20.14 | m | m | [26] 20.14 | m | m |
| 7 | Content-Type | [26] 20.15 | m | m | [26] 20.15 | i | i |
| 8 | CSeq | [26] 20.16 | m | m | [26] 20.16 | m | m |
| 9 | Date | [26] 20.17 | m | m | [26] 20.17 | c1 | c1 |
| 9A | Expires | [26] 20.19 | m | m | [26] 20.19 | i | i |
| 10 | From | [26] 20.20 | m | m | [26] 20.20 | m | m |
| 10A | Geolocation-Error | [89] 4.3 | c17 | c17 | [89] 4.3 | c18 | c18 |
| 10B | History-Info | [66] 4.1 | c16 | c16 | [66] 4.1 | c16 | c16 |
| 11 | MIME-Version | [26] 20.24 | m | m | [26] 20.24 | i | i |
| 12 | Organization | [26] 20.25 | m | m | [26] 20.25 | c2 | c2 |
| 12A | P-Access-Network-Info | [52] 4.4, [52A] 4, [234] 2 | c13 | c13 | [52] 4.4, [52A] 4, [234] 2 | c14 | c14 |
| 12B | P-Asserted-Identity | [34] 9.1 | c5 | c5 | [34] 9.1 | c6 | c6 |
| 12C | P-Charging-Function-Addresses | [52] 4.5, [52A] 4 | c11 | c11 | [52] 4.5, [52A] 4 | c12 | c12 |
| 12D | P-Charging-Vector | [52] 4.6, [52A] 4 | c9 | c9 | [52] 4.6, [52A] 4 | c10 | c10 |
| 12F | P-Preferred-Identity | [34] 9.2 | x | x | [34] 9.2 | c4 | n/a |
| 12G | Privacy | [33] 4.2 | c7 | c7 | [33] 4.2 | c8 | c8 |
| 12H | Relayed-Charge | 7.2.12 | n/a | c22 | 7.2.12 | n/a | c22 |
| 12I | Reply-To | [26] 20.31 | m | m | [26] 20.31 | i | i |
| 12J | Require | [26] 20.32 | m | m | [26] 20.32 | c15 | c15 |
| 13 | Server | [26] 20.35 | m | m | [26] 20.35 | i | i |
| 13AA | Service-Interact-Info | Subclause 7.2.14 | n/a | c25 | Subclause 7.2.14 | n/a | c25 |
| 13A | Session-ID | [162] | c21 | c21 | [162] | c21 | c21 |
| 14 | Timestamp | [26] 20.38 | i | i | [26] 20.38 | i | i |
| 15 | To | [26] 20.39 | m | m | [26] 20.39 | m | m |
| 16 | User-Agent | [26] 20.41 | m | m | [26] 20.41 | i | i |
| 17 | Via | [26] 20.42 | m | m | [26] 20.42 | m | m |
| 18 | Warning | [26] 20.43 | m | m | [26] 20.43 | i | i |
| c1: IF A.162/9 THEN m ELSE i - - insertion of date in requests and responses. c2: IF A.162/19A OR A.162/19B THEN m ELSE i - - reading, adding or concatenating the Organization header. c3: IF A.162/19C OR A.162/19D THEN m ELSE i - - reading, adding or concatenating the Call-Info header. c4: IF A.162/30A THEN m ELSE n/a - - act as first entity within the trust domain for asserted identity. c5: IF A.162/30 THEN m ELSE n/a - - extensions to the Session Initiation Protocol (SIP) for asserted identity within trusted networks. c6: IF A.162/30A or A.162/30B THEN m ELSE i - - extensions to the Session Initiation Protocol (SIP) for asserted identity within trusted networks or subsequent entity within trust network that can route outside the trust network. c7: IF A.162/31 THEN m ELSE n/a - - a privacy mechanism for the Session Initiation Protocol (SIP). c8: IF A.162/31D OR A.162/31G THEN m ELSE IF A.162/31C THEN i ELSE n/a - - application of the privacy option "header" or application of the privacy option "id" or passing on of the Privacy header transparently. c9: IF A.162/45 THEN m ELSE n/a - - the P-Charging-Vector header extension. c10: IF A.162/46 THEN m ELSE IF A.162/45 THEN i ELSE n/a - - adding, deleting, reading or modifying the P-Charging-Vector header before proxying the request or response or the P-Charging-Vector header extension. c11: IF A.162/44 THEN m ELSE n/a - - the P-Charging-Function-Addresses header extension. c12: IF A.162/44A THEN m ELSE IF A.162/44 THEN i ELSE n/a - - adding, deleting or reading the P-Charging-Function-Addresses header before proxying the request or response, or the P-Charging-Function-Addresses header extension. c13: IF A.162/43 THEN x ELSE IF A.162/41 THEN m ELSE n/a - - act as subsequent entity within trust network for access network information that can route outside the trust network, the P-Access-Network-Info header extension. c14: IF A.162/43 THEN m ELSE IF A.162/41 THEN i ELSE n/a - - act as subsequent entity within trust network for access network information that can route outside the trust network, the P-Access-Network-Info header extension. c15: IF A.162/11 OR A.162/13 THEN m ELSE i - - reading the contents of the Require header before proxying the request or response or adding or modifying the contents of the Require header before proxying the request or response for methods other than REGISTER. c16: IF A.162/57 THEN m ELSE n/a - - an extension to the session initiation protocol for request history information. c17: IF A.162/70 THEN m ELSE n/a - - SIP location conveyance. c18: IF A.162/70A THEN m ELSE IF A.162/70B THEN i ELSE n/a - - addition or modification of location in a SIP method, passes on locations in SIP method without modification. c21: IF A.162/101 THEN m ELSE n/a - - the Session-ID header. c22: IF A.162/121 THEN m ELSE n/a - - the Relayed-Charge header field extension. c23: IF A.162/43 THEN x ELSE IF A.162/123 THEN m ELSE n/a - - act as subsequent entity within trust network for access network information that can route outside the trust network, the Cellular-Network-Info header extension. c24: IF A.162/43 THEN m ELSE IF A.162/123 THEN i ELSE n/a - - act as subsequent entity within trust network for access network information that can route outside the trust network, the Cellular-Network-Info header extension. c25: IF A.162/130 THEN m ELSE n/a - - Dynamic services interactions. |  |  |  |  |  |  |  |

Prerequisite A.163/9B - - MESSAGE response

Prerequisite: A.164/102 - - Additional for 2xx response

Table A.218D: Supported header fields within the MESSAGE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 0A | Accept-Resource-Priority | [116] 3.2 | c4 | c4 | [116] 3.2 | c4 | c4 |
| 1 | Allow-Events | [28] 8.2.2 | m | m | [28] 8.2.2 | c1 | c1 |
| 2 | Authentication-Info | [26] 20.6 | m | m | [26] 20.6 | i | i |
| 3 | Feature-Caps | [190] | c6 | c6 | [190] | c6 | c6 |
| 3A | Reason | [294] | o | c7 | [294] | o | c7 |
| 4 | P-Visited-Network-ID | [52B] 3 | o | o | [52B] 3 | o | o |
| 6 | Supported | [26] 20.37 | m | m | [26] 20.37 | i | i |
| c1: IF A.4/20 THEN m ELSE i - - SIP specific event notification extension. c4: IF A.162/80A THEN m ELSE n/a - - inclusion of MESSAGE, SUBSCRIBE, NOTIFY in communications resource priority for the session initiation protocol. c6: IF A.162/110 THEN m ELSE n/a - - indication of features supported by proxy. c7: IF A.162/48C THEN o ELSE n/a - - use of the Reason header field in Session Initiation Protocol (SIP) responses (carrying STIR codes). |  |  |  |  |  |  |  |

Prerequisite A.163/9B - - MESSAGE response

Prerequisite: A.164/103 OR A.164/104 OR A.164/105 OR A.164/106 - - Additional for 3xx – 6xx response

Table A.218DA: Supported header fields within the MESSAGE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 1 | Error-Info | [26] 20.18 | m | m | [26] 20.18 | i | i |
| 1A | Reason | [294] | o | c2 | [294] | o | c2 |
| 2 | Response-Source | 7.2.17 | n/a | c1 | 7.2.17 | n/a | c1 |
| c1: IF A.162/125 THEN o ELSE n/a - - use of the Response-Source header field in SIP error responses. c2: IF A.4/38C THEN o ELSE n/a - - use of the Reason header field in Session Initiation Protocol (SIP) responses (carrying STIR codes). |  |  |  |  |  |  |  |

Prerequisite A.163/9B - - MESSAGE response

Prerequisite: A.164/103 OR A.164/35 - - Additional for 3xx or 485 (Ambiguous) response

Table A.218E: Supported header fields within the MESSAGE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 2 | Contact | [26] 20.10 | m | m | [26] 20.10 | c1 | c1 |
| c1: IF A.162/19E THEN m ELSE i - - deleting Contact headers. |  |  |  |  |  |  |  |

Prerequisite A.163/9B - - MESSAGE response

Prerequisite: A.164/14 - - Additional for 401 (Unauthorized) response

Table A.218F: Supported header fields within the MESSAGE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 3 | Proxy-Authenticate | [26] 20.27 | m | m | [26] 20.27 | m | m |
| 6 | WWW-Authenticate | [26] 20.44 | m | m | [26] 20.44 | i | i |

Prerequisite A.163/9B - - MESSAGE response

Prerequisite: A.164/17 OR A.164/23 OR A.164/30 OR A.164/36 OR A.164/42 OR A.164/45 OR A.164/50 OR A.164/51 - - Additional for 404 (Not Found), 413 (Request Entity Too Large), 480(Temporarily not available), 486 (Busy Here), 500 (Internal Server Error), 503 (Service Unavailable), 600 (Busy Everywhere), 603 (Decline) response

Table A.218G: Supported header fields within the MESSAGE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 4 | Retry-After | [26] 20.33 | m | m | [26] 20.33 | i | i |

Table A.218H: Void

Prerequisite A.163/9B - - MESSAGE response

Prerequisite: A.164/20 - - Additional for 407 (Proxy Authentication Required) response

Table A.218I: Supported header fields within the MESSAGE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 3 | Proxy-Authenticate | [26] 20.27 | m | m | [26] 20.27 | m | m |
| 6 | WWW-Authenticate | [26] 20.44 | m | m | [26] 20.44 | i | i |

Table 218IA: Void

Prerequisite A.163/9B - - MESSAGE response

Prerequisite: A.164/25 - - Additional for 415 (Unsupported Media Type)

Table A.218J: Supported header fields within the MESSAGE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 1 | Accept | [26] 20.1 | m | m | [26] 20.1 | i | i |
| 2 | Accept-Encoding | [26] 20.2 | m | m | [26] 20.2 | i | i |
| 3 | Accept-Language | [26] 20.3 | m | m | [26] 20.3 | i | i |

Prerequisite A.163/9B - - MESSAGE response

Prerequisite: A.164/26A - - Additional for 417 (Unknown Resource-Priority) response

Table A.218JA: Supported header fields within the MESSAGE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 1 | Accept-Resource-Priority | [116] 3.2 | c1 | c1 | [116] 3.2 | c1 | c1 |
| c1: IF A.162/80A THEN m ELSE n/a - - inclusion of MESSAGE, SUBSCRIBE, NOTIFY in communications resource priority for the session initiation protocol. |  |  |  |  |  |  |  |

Prerequisite A.163/9B - - MESSAGE response

Prerequisite: A.164/27 - - Additional for 420 (Bad Extension) response

Table A.218K: Supported header fields within the MESSAGE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 5 | Unsupported | [26] 20.40 | m | m | [26] 20.40 | c3 | c3 |
| c3: IF A.162/18 THEN m ELSE i - - reading the contents of the Unsupported header before proxying the 420 response to a method other than REGISTER. |  |  |  |  |  |  |  |

Prerequisite A.163/9B - - MESSAGE response

Prerequisite: A.164/28 OR A.164/41A - - Additional for 421 (Extension Required), 494 (Security Agreement Required) response

Table A.218L: Supported header fields within the MESSAGE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 3 | Security-Server | [48] 2 | c1 | c1 | [48] 2 | n/a | n/a |
| c1: IF A.162/47 THEN m ELSE n/a - - security mechanism agreement for the session initiation protocol. |  |  |  |  |  |  |  |

Table A.218M: Void

Prerequisite A.163/9B - - MESSAGE response

Prerequisite: A.164/29H - - Additional for 470 (Consent Needed) response

Table A.218MA: Supported header fields within the MESSAGE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 1 | Permission-Missing | [125] 5.9.3 | m | m | [125] 5.9.3 | m | m |

Prerequisite A.163/9B - - MESSAGE response

Prerequisite: A.164/46 - - Additional for 504 (Server Time-out) response

Table A.218MB: Supported header fields within the MESSAGE response

| Item | Header field | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 1 | Restoration-Info | subclause 7.2.11 | n/a | c1 | subclause 7.2.11 | n/a | n/a |
| c1: IF A.4/110 THEN o ELSE n/a - - HSS based P-CSCF restoration. |  |  |  |  |  |  |  |

Prerequisite A.163/9B - - MESSAGE response

Table A.218N: Supported message bodies within the MESSAGE response

| Item | Header | Sending |  |  | Receiving |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. | RFC status | Profile status | Ref. | RFC status | Profile status |
| 1 |  |  |  |  |  |  |  |

# *** End of Changes ***