3GPP TSG-CT WG1 Meeting #157	C1-256228

Sophia Antipolis, France, 13-17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.501 | CR | 7024 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Additional information IE setting for UL UPP-CMI container |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GProtoc19, 5G_eLCS_Ph3 |  |  |  |  |  | Date: |  |  | 2025-10-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In TS 24.501 clause 5.4.5.2.2 for UE-initiated NAS transport procedure to transport a UPP-CMI container, it specifies that “In case j in subclause 5.4.5.2.1, the UE shall: - set the Payload container type IE to "UPP-CMI container"; - set the Payload container IE to the UPP-CMI container; and - set the Additional information IE to the routing information, if provided by AMF in a previous procedure or provided by the upper layer location services application.” Based on TS 24.572, the following UL NAS messages can be sent in UPP-CMI container during UE-initiated NAS transport procedure: - USER PLANE CONNECTION ESTABLISHMENT COMPLETE message; - USER PLANE CONNECTION RELEASE COMPLETE message; - USER PLANE CONNECTION ESTABLISHMENT REQUEST message; and - USER PLANE CONNECTION RELEASE REQUEST message. For case 1), 2) and 4), the UE sets the Additional information IE as provided by AMF in previous message. For case 3) during UE requested user plane connection establishment procedure, as specified in TS 23.273 clause 6.18.2 step 2: “[Conditional] AMF selects an LMF which is capable to establish a user plane session for positioning with the UE if the UE is authorized based on UE Subscription to use the user plane positioning and either has no existing user plane connection context with another LMF or has indicated in the 5GMM capabilities the support of multiple LCS-UPP connections. Otherwise, the AMF rejects the request. AMF may either query the NRF or based on local configuration to discover and select a proper LMF.” The highlight indicates the condition for the AMF to perform LMF selection for UE initiated user plane connection establishment, and stage 2 requirement does not mention that UE provides LMF information to AMF. And when the condition is not met, the AMF rejects the request. So according to stage 2, for case 3), the UE doesn’t provide LMF information and the AMF performs LMF selection if conditons are met. Also in TS 24.572 clause 6.2.2.1.1, for case 3), it also does not mention that UE provides LMF information to AMF. So the condition actually does not refer to any valid case and it should be removed to avoid confusion. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For UE-initiated NAS transport procedure to transport a UPP-CMI container, the UE shall set the Additional information IE to the routing information, if provided by AMF in a previous procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | An invalid condition for UE to include Additional information IE during UE-initiated NAS transport procedure to transport a UPP-CMI container remains in the spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.5.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

5.4.5.2.2	UE-initiated NAS transport procedure initiation

In the connected mode, the UE initiates the NAS transport procedure by sending the UL NAS TRANSPORT message to the AMF, as shown in figure 5.4.5.2.2.1.

In case a in subclause 5.4.5.2.1, the UE shall:

-	include the PDU session information (PDU session ID, old PDU session ID, S-NSSAI, mapped S-NSSAI (if provided by the network), DNN, request type, alternative S-NSSAI, MA PDU session information, non-3GPP access path switching indication), if available;

-	set the Payload container type IE to "N1 SM information"; and

-	set the Payload container IE to the 5GSM message.

The UE shall set the PDU session ID IE to the PDU session ID. If an old PDU session ID is to be included, the UE shall set the Old PDU session ID IE to the old PDU session ID.

If an alternative S-NSSAI is to be included, the UE shall set the Alternative S-NSSAI IE to the alternative S-NSSAI and shall set the S-NSSAI IE to the S-NSSAI to be replaced.

If an S-NSSAI is to be included, the UE shall set the S-NSSAI IE to the S-NSSAI selected for the PDU session from the allowed NSSAI for the current PLMN or SNPN, associated with the mapped S-NSSAI (if provided by the network).

If a DNN is to be included, the UE shall set the DNN IE to the DNN. 5GSM procedures specified in clause 6 describe conditions for inclusion of the S-NSSAI, mapped S-NSSAI (if provided by the network), and the DNN.

If a request type is to be included, the UE shall set the Request type IE to the request type. The request type is not provided along 5GSM messages other than the PDU SESSION ESTABLISHMENT REQUEST message and the PDU SESSION MODIFICATION REQUEST message.

If an MA PDU session information is to be included, the UE shall set the MA PDU session information IE to the MA PDU session information. The MA PDU session information is not provided along 5GSM messages other than the PDU SESSION ESTABLISHMENT REQUEST message and the PDU SESSION MODIFICATION REQUEST message as specified in 3GPP TS 24.193 [13B].

If the UE supports the non-3GPP access path switching for the PDU session and the AMF has indicated its support for the non-3GPP access path switching, the UE shall include the Non-3GPP access path switching indication information element and set the NAPS bit to "non-3GPP access path switching supported". The non-3GPP access path switching indication is not provided along 5GSM messages other than the PDU SESSION ESTABLISHMENT REQUEST message.

In case b in subclause 5.4.5.2.1, the UE shall:

-	set the Payload container type IE to "SMS"; and

-	set the Payload container IE to the SMS payload.

Based on the UE preferences regarding access selection for mobile originated (MO) transmission of SMS over NAS as described in 3GPP TS 23.501 [8]:

-	when SMS over NAS is preferred to be sent over 3GPP access: the UE attempts to deliver MO SMS over NAS via the 3GPP access if the UE is registered over both 3GPP access and non-3GPP access. If the delivery of SMS over NAS via the 3GPP access is not available, the UE attempts to deliver MO SMS over NAS via the non-3GPP access; and

-	when SMS over NAS is preferred to be sent over non-3GPP access: the UE attempts to deliver MO SMS over NAS via the non-3GPP access if the UE is registered over both 3GPP access and non-3GPP access. If the delivery of SMS over NAS via the non-3GPP access is not available, the UE attempts to deliver MO SMS over NAS via the 3GPP access.

In case c in subclause 5.4.5.2.1, the UE shall:

-	set the Payload container type IE to "LTE Positioning Protocol (LPP) message container";

-	set the Payload container IE to the LPP message payload; and

-	set the Additional information IE to the routing information provided by the upper layer location services application.

In case c1 in subclause 5.4.5.2.1, the UE shall:

-	set the Payload container type IE to "SLPP message container";

-	set the Payload container IE to the SLPP message payload; and

-	set the Additional information IE to the routing information provided by the upper layer location services application.

In case d in subclause 5.4.5.2.1, the UE:

-	shall set the Payload container type IE to "SOR transparent container"; and

-	shall set the Payload container IE to the UE acknowledgement due to successful reception of steering of roaming information, and;

i)	shall set the ME support of SOR-CMCI indicator to "SOR-CMCI supported by the ME" ;

ii)	shall set the ME support of SOR-SNPN-SI indicator to "SOR-SNPN-SI supported by the ME" if the UE supports access to an SNPN using credentials from a credentials holder and the UE is operating in SNPN access operation mode or may set the ME support of SOR-SNPN-SI indicator to "SOR-SNPN-SI supported by the ME" if the UE supports access to an SNPN using credentials from a credentials holder and the UE is not operating in SNPN access operation mode; and

iii)	shall set the ME support of SOR-SNPN-SI-LS indicator to "SOR-SNPN-SI-LS supported by the ME" if the UE supports access to an SNPN providing access for localized services in SNPN,

in the Payload container IE carrying the acknowledgement (see 3GPP TS 23.122 [5]).

In case e in subclause 5.4.5.2.1, the UE shall:

-	set the Payload container type IE to "UE policy container"; and

-	set the contents of the Payload container IE as specified in annex D.

In case f in subclause 5.4.5.2.1, the UE shall:

-	set the Payload container type IE to "UE parameters update transparent container"; and

-	set the contents of the Payload container IE to the UE acknowledgement due to successful reception of UE parameters update data (see 3GPP TS 23.502 [9]). If the UE supports UE parameters update header protection, the UE shall set the UHP bit to "UE parameters update header protection supported" in the UE parameters update header.

In case g in subclause 5.4.5.2.1, the UE shall:

-	set the Payload container type IE to "Location services (LCS) message container";

-	set the Payload container IE to the LCS message payload;

-	set the Additional information IE to the routing information, if preconfigured or provided by AMF in a previous procedure or provided by the upper layer location services application; and

-	include the Payload container information IE with the PRU bit set to "Payload container related to PRU" if the location services message payload is related to PRU (see 3GPP TS 24.080 [13A]).

NOTE:	The AMF may provide the routing information to the UE during the PRU association procedure or the PRU disassociation procedure as specified in 3GPP TS 23.273 [6B].

In case h in subclause 5.4.5.2.1:

-	if the UE is not located outside the LADN service area, the UE shall:

-	include the PDU session ID, and Release assistance indication (if available);

-	set the Payload container type IE to "CIoT user data container"; and

-	set the Payload container IE to the user data container; or

-	if the UE is located outside the LADN service area, the UE shall not perform the UE-initiated NAS transport procedure to send CIoT user data via the control plane for a PDU session for LADN.

In case i in subclause 5.4.5.2.1, the UE shall:

-	set the Payload container type IE to "Service-level-AA container"; and

-	set the Payload container IE to the Service-level-AA container.

In case j in subclause 5.4.5.2.1, the UE shall:

-	set the Payload container type IE to "UPP-CMI container";

-	set the Payload container IE to the UPP-CMI container; and

-	set the Additional information IE to the routing information, if provided by AMF in a previous procedure.

In case k in subclause 5.4.5.2.1, the UE shall:

-	set the Payload container type IE to "Multiple payloads"; and

-	set each payload container entry of the Payload container IE (see subclause 9.11.3.39), as follows:

i)	set the payload container type field of the payload container entry to a payload container type value set in the Payload container type IE as specified in cases a) to j) above;

ii)	set the payload container entry contents field of the payload container entry to the payload container contents set in the Payload container IE as specified in cases a) to j) above, and

iii)	set the optional IE fields, if any, to the optional associated payload routing information as specified in cases a to j above.

Figure 5.4.5.2.2.1: UE-initiated NAS transport procedure

* * * End of Change * * * *