3GPP TSG-CT WG1 Meeting #157	C1-256240

Sophia-Antipolis, France 13-17 October 2025	was C1-xxxxxx

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.554 | CR | 0847 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Reslove the EN related to U2U path information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph3 |  |  |  |  |  | Date: |  |  | 2025-10-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There is an EN in TS 24.554: “Editor's note (5G_ProSe_Ph3, CR#0704): Whether the path info field also includes source layer-2 ID of discoverer Prose end UE (specified in 5.8.6.3 but not specified in 6.3.2.6.3.1 of TS 23.304) is FFS.” According to CR 0558, the source layer-2 ID of discoverer ProSe End UE is removed from path information. So the EN needs to be removed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the related EN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EN related to U2U path info is in the spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.2.24 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

11.2.24	List of multi-hop UE-to-UE relay paths

The purpose of the list of multi-hop UE-to-UE relay paths information element is to carry the direct discovery set as specified in 3GPP TS 33.503 [34].

The list of multi-hop UE-to-UE relay paths information element is a type 6 information element.

The list of multi-hop UE-to-UE relay paths information element is coded as shown in figure 11.2.24.1 and table 11.2.24.1.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| List of multi-hop UE-to-UE relay paths IEI |  |  |  |  |  |  |  | octet 1 |
| Length of list of multi-hop UE-to-UE relay paths contents |  |  |  |  |  |  |  | octet 2 octet 3 |
| Path info #1 |  |  |  |  |  |  |  | octet 4 octet a |
| Path info #2 |  |  |  |  |  |  |  | octet (a+1)* octet b |
| …… |  |  |  |  |  |  |  | octet (b+1)* octet c |
| Path info #n |  |  |  |  |  |  |  | octet (c+1)* octet d |

Figure 11.2.24.1: List of multi-hop UE-to-UE relay paths information element

Table 11.2.24.1: List of multi-hop UE-to-UE relay paths information element

Path info (octet 3 to a) The path info is coded as in figure 11.2.24.2 and in table 11.2.24.2. Each path info represents the path information to one or more 5G ProSe multi-hop layer-3 end UE(s).

| 8 | 7 | 6 | 5 |  | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hop count |  |  |  | Spare |  |  |  | HL | octet 4 |
| Hop limit |  |  |  |  |  |  |  |  | octet 5* |
| Length of the list of protected user infos |  |  |  |  |  |  |  |  | octet 6 octet 7 |
| List of protected user infos |  |  |  |  |  |  |  |  | octet 8* octet m* |
| Length of the path of relays |  |  |  |  |  |  |  |  | octet m+1* octet m+2* |
| Path of relays |  |  |  |  |  |  |  |  | octet m+3* octet n* |

Figure 11.2.24.2: Path info

Table 11.2.24.2: Path info

| HL (bit 1 of octet 4) |  |
| --- | --- |
| Bit 1 |  |
| 0 | Hop limit field is not present |
| 1 | Hop limit field is present |
| Hop count (bit 5 to 8 of octet 4) The hop count field indicates the number of hops that the message is already relayed. |  |
| List of protected user infos (octet 8 to m) The list of protected user infos field contains the protected user info(s) of the 5G ProSe multi-hop layer-3 end UE(s) that can be reached via the path of relays as indicated in "Path of relays" field (octet m+1 to n) and is coded as in figure 11.2.17.1 starting from octet 4. |  |
| Path of relays (octet m+3 to n) The path of relays field indicates is a list of intermediate 5G ProSe multi-hop layer-3 UE-to-UE relays that need to be traversed to reach the 5G ProSe multi-hop layer-3 end UE(s) as indicated in list of protected user infos field (octet 8 to m) and is coded as in figure 11.3.47.1 starting from octet 3. The order of the list starts from the current 5G ProSe multi-hop layer-3 UE-to-UE relay and ends with the last hop before reaching the 5G ProSe multi-hop layer-3 end UE(s). |  |

* * * End of Changes * * * *