3GPP TSG-CT WG1 Meeting #157                       C1-256245

Sophia-Antipolis, France 13-17 October 2025

Source:	CATT

Title:	Discussion on 6G NAS functions related to UE in idle mode

Agenda item:	20.2

Document for:	DISCUSSION

1. Background

A new Rel-20 SID on 6G System –Study on 6G Use Cases and Service Requirements (FS_6G_REQ) in SP-241391 has been approved. The evolution to 6G, as outlined in 3GPP TR 22.870, 6G will introduce new services and capabilities, such as artificial intelligence, sensing, and computing, which require enhancement of the existing mechanism in to efficiently manage evolving technical demands and service requirements. TSG RAN has also initiated the FS_6G_RAN_Scen_Req study item to develop requirements for 6G Radio.

The existing mechanisms for network selection, access technology prioritization, and access control, while robust in 5G, may not be sufficient to handle the dynamic, multi-dimensional, and intent-driven requirements of 6G use cases. This paper is proposed to start the 6G NAS functions study related to UE in idle mode, including 6G network selection and 6G access control.

2. Discussion

2.1 6G network selection

According to 3GPP TR 22.870, in order to meet the new requirements and new services of 6G network, it is suggested to carry out the research on network selection for 6GR, multi-orbit satellite access types and network slice.

2.1.1 6GR

The 6G system will support existing 3GPP access technologies, including one or more NR and E-UTRA as well as non-3GPP access technologies and new access technologies, e.g., 6G RAT. Therefore, the network selection mechanism needs to be studied how to support 6GR.

According to the discussion from RAN, 6G will have a new RAT (6GR), so it is proposed to:

Proposal 1. Study how to support 6GR during PLMN selection.

Proposal 2. Study how to support the network selection procedures, messages and corresponding network and UE behaviour for 6GR.

2.1.2 Multi-orbit satellite access types

As identified in the SA1 study and service requirements on a 5G system with satellite access supporting multiple satellite orbit types (see TR 22.887 and TS 22.261), multi-orbit expands the existing satellite access scenarios. In the discussion on the satellite topic in 6G SID NWM, 16 companies support enhancements for multi-orbit NTN or service continuity between NTN and NTN.

SA1 has agreed the new requirement in TS 22.261 clause 6.46.12:

“Subject to regulatory requirements and operator’s policy, a 5G system with satellite access supporting multiple satellite orbit types with different characteristics (e.g., altitude, orbital characteristics, satellite capabilities) shall be able to assist a UE (e.g., provide the UE with the necessary information) to acquire and access the suitable satellite among those available.”

“NOTE 1:	The information provided to the UE can include satellites information operated by roaming partners.”

“Subject to regulatory requirements and operator policies, the 5G network with satellite access supporting multiple satellite orbit types with different characteristics (e.g., altitude, orbital characteristics, satellite capabilities) shall support the selection of suitable satellites for the UE…”

The current mechanism of PLMN selection limits the UEs to only use "satellite NG-RAN" as access technology employed by the network operators, which is applicable for all of the satellite access scenarios. This mechanism does not support the utilization of multiple satellite orbit types (such as GEO, MEO, LEO) to assist a UE to access the suitable satellite.

Observation 1: It’s necessary for UE to know the information about the multiple satellite access types during the PLMN selection procedures.

Observation 2: The information of multiple satellite  access types can include roaming partners’ information.

As shown in the following figure 1, when UE can receive signals from multiple satellite orbits simultaneously, the current mechanism is to select based on signal strength and cannot choose satellite access based on different orbit types.

Figure 1: UE receiving signals from multiple satellite orbits simultaneously

In TS 23.122, the access technologies related with satellite access only include satellite NG-RAN and satellite E-UTRAN:

“Access Technology: The access technology associated with a PLMN or SNPN. The MS uses this information to determine what type(s) of radio carrier to search for when attempting to select a specific PLMN or SNPN (e.g., GSM, UTRAN, GSM COMPACT, E-UTRAN, NG-RAN, satellite NG-RAN or satellite E-UTRAN). A PLMN may support more than one access technology. SNPNs only support NG-RAN.”

“The ME shall utilise all the information stored in the SIM related to the PLMN selection; e.g. "HPLMN Selector with Access Technology", "User Controlled PLMN Selector with Access Technology", "Forbidden PLMNs", "Equivalent HPLMN", see 3GPP TS 31.102 [40]. The ME shall also utilise the extension of the "forbidden PLMNs" list that it has stored locally on the ME if available.

The ME shall either utilise the "Operator controlled PLMN Selector with Access Technology" that it has stored locally on the ME, or the Operator controlled PLMN Selector with Access Technology" stored in the SIM, for the purposes of PLMN selection.

The "HPLMN Selector with Access Technology", "User Controlled PLMN Selector with Access Technology" and "Operator Controlled PLMN Selector with Access Technology" data files in the SIM include associated access technologies for each PLMN entry, see 3GPP TS 31.102 [40]. The PLMN/access technology combinations are listed in priority order. If an entry indicates more than one access technology, then no priority is defined for the access technologies within this entry and the priority applied to each access technology within this entry is an implementation issue. If no particular access technology is indicated in an entry, it shall be assumed that all access technologies supported by the ME apply to the entry. If an entry only indicates access technologies not supported by the ME, the entry shall be ignored. If an entry indicates at least one access technology supported by the ME, the entry shall be used in the PLMN selection procedures if the other criteria defined for the specific PLMN selection procedures are fulfilled.“

Observation 3: The NAS layer can only indicate” satellite NG-RAN” and “satellite E-UTRAN” and cannot further indicate the information of orbit type (e.g. GEO, MEO, LEO).

Observation 4: The NAS layer indicates PLMN/access technology combinations in order and the same PLMN entry cannot be prioritized in the order of access technology.

According to clause 4.2 of TS 38.304, it explains how to use NAS layer information:

Table 4.2-1: Functional division between AS and NAS in RRC_IDLE state and RRC_INACTIVE state

| RRC_IDLE and RRC_INACTIVE state Process | UE Non-Access Stratum | UE Access Stratum |
| --- | --- | --- |
| PLMN Selection and SNPN Selection | For a UE not operating in SNPN access mode, perform the following: Maintain a list of PLMNs in priority order according to TS 23.122 [9]. Select a PLMN using automatic or manual mode as specified in TS 23.122 [9] and request AS to select a cell belonging to this PLMN. For each PLMN, associated RAT(s) may be set. Evaluate reports of available PLMNs and any associated CAG-IDs from AS for PLMN selection. Maintain a list of equivalent PLMN identities. Maintain a list of "PLMNs not allowed to operate at the present UE location". Maintain applicable disaster roaming information for available PLMNs including potential disaster PLMNs for available PLMNs. … | For a UE not operating in SNPN access mode, search for available PLMNs. If associated RAT(s) is (are) set for the PLMN, search in this (these) RAT(s) and other RAT(s) for that PLMN as specified in TS 23.122 [9]. For a UE operating in SNPN access mode, search for available SNPNs only consider NR cells. Perform measurements to support PLMN/SNPN selection. Synchronise to a broadcast channel to identify found PLMNs/SNPNs. Report available PLMNs and any associated CAG-IDs with associated RAT(s) to NAS on request from NAS or autonomously. Report applicable disaster roaming information for available PLMNs autonomously including potential disaster PLMNs… |
| Cell Selection | Control cell selection for example by indicating RAT(s) associated with the selected PLMN to be used initially in the search of a cell in the cell selection. Maintain a list of "Forbidden Tracking Areas" and provide the list to AS. Maintain a list of "PLMNs not allowed to operate at the present UE location" and provide the list to AS. For a UE not operating in SNPN access mode: Maintain Allowed CAG list and optional CAG-only indication along with associated PLMN ID(s) on which the UE is allowed access and provide these lists to AS. To support manual CAG selection, select a CAG and request AS to select a cell belonging to this CAG. | Perform measurements needed to support cell selection. Detect and synchronise to a broadcast channel. Receive and handle broadcast information. Forward NAS system information to NAS. Search for a suitable cell. The cells broadcast one or more 'PLMN identity' or 'SNPN identity' (for a UE operating in SNPN access mode) in the system information. Respond to NAS whether such cell is found or not. If associated RATs is (are) set for the PLMN, perform the search in this (these) RAT(s) and other RATs for that PLMN as specified in TS 23.122 [9]. If a cell is found which satisfies cell selection criteria, camp on that cell. |

Observation 5: If the NAS layer provides the selected PLMN ID and associated RAT(s) to the AS layer, AS layer will take this (these) RAT as guidance.

Note: 4G and 5G have similar mechanisms as mentioned above.

According to TS 38.331, SIB 19 information including orbital parameters (i.e., orbital-r17) in NTN-Config IE can indicate the multiple satellite access types to the UE:

SIB19 information element

-- ASN1START

-- TAG-SIB19-START

SIB19-r17 ::= SEQUENCE {

ntn-Config-r17                           NTN-Config-r17                                  OPTIONAL,       -- Need R

t-Service-r17                            INTEGER (0..549755813887)                       OPTIONAL,       -- Need R

referenceLocation-r17                    ReferenceLocation-r17                           OPTIONAL,       -- Need R

distanceThresh-r17                       INTEGER(0..65525)                               OPTIONAL,       -- Need R

ntn-NeighCellConfigList-r17              NTN-NeighCellConfigList-r17                     OPTIONAL,       -- Need R

lateNonCriticalExtension                 OCTET STRING                                    OPTIONAL,

...,

[[

ntn-NeighCellConfigListExt-v1720         NTN-NeighCellConfigList-r17                     OPTIONAL        -- Need R

]],

[[

movingReferenceLocation-r18              ReferenceLocation-r17                           OPTIONAL,       -- Need R

ntnCovEnh-r18                            NTN-CovEnh-r18                                  OPTIONAL,       -- Need R

satSwitchWithReSync-r18                  SatSwitchWithReSync-r18                         OPTIONAL        -- Need R

]]

}

According to TS 36.331, SIB 31 information including orbital parameters (i.e., orbitalParameters) in servingSatelliteInfo-r17 IE can indicate the multiple satellite access types to the UE:

SystemInformationBlockType31-r17 ::= SEQUENCE {

servingSatelliteInfo-r17		ServingSatelliteInfo-r17,

lateNonCriticalExtension		OCTET STRING					OPTIONAL,

...,

[[	servingSatelliteInfo-v1820	ServingSatelliteInfo-v1820		OPTIONAL	-- Need OR

]]

}

Observation 6: AS layer can know the orbit type based on SIB19 (5G) or SIB 31 (4G).

Based on the above analysis, the existing PLMN selection mechanism is unable to indicate the AS layer to select the satellites access based on different orbit types, it is proposed to:

Proposal 3. Study how to support and differentiate multiple satellite access types (i.e., GEO, MEO, LEO) for 6GS during PLMN selection procedure.

The mechanism for multiple satellite access types should be applied to 5GS.

2.1.3 Network slicing

3GPP has introduced network slicing as a mandatory feature for 5GS and will continue to support the network slicing in 6G system as described in clause 5.7.5 of 3GPP TR 22.870:

“[PR 5.7.5.2-1] The 6G system should support potential enhancement of network slicing, e.g.:

-	Create slices quickly without much overhead/complexity by leveraging automated operations

-	Scale and manage the network slices efficiently

-	Improve the mechanism to select and access network slice(s)

Editor’s Note: this requirement is FFS.”

The stage-1 service requirements has been provided for EASNS (Enhanced Access to and Support of Network Slice) work item in Rel-18, and reflected in clause 6.1.2.1 of TS 22.261:

For a roaming UE activating a service/application requiring a network slice not offered by the serving network but available in the area from other network(s), the HPLMN shall be able to provide the UE with prioritization information of the VPLMNs with which the UE may register for the network slice.

CT1 has discussed how to achieve this requirement in several meetings and sent LS (C1-236565) to SA1 to ask related questions. Based on SA1 reply LS (C1-239237) indicating that SA1 removed the requirement on slice-based PLMN selection from TS 22.261 for Rel-18, CT1 stopped the corresponding work at CT1#145.

Some companies have analysed the requirement for service-based and slice-based network selection in Rel-18, such as wearable or IoT devices to do registration for specific service(s) to the network and also have proposed the related solutions (e.g. including network slicing info in the PLMN selection procedure and others). Some companies consider that Slice-based PLMN Selection can be prioritized over legacy PLMN selection.

Therefore, it is proposed to investigate and study whether and how to take network slicing availability info into account for VPLMN selection in 6GS at the initial stage of 6G R20.

Observation 7: The current network selection mechanism does not take network slices information into account.

Proposal 4. Study whether and how to take network slicing availability info into account for VPLMN selection in 6GS.

2.2 6G access control

As described in clause 5.7.6 of 3GPP TR 22.870, “the UAC shall be enhanced to address the new services (such as sensing, AI application, computing), enabling network operators to control and manage the network in congestion scenarios” and has the potential new requirements:

“[PR 5.7.6.2-1] The 6G system shall support Unified Access Control requirements as specified in [14].

[PR 5.7.6.2-2] The 6G system shall support suitable access categories to manage the access attempt for the new services (such as sensing, AI application, computing) in congestion scenarios.”

The existing access control mechanism is defined in clause 4.5 of 3GPP TS 24.501 and 5GS shall be able to prevent UEs from accessing the network using relevant barring parameters that vary depending on Access Identity and Access Category as defined in clause 6.22 of 3GPP TS 22.261, which does not include the new services (such as sensing, AI application, computing).

Observation 8: Access control mechanism in 5GS does not meet the requirements for 6G new services (such as sensing, AI application, computing).

Proposal 5. Study whether and how to support access control to manage the access attempt for the new services (such as sensing, AI application, computing) in congestion conditions.

3. Conclusion

In the above the following observation were made:

Observation 1: It’s necessary for UE to know the information about the multiple satellite access types during the PLMN selection procedures.

Observation 2: The information of multiple satellite  access types can include roaming partners’ information.

Observation 3: The NAS layer can only indicate” satellite NG-RAN” and “satellite E-UTRAN” and cannot further indicate the information of orbit type (e.g. GEO, MEO, LEO).

Observation 4: The NAS layer indicates PLMN/access technology combinations in order and the same PLMN entry cannot be prioritized in the order of access technology.

Observation 5: If the NAS layer provides the selected PLMN ID and associated RAT(s) to the AS layer, AS layer will take this (these) RAT as guidance.

Observation 6: AS layer can know the orbit type based on SIB19 (5G) or SIB 31 (4G).

Observation 7: The current network selection mechanism does not take network slices information into account.

Observation 8: Access control mechanism in 5GS does not meet the requirements for 6G new services (such as sensing, AI application, computing).

Based on the observation, it is proposed to study the following 2 issues on 6G NAS functions related to UE in idle mode:

Proposal 1. Study how to support 6GR during PLMN selection.

Proposal 2. Study how to support the network selection procedures, messages and corresponding network and UE behaviour for 6GR.

Proposal 3. Study how to support and differentiate multiple satellite access types (i.e., GEO, MEO, LEO) for 6GS during PLMN selection procedure.

The mechanism for multiple satellite access types should be applied to 5GS.

Proposal 4. Study whether and how to take network slicing availability info into account for VPLMN selection in 6GS.

Proposal 5. Study whether and how to support access control to manage the access attempt for the new services (such as sensing, AI application, computing) in congestion conditions.