3GPP TSG-CT WG1 Meeting #157	C1-256246

Sophia-Antipolis, France, 13-17 October 2025

Source:	Samsung

Title:	Resolve references in Data Source Registration service operations

Spec:	3GPP TS 24.550 v1.1.0

Agenda item:	19.42

Document for:	Agreement

1. Introduction

This pCR resolves the ENs in 3GPP TS 24.550.

2. Reason for Change

In the TS 24.550v1.1.0, in Data Source Registration service operations the references to the clause numbers are under "TBD" and needs to be resolved with the correct clause numbers defined in 24.550, which is addressed in this pCR.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 24.550 v1.1.0.

* * * First Change * * * *

###### 5.3.3.2.2.2	SM client registering as data source on SM server

The SM client shall send an HTTP POST request to the SM server on the resource URI identifying the "datasources-reg-lists" collection resource as specified in clause "6.2.1.3.2.2". The body of the HTTP POST request shall include the "DataSourceRegReq" data structure as specified in clause "6.2.1.3.2.3.1".

Upon reception of the HTTP POST request from the SM client, the SM server shall:

a)	validate if the SM client is authorized to perform the data source registration; if the SM client is unauthorized, the HTTP "403 Forbidden" response is sent; or

b)	if the SM client is authorized, shall perform the data source registration. If the registration request operation:

1)	is successful, shall further generate a unique data source registration identifier for the newly registered data source and send the HTTP "201 Created" response with the response body including the "DataSourceRegReq" data structure as specified in the clause "6.2.1.6.2.2"; or

2)	fails, shall send the POST response set with the appropriate HTTP status code indicating the "failure" and the data structure as described in the clause "6.2.1.7".

* * * Next Change * * * *

###### 5.3.3.2.3.2	SM client updating the data source registration on SM server

The SM client shall send an HTTP PATCH request (for partial update) or HTTP PUT request (for full replacement) to the SM server on the resource URI identifying the "Individual data source registration" resource as specified in clause "6.2.1.4.2". The body of the HTTP PUT shall include the "DataSourceRegReq" data structure as specified in clause "6.2.1.6.2.2" or the HTTP PATCH request shall include the "DataSourcePatchRegReq" data structure as specified in clause "6.2.1.6.2.3".

Upon reception of the HTTP PATCH or PUT request from the SM client, the SM server shall:

a)	validate if the SM client is authorized to perform the data source registration update; if the SM client is unauthorized, the HTTP "403 Forbidden" response is sent; or

b)	if the SM client is authorized, shall check for the data source registration resource. If the matching data source registration resource is:

1)	found, the SM server shall:

i)	update the SM data source profile attribute(s) of the matching data source registration resource with the received SM data source profile attribute(s) information for the case of HTTP PATCH request; or

ii)	replace the SM data source profile of the matching data source registration resource with the received SM data source profile information for the case of HTTP PUT request; and

iii)	the SM server shall send the HTTP "200 OK" response with the response body including the "DataSourceRegReq" data structure as specified in the clause "6.2.1.6.2.2"; or

2)	not found, return the HTTP "404 Not Found" response to the SM client.

* * * Next Change * * * *

###### 5.3.3.2.4.2	SM client deregistering the data source registration on SM server

The SM client shall send an HTTP DELETE request to the SM server on the resource URI identifying the "Individual data source registration" collection resource as specified in clause "6.2.1.4.2".

Upon reception of the HTTP DELETE request from the SM client, the SM server shall:

a)	validate if the SM client is authorized to perform the data source deregistration; if the SM client is unauthorized, the HTTP "403 Forbidden" response is sent; or

b)	if the SM client is authorized, shall check for the registration resource. If the matching registration is:

1)	not found, return HTTP "404 Not Found" response to the SM client; or

2)	found, deregister the identified spatial map data source registration and return the HTTP "204 No Content" response to the SM client.

* * * End of Changes * * * *