3GPP TSG-CT WG1 Meeting #157	C1-256250

# Sophia Antipolis, France, 13-17 October 2025	(was C1-25xxxx)

Source:	Apple

Title:	Pseudo-CR on "AIoT NAS message type extension for future use"

Spec:	3GPP TS 24.369

Agenda item:	19.70

Document for:	Decision

1. Introduction

--

2. Reason for Change

Current AIoT NAS message layout has not means to enhance the protocol in future releases.

3. Conclusions

To ensure that the AIoT NAS message layout can be enhanced in future releases, it is proposed to reserve bit 7 and 8 for possible future use.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.369 v1.0.0.

* * * First Change * * * *

### 7.1.1	General

Within the AIoT NAS protocol defined in the present document, every message is a standard L3 message as defined in 3GPP TS 24.007 [3]. This means that the message consists of the following parts:

1)	message type;

2)	security parameters;

3)	other information elements, as required.

The organization of an AIoT NAS message is illustrated in the example shown in figure 7.1.1-1.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | Message type |  |  |  |  |  | octet 1 |
| Security parameters |  |  |  |  |  |  |  | octet 2 octet a |
|  |  |  |  |  |  |  |  | octet a+1 |
| Other information elements as required |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | octet n |

Figure 7.1.1-1: General message organization example for an AIoT NAS message

Editor's note:	Additional content of AIoT NAS general message organization is FFS and can be added as normative requirements are available.

Editor's note:	The position of the Message type in the AIoT message as well as the position of any potential security parameters are FFS.

* * * Next Change * * * *

### 7.2.2	Message type

The Message type IE and its use are defined in 3GPP TS 24.007 [3]. Table 7.2.2.1 defines the value part of the Message type IE used in the AIoT NAS protocol.

Table 7.2.2-1: Message types for AIoT NAS

| Bits |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6 | 5 | 4 | 3 | 2 | 1 |  |  |  |  |
| 0 | 0 | 0 | 0 | 0 | 1 |  | Inventory report |  |  |
| 0 | 0 | 0 | 0 | 1 | 0 |  | Read command |  |  |
| 0 | 0 | 0 | 0 | 1 | 1 |  | Read complete |  |  |
| 0 | 0 | 0 | 1 | 0 | 0 |  | Read command reject |  |  |
| 0 | 0 | 0 | 1 | 0 | 1 |  | Write command |  |  |
| 0 | 0 | 0 | 1 | 1 | 0 |  | Write complete |  |  |
| 0 | 0 | 0 | 1 | 1 | 1 |  | Write command reject |  |  |
| 0 | 0 | 1 | 0 | 0 | 0 |  | Permanent disable command |  |  |
| 0 | 0 | 1 | 0 | 0 | 1 |  | Permanent disable complete |  |  |
| Bits 8 and 7 are encoded as "0"; other values are reserved for possible future protocol extensions. A protocol entity expecting an AIoT NAS message with bits 8 and 7 encoded as "0" and receiving a message containing bit 8 or bit 7 encoded as "1" shall diagnose an "unknown or unforeseen message type" error and treat the message as specified in clause 6.3. |  |  |  |  |  |  |  |  |  |

Editor's note:	Whether the range for message type of 8 bits is needed or a shorter range is sufficient is FFS. If a shorter range is sufficient, remaining bits in the IE can be used for other mandatory parameters, e.g. part of the security parameters.

* * * End of Changes * * * *