3GPP TSG-CT WG1 Meeting #157	C1-256259

# Sophia Antipolis, France, 13-17 October 2025

Source:	China Mobile

Title:	Discussion on the issue regarding command not supported

Agenda item:	19.70

Document for:	DECISION

# 1. Abstract

This paper discusses the issue on command not support and suggests a solution for this issue.

# 2. Discussion

## 2.1 Stage 2 requirement on handling the case of command not supported

According to TS 23.369:

5.2.2.2	Read and Write Commands

It is optional for an AIoT Device to support of Read and Write Commands.

...

If an AIoT Device does not support Read Request or Write Request commands, or the parameters in the commands are invalid the AIoT Device shall reject the command.

Observation#1: In stage 2 context, the AIoT device can distinguish read/write command and is aware of whether it supports the requested command or not.

## 2.2 The analysis on HTTP, SIP and Diameter protocol in the similar case

The HTTP, SIP and Diameter protocols widely used for the client/server communication can apply for simple logic terminals. The similar case e.g. method not supported and command not supported are considered in the above protocols.

- 1)HTTP

According to RFC 7231 Hypertext Transfer Protocol (HTTP/1.1): Semantics and Content:

4.  Request Methods

4.1.  Overview

...

All general-purpose servers MUST support the methods GET and HEAD.

All other methods are OPTIONAL.

Additional methods, outside the scope of this specification, have

been standardized for use in HTTP.  All such methods ought to be

registered within the "Hypertext Transfer Protocol (HTTP) Method

Registry" maintained by IANA, as defined in Section 8.1.

...

The set of methods allowed by a target resource can be listed in an

Allow header field (Section 7.4.1).  However, the set of allowed

methods can change dynamically.  When a request method is received

that is unrecognized or not implemented by an origin server, the

origin server SHOULD respond with the 501 (Not Implemented) status

code.  When a request method is received that is known by an origin

server but not allowed for the target resource, the origin server

SHOULD respond with the 405 (Method Not Allowed) status code.

Observation#2: In HTTP, a server receiving a request method which is unrecognized or not implemented responds with 501 status code.

- 2)SIP

According to RFC 3261 SIP: Session Initiation Protocol

21.5.2 501 Not Implemented

The server does not support the functionality required to fulfill the

request.  This is the appropriate response when a UAS does not

recognize the request method and is not capable of supporting it for

any user.  (Proxies forward all requests regardless of method.)

Note that a 405 (Method Not Allowed) is sent when the server

recognizes the request method, but that method is not allowed or

supported.

Observation#3: SIP follows the same way as HTTP i.e. a server receiving a request method which is unrecognized or not implemented responds with 501 status code.

3)Diameter

According to RFC 3261 Diameter Base Protocol

7.1.3.  Protocol Errors

Errors that fall within the Protocol Error category SHOULD be treated

on a per-hop basis, and Diameter proxies MAY attempt to correct the

error, if it is possible.  Note that these errors MUST only be used

in answer messages whose 'E' bit is set.

DIAMETER_COMMAND_UNSUPPORTED 3001

This error code is used when a Diameter entity receives a message

with a Command Code that it does not support.

7.2.  Error Bit

The 'E' (Error Bit) in the Diameter header is set when the request

caused a protocol-related error (see Section 7.1.3).  A message with

the 'E' bit MUST NOT be sent as a response to an answer message.

Note that a message with the 'E' bit set is still subjected to the

processing rules defined in Section 6.2.  When set, the answer

message will not conform to the CCF specification for the command;

instead, it and will conform to the following CCF:

Message Format

<answer-message> ::= < Diameter Header: code, ERR [, PXY] >

0*1< Session-Id >

{ Origin-Host }

{ Origin-Realm }

{ Result-Code }

[ Origin-State-Id ]

[ Error-Message ]

[ Error-Reporting-Host ]

[ Failed-AVP ]

[ Experimental-Result ]

* [ Proxy-Info ]

* [ AVP ]

Note that the code used in the header is the same than the one found

in the request message, but with the 'R' bit cleared and the 'E' bit

set.  The 'P' bit in the header is set to the same value as the one

found in the request message.

Observation#4: In Diameter, a Diameter entity receives a message with a Command Code that it does not support responds with an error message containing the same command code as the request, an error bit and an error code DIAMETER_COMMAND_UNSUPPORTED 3001.

## 2.3 The analysis on 5GMM status procedure and message

According to TS 24.501 5.4.6/8.2.29/9.7:

5.4.6	5GMM status procedure

5.4.6.1	General

The purpose of the 5GMM status procedure is to report at any time in the 5GMM STATUS message certain error conditions detected upon receipt of 5GMM protocol data in the AMF or in the UE. The 5GMM STATUS message can be sent by both the AMF and the UE (see example in figure 5.4.6.1).

Figure 5.4.6.1: 5GMM status procedure

8.2.29	5GMM status

8.2.29.1	Message definition

The 5GMM STATUS message is sent by the UE or by the AMF at any time to report certain error conditions. See table 8.2.29.1.1.

Message type:	5GMM STATUS

Significance:	local

Direction:	both

Table 8.2.29.1.1: 5GMM STATUS message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | Security header type | Security header type 9.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | 5GMM STATUS message identity | Message type 9.7 | M | V | 1 |
|  | 5GMM cause | 5GMM cause 9.11.3.2 | M | V | 1 |

9.7	Message type

The Message type IE and its use are defined in 3GPP TS 24.007 [11]. Tables 9.7.1 and 9.7.2 define the value part of the message type IE used in the 5GS mobility management protocol and 5GS session management protocol.

Table 9.7.1: Message types for 5GS mobility management

| Bits |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| 0 | 1 | - | - | - | - | - | - |  | 5GS mobility management messages |
| 0 | 1 | 0 | 0 | 0 | 0 | 0 | 1 |  | Registration request |
| 0 | 1 | 0 | 0 | 0 | 0 | 1 | 0 |  | Registration accept |
| 0 | 1 | 0 | 0 | 0 | 0 | 1 | 1 |  | Registration complete |
| 0 | 1 | 0 | 0 | 0 | 1 | 0 | 0 |  | Registration reject |
| 0 | 1 | 0 | 0 | 0 | 1 | 0 | 1 |  | Deregistration request (UE originating) as specified in subclause 8.2.12) |
| 0 | 1 | 0 | 0 | 0 | 1 | 1 | 0 |  | Deregistration accept (UE originating) as specified in subclause 8.2.13) |
| 0 | 1 | 0 | 0 | 0 | 1 | 1 | 1 |  | Deregistration request (UE terminated) as specified in subclause 8.2.14) |
| 0 | 1 | 0 | 0 | 1 | 0 | 0 | 0 |  | Deregistration accept (UE terminated) as specified in subclause 8.2.15) |
| 0 | 1 | 0 | 0 | 1 | 1 | 0 | 0 |  | Service request |
| 0 | 1 | 0 | 0 | 1 | 1 | 0 | 1 |  | Service reject |
| 0 | 1 | 0 | 0 | 1 | 1 | 1 | 0 |  | Service accept |
| 0 | 1 | 0 | 0 | 1 | 1 | 1 | 1 |  | Control plane service request |
| 0 | 1 | 0 | 1 | 0 | 0 | 0 | 0 |  | Network slice-specific authentication command |
| 0 | 1 | 0 | 1 | 0 | 0 | 0 | 1 |  | Network slice-specific authentication complete |
| 0 | 1 | 0 | 1 | 0 | 0 | 1 | 0 |  | Network slice-specific authentication result |
| 0 | 1 | 0 | 1 | 0 | 1 | 0 | 0 |  | Configuration update command |
| 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |  | Configuration update complete |
| 0 | 1 | 0 | 1 | 0 | 1 | 1 | 0 |  | Authentication request |
| 0 | 1 | 0 | 1 | 0 | 1 | 1 | 1 |  | Authentication response |
| 0 | 1 | 0 | 1 | 1 | 0 | 0 | 0 |  | Authentication reject |
| 0 | 1 | 0 | 1 | 1 | 0 | 0 | 1 |  | Authentication failure |
| 0 | 1 | 0 | 1 | 1 | 0 | 1 | 0 |  | Authentication result |
| 0 | 1 | 0 | 1 | 1 | 0 | 1 | 1 |  | Identity request |
| 0 | 1 | 0 | 1 | 1 | 1 | 0 | 0 |  | Identity response |
| 0 | 1 | 0 | 1 | 1 | 1 | 0 | 1 |  | Security mode command |
| 0 | 1 | 0 | 1 | 1 | 1 | 1 | 0 |  | Security mode complete |
| 0 | 1 | 0 | 1 | 1 | 1 | 1 | 1 |  | Security mode reject |
| 0 | 1 | 1 | 0 | 0 | 1 | 0 | 0 |  | 5GMM status |
| 0 | 1 | 1 | 0 | 0 | 1 | 0 | 1 |  | Notification |
| 0 | 1 | 1 | 0 | 0 | 1 | 1 | 0 |  | Notification response |
| 0 | 1 | 1 | 0 | 0 | 1 | 1 | 1 |  | UL NAS transport |
| 0 | 1 | 1 | 0 | 1 | 0 | 0 | 0 |  | DL NAS transport |
| 0 | 1 | 1 | 0 | 1 | 0 | 0 | 1 |  | Relay key request |
| 0 | 1 | 1 | 0 | 1 | 0 | 1 | 0 |  | Relay key accept |
| 0 | 1 | 1 | 0 | 1 | 0 | 1 | 1 |  | Relay key reject |
| 0 | 1 | 1 | 0 | 1 | 1 | 0 | 0 |  | Relay authentication request |
| 0 | 1 | 1 | 0 | 1 | 1 | 0 | 1 |  | Relay authentication response |

Observation#5: Not all the 5GMM NAS messages belong to request/response mechanism.

Observation#6: The distinctive advantage of 5GMM status message is that it can report protocol errors of response message, e.g. a rejection, and a message without a response.

# 3. The view on the issue regarding command not supported

Based on the analysis in clause 2, the views on this issue are as follows:

1)The AIoT device is required to distinguish read/write command and is aware of whether it supports the requested read/write command or not. When it does not support read or write command, it needs to response to the AIOTF/AF to avoid repeatedly sending unmatched command.

2)For unsupported requested method/requested command, the way HTTP, SIP and Diameter protocol use is to return an error response (e.g. status code, error bit and error code) to clearly inform the message/request sender, which is similar to returning a rejection to a request.

3)In this phase, the READ/WRITE COMMAND REJECT message already defined to carry cause values for other scenarios can carry one more cause value for command not support when an AIoT device does not support read or write command. This mechanism can be also further used for an AIoT device to reject a new extended command.

4)A separate status message or general rejection message could take more advantages, e.g. reporting errors of a rejection or a message without response, carry more parameters, for more complex scenarios and procedures in the next phase, and can be designed based on a detailed new requirement later.

- Proposal

According to the analysis in clause 2 and the view in clause 3, it is proposed to reuse READ/WRITE COMMAND REJECT message as the solution to this issue.

- This DP is related to C1-256260.