3GPP TSG-CT WG1 Meeting #157	C1-256291

# Sophia Antipolis, France 13 – 17 October 2025

Source:	ZTE

Title:	Pseudo-CR on unsuccessful authentication of the AIoT device

Spec:	3GPP TS 24.369 v1.0.0

Agenda item:	19.70

Document for:	Agreement

1. Introduction

<Introduction part (optional)>

2. Reason for Change

Authentication of the AIoT device is performed by the network in inventory procedure. The network authenticates the AIoT device by verifying the RESAIOT received from the inventory report message. Therefore, authentication failure only occurs in the network side in inventory procedure.

3. Conclusions

Failure of authentication case should be removed from the abnormal cases in the AIoT device in inventory procedure.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.369 v1.0.0.

* * * First Change * * * *

### 5.2.5	Abnormal cases in the AIoT device

The following abnormal cases can be identified:

a)	Collision of AIoT paging.

If a new AIoT paging is received while an inventory procedure is in progress, the AIoT device shall proceed with the ongoing inventory procedure and shall process the new AIoT paging after the completion of the inventory procedure.

* * * Next Change * * * *

### 5.2.6	Abnormal cases in the AIOTF

The following abnormal cases can be identified:

a)	Lower layer failure

In the case the lower layer indication of an inventory failure received as specified by 3GPP TS 38.413 [10] from all the selected NG-RAN(s), the AIOTF shall abort the inventory procedure.

In the case the lower layer indication of no inventory response from all the selected NG-RAN(s), the AIOTF shall abort the inventory procedure.

* * * End of Changes * * * *