3GPP TSG-CT WG1 Meeting #157	C1-256297

Sophia Antipolis, France, 13-17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.558 | CR | 0172 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on the EASInfoProvResp and Eees_EECRegistration service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NBI19 |  |  |  |  |  | Date: |  |  | 2025-10-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The following issues have been identified in the current specification: - The know consumers for the Eees_AppContextRelocation and Eees_EASInformationProvisioning API in clause 5.1 are incorrect. - The description for Eees_EECRegistration service can be improved and aligned with other services. Additionally, missing attribute names shoud be supplemented. - In clause 5.7.2.2.2, the desctiption of EAS executed ACR scenario is incorrect, since there is no "EAS executed ACR scenario" IE included in "selAcrScenarios" attribute. - The EASInfoProvResp data type name is incorrect in clauses 6.6.5.1 and 6.6.5.2.3, and misalignment with the OpenAPI file. - Additional editorial and format issues. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Fix the above issues. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1, 5.2.1, 5.2.2.2.2, 5.2.2.3.2, 5.2.2.4.2, 5.7.2.2.2, 6.6.5.1, 6.6.5.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR does not impact on the OpenAPI file. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

# Proposed changes:

# *** 1st Change ***

5.1	Introduction

The table 5.1-1 lists the Edge Enabler Server APIs below the service name. A service description clause for each API gives a general description of the related API.

Table 5.1-1: List of EES Service APIs

| Service Name | Service Operations | Operation Semantics | Know Consumer(s) |
| --- | --- | --- | --- |
| Eees_EECRegistration | Request | Request/Response | EEC |
|  | Update | Request/Response | EEC |
|  | Deregister | Request/Response | EEC |
| Eees_EASDiscovery | EasDiscRequest | Request/Response | EEC |
|  | Subscribe | Subscribe/Notify | EEC |
|  | Notify |  | EEC |
|  | UpdateSubscription | Subscribe/Notify | EEC |
|  | Unsubscribe | Subscribe/Notify | EEC |
| Eees_ACREvents | Subscribe | Subscribe/Notify | EEC |
|  | Notify |  | EEC |
|  | UpdateSubscription | Subscribe/Notify | EEC |
|  | Unsubscribe | Subscribe/Notify | EEC |
| Eees_AppContextRelocation | Determine | Request/Response | EEC |
|  | Initiate | Request/Response | EEC, EES, EAS |
| Eees_EASInformationProvisioning | Declare | Request/Response | EEC |
| Eees_UEIdentifier | Get | Request/Response | EEC (NOTE) |
| NOTE: The procedure of the Get service operation when initiated by the EEC is defined in clause 5.6. The corresponding Eees_UEIdentifier API is specified in clause 8.3 of 3GPP TS 29.558 [4]. |  |  |  |

Table 5.1-2 summarizes the corresponding Edge Enabler Server APIs defined in this specification.

Table 5.1-2: API Descriptions

| Service Name | Clause | Description | OpenAPI Specification File | apiName | Annex |
| --- | --- | --- | --- | --- | --- |
| Eees_EECRegistration | 6.2 | Eees EEC Registration. | TS24558_Eees_EECRegistration.yaml | eees-eecregistration | A.2 |
| Eees_EASDiscovery | 6.3 | EES Service for the EAS Discovery. | TS24558_Eees_EASDiscovery.yaml | eees-easdiscovery | A.3 |
| Eees_ACREvents | 6.4 | EES Service for ACR Events subscription. | TS24558_Eees_ACREvents.yaml | eees-acrevents | A.4 |
| Eees_AppContextRelocation | 6.5 | EES Service to request ACR determination | TS24558_Eees_AppContextRelocation.yaml | eees-appctxtreloc | A.5 |
| Eees_EASInformationProvisioning | 6.6 | EES Service to declare EAS Information | TS24558_Eees_EASInformationProvisioning.yaml | eees-easinfoprov | A.6 |

# *** Next Change ***

5.2.1	Service Description

The Eees_EECRegistration API, as defined in 3GPP TS 23.558 [2], enable a service consumer (e.g., EEC) to:

-	register itself to a given EES;

-	update its registration information at EES; and

-	deregister itself from a given EES.

# *** Next Change ***

5.2.2.2.2	EEC registering to EES using Eees_EECRegistration_Request operation

For an EEC to register at the EES, the EEC shall send an HTTP POST message to the EES on the "EEC Registrations" collection resource to create the resource associated to or representing the EEC. The body of the HTTP POST message shall include the EEC ID, may include UE identifier, AC Profile(s), proposed expiration time for the registration, EEC context ID obtained from a previous registration, the ACR scenario(s) supported by the EEC for service continuity, as specified in clause 6.2.2.2.3.1. If EEC context ID is included in the body of the HTTP POST message, it shall also include Source EES ID and Source EES Endpoint of the EES that provided EEC context ID. If the EdgeApp_2 feature is supported, the EEC may include indication for UE mobility support requirement within the "ueMobilityReq" attribute and UE type to the EES.

Upon receiving the HTTP POST message from the EEC, the EES shall:

a)	Process the EEC registration request information;

b)	verify if the EEC is authorized to register itself at EES; and

c)	if the EEC is authorized to register with EES, then;

1)	if the AC Profile(s) is included in the HTTP POST message, the EES further determines whether the registered EAS(s) fulfils the requirements that were indicated in the AC Profile(s):

i)	if the "acSvcContSupp" attribute is included in the AC Profile, the EEC, EES, and the matching EAS have to support which ACR scenarios indicated within the ACRScenario data type; and

ii)	For each AC Profile, if "eass" attribute is included in the AC Profile, the EES identifies the matching EAS such that the matching EAS shall:

A)	be identified by the "easId" attribute; and

B)	suffice all information included in the "minimumReqSvcKPIs" attribute.

NOTE 1:	With respect to "expectedSvcKPIs" attribute, it is up to the EES implementation on how to identifies the matching EAS.

iii)	if the EdgeApp_2 feature is supported:

A)	for each AC Profile, if the "easBundleInfos" attribute is included in the AC Profile, the EES identifies the matching list of EAS bundle to which the EAS identified by the "easId" attribute information belongs; and

B)	if the EEC includes EAS selection request indication, the EES shall select EASs from the list of matching EASs based on EES local policies and provide the information to the EEC in the registration response as part of "discoveredEas" attribute. If the "easBundleInfos" attribute is included, then the EES determines whether all or a subset of the EAS(s) in the bundle are registered and instantiated. If only a subset of bundle EASs is found, the EES may determine whether instantiable but not yet instantiated EASs match the subset of remaining (i.e. not yet found) bundle EASs and the EES may trigger the ECSP management system to instantiate the EAS.

When a matching EAS is identified, the EES determines that the corresponding requirements are fulfilled and are supported for the new resource.

When a matching EAS is not identified for even one AC profile, the EES shall reject the request message by sending an HTTP response to the EEC with a status code set to 404 Not Found and indicate the "RESOURCE_NOT_FOUND" error in the "cause" attribute of the "ProblemDetails" structure.

2)	if the received EEC registration request contains an EEC context ID, a source EES endpoint, the EES retrieves the EEC's context from the source EES according to the procedures specified in clause 5.10 of 3GPP TS 29.558 [4];

3)	the EES creates a new resource with the EEC registration information as specified in clause 6.2.2.1, and assigns and stores new EEC context ID;

i)	if the EES cannot reserve the necessary resources while meeting the capability requirements of the existing registered EECs, the EES shall determine the EEC Context information stale and send a failure response with a corresponding cause as specified in clause 6.2.2.2.3.1; and

ii)	otherwise, the EES shall return the EEC registration information in the response message. The response message may include expiration time to indicate when the EEC registration will automatically expire. The response message may include a newly assigned EEC context ID. The URI of the created resource shall be returned in the "Location" HTTP header. If the EEC registration request contains AC Profile(s), and the EES determines that the requirements indicated in the AC profile(s) cannot be fulfilled for some of the AC profile(s), the EES shall include "unfulfillAcProfs" or "unfulfilledAcProfs" attribute containing the list of ACIDs of such AC Profile(s) and appropriate reasons as specified in clause 6.2.5.2.2; and

4)	if the EdgeApp_2 feature is supported;

i)	if the received EEC registration request contains UE mobility support requirement information within the "ueMobilityReq" attribute to the EES, the EES shall store the same in the EEC context. If UE mobility support requirement within the "ueMobilityReq" attribute is set to true, the EES shall subscribe to UE’s location or analytics information using 3GPP core network capabilities and in case of false the EES as per ECSP policy and EAS requirements may decide to fetch one time UE location or subscribe to NEF or NWDAF for UE location information or its analytics; and

ii)	if the received EEC registration request contains UE type information, the EES shall check if the UE is a constrained device and shall store the received UE type information in the EEC context. The EES may use this information to apply UE-type-specific local policies.

NOTE 2:	The "unfulfilledAcProfs" attribute can only be provided if there is only a single unfulfilled AC profile.

The EEC stores the new EEC context ID and uses it when it registers with another EES.

If the expiration time is provided, then to maintain the registration, the EEC shall send a registration update request (as described in clause 5.2.2.3) prior to the expiration time. If a succesful registration update request is not received prior to the expiration time, then the EES shall treat the EEC as implicitly deregistered and remove the corresponding EEC registration resource.

# *** Next Change ***

5.2.2.3.2	EEC updating registration information using Eees_EECRegistration_Update operation

To update the EEC registration information at the EES, the EEC shall send an HTTP PATCH request (for partial update) or HTTP PUT message (for fully replacement) to the EES on resource URI identifying the Individual EEC registration resource representation as specified in clause 6.2.2.3.3.3 for an HTTP PATCH message or in clause 6.2.2.3.3.1 for an HTTP PUT message.

The PATCH message includes the parameters (AC profiles or proposed expiry time) that need to be replaced in the existing registration information.

The PUT message shall replace all properties of the existing resource with the EEC registration information in the request. The value of the eecId provided during the EEC registration shall not be changed.

Upon receiving the HTTP PATCH or PUT message from the EEC, if the resource URI does not exist, the EES shall respond 404 Not Found error to the EEC. Otherwise, the EES shall:

a)	check the registration update message from the EEC to see if the EEC is authorized to modify the requested registration resource; and

b)	if the EEC is authorized to update the registration information and the eecId information in the request and the resource match, then the EES shall;

1)	if the AC Profile(s) is included in the HTTP PATCH or PUT message, the EES further determines whether the registered EAS(s) fulfils the requirements that were indicated in the AC Profile(s):

i)	if "acSvcContSupp" attribute is included in the AC Profile, the EEC, EES and the matching EAS have to support the updated ACR scenario(s) indicated within the ACRScenario data typen; and

ii)	For each AC Profile, if EAS(s) information is included in the AC Profile, the EES identifies the matching EAS such that the matching EAS shall:

A)	be identified by the "easId" attribute; and

B)	suffice the updated information included in the "minimumReqSvcKPIs" attribute.

NOTE 1:	With respect to "expectedSvcKPIs" attribute, it is up to the EES implementation on how to identifies the matching EAS.

iii)	if the EdgeApp_2 feature is supported:

A)	for each AC Profile, if the "easBundleInfos" attribute is included in the AC Profile, the EES identifies the matching the updated list of EAS bundle to which the EAS identified by the "easId" attribute belongs; and

B)	if the EEC includes EAS selection request indication, the EES shall select EASs from list of matching EASs based on EES local policies and provide the information to the EEC in the registration modification response as part of "discoveredEas" attribute. If "easBundleInfos" attribute is included, then the EES determines whether all or a subset of the EAS(s) in the bundle are registered and instantiated. If only a subset of bundle EASs is found, the EES may determine whether instantiable but not yet instantiated EASs match the subset of remaining (i.e. not yet found) bundle EASs and the EES may trigger the ECSP management system to instantiate the EAS.

When a matching EAS is identified for atleast one AC profile, the EES determines that the corresponding requirements are fulfilled and are supported and shall update the resource identified by Resource URI of the EEC registration information with the updated EEC registration information received in the HTTP PATCH or PUT request message.

2)	return the updated EEC registration information in the response. In the response message, the EES may send "200 OK" response code to provide an updated expiration time to indicate to the EEC when the updated registration will automatically expire. Otherwise, the EES sends "204 No Content" response code. If the EEC registration request contains AC Profile(s), and the EES determines that the requirements indicated in the AC profile(s) cannot be fulfilled for some of the AC profile(s), the EES shall include "unfulfillAcProfs" or "unfulfilledAcProfs" attribute containing the list of ACIDs of such AC Profile(s) and appropriate reasons as specified in clause 6.2.5.2.2.

3)	if the EdgeApp_2 feature is supported;

i)	if the received EEC registration update request contains UE mobility support requirement information within the "ueMobilityReq" attribute to the EES, the EES shall update the same in the EEC context. If UE mobility support requirement within the "ueMobilityReq" attribute is set to true, the EES shall subscribe to UE’s location or analytics information using 3GPP core network capabilities and in case of false the EES as per ECSP policy and EAS requirements may decide to fetch one time UE location or subscribe to NEF or NWDAF for UE location information or its analytics; and

ii)	if the received EEC registration update request contains UE type information, the EES shall check if the UE is a constrained device and shall store the received UE type information in the EEC context. The EES may use this information to apply UE-type-specific local policies.

NOTE 2:	The "unfulfilledAcProfs" attribute can only be provided if there is only a single unfulfilled AC profile.

If the expiration time is provided, the EEC shall send a registration update request prior to the expiration time if the EEC wants to maintain the registration. If a successful registration update request is not received prior to the expiration time, the EES shall treat the EEC as implicitly de-registered and remove the corresponding EEC registration resource.

# *** Next Change ***

5.2.2.4.2	EEC deregistering from EES using Eees_EECRegistration_Deregister operation

To deregister itself from the EES, the EEC shall send HTTP DELETE message to the EES, on the resource URI identifying the Individual EEC registration resource representation as specified in clause 6.2.2.3.3.3. Upon receiving the HTTP DELETE request, the EES shall:

a)	verify the identity of the EEC and check if the EEC is authorized to deregister the EEC registration information;

b)	if the EEC is authorized to deregister the EEC registration information, then the EES shall

1)	if the resource identified by "registrationId" attribute is not found, return "404 Not Found" error message to the EEC;

2)	otherwise, deregister the EEC profile from the EES and delete the resource representing EEC registration information; and

3)	return the "204 No Content" message to the EEC, indicating the successful deregistration of the EEC information.

# *** Next Change ***

5.7.2.2.2	EEC exchanging EAS information in EES using Eees_EASInformationProvisioning_Declare operation

To declare information about selected EAS or ACR scenario selection or both, the EEC shall send an HTTP POST request to the EES, with the request URI set to "{apiRoot}/eees-easinfoprov/<apiVersion>/declare" and the request body including the "EASInfoProvReq" data strcutre as described in the clause 6.6.3.2.2. The "EASInfoProvReq" shall include EECID and ACID, and may include request type, selected EASID, selected EAS endpoint, list of DNAIs, service area, CAS information, list of EES that are associated with a direct EAS bundle, and EEC security credentials.

If the request type is for:

a)	ACR scenario announcement by the EEC, the request includes "selAcrScenarios" attribute to indicate the ACR scenario list supported by the EEC;

b)	ACR scenario selection by the EES, the request may include the "acProf" attribute for which the selected EAS provides a service, and the "selAcrScenarios" attribute to indicate the ACR scenario list supported by the EEC; or

c)	EAS selection, the request may include the "appGrpId" attribute of a common EAS, and "eesList" attribute to indicate the list of EES that supports announcement of a common EAS.

If the EEC has selected an EAS that is instantiable but not yet instantiated, the EEC includes the"selEasIds" attribute to indicate the selected EASID without including the selected EAS endpoint in the request.

Upon receiving the HTTP POST message from the EEC, the EES shall:

a)	process the EAS information declaration request;

b)	verify if the EEC is authorized to declare EAS information to the EES;

c)	if the EEC is authorized to declare EAS information, then if the request is for:

1)	ACR scenario selection by the EES, the EES selects the ACR scenario list based on the AC/EEC/EES(s)/EAS(s) service continuity support;

2)	ACR scenario announcement by the EEC or selection by the EES, the EES may send the ACR Selection notification to the selected EAS or the selected bundled EAS(s) that have subscribed. Additionally for the EAS bundles scenario, if the "selAcrScenarios" attribute includes the "EEC_EXECUTED_VIA_SOURCE_EES", "EEC_EXECUTED_VIA_TARGET_EES", or "SOURCE_EES_EXECUTED" for EAS executed ACR scenario, the EES includes "dnais" and "svcArea" attribute of the selected EAS(s) in the ACR selection notification;

A)	if the request contains the "selEasIds" attribute to indicate the selected EAS ID and the "selEasEndPoints" attribute to indicate the selected EAS endpoint is not included, the EES verifies if instantiation of EAS is needed and may trigger the ECSP management system to instantiate the EAS;

B)	if the request contains "selEasIds" attribute to indicate the selected EAS ID and "selEasEndPoints" attribute to indicate the selected EAS Endpoint, the EES may apply the EAS traffic influence with the N6 routing information of the EAS in the 3GPP Core Network, based on application KPIs and if the EAS traffic influence was not done before.

NOTE 1:	EES can also influence the EAS traffic in advance.

NOTE 2:	The common supported ACR scenarios is decided as part of the EAS discovery and selection procedure.

3)	if the request contains "eecSvcContSupp" attribute to indicate service continuity and the EEC context has been established, the EES includes this information into the EEC context.

4)	if "appGrpId" attribute is included in the request, the EES considers the provided EAS as Common EAS for the Application Group; and

5)	for the common EAS scenario, if the request contains the"appGrpId" attribute, then the application group related information may be used for further common EAS announcement(s) between EES(s). In case of ECS that plays the role of center of information is:

A)	 not available and the request message does not contain the "eesList" attribute of EESs for a certain Application Group ID, then the EES determines the other EESs to which common EAS announce request needs to be sent as specific in clause 6.3.2.2.2 of 3GPP TS 29.558[4]; or

B)	available, then EEC selected EAS is used in interaction between the EES and ECS, to determine the common EAS as specified in clause 6.4 of 3GPP TS 29.558 [4]. If a different EAS is determined as the common EAS, then it shall be shared in the EAS information provisioning response. If the common EAS is registered to another EES, then the EES endpoint of the EES where the common EAS is registered is also included in the EAS information provisioning response.

Upon success, the EES shall send response:

a)	with an HTTP "200 OK" status code and the POST response body including the EASInfoProvResp data type as described in the clause 6.6.5.2.3 if the EEC context is established and skip the below procedure; and

b)	with an HTTP "204 No Content" status code.

On failure, the appropriate HTTP status code indicating the error shall be returned with additional error information in the POST response body.

NOTE 3:	The common supported ACR scenarios is decided as part of the EAS discovery and selection procedure.

# *** Next Change ***

6.6.5.1	General

This clause specifies the application data model supported by the Eees_EASInformationProvisioning API.

Table 6.6.5.1-1 specifies the data types defined specifically for the Eees_EASInformationProvisioning API service.

Table 6.6.5.1-1: Eees_EASInformationProvisioning API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| EasInfoProvReq | 6.6.5.2.2 | Describes the parameters shared to perform EAS Information Provision related operations. |  |
| EasInfoProvReqType | 6.6.5.3.3 | Represents the type of EAS Information Provisioning Request. |  |
| EASInfoProvResp | 6.6.5.2.3 | Information about the EAS information provisioning response. |  |
| InstantiatedEASInfo | 6.6.5.2.4 | Contains the instantiated EAS information. |  |

Table 6.6.5.1-2 specifies data types re-used by the Eees_EASInformationProvisioning API service from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Eees_EASInformationProvisioning.

Table 6.6.5.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| ACRScenario | 3GPP TS 29.558 [4] | To represent the selected ACR scenarios in ACR_SELECTION event. |  |
| Dnai | 3GPP TS 29.571 [5] | Identifies a DNAI. |  |
| EndPoint | 3GPP TS 29.558 [4] | Represents the endpoint information of an EAS. |  |
| LocationArea5G | 3GPP TS 29.122 [3] | Used to define the geographic and topological area served by EAS. |  |

# *** Next Change ***

6.6.5.2.3	Type: EASInfoProvResp

Table 6.6.5.2.3-1: Definition of type EASInfoProvResp

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| selAcrScenarioList | array(ACRScenario) | O | 1..N | The list of ACR scenarios (or the list of ACR scenarios for EAS bundles) selected by the EES. (NOTE 1) |  |
| instEasInfo | InstantiatedEASInfo | O | 0..1 | The instantiated EAS information. (NOTE 2) |  |
| comEasEndpoint | EndPoint | O | 0..1 | The common EAS endpoint provided if the EES has determined a different common EAS. (NOTE 3) |  |
| comEesEndpoint | EndPoint | O | 0..1 | The common EES endpoint of the common EAS provided if the common EAS is registered to a different EES. (NOTE 3) |  |
| NOTE 1: Only present if the "reqType" attribute is set to "ACR_SCENARIO_SELECTION_REQUEST". NOTE 2: Only present if request does not include the selected EAS endpoint. NOTE 3: Only present if the "reqType" attribute set tois set to "EAS_SELECTION". |  |  |  |  |  |

# *** End of Changes ***