3GPP TSG-CT WG1 Meeting #157	C1-256342

# Sophia Antipolis, France, 13-17 October 2025

Source:	vivo

Title:	Discussion on the NAS overhead to transport voice packets over NB-IoT NTN

Agenda item:	20.3

Document for:	DISCUSSION

1 Introduction

SA2 is studying on the support for IMS voice call over GEO via NB-IoT NTN connecting to EPC in a Rel-20 study. An LS (C1-256067) has been sent to CT1 with a question regarding the NAS overhead. This paper analyses and gives the answer to the question.

Question 8 (To CT1): In addition to question 7 related to security, is there any further possibility to reduce the NAS overhead further than what the rel.19 NORDAT_CP WI provided in order to transport voice packets over NB-IoT NTN?

2. Discussion

In Rel-19 NORDAT_CP work, CT1 has agreed on a new message for data transfer using control plane CIoT optimization with reduced overhead. The new EMM TRANSPORT message content specified in TS 24.301 is as below:

Table 8.2.35.1.1: EMM TRANSPORT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Protocol discriminator | Protocol discriminator 9.2 | M | V | 1/2 |
|  | Security header type | Security header type 9.3.1 | M | V | 1/2 |
|  | Message authentication code | Message authentication code 9.5 | M | V | 4 |
|  | Sequence number | Sequence number 9.6 | M | V | 1 |
| 79 | Data container (NOTE) | Data container 9.9.3.74 | O | V | 2-n |
| NOTE: Data container is encoded in all remaining octets of the NAS message after the Sequence number, i.e., up to the end of the NAS PDU provided by lower layers. |  |  |  |  |  |

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data type |  |  | DDX |  | EPS bearer identity |  |  | octet 1 |
| Data payload |  |  |  |  |  |  |  | octet 2 octet n |

Figure 9.9.3.74.1: Data container IE for Data type "Control plane user data"

For transport of user data over control plane, the NAS protocol overhead comprises NAS message overhead and NAS security overhead. NAS security overhead is five octets and falls under the remit of SA3. CT1 can only focus on the NAS message header.

Observation 1: The possibility of reducing the overhead on the octet 1 and octet 7 of EMM transport message can be analysed.

| Information Element | purpose |
| --- | --- |
| Protocol discriminator | It is used to identify L3 EMM protocol and used by routing function of different sublayers to distribute messages to corresponding entities of SAPs. |
| Security header type | It is used to identify this non-standard L3 message for efficient data transfer using control plane CIoT optimization with reduced overhead. |
| Data type | It is used to indicate the date type of user data. |
| Downlink data expected | It is used by the MME to identify whether current NAS connection is maintained or not. |
| EPS bearer identity | It is used by the MME to identify the target SGW for forwarding user data. |

Observation 2: The fields in the EMM transport message are used for message identification and routing.

If these fields are reduced further, there is no way to ensure the NAS message is correctly identified and user data is correctly routed.

Proposal: CT1 answers to SA2 that CT1 thinks there is no further possibility to reduce the NAS overhead further than what the rel.19 NORDAT_CP WI provided in order to transport voice packets over NB-IoT NTN.

3. Observation and Proposal

Observation 1: The possibility of reducing the overhead on the octet 1 and octet 7 of EMM transport message can be analysed.

Observation 2: The fields in the EMM transport message are used for message identification and routing.

Proposal: CT1 answers to SA2 that CT1 thinks there is no further possibility to reduce the NAS overhead further than what the rel.19 NORDAT_CP WI provided in order to transport voice packets over NB-IoT NTN.

4. Conclusion

If CT1 can reach an agreement on the proposal, it is proposed to reflect the proposal in the reply LS to SA2 in C1-256131.