3GPP TSG-CT WG1 Meeting #157	C1-256359

Sophia Antipolis, France, 13th – 17th October 2025

Source:	InterDigital

Title:	Split operation event subscription service

Spec:	3GPP TS 24.560 v1.0.0

Agenda item:	19.41

Document for:	Agreement

1. Introduction

<Introduction part (optional)>

2. Reason for Change

The pCR adds split operation event subscription service, as defined in stage-2 TS 23.482

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.560 v1.0.0.

* * * First Change * * * *

## 5.x	Split operation event subscription service

### 5.x.1	Service description

The Aimles_SplitOpEvent API, as defined in 3GPP TS 23.482 [4], allows an AIMLE client to subscribe, unsubscribe, and to update an existing subscription with the AIMLE server for events notification related to split AI/ML operation.

### 5.x.2	Service operations

#### 5.x.2.1	Introduction

The service operation defined for Aimles_SplitOpEvent API is shown in the table 5.x.2.1-1.

Table 5.x.2.1-1: Operations of the Aimles_SplitOpEvent API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Aimles_SplitOpEvent_Subscribe | This service operation is used by the AIMLE client to subscribe with the AIMLE server to be notified of events related to split AI/ML operation. | AIMLE client |
| Aimles_SplitOpEvent_Notify | This service operation is used by the AIMLE client to receive split operation notification from AIMLE server. | AIMLE client |
| Aimles_SplitOpEvent_UpdateSubscription | This service operation is used by the AIMLE client to update a subscription with the AIMLE server. | AIMLE client |
| Aimles_SplitOpEvent_Unsubscribe | This service operation is used by the AIMLE client to delete a subscription with the AIMLE server. | AIMLE client |

#### 5.x.2.3	Aimles_SplitOpEvent_Subscribe

##### 5.x.2.3.1	General

This service operation is used by AIMLE client to subscribe with the AIMLE server to be notified of events related to split AI/ML operation.

##### 5.x.2.3.2	AIML operation for split operation subscription

To subscribe with the AIML server for split operation event notification, the AIMLE client shall send an HTTP POST request to AIMLE server as specified in clause 6.x.3.2.3. The body of the POST message shall include the SplitOpEventSub data structure as specified in clause 6.x.3.2.3.1.

Upon receipt of the HTTP POST request from AIMLE client:

a)	the AIMLE server shall verify the identity of the AIMLE client and determine if the AIMLE client is authorized for the request;

b)	if the AIMLE client:

1)	is not authorized for split operation subscription, the AIMLE server shall respond to the AIMLE client with an appropriate error status code; or

2)	is authorized to request ML split operation then the AIMLE server may create the subscription and store the subscription information. If the required AIML server:

i)	has created the subscription, then in the response the AIMLE server shall include an indication of success, the subscription identity and may include an expiration time to maintain the subscription; or

ii)	has not created the subscription, then in the response the AIMLE server shall include an indication of failure that the subscription is not created and shall include an appropriate error response as specified in clause 6.x.7.

If the AIMLE client determined the received HTTP POST request needs to be redirected, the AIMLE client may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative URI representing the end point of an alternative AIMLE client towards which the HTTP POST request should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [5].

#### 5.x.2.4	Aimles_SplitOpEvent_Notify

##### 5.x.2.4.1	General

This service operation is used by AIMLE server to inform the AIMLE client about the a split operation event.

##### 5.x.2.4.2	AIML operation for split operation notification

To indicate a split operation event to a subscriber, i.e, AIMLE client, the AIMLE server shall send an HTTP POST request to AIMLE client as specified in clause 6.x.5. The body of the POST message shall include the SplitOpEventNotif data structure as specified in clause 6.x.5.2.3.1.

Upon receipt of the HTTP POST request:

a)	the AIMLE client shall verify the identity of the AIMLE server and determine if the AIMLE server is authorized to notify the AIMLE client about the split operation event; and

b)	if the AIMLE server:

1)	is not authorized, the AIMLE client shall respond to the AIMLE server with an appropriate error status code; or

2)	is authorized, the AIMLE client shall respond to the AIMLE server with:

i)	if the HTTP POST request is handled successfully, an HTTP "204 No Content" status code; and

ii)	if the HTTP POST request is not handled successfully, an appropriate error response as specified in clause 6.x.7.

#### 5.x.2.5	Aimles_SplitOpEvent_UpdateSubscription

##### 5.x.2.5.1	General

This service operation is used by AIMLE client to update subscription with the AIMLE server for split operation event notification.

##### 5.x.2.5.2	AIML operation for split operation subscription update

To update subscription with the AIML server for split operation, the AIMLE client shall send an HTTP PATCH request (for partial update) to AIMLE server as specified in clause 6.x.3.3.3 or HTTP PUT request (for full replacement) to AIMLE server as specified in clause 6.x.3.3.3.2. The body of the PATCH message shall include the SplitOpEventSubPatch data structure as specified in clause 6.x.6.2.3 or the body of the PUT message shall include the SplitOpEventSub data structure as specified in clause 6.x.6.2.2.

Upon receipt of the HTTP PATCH or PUT request from AIMLE client:

a)	the AIMLE server shall verify the identity of the AIMLE client and determine if the AIMLE client is authorized for the request;

b)	if the AIMLE client:

1)	is not authorized for split operation subscription update, the AIMLE server shall respond to the AIMLE client with an appropriate error status code; or

2)	is authorized to request split operation then the AIMLE server may update the subscription information. If the AIML server:

i)	has updated the subscription, then in the response the AIMLE server shall include an indication of success, and may include an expiration time; or

ii)	has not updated the subscription, then in the response the AIMLE server shall include an indication of failure and may include an appropriate error response as specified in clause 6.x.7.

If the AIMLE client determined the received HTTP POST request needs to be redirected, the AIMLE client may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative URI representing the end point of an alternative AIMLE client towards which the HTTP POST request should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [5].

#### 5.x.2.6	Aimles_SplitOpEvent_Unsubscribe

##### 5.x.2.6.1	General

This service operation is used by AIMLE client to unsubscribe with the AIMLE server for ML split operation.

##### 5.x.2.6.2	AIML operation to unsubscribe for split operation

To unsubscribe with the AIML server for split operation, the AIMLE client shall send an HTTP DELETE request to AIMLE server as specified in clause 6.x.3.3.3.4.

Upon receipt of the HTTP DELETE request from AIMLE client:

a)	the AIMLE server shall verify the identity of the AIMLE client and determine if the AIMLE client is authorized for the request;

b)	if the AIMLE client:

1)	is not authorized to unsubscribe for split operation, the AIMLE server shall respond to the AIMLE client with an appropriate error status code; or

2)	is authorized to request split operation then the AIMLE server may cancel the subscription. If the AIML server:

i)	has cancelled the subscription, then in the response the AIMLE server shall include an indication of success; or

ii)	has not cancelled the subscription, then in the response the AIMLE server shall include an indication of failure and may include an appropriate error response as specified in clause 6.x.7.

If the AIMLE client determined the received HTTP POST request needs to be redirected, the AIMLE client may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative URI representing the end point of an alternative AIMLE client towards which the HTTP POST request should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [5].

* * * End of Change * * * *