3GPP TSG-CT WG1 Meeting #157	C1-256382

Sophia Antipolis, France, 13-17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.543 | CR | 0110 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolution of editor's note under clause A.2.4.12 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT1 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEALDD_Ph2 |  |  |  |  |  | Date: |  |  | 2025-10-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The specification contains the following editor’s note under clause A.2.4.12, quote: Editor's note [WID: SEALDD_Ph2, CR#: 0085]: The data type L4sFeedbackCapability to support L4S mechanism will be checked to be aligned with the latest stage-2 requirements. SA6 agreed at their last meeting the CR in S6-253529 on “Correction to L4S Feedback capability”. This CR provides an update to the definition of the L4S Feedback capability which has to be considered by stage 3 in TS 24.543. The changes introduced by the approved SA6 CR are, quote: Considering the specification of the L4S capability in TS 24.543, one can notice that CT1 already provides L4S feedback and L4S-based congestion control by using the L4sFeedbackCapability data structure. The only thing is to align descrption and terminology with stage 2 in TS 23.433. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The L4sFeedbackCapability data structure and related parametersare updated for HTTP procedures to align with stage-2 requirements in TS 23.433 clause 9.2.3.3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment between stage 2 and stage 3 remains. Editor’s note remains in the specification after freezing the release 19. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.2.3, A.2.4.2, A.2.4.12, A.2.4.14, A.3.1.3.1, A.3.1.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 7.2.2.3	SDDM client CoAP procedure

In order to request an SEALDD regular transmission connection establishment to the SDDM-S, the SDDM-C shall send a CoAP POST request message to the SDDM-S according to procedures specified in IETF RFC 7252 [14]. In the CoAP POST request, the SDDM-C:

a)	shall include a CoAP URI set to the URI corresponding to the identity of the SDDM-S as specified in clause A.4.1.1 with:

1)	the "apiRoot" set to the SDDM-S URI;

b)	shall include Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=establishment-req";

c)	shall include an "EstablishmentRequest" object:

1)	shall include a "requestorId" attribute set to "sealddclient";

2)	shall include a "sealddFlowId" attribute set to the identity of the SDDM flow used by the SDDM-C and SDDM-S to identify the application traffic;

3)	shall include a "serverId" attribute set to the information of the VAL server;

4)	shall include an "endpointId" attribute set to the information of the endpoint of the selected VAL server to which the SDMM regular transmission connection establishment request has to be sent;

5)	may include a "valServiceId" attribute set to the VAL service identity of the vertical application;

6)	may include a "userPlaneAddress" attribute specifying the identity of the IP address of the traffic;

7)	may include a "portNumber" attribute specifying the identity of the port number of the traffic;

8)	may include a "url" attribute specifying the address of a given unique resource on the Web for the traffic;

9)	may include a "transportLayerProtocol" attribute specifying the transport layer protocol for the traffic;

10)	may include a "valUserId" attribute set to the identity of the VAL user or the identity of the SDDM-C acting as the VAL UE and performing the request; and

11)	may include a "batPeriodAdaptCap" attribute to indicate a BAT and periodicity adaptation capability or a "transmisAssistInfo" attribute specifying a transmission assistance information. In the "transmisAssistInfo" attribute the SDDM-C:

i)	shall include at least one of the following attributes:

A)	a "bat" attribute specifying the arrival time of the first packet of the data burst; and

B)	a "periodicity" attribute specifying the time period between the start of two bursts;

ii)	if the "bat" attribute is included, may include a "batWindow" attribute containing the acceptable earliest and latest arrival time of the first packet of the data burst; and

iii)	if the "bat", "batWindow" and "periodicity" attributes are included, may include a "periodRange" attribute specifying the periodicity range. In the "periodRange" attribute the SDDM-C shall include:

A)	a "lowerBound" attribute set to the lower bound of the periodicity and an "upperBound" attribute set to the upper bound of the periodicity of the start two bursts; or

B)	a "periodicityValues" attribute set to the acceptable periodicity values;

12)	may include an "l4sSupportingCapability" element set to the L4S support capability (i.e. ECN identification, L4S feedback and L4S-based congestion control); and

NOTE 1:	The L4S support capability is used for the SEALDD enabled congestion control for VAL applications (see 3GPP TS 23.433 [2] clause 9.8.2.2).

13)	may include an "xrAppDevCap" attribute to indicate an XR application device capability information. In the "xrAppDevCap" attribute, the SDDM-C shall include:

i)	an "xrMediaCap" attribute containing media capabilities (e.g. media resolution, media codec, media frame rate, media projection, media decoding time, mono vs. stereo 360 video); and

ii)	an "xrDisplayCap" attribute containing display capabilities (e.g. display resolution, max display refresh rate, horizontal and vertical field of view, eye to screen distance, lens separation distance).

NOTE 2:	The XR application device capability is used for the SEALDD enabled bandwidth control for different VAL users (see 3GPP TS 23.433 [2] clause 9.8.2.1).

d)	shall send the request protected with the relevant ACE profile (OSCORE profile or DTLS profile) as described in 3GPP TS 24.547 [7].

Upon receiving a CoAP POST request where the CoAP URI of the CoAP POST request identifies the establishment resource as specified in clause A.3.1.1, and containing:

a)	a Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=establishment-req", and

b)	an "EstablishmentRequest" object;

the SDDM-C shall generate a CoAP POST response according to IETF RFC 7252 [14]. In the CoAP POST response message, the SDDM-C:

a)	shall include a Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=establishment-res";

b)	shall attempt to create the SDDM regular transmission connection resource pointed at by the CoAP URI with the content of "EstablishmentRequest" object received in the request and:

1)	if successfully created, shall include an "EstablishmentResponse" object in the CoAP POST 2.01 (Created) response message;

i)	shall include a "result" attribute set to "success";

ii)	may include a "userPlaneAddress" attribute specifying the identity of the IP address of the traffic;

iii)	may include a "portNumber" attribute specifying the identity of the port number of the traffic;

iv)	may include a "url" attribute specifying the address of a given unique resource on the Web for the traffic;

v)	may include a "transportLayerProtocol" attribute specifying the transport layer protocol for the traffic; and

vi)	may include a "batPeriodAdaptCap" attribute to indicate a BAT and periodicity adaptation capability or a "transmisAssistInfo" attribute specifying a transmission assistance information. In the "transmisAssistInfo" attribute the SDDM-C:

A)	shall include at least one of the following attributes:

I)	a "bat" attribute specifying the arrival time of the first packet of the data burst; and

II)	a "periodicity" attribute specifying the time period between the start of two bursts;

B)	if the "bat" attribute is included, may include a "batWindow" attribute containing the acceptable earliest and latest arrival time of the first packet of the data burst; and

C)	if the "bat", "batWindow" and "periodicity" attributes are included, may include a "periodRange" attribute specifying the periodicity range. In the "periodRange" attribute the SDDM-C shall include:

I)	a "lowerBound" attribute set to the lower bound of the periodicity and an "upperBound" attribute set to the upper bound of the periodicity of the start two bursts; or

II)	a "periodicityValues" attribute set to the acceptable periodicity values;

2)	otherwise, shall include an "EstablishmentResponse" object with a "result" attribute set to "failure" and a "cause" attribute specifying the cause of the failure of the operation, e.g. VAL client error in the CoAP POST response; and

c)	shall send the CoAP POST response towards the SDDM-S.

* * * Next Change * * * *

### A.2.4.2	Type: EstablishmentRequest

Table A.2.4.2.1: Definition of type EstablishmentRequest

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| requestorId | RequestorId | M | 1 | Identity of the requestor of the establishment request (NOTE 1). |  |
| sealddflowId | Uinteger | M | 1 | Identity of SDDM flow used by the SDDM-C and SDDM-S to identify the application traffic. |  |
| endpointId | string | M | 1 | Identity of the endpoint of the selected VAL server to which the establishment request has to be sent. |  |
| sealddCommunicationLifetime | Uinteger | O | 0..1 | Information of the data delivery communication lifetime in milliseconds (NOTE 2). |  |
| valServiceId | string | O | 0..1 | Identity of the VAL service enabled by the SDD regular transmission connection. |  |
| userPlaneAddress | string | O | 0..1 | Identity of the IP address of the traffic. |  |
| portNumber | Uinteger | O | 0..1 | Identity of the port number of the traffic. |  |
| url | string | O | 0..1 | Identity of the address of a given unique resource on the Web for the traffic. |  |
| transportLayer | string | O | 0..1 | Identity of the transport layer protocol for the traffic. |  |
| valTgtUe | ValTargetUe | O | 0..1 | VAL user to whom the establishment request is applied. |  |
| l4sSupportingCapability | L4sSupportingCapability | O | 0..1 | Information of the L4S support capability (i.e. ECN identification, L4S feedback and L4S-based congestion control). |  |
| batPeriodAdaptCap | boolean | O | 0..1 | Indicates a BAT and periodicity adaptation capability of the SEALDD client side. (NOTE 3, NOTE 4) |  |
| transmisAssistInfo | TransmissionAssistInfo | O | 0..1 | Indicates a transmission assistance information for uplink SEALDD traffic. It includes a burst arrival time (BAT), BAT time window, periodicity, and periodicity range. (NOTE 3, NOTE 4) |  |
| xrAppDevCap | XrAppDevCapability | O | 0..1 | Indicates an XR application device capability information i.e. media capabilities and display capabilities. |  |
| NOTE 1: Thie requestorId attribute shall be set to either "SEALDDSERVER" if the requesting entity is the SDDM-S or."SEALDDCLIENT" if the requesting entity is the SDDM-C. NOTE 2: The sealddCommunicationLifetime attribute shall be included when the requesting entity is the SDDM-S. This attribute shall be included when the requesting entity is the SDDM-C. NOTE 3: This attribute may only be included in the SDDM-C initiated request. NOTE 4: The "batPeriodAdaptCap" attribute and "transmisAssistInfo" attribute are mutually exclusive. |  |  |  |  |  |

* * * Next Change * * * *

### A.2.4.12	Type: L4sSupportingCapability

Table A.2.4.12.1: Definition of type L4sSupportingCapability

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| ecnMarking | EcnMarking | M | 1 | Contais the ECN marking. |  |
| l4sFeedbackAndCongestionControl | L4sFeedbackAndCongestionControl | M | 1 | Contains the L4S feedback capability and L4S-based congestion control parameters. |  |

* * * Next Change * * * *

### A.2.4.14	Type: L4sFeedbackAndCongestionControl

Table A.2.4.14.1: Definition of type L4sFeedbackAndCongestionControl

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| perPacketCeReporting | boolean | M | 1 | Indicates that the entity can report congestion-experienced (CE) indications on a per-packet basis. |  |
| feedbackInterval | Uinteger | O | 0..1 | Indicates the time between feedback transmissions (how often they can report without overwhelming control channels) (NOTE). |  |
| maxFeedbackFrequency | Uinteger | O | 0..1 | Indicates the fastest rate at which feedback may be sent (NOTE). |  |
| protocol | string | O | 0..1 | Indicates the transport protocol (e.g. "RTCP", "in-band") (NOTE). |  |
| NOTE: This attribute may only be included if the "perPacketCeReporting" attribute indicates the value "1" (true). |  |  |  |  |  |

* * * Next Change * * * *

#### A.3.1.3.1	General

Table A.3.1.3.1.1 specifies the data types defined specifically for the SDD_RegularTransmissionConnection API service provided by SDDM-S.

Table A.3.1.3.1.1: SDD_RegularTransmissionConnection API provided by SDDM-S specific data types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| ValTargetUe | A.2.2 | Information identifying a VAL user ID or VAL UE ID. |  |
| EstablishmentResponse | A.2.4.1 | Information identifying an SDD regular transmission connection establishment response. |  |
| EstablishmentRequest | A.2.4.2.1 | Information identifying an SDD regular transmission connection establishment request. |  |
| PeriodicityRange | A.2.4.7 | Contains the acceptable periodicity range or periodicity value(s). |  |
| ReleaseRequest | A.3.1.3.2.3 | Information identifying an SDD regular transmission connection release request. |  |
| TimeWindow | A.2.4.8 | Contains start time and stop time. |  |
| TransmissionAssistInfo | A.2.4.6 | Contains transmission assistance information for uplink SEALDD traffic. |  |
| L4sSupportingCapability | A.2.4.12 | Information identifying the L4S support capability (i.e. ECN identification, L4S feedback and L4S-based congestion control). |  |

Table A.3.1.3.1.2 specifies the simple data types defined specifically for the SDD_RegularTransmissionConnection API service provided by SDDM-S.

Table A.3.1.3.1.2: SDD_RegularTransmissionConnection API provided by SDDM-S specific simple data types

| Data type | Section defined | Description |
| --- | --- | --- |
| Uinteger | A.2.3 | Unsigned integer. |
| ServerId | A.2.5 | String representing a unique identifier of a VAL server. |

Table A.3.1.3.1.3 specifies the enumerations defined specifically for the SDD_RegularTransmissionConnection API service provided by SDDM-S.

Table A.3.1.3.1.3: SDD_RegularTransmissionConnection API provided by SDDM-S specific enumeration

| Data type | Section defined | Description |
| --- | --- | --- |
| RequestorId | A.2.6.1 | Information identifying a VAL user ID or VAL UE ID. |
| ResultOp | A.2.6.2 | Information identifying the result of an operation. |
| Cause | A.2.6.3 | Information identifying the reason of the cause of the failure of an operation. |

* * * Next Change * * * *

#### A.3.1.5.2	CDDL document

;;; EstablishmentRequest

;;+ Represents a request for establishing an SDDM regular transmission connection.

EstablishmentRequest = {

requestorId: RequestorId

sealddFlowId: Uinteger

serverId: ServerId

endpointId: string

? sealddCommunicationLifetime: Uinteger

? valServiceId: string

? userPlaneAddress: string

? portNumber: Uinteger

? url: string

? transportLayer: string

? valTgtUe: ValTargetUe

? l4sSupportingCapability: L4sSupportingCapability

}

;;; EstablishmentResponse

;;+ Represents a response of establishing an SDDM regular transmission connection.

EstablishmentResponse = {

result: ResultOp

? cause: Cause

? userPlaneAddress: string

? portNumber: Uinteger

? url: string

.?.batAndPeriodicityCapability: boolean

.?.transmisAssistInfo: TransmissionAssistInfo

? transportLayer: string

}

;;; ReleaseRequest

;;+ Represents a request for releasing an SDDM regular transmission connection.

ReleaseRequest = {

serverId: ServerId

sealddFlowId: Uinteger

}

;;; RequestorId

;;+ Indicates requestor of an operation.

RequestorId = "SEALDDCLIENT" / "SEALDDSERVER"

;;; Uinteger

;;+ Unsigned Integer, i.e. only value 0 and integers above 0 are permissible.

Uinteger = int .ge 0

;;; ValTargetUe

;;+ Represents information identifying a VAL user ID or a VAL UE ID.

valUserId = {

valUserId: text                 ; Unique identifier of a VAL user.

}

valUeId = {

valUeId: text                   ; Unique identifier of a VAL UE.

}

ValTargetUe = valUserId / valUeId

;;; ServerId

;;+ Represents information identifying a unique server.

serverId = text

;;; ResultOp

;;+ Represents the result of an operation.

ResultOp = "SUCCESS" / "FAILURE"

;;; Cause

;;+ Represents the cause of failure of an operation.

Cause = "VAL CLIENT ERROR" / "SEALDD POLICY MISMATCH" / "OTHER"

;;; TransmissionAssistInfo

;;+ Indicates a transmission assistance information for uplink SEALDD traffic.

TransmissionAssistInfo = {

? bat: DateTime

? periodicity: Uinteger

? batWindow: TimeWindow

? periodRange: PeriodicityRange

}

;;; TimeWindow

;;+ Indicates the acceptable earliest and latest arrival time of the first packet of the data burst. The start time contains the earliest acceptable arrival time, and the stop time contains the latest acceptable arrival time.

TransmissionAssistInfo = {

? startTime: DateTime

? stopTime: DateTime

}

;;; PeriodicityRange

;;+ Indicates the acceptable time period range between the start of two bursts or the acceptable periodicity value(s).

PeriodicityRange = {

? lowerBound: Uinteger

? upperBound: Uinteger

? periodicityValues: [* Uinteger]

}

;;; L4sSupportingCapability

;;+ Indicates the L4S support capability (i.e. ECN identification, L4S feedback and L4S-based congestion control).

L4sSupportCapability = {

ecnMarking: EcnMarking

l4sFeeedbackAndCongestionControl: L4sFeedbackAndCongestionControl

}

;;; EcnMarking

;;+ Indicates the ECN marking.

EcnMarking = {

supportsEct0: boolean

? ect0Value: string

supportsEct1: boolean

? ect1Value: string

}

;;; L4sFeedbackAndCongestionControl

;;+ Indicates the L4S feedback capability to report congestion-experienced (CE) and L4S-based congestion control parameters.

L4sFeedback = {

perPacketCeReporting: boolean

? feedbackInterval: Uinteger      ; How often feedback is sent.

? maxFeedbackFrequency: Uinteger  ; The fastest rate at which feedback may be sent.

? protocol: string                ; Transport protocol e.g. "RTCP", "in-band".

}

* * * End of Changes * * * *