3GPP TSG-CT WG1 Meeting #157	C1-256396

# Sophia Antipolis, France, 13-17 October 2025

Source:	Ericsson, CATT, ZTE

Title:	Multiple LCS secured user plane connections support

Agenda item:	19.4

Document for:	DISCUSSION

# 1. Background

SA2 agreed to support multiple LCS-UPP connections per UE in Rel19, with an EN on whether LMF identifier (other than LMF user plane address) is needed to guarantee UE has a unique ID for UP connection association, will be decided in CT1.

2 main proposals were discussed in CT1 and didn’t reach consensus.

# 2. Discussion

2 solutions were discussed in #156 meeting on supporting the Multiple LCS secured user plane connections:

Solution 1 (C1-254777, C1-254778, C1-254779): Reuse of routing information which is in Additional information IE in UL NAS TRANSPORT or DL NAS TRANSPORT message for UE to identify the LCS secured user plane connection.

Solution 2 (C1-255147): UE to allocate a LCS secured user plane connection id for each established LCS secured user plane connection.

Here is the summary of the changes needed for these 2 solutions:

|  | New parameter needed during user plane connection establishment procedure? | New parameter needed in network initiated user plane connection modification procedure? | New parameter needed in network initiated user plane connection release procedure? | Sent the parameter for the old LCS secured user plane connection / or LMF identifier during LMF relocation? |
| --- | --- | --- | --- | --- |
| Solution 1 | no | no | no | yes(16 octets) |
| Solution 2 | yes(3 octets) | yes(3 octets) | yes(3 octets) | yes(3 octet) |

For the UE allocates new LCS-UP connection ID solution, UPP-CM messages need to be extended with the new LCS-UP connection ID also in procedures other that for LMF relocation, and new LCS-UP connection ID needs to be stored in the LMF.

Observation 1: Solution 2 has many more impacts for network and the UE, and requires more changes to the existing functionality.

Observation 2: Solution 2 consumes extra radio resource to transport since:

1). during the user plane connection establishment procedure, UE notifies the LCS-UP connection ID to the LMF;

2). after the LCS secured user plane connection is established, all the DL and UL UPP-CM messages shall include the LCS-UP connection ID of the LCS secured user plane connection.

Observation 3: For solution 2, the UE would still need to keep the routing information for each LCS secured user plane connection to ensure routing of UL UPP-CM message to the correct LMF.

Proposal 1: It is proposed to add the clarification about the UE uses the routing information included in the Additional information IE of the DL NAS TRANSPORT message to identify the established LCS secured user plane connection.

# 3. Conclusion and proposal

This paper compares 2 solutions (UE allocates new LCS-UP connection ID solution and reusing routing information in the Additional information IE solution) after the multiple LCS-UPP connections per UE in Rel19 is supported, which identifier shall be used for UE to identify the LCS secured user plane connection. Proposal #1 is captured in C1-256397, C1-256398 and C1-256399.