3GPP TSG-CT WG1 Meeting #157	C1-256398

Sophia Antipolis, France, 13-17 October 2025	was C1-255302

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.572 | CR | 0116 | rev | 2 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Include routing information for LMF relocation on new user plane connection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, CATT, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19, 5G_eLCS_Ph3 |  |  |  |  |  | Date: |  |  | 2025-10-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | SA2 discussed and agreed to support the multiple LCS-UPP connections per UE in Rel19. Since there can be more than one LCS secured user plane connections in the UE, when the UE receives an UPP-CM message, the UE needs to identify the UPP-CM message is for which LCS secured user plane connection. Considering that when a LCS secured user plane connection is established, the routing information of the LMF which is for the UPP-CM message is always included in the Additional information IE of the UL NAS TRANSPORT message and DL NAS TRANSPORT message, and since only one LCS secured user plane connection is supported per LMF, the routing information can be used for UE to identify the LCS secured user plane connection. For LMF relocation scenario, if there is no existing user plane connection exists between the UE and the target LMF, it is proposed to include the routing information of the source LMF in the USER PLANE CONNECTION ESTABLISHMENT COMMAND message. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Include the routing information of the source LMF in the USER PLANE CONNECTION ESTABLISHMENT COMMAND message for the LMF relocation scenario and there is no existing user plane connection exists between the UE and the target LMF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE has no clue about which user plane connection to replace for the LMF relocation scenario if there is no existing user plane connection between the UE and the target LMF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 6.2.1.1.3, 10.3.1.1, 10.3.1.X (new), 11.3.xx (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[3]	3GPP TS 24.571: "5G System (5GS) Control plane Location Services (LCS) procedures; Stage 3".

[4]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[5]	3GPP TS 23.271: "Functional stage 2 description of Location Services (LCS)".

[6]	3GPP TS 38.305: "Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[7]	3GPP TS 24.007: "Mobile radio interface signalling layer 3; General aspects".

[8]	3GPP TS 33.501: "Security architecture and procedures for 5G System".

[9]	3GPP TS 24.526: "UE policies for 5G System (5GS); Stage 3".

[10]	3GPP TS 23.003: "Numbering, addressing and identification".

[11]	3GPP TS 24.080: "Mobile radio interface layer 3 supplementary services specification; Formats and coding".

[12]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[13]	3GPP TS 37.355: "Technical Specification Group Radio Access Network; LTE Positioning Protocol (LPP)".

[14]	3GPP TS 24.008: "Mobile Radio Interface Layer 3 specification; Core Network Protocols; Stage 3".

[15]	3GPP TS 29.641: "Technical Specification Group Core Network and Terminals; 3GPP registry for Service Names and Port Numbers".

[16]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

* * * Next Change * * * *

##### 6.2.1.1.3	Network initiated user plane connection establishment procedure accepted by the UE

Upon receipt of a USER PLANE CONNECTION ESTABLISHMENT COMMAND message from the LMF, the UE shall stop timer T5011, if running, and timer T5014, if running, delete the stored LCS-UP binding ID, if any, and store the LCS-UP binding ID and the LMF LCS-UP address received in the USER PLANE CONNECTION ESTABLISHMENT COMMAND message.

If the USER PLANE CONNECTION ESTABLISHMENT COMMAND message can be accepted, the UE shall:

a)	establish a PDU session providing PDU connectivity service between the UE and the LMF, if not available, as described in clause 4.2.2;

b)	establish a TLS connection between the UE and the LMF, as described in clause 4.3 after the required PDU connectivity service between the UE and the LMF is available; and

c)	perform an LCS-UP connection binding procedure as described in clause 7.3.4 after the successful TLS connection establishment.

After the LCS-UP connection binding procedure is completed successfully as described in clause 7.3.4, the UE shall:

a)	create the USER PLANE CONNECTION ESTABLISHMENT COMPLETE message according to clause 10.3.2;

b)	send the USER PLANE CONNECTION ESTABLISHMENT COMPLETE message to the LMF; and

c)	consider the LCS secured user plane connection between the UE and the LMF is established.

Additionally, if the Source LMF routing information IE is included in the USER PLANE CONNECTION ESTABLISHMENT COMMAND message for the LMF relocation as specified in subclause 6.18.3 in 3GPP TS 23.273 [2], after the LCS secured user plane connection between the UE and the LMF is established successfully, the UE shall identify the source LMF's LCS secured user plane connection, associate all location sessions associated with the source LMF's LCS secured user plane connection to the new LCS secured user plane connection, and stop using the source LMF's LCS secured user plane connection.

* * * Next Change * * * *

#### 10.3.1.1	Message definition

The USER PLANE CONNECTION ESTABLISHMENT COMMAND message is sent by the LMF to the UE to transport the user plane connection information and trigger the UE to establish the LCS secured user plane connection towards the LMF. See table 10.3.1.1.1.

Message type:	USER PLANE CONNECTION ESTABLISHMENT COMMAND

Significance:	dual

Direction:	network to UE

Table 10.3.1.1.1: USER PLANE CONNECTION ESTABLISHMENT COMMAND message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | USER PLANE CONNECTION ESTABLISHMENT COMMAND message identity | Message type 11.1.3 | M | V | 1 |
|  | LCS-UP binding ID | LCS-UP binding ID 11.3.4 | M | LV | 5-256 |
|  | LMF LCS-UP address | LMF LCS-UP address 11.3.1 | M | LV | 3-256 |
| XY | Source LMF routing information | Source LMF routing information 11.3.xx | O | TLV | 18 |

* * * Next Change * * * *

#### 10.3.1.X	Source LMF routing information

The LMF shall include this information element to the UE for modification of LCS secured user plane connection if there is no existing LCS secured user plane connection between the UE and the target LMF for the LMF relocation as specified in subclause 6.18.3 in 3GPP TS 23.273 [2].

* * * Next Change * * * *

### 11.3.xx	Source LMF routing information

The purpose of the Source LMF routing information information element is to identify the source LCS secured user plane connection between the UE and target LMF for the LMF relocation as specified in subclause 6.18.3 in 3GPP TS 23.273 [2].

The Source LMF routing information information element is coded as shown in figure 11.3.xx.1 and table 11.3..xx.1.

The Source LMF routing information information element is a type 4 information element with a length of 18 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source LMF routing information IEI |  |  |  |  |  |  |  | octet 1 |
| Length of Source LMF routing information contents |  |  |  |  |  |  |  | octet 2 |
| Source LMF routing information value |  |  |  |  |  |  |  | octets 3-18 |

Figure 11.3.xx.1: Source LMF routing information information element

Table 11.3.xx.1: Source LMF routing information information element

Source LMF routing information value (octet 3 to octet 18)

The Source LMF routing information value is set to the source LMF Routing Id as specified in 3GPP TS 29.572 [16].

* * * End of Changes * * * *