3GPP TSG-CT WG1 Meeting #156	C1-256472

Goteberg, Sweden 25-29 August 2025

Source:	China Mobile

Title:	Pseudo-CR on Subscription request sent to the MMTel Enabler server

Spec:	3GPP TS 24.392 v1.0.0

Agenda item:	19.62

Document for:	Approval

1. Introduction

This pCR is proposed to add the procedures on Subscription request sent to the MMTel Enabler server. This procedure is specified in S6-253663

This procedure is theVAL client subscribes to the VAL server for receiving notifications, i.e. step 7 specified in clause 17.3.3 of 3GPP TS 23.434., and is out of scope of SEAL Notification Management procedures.

2. Reason for Change

Add the Subscription request sent to the MMTel Enabler server.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.392 v1.0.0.

* * * First Change * * * *

6.2.2.1.2	Procedure at the MMTel enabler client

In order to request DC application profiles based on notification, the MMTel Enabler client interacts with SNM-C in the same UE, or acts as SNM-S, and:

a)	shall utilize Notification channel creation procedures as specified in clause 6.2.2.1 of 3GPP TS 24.542 [3] to create the notification channel, with following clarification:

1)	In step e) of clause 6.2.2.1 of 3GPP TS 24.542 [3],

i)	shall set the channel type to PUSH;

ii)	shall set the PUSH channel details parameter with the PUSH callback-URL;

b)	after the Notification channel is created, the MMTel Enabler client subscribes to the MMTel Enabler Server for receiving notifications. In order to subscribe the notification, the MMTel Enabler client shall send an HTTP POST request to the MMTel enabler server:  In the HTTP POST request the MMTel Enabler client shall include a Request-URI set to ("/") with the following query parameters:

1)	shall include a <MMTel Enabler Client ID> element to indicate the requesting MMTel Enabler Client;

2)	shall include a <MMTel Enabler Client version> element to indicate the version of MMTel Enabler Client;

3)	shall include a <Validity Duration> element to indicate how long the subscription will last (i.e. subscription lifetime) as requested by the MMTel Enabler client; and

4)	the callback URL returned from the notification management server is included in this subscription request sent to the MMTel Enabler server; and

c)	upon receiving the HTTP POST request over a call back URI which was given to MMTel Enabler server, the MMTel Enabler client shall use the procedures specified in clause 6.2.1.1 to request the DC application profiles.

If the MMTel Enabler server acts as SNM-S to provide the notification service, the MMTel Enabler Server knows the call back URI of the MMTel Enabler client during the setup of bootstrap data channel. In this case, the step a) and  step b) are skipped.

* * * Next Change * * * *

6.2.2.1.3	Procedure at the MMTel enabler server

In order to notify the MMTel Enabler client to request DC application profiles, the MMTel Enabler server interacts with SNM-S or acts as SNM-S, and:

a)	shall utilize Notification channel creation procedures as specified in clause 6.2.2.2 of 3GPP TS 24.542 [3] to create the notification channel, with following clarification:

1)	In step e) of clause 6.2.2.1 of 3GPP TS 24.542 [3], the channel type is PUSH;

b)	upon receiving the HTTP POST subscription request sent from MMTel Enabler client containing the callback URL, the MMTel Enabler server:

1)	 shall create the subscription and set the subscription lifetime to the value included in the <Validity Duration> element; and

2)	shall store this PUSH callback-URL; and

3)	shall delete the subscription if the subscription lifetime is expired; and

c)	if the DC application profiles is updated, the MMTel Enabler server shall utilize PUSH notification messages  procedures as specified in clause 6.2.3.1.2 of 3GPP TS 24.542 [3] to notify the MMTel Enabler client, with following clarification:

1)	only one notification message should be included in this notification message payload;

2)	may include a <Urgent> element in the "VAL Notification Message" parameter, i.e. the payload of the notification message, to indicate whether the operation is urgent to UE, i.e. whether the Root application and profile need to be updated immediately.

If the MMTel Enabler server acts as SNM-S to provide the notification service, the MMTel Enabler Server knows the call back URI of the MMTel Enabler client during the setup of bootstrap data channel. In this case, the step a) and  step b) are skipped.

* * * Next Change * * * *

6.2.2.2.2	Procedure at the MMTel enabler client

In order to receive DC application profiles from MMTel Enabler Server based on notification, the MMTel Enabler client interacts with SNM-C in the same UE, or acts as SNM-S, and:

a)	shall utilize Notification channel creation procedures as specified in clause 6.2.2.1 of 3GPP TS 24.542 [3] to create the notification channel, with following clarification:

1)	In step e) of clause 6.2.2.1 of 3GPP TS 24.542 [3],

i)	shall set the channel type to PUSH; and

ii)	shall set the PUSH channel details parameter with the PUSH callback-URL;

b)	after the Notification channel is created, the MMTel Enabler client subscribes to the MMTel Enabler Server for receiving notifications. In order to subscribe the notification, the MMTel Enabler client shall send an HTTP POST request to the MMTel enabler server:  In the HTTP POST request the MMTel Enabler client shall include a Request-URI set to ("/") with the following query parameters:

1)	shall include a <MMTel Enabler Client ID> element to indicate the requesting MMTel Enabler Client;

2)	shall include a <MMTel Enabler Client version> element to indicate the version of MMTel Enabler Client;

3)	shall include a <Validity Duration> element to indicate how long the subscription will last (i.e. subscription lifetime) as requested by the MMTel Enabler client; and

4)	the callback URL returned from the notification management server is included in this subscription request sent to the MMTel Enabler server; and

c)	upon receiving the HTTP POST request over a call back URI which was given to MMTel Enabler server, the MMTel enabler client shall parse the HTTP POST request received from the MMTel enabler server and locally stores the DC application profiles included in this HTTP POST request. Then the MMTel enabler client displays the related application information from the DC application profile on the Root application.

If the MMTel Enabler server acts as SNM-S to provide the notification service, the MMTel Enabler Server knows the call back URI of the MMTel Enabler client during the setup of bootstrap data channel. In this case, the step a) and  step b) are skipped.

* * * Next Change * * * *

6.2.2.2.3	Procedure at the MMTel enabler server

In order to notify the MMTel Enabler client the DC application profiles, the MMTel Enabler server interacts with SNM-S or acts as SNM-S, and:

a)	shall utilize Notification channel creation procedures as specified in clause 6.2.2.2 of 3GPP TS 24.542 [3] to create the notification channel, with following clarification:

1)	In step e) of clause 6.2.2.1 of 3GPP TS 24.542 [3], the channel type is PUSH;

b)	upon receiving the HTTP POST subscription request sent from MMTel Enabler client containing the callback URL, the MMTel Enabler server:

1)	shall create the subscription and set the subscription lifetime to the value included in the <Validity Duration> element;

2)	shall store this PUSH callback-URL; and

3)	shall delete the subscription if the subscription lifetime is expired; and

c)	if the DC application profiles is updated, the MMTel Enabler server shall utilize PUSH notification messages  procedures as specified in clause 6.2.3.1.2 of 3GPP TS 24.542 [3] to notify the MMTel Enabler client, with following clarification:

1)	only one notification message should be included in this notification message payload; and

2)	shall include the information elements as specified in step b) of clause 6.2.1.2 encoded in JSON format as specified in clause 7.1.2, in the "VAL Notification Message" parameter.

NOTE:	The MMTel service provider can provide the MMTel Enabler Client with different DC application profile list based on the <MMTel Enabler Client version> element included in the HTTP POST request specified in step b) of clause 6.2.2.2.2.

If the MMTel Enabler server acts as SNM-S to provide the notification service, the MMTel Enabler Server knows the call back URI of the MMTel Enabler client during the setup of bootstrap data channel. In this case, the step a) and  step b) are skipped.