3GPP TSG-CT WG1 Meeting #157	C1-256475

# Gothenburg, Sweden, August 25 – 29, 2025

Source:	Lenovo

Title:	Pseudo-CR on AIoT privacy using T-IDs

Spec:	3GPP TS 24.369 V1.0.0

Agenda item:	19.70

Document for:	Agreement

1. Introduction

This PCR introduces privacy support using a Temporary ID (T-ID) as specified in 3GPP TS 33.369.

2. Reason for Change

The current version of TS 24.369 does not support privacy mechanisms based on a T-ID. This PCR adds text to specify support for privacy using a T-ID as defined in 3GPP TS 33.369.

3. Conclusions

This PCR adds the necessary text to support privacy using a T-ID.

4. Proposal

It is proposed to adopt the following changes to 3GPP TS 24.369 V1.0.0.

*** First Change ***

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AIoT	Ambient Internet of Things

AIOTF	AIoT Function

NAS	Non-Access-Stratum

T-ID	Temporary Identifier

*** Next Change ***

## 4.1	Overview

The AIoT NAS protocol described in the present document provide signalling between the AIoT device and the AIOTF, for 5GS. The AIoT NAS as part of the protocol stacks for support of AIoT services in 5GS is described in 3GPP TS 23.369 [2].

The main function of the AIoT NAS protocol is to support the AIoT procedures (see 3GPP TS 23.369[2]):

a)	inventory procedure; and

b)	command procedure.

For the support of the above functions, the procedures are supplied within procedures for AIoT NAS, in clause 5.

The AIoT NAS messages are transported between the AIoT device and the AIOTF via AIOT1 reference point, and this transport does not involve N1 reference point. The AIoT NAS protocol specified in the present document is unrelated to the 5GMM NAS and 5GSM NAS protocols specified in 3GPP TS 24.501 [4].

Editor's note:	AIoT NAS security functions, e.g. integrity protection and ciphering of AIoT NAS signalling messages, is FFS and depends on SA3 progress and available normative requirements in 3GPP TS 33.369 [6].

The AIoT NAS protocol for 5GS follows the protocol architecture model for layer 3 as described in 3GPP TS 24.007 [3].

The AIoT NAS protocol supports privacy of AIoT device identifiers by using filter information or Temporary Identifiers (T-IDs) as specified in 3GPP TS 33.369 [6]. When T-ID is used, the AIoT device and the AIOTF shall follow the procedures for T-ID generation, update, and synchronization defined in 3GPP TS 33.369 [6], clause 5.4 and Annex B.

*** Next Change ***

### 4.2.6	Privacy of AIoT device identifiers

The AIoT NAS protocol supports privacy of AIoT device identifiers by enabling the use of filtering information or Temporary Identifiers (T-IDs), as specified in 3GPP TS 33.369 [6].

When filtering information is provided by the network in the paging message, the AIoT device shall determine whether to respond based on a match between the filtering information and its AIoT device permanent identifier.

When T-ID is provided by the network in the paging message, the AIoT device shall determine whether to respond based on a match between the received T-ID and its local T-ID according to the received T-ID handling type. The following T-ID handling types are supported:

a)	"stored no update": The AIoT device matches the received T-ID against a locally stored T-ID.

b)	"stored update without command": The AIoT device matches the received T-ID against a locally stored T-ID. Upon a successful match, it generates new T-ID as specified in annex B.1 of 3GPP TS 33.369 and stores the new T-ID locally.

c)	"stored update with command": The AIoT device matches the received T-ID against a locally stored T-ID. Upon a successful match and successfully receiving an AIoT NAS command message, it generates new T-ID as specified in annex B.1 of 3GPP TS 33.369 and stores the new T-ID locally.

d)	"concealed from stored": The AIoT device generates a T-ID from a locally stored T-ID as specified in annex B.1 of 3GPP TS 33.369 and matches the received T-ID against the generated T-ID.

e)	"concealed from permanent": The AIoT device generates a T-ID from its AIoT device permanent identifier as specified in annex B.1 of 3GPP TS 33.369 and matches the received T-ID against the generated T-ID.

In all cases above, the AIoT device does not include its AIoT device identifier in the INVENTORY REPORT message. The AIOTF identifies the AIoT device based on the authentication response (RESAIOT) received in the INVENTORY REPORT message.

*** Next Change ***

### 5.2.1	General

The inventory procedure is used to discover one or more AIoT devices:

a)	if the AIoT identification information is not provided to NG-RAN, all the AIoT devices are to be discovered. In this case, an indication from lower layer is provided as specified in clause 5.2 of 3GPP TS 38.391 [8];

b)	if the AIoT identification information is provided to NG-RAN and the AIoT identification information includes the filtering information, a group of AIoT devices that matches the AIoT identification information are to be discovered; and

c)	if the AIoT identification information is provided to NG-RAN and the AIoT identification information includes an AIoT device identifier, a specific AIoT device that matches the AIoT identification information is to be discovered.

d)	if the AIoT identification information is provided to NG-RAN and the AIoT identification information includes a T-ID, a specific AIoT device that matches the T-ID is to be discovered.

*** Next Change ***

### 5.2.3	Inventory procedure completion

The AIoT device determines to respond to the AIoT paging:

a)	based on the indication from lower layer as specified in 3GPP TS 38.391 [8]; or

b)	based on whether the AIoT identification information received from the lower layer is matched or not.

If the AIoT device determines to respond to the AIoT paging, the AIoT device shall send an INVENTORY REPORT message to the AIOTF.

The AIoT device shall derive a RANDAIOT_d value as specified in 3GPP TS 33.369 [6] and shall include the derived RANDAIOT_d  value in the in the RANDAIOT_d IE in the INVENTORY REPORT message.

The AIoT device shall derive a RESAIOT value, using the long-term configured KAIoT and the value of the RANDAIOT_n included in the paging message provided by lower layers as specified in 3GPP TS 33.369 [6]. The derived RESAIOT value shall be included in the in the RESAIOT IE in the INVENTORY REPORT message.

The AIoT device shall include the AIoT device identity IE in the INVENTORY REPORT message.

Editor's note:	Details of the AIoT device identity IE is FFS and depends on SA3 progress.

The AIoT device determines whether the AIoT identification information is matched or not as follows:

Editor's note:	It is FFS how to determine the component of AIoT identification information for AIoT device, and this may need coordination with RAN2/3.

a)	if the AIoT identification information includes an AIoT device identifier:

Editor's note:	Details of AIoT device identifier should be aligned with security consideration from SA3. How to match the AIoT device identifier depends on the AIoT device identifier included.

b)	if the AIoT identification information includes filtering information (see clause 31.3 of 3GPP TS 23.003 [5]):

1)	based on ID type field included in the filtering information, the AIoT device determines the AIoT identification information is matched if:

A)	the PLMN ID field is not included in the filtering information, or the PLMN ID field is included in the filtering information and:

i)	the PLMN ID field is included in the AIoT device permanent identifier; and

ii)	the PLMN ID field in the AIoT device permanent identifier is the same as the PLMN ID field within the filtering information;

B)	the NID field is not included in the filtering information, or the NID field is included in the filtering information and:

i)	the NID field is included in the AIoT device permanent identifier; and

ii)	the NID field in the AIoT device permanent identifier is the same as the NID field within the filtering information;

C)	the third party ID field is not included in the filtering information, or the third party ID field is included in the filtering information and:

i)	the third party ID field is included in the AIoT device permanent identifier; and

ii)	the third party ID field in the AIoT device permanent identifier is the same as the third party ID field within the filtering information; and

D)	the identification information filters field is not included in the filtering information, or the identification information filters field is included in the filtering information and for each identification information filter, the component value within the identification information filter is the same as the corresponding bitstring from the AIoT device permanent identifier based on offset field and length field; or

2)	otherwise, the AIoT device determines the AIoT identification information is not matched.

NOTE 1:	The AIoT identification information corresponds to paging ID, as defined in 3GPP TS 38.391 [8]).

Editor's note:	The reference to filtering information (e.g. IE name) will be updated according to CT4 and RAN2 progress.

c)	if the AIoT identification information includes a T‑ID along with a T-ID handling type:

1)	The AIoT device shall derive a local T-ID according to the indicated T-ID handling type as specified in clause 4.2.6 and 3GPP TS 33.369 [6] and compare it with the received T-ID in the paging message; and

2)	if the resulting T-ID matches the received T-ID, the AIoT device shall consider the identification information as matched.

Based on whether the AIoT identification information is matched:

a)	if the AIoT identification information is matched, the AIoT device shall indicate to the lower layer that the AIoT identification information is matched, and determine to respond to the AIoT paging; or

b)	otherwise, the AIoT device shall ignore the AIoT identification information, indicate to the lower layer that the filtering information is not matched, and determine not to respond to the AIoT paging.

Upon all the selected NG-RAN indicating the inventory procedure has been completed, the inventory procedure is considered as completed by the AIOTF as specified in 3GPP TS 23.369 [2].

NOTE:	How NG-RAN indicates the inventory procedure has been completed is defined in 3GPP TS 38.300 [7].

Editor's note:	The reference to TS 38.300 may be updated according to RAN2 progress.

Editor's note:	AIoT .device authentication of the network is FFS.

*** End Changes ***