3GPP TSG-CT WG1 Meeting #157	C1-256624

# Sophia Antipolis, France, 13-17 October 2025

Title:	LS on handling of inventory and command collision

Response to:	-

Release:	Release19

Work Item:	AmbientIoT-CT

Source:	CT1

To:	RAN2

Cc:	SA3

Contact Person:

Name:	Yizhong Zhang

Tel. Number:	-

E-mail Address:	yizhong.zhang@vivo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	N/A

1. Overall Description:

CT1 notices that in R2-2504935 (LS on parallel service request), RAN2 has agreed on the following on support of parallel service requests:

Agreements on parallel service request

- Rel-19 devices are not expected to receive parallel service request for overlapping reader scenario based on network implementation. Capture this in RAN2 stage 2 specification.

- The Rel-19 device always responds to the new service indicated by the received paging message applicable for that device. Capture this in RAN2 stage 3 specification.

Based on above RAN2 agreement, CT1 has discussed the following two scenarios:

a)	Collision between inventory procedure and inventory procedure: The AIoT NAS layer receives a new inventory from lower layer when the AIoT NAS layer is performing the inventory procedure

b)	Collision between inventory procedure and command procedure: The AIoT NAS layer receives a new inventory from lower layer when the AIoT NAS layer is performing the command procedure

CT1 concluded that there can be an attack scenario of false AIoT pagings that would abort any ongoing valid paging or command, and thereby temporarily or continuously block valid inventory procedure and command procedure execution by the AIoT devices.

Hence, CT1 kindly ask RAN2 to provide their view on whether it is possible for the AIoT device to respond to the new AIoT paging after completion of an ongoing inventory or command when collision happens.

2. Actions:

To RAN2 group.

ACTION: 	CT1 kindly ask RAN2 to provide their view on whether it is possible for the AIoT device to respond to the new AIoT paging after completion of an ongoing inventory or command when collision happens.

3. Date of Next CT1 Meetings:

CT1#158                           17th November- 21st November 2025............Dallas, US

CT1#159                           9th February- 13th February 2026..................India