3GPP TSG-CT WG1 Meeting #157	C1-256631

Sophia Antipolis, France, 13-17 October 2025

Source:	vivo, InterDigital

Title:	Clarification on integrity verification failure

Spec:	3GPP TS 24.369 v1.0.0

Agenda item:	19.70

Document for:	Decision

1. Introduction

<Introduction part (optional)>

2. Reason for Change

For the delayed AIoT command response issue, RAN2 agrees to go with the AS based solution (i.e. AIoT MAC layer will schedule more resource for transmission). However, whether to schedule more resource for transmission also depends on whether the delayed AIoT command response is the only case for "0 SDU". The following is agreement of RAN2#131:

If the security of command fails (e.g. integrity verification failure), the AIoT device should discard the command message. However, if the NAS layer silently discards the command message, it will create another case for "0 SDU". AIoT MAC layer hence is not able to distinguish whether it is a case for delayed AIoT command or security failure. The motivation to distinguish is that for integrity verification failure case it is a waste of resource for scheduling more transmission occasion.

Hence, it is proposed that the AIoT NAS layer can indicate to the lower layer in case of security failure.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.369 v1.0.0.

* * * First Change * * * *

4.2.4	Integrity protection of AIoT NAS signalling messages

Editor's note: Integrity protection of AIoT NAS signalling messages is FFS, pending available stage 2 requirements.

In case of an AIoT NAS message fails the integrity check by the AIoT NAS, the AIoT device shall discard the AIoT NAS message and provide an indication to the lower layers.

In case of an AIoT NAS message fails the integrity check by the AIoT NAS, the AIOTF shall discard the AIoT NAS message.

* * * End of Changes * * * *