3GPP TSG-CT WG3 Meeting #142	C3-253496

Goteborg, Sweden, 25 – 29 August 2025	(Revision of C3-253412)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.213 | CR | 0752 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolve ENs for IPD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | IPD |  |  |  |  |  | Date: |  |  | 2025-08-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Upon the IP Domain can be managed by the operator for uniqueness, hence consider to add based the operator’s policy to resolve below 2 editor’s note for operator policy to control the uniqueness with flexiblity. Editor’s Note: FFS SCEF supports to translate the UE public IPv4 address into the local configured public address-related identifiers depends on SA2 requirement. Editor’s Note: It is FFS that the public address-related identifiers can be the identifier of the UPF, the IP index supported by the UPF, the identifier of the NAT, the NAT domain name, etc |  |  |  |  |  |  |  |  |
| Summary of change: |  | Resolved the IPD related editor’s notes by adding the condition base on the operator's policy to manage the flexibility and uniqueness. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | IPD related editor’s notes are not resolved yet. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | L.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

- Additional discussion(if needed):

- Proposed changes:

- *** 1st Change ***

# L.2	SCEF use case on the IP domain translation from public IPv4 address and public address-related identifier

Figure L.2-1: SCEF translate public IPv4 address to public address-related identifier in IP domain

When the SCS/AS applies network service (e.g. step 1 of clause 4.3.1.2.2.1) in the consumer scenario, the GPSI/MSISDN of the UE may not be available. IP domain may be used for when private IPv4 addresses has been reused across different NATs.

1. 	The PCEF initiates PCC procedure to the PCRF with the UE private IP within the "Framed-IP-Address" AVP assigned by the network (e.g., the PCEF, the Radius server, the static IP address subscribed in the HSS, etc.), the PDN information within the Called-Station-Id AVP. The PCRF delivers policy rule information to the PCEF.

2.	The PCRF establishes policy association for UE and create a resource for the session.

3.	The SCS/AS invokes the AsSessionWithQoS API for the required QoS based on the application and service requirement, with UE private IPv4 address in the "Framed-IP-Address" AVP and optionally UE public IPv4 address within the "IP-Domain-Id" AVP, also optionally DNN within "dnn" attribute.

NOTE:	The UE public IPv4 address can refer to the IPv4 address assigned to the UE by the NAT after the NAT performs the public IPv4 address mapping to the private IPv4 address translation.

4.	If the UE public IPv4 address is received with the "ipDomain" attribute, the SCEF may based on the operator's policy to translate the UE public IPv4 address into public address-related identifiers (i.e., the public address associated unique identifiers which can be configured based on the operator's policy) and address the PCRF via the DRA or based on local configuration.

5.	The SCEF uses the AAR command to requests policy from the PCRF with UE private IPv4 address within the "Framed-IP-Address" AVP, public address-related identifiers within the "IP-Domain-Id" AVP and optionally PDN information within the Called-Station-Id AVP. And the PCRF binds the request to the session and then update the policy to the current unique session and deliver the policy to the PCEF.

6.-	The PCRF binds the request to the session, and then update the policy to the current unique session and deliver the policy to the SMF by using the RAR command.

*** End of Changes ***