3GPP TSG-CT4 Meeting #131	C4-254016

# Sophia-Antipolis, France, 13th Oct 2025 - 17th Oct 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.510 | CR | 1240 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of Compression support in Discovery Response from NRF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Oracle Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | SBIProtoc16 |  |  |  |  |  | Date: |  |  | 2025-10-06 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The overall behavior of sending compressed responses is documented in TS 29.500, clause 6.9.2.2; this is an optional behavior, but it is expected that most, if not all, producers will support this, since it is very basic and typical behavior of most HTTP servers. However, this leads to implementation based on inference problems where although not explicitly mentioned, NRF can chose to not support zipping the responses |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR proposes to add an explicit section under section 6.2 Nnrf_NFDiscovery Service API section of 29.510 detailing out the support for Content-Type : Gzip |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Implementation of various NFs based on understanding of the specification can be different, causing functional issues |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR does not impact any yaml files |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

##### 6.2.2.2.X	Accept-Encoding

The NRF should support gzip coding (see IETF RFC 1952 [30]) in HTTP responses and indicate so in the Accept-Encoding header, as described in clause 6.9 of 3GPP TS 29 500 [4].

NF Service Consumers of the NFDiscovery API should support gzip coding in HTTP responses. NF Service Consumers should also support gzip coding in the responses of notification requests sent by the NRF

# * * * * End of changes * * * *