3GPP TSG-CT WG4 Meeting #131	C4-254045

# Sophia Antipolis, France; 13th – 17th October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.274 | CR | 2132 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update and replace obsoleted HTTP RFC 2460 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-09-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | IETF RFC 2460: "Internet Protocol, Version 6 (IPv6) Specification" is obsoleted by 8200. The changes between the obsoleted RFC 2460 and the new RFC 8200 are listed in the Appendix B of the the new RFC 8200. The main changes are the following: - Clarified that extension headers (except for the Hop-by-Hop Options header) are not processed, inserted, or deleted by any node along a packet's delivery path. RFC 8200: Extension headers (except for the Hop-by-Hop Options header) are not processed, inserted, or deleted by any node along a packet's delivery path, until the packet reaches the node (or each of the set of nodes, in the case of multicast) identified in the Destination Address field of the IPv6 header. 3GPP TS 29.274 does not specify insertion IPv6 extension headers by network entities. Hence this does not impact to this spec. - Section 4: clarify how extension headers are numbered and which are upper-layer headers as below: RFC 8200: Extension headers are numbered from IANA IP Protocol Numbers [IANA-PN], the same values used for IPv4 and IPv6. When processing a sequence of Next Header values in a packet, the first one that is not an extension header [IANA-EH] indicates that the next item in the packet is the corresponding upper-layer header. A special "No Next Header" value is used if there is no upper-layer header. This spec does not mention IPv6 extension header and this clarification has no impact to this spec. - The requirements for the Hop-by-Hop Options header were changed to make it optional to examine/process by nodes (only if they are explicitly configured to do so) along a packet's delivery path. - RFC 2460: The Hop-by-Hop Options header is used to carry optional information that must be examined by every node along a packet's delivery path. - RFC 8200: The Hop-by-Hop Options header is used to carry optional information that may be examined and processed by every node along a packet's delivery path. The hop-by-hop headers are a IPv6 level information. This spec does not mention IPv6 extension header and this clarification has no impact to this spec. - Incorporate the updates from RFCs 5095 and 5871 related to RH0, as defined in RFC 5095, the RH0 has security issues, so that it has been deprecated. This spec does not mention IPv6 extension header and this clarification has no impact to this spec. - Clause 4.5 on IPv6 fragmentation is updated based on the updates in RFCs 5722, 6946, 7112 and 8021, including: - Require that IPv6 nodes must not create overlapping fragments. Also, when reassembling an IPv6 datagram, if one or more its constituent fragments is determined to be an overlapping fragment, the entire datagram (and any constituent fragments) must be silently discarded. This is a clarification to the IPv6 layer of the stack on how to do the fragmentation. This does not impact this spec. - Require that all headers through the first Upper-Layer header are in the first fragment. The new error case added is about the silent discarding of overlapping fragments during re-assembly. This is a clarification to the IPv6 layer of the stack on how to do the reassembly. This does not impact this spec. - Added text to the Fragment header process on handling exact duplicate fragments. This is an IPv6 level clarification for implementation. This does not impact this spec. Removed the paragraph in Section 5 that required including a Fragment header to outgoing packets if an ICMP Packet Too Big message reports a Next-Hop MTU less than 1280. This does not impact this spec. - Incorporated the update made by RFC 6935 ("IPv6 and UDP Checksums for Tunneled Packets") in Section 8. Added an exception to the default behavior for the handling of UDP packets with zero checksums for tunnels. This spec does not mention the handling of UDP packets with zero checksums for tunnels and this clarification has no impact to this spec. - Added clarification noting that some fields in the IPv6 header may also vary across the fragments being reassembled in Section 4.5. RFC 8200: Other fields in the IPv6 header may also vary across the fragments being reassembled. Specifications that use these fields may provide additional instructions if the basic mechanism of using the values from the Offset zero fragment is not sufficient. For example, Section 5.3 of [RFC3168] describes how to combine the Explicit Congestion Notification (ECN) bits from different fragments to derive the ECN bits of the reassembled packet. This does not impact this spec. There is hence no impact on this specification that references this RFC for the overall definition of the Internet Protocol, Version 6 (IPv6). It is hence needed to update the corresponding references accordingly to avoid keeping an obsoleted RFC referenced in this specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Reference to obsoleted RFC is replaced with the new reference in the References clause and within the text. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification referes to obsoleted RFC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 10.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.003: "Numbering, addressing and identification".

[3]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[4]	3GPP TS 29.060: "General Packet Radio Service (GPRS); GPRS Tunnelling Protocol (GTP) across the Gn and Gp interface".

[5]	3GPP TS 24.008: "Mobile radio interface Layer 3 specification; Core network protocols; Stage 3".

[6]	IETF RFC 791 (STD 0005): "Internet Protocol", J. Postel.

[7]	IETF RFC 768 (STD 0006): "User Datagram Protocol", J. Postel.

[8]	3GPP TS 32.251: "Telecommunication Management; Charging Management; Packet Switched (PS) domain charging.

[9]	3GPP TS 32.298: "Telecommunication Management; Charging Management; Charging Data Record (CDR) parameter classification.

[10]	3GPP TS 36.413: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 Application Protocol (S1AP)".

[11]	3GPP TS 33.102: "3G security; Security architecture".

[12]	3GPP TS 33.401: "3GPP System Architecture Evolution (SAE); Security architecture".

[13]	3GPP TS 29.281: "General Packet Radio System (GPRS) Tunnelling Protocol User Plane (GTPv1-U)".

[14]	3GPP TS 29.276: "3GPP Evolved Packet System (EPS); Optimized handover procedures and protocols between E-UTRAN Access and cdma2000 HRPD Access; Stage 3".

[15]	3GPP TS 29.280: "Evolved Packet System (EPS); 3GPP Sv interface (MME to MSC, and SGSN to MSC) for SRVCC".

[16]	IETF RFC 8200: "Internet Protocol, Version 6 (IPv6) Specification".

[17]	3GPP TS 23.007: "Restoration procedures".

[18]	3GPP TS 32.422: "Telecommunication management; Subscriber and equipment trace; Trace control and configuration management ".

[19]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[20]	3GPP TS 36.414: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 data transport".

[21]	3GPP TS 23.272: "Circuit Switched (CS) fallback in Evolved Packet System (EPS); Stage 2".

[22]	3GPP TS 29.118: "Mobility Management Entity (MME) - Visitor Location Register (VLR) SGs interface specification".

[23]	3GPP TS 24.301: "Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS); Stage 3".

[24]	void

[25]	ITU-T Recommendation E.164: "The international public telecommunication numbering plan".

[26]	3GPP TS 29.275: "Proxy Mobile IPv6 (PMIPv6) based Mobility and Tunnelling protocols; Stage 3".

[27]	3GPP TS 44.018: "Mobile radio interface layer 3 specification; Radio Resource Control Protocol".

[28]	3GPP TS 48.008: "Mobile Switching Centre - Base Station System (MSC-BSS) interface; Layer 3 specification".

[29]	3GPP TS 29.212: "Policy and Charging Control (PCC); Reference points".

[30]	3GPP TS 24.007: "Mobile radio interface signalling layer 3; General Aspects".

[31]	IETF RFC 1035: "Domain Names - Implementation and Specification".

[32]	3GPP TS 29.303: "Domain Name System Procedures; Stage 3".

[33]	3GPP TS 25.413: "UTRAN Iu interface Radio Access Network Application Part (RANAP) signalling".

[34]	3GPP TS 48.018: "General Packet Radio Service (GPRS); Base Station System (BSS) - Serving GPRS Support Node (SGSN); BSS GPRS protocol (BSSGP)".

[35]	3GPP TS 23.060: "General Packet Radio Service (GPRS); Service description; Stage 2".

[36]	3GPP TS 32.295: "Telecommunication management; Charging management; Charging Data Record (CDR) transfer".

[37]	3GPP TS 23.246: "Multimedia Broadcast/Multicast Service (MBMS); Architecture and functional description".

[38]	3GPP TS 29.061: "Interworking between the Public Land Mobile Network (PLMN) supporting packet based services and Packet Data Networks (PDN) ".

[39]	IETF RFC 3588: "Diameter Base Protocol ".

[40]	IETF RFC 4607: "Source-Specific Multicast for IP".

[41]	3GPP TS 29.002: "Mobile Application Part (MAP) specification".

[42]	3GPP TS 29.010: "Information element mapping between Mobile Station - Base Station System (MS - BSS) and Base Station System - Mobile-services Switching Centre (BSS - MSC); Signalling procedures and the Mobile Application Part (MAP)".

[43]	3GPP TS 23.216: "Single Radio Voice Call Continuity (SRVCC); Stage 2".

[44]	3GPP TS 32.423: "Telecommunication management; Subscriber and equipment trace: Trace data definition and management".

[45]	3GPP TS 23.402: "Architecture enhancements for non-3GPP accesses.

[46]	3GPP TR 25.999: "HSPA Evolution (FDD)".

[47]	3GPP TS 23.292: "IP Multimedia Subsystem (IMS) centralized services".

[48]	3GPP TS 23.203: "Policy and charging control architecture; Stage 2".

[49]	ITU-T Recommendation X.691 (07/2002): "Information technology – ASN.1 encoding rules: Specification of Packed Encoding Rules (PER)".

[50]	3GPP TS 33.402: "3GPP System Architecture Evolution (SAE); Security aspects of non-3GPP accesses".

[51]	3GPP TS 23.139: "3GPP system - fixed broadband access network interworking; Stage 2".

[52]	IEEE Std 802.11-2012: "IEEE Standard for Information technology - Telecommunications and information exchange between systems - Local and metropolitan area networks - Specific requirements - Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications".

[53]	IETF RFC 5905: "Network Time Protocol Version 4: Protocol and Algorithms Specification".

[54]	3GPP TS 32.299: "Telecommunication Management; Charging Management; Diameter charging applications.

[55]	3GPP TS 23.251: " Network Sharing; Architecture and Functional Description".

[56]	3GPP TS 23.271: "Location Services".

[57]	3GPP TS 29.173: "Diameter-based SLh interface for Control Plane LCS".

[58]	IETF RFC 5453: "Reserved IPv6 Interface Identifiers".

[59]	IETF RFC 4776: "Dynamic Host Configuration Protocol (DHCPv4 and DHCPv6) Option for Civic Addresses Configuration Information".

[60]	IETF RFC 3046: "DHCP Relay Agent Information Option".

[61]	3GPP TS 23.380: "IMS Restoration Procedures".

[62]	3GPP TS 22.153: "Multimedia Priority Service".

[63]	3GPP TS 24.302: "Access to the 3GPP Evolved Packet Core (EPC)
via non-3GPP access networks; stage 3".

[64]	ITU-T Recommendation E.212: "The international identification plan for mobile terminals and mobile users".

[65]	IETF RFC 2474: "Definition of the Differentiated Services Field (DS Field) in the IPv4 and IPv6 Headers".

[66]	3GPP TS 24.244: "Wireless LAN control plane protocol for trusted WLAN access to EPC".

[67]	IETF RFC 7296: "Internet Key Exchange Protocol Version 2 (IKEv2)".

[68]	3GPP TS 29.273: "3GPP EPS AAA Interfaces".

[69]	3GPP TS 29.336: "Home Subscriber Server (HSS) diameter interfaces for interworking with packet data networks and applications".

[70]	3GPP TS 29.272: "Mobility Management Entity (MME) and Serving GPRS Support Node (SGSN) related interfaces based on Diameter protocol".

[71]	3GPP TS 23.161: "Network-Based IP Flow Mobility (NBIFOM); Stage 2".

[72]	3GPP TS 23.303: "Proximity-based services (ProSe); Stage 2".

[73]	3GPP TS 24.161: "Network-Based IP Flow Mobility (NBIFOM); Stage 3".

[74]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications".

[75]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS)".

[76]	3GPP TS 36.323: "Evolved Universal Terrestrial Radio Access (E-UTRA); Packet Data Convergence Protocol (PDCP) specification".

[77]	IETF RFC 4995: "The RObust Header Compression (ROHC) Framework".

[78]	3GPP TS 43.020: "Security related network functions".

[79]	3GPP TS 36.444: "EUTRAN M3 Application Protocol (M3AP)".

[80]	3GPP TS 29.244: "Interface between the Control Plane and the User Plane of EPC Nodes; stage 3".

[81]	IETF RFC 2474: "Definition of the Differentiated Services Field (DS Field) in the IPv4 and IPv6 Headers".

[82]	3GPP TS 23.501:"System Architecture for the 5G System; Stage 2".

[83]	3GPP TS 23.502:"Procedures for the 5G System; Stage 2".

[84]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[85]	3GPP TS 33.250: "Security assurance specification for the PGW network product class".

[86]	3GPP TS 33.501:" Security Architecture and Procedures for 5G System".

[87]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[88]	3GPP TS 29.502: "5G System; Session Management Services; Stage 3".

[89]	3GPP TS 23.287: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services".

[90]	3GPP TS 23.256: "Support of Uncrewed Aerial Systems (UAS) connectivity, identification and tracking; Stage 2".

[91]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[92]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[93]	3GPP TS 24.193: "Access Traffic Steering, Switching and Splitting; Stage 3".

[94]	IETF RFC 3986: "Uniform Resource Identifier (URI): Generic Syntax".

* * * * Next Change * * * *

10.1	IP Version

GTPv2 entities shall support both versions of the Internet Protocol, version 4 (IPv4) as defined by IETF RFC 791 [6], and version 6 (IPv6) as defined by IETF RFC 8200 [16].

* * * * End of Changes * * * *