3GPP TSG-CT WG4 Meeting #131	C4-254046

# Sophia Antipolis, France; 13th – 17th October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.003 | CR | 0725 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates on filtering information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AmbientIoT-CT |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As defined in TS 23.369 below: The Filtering Information is used to identify or filter one or multiple AIoT Device(s). The Filtering Information is constructed by one or multiple filtering elements and each filtering element corresponds to a component (i.e. ID Type, PLMN Identifier, NID, Third Party Identifier or Identification Information) of the AIoT Device Permanent Identifier as defined in clause 5.7 or corresponds to the stored/concealed type of temporary ID as defined in clause 5.X. Each filtering element shall include: - Component Type Information indicating a component of the AIoT Device Permanent Identifier that is used to match the bitstring or indicating the stored/concealed type of temporary ID that is used to match the bitstring. - A bitstring which is used to compare with the component of the AIoT Device Permanent Identifier, stored type of temporary ID or concealed type of temporary ID. - An Offset from the beginning of the indicated component of the AIoT Device Permanent Identifier, which indicates the start location in the indicated component to be used to compare with the corresponding bitstring. When the Component Type Information of a filtering element indicates a PLMN ID, NID, Third Party Identifier, stored type of temporary ID or concealed type of temporary ID component, then the Offset is not included in the filtering element, i.e. the whole component is compared with the bitstring. The stored/concealed type of temporary ID has been defined for filtering information. It proposes to update the structure of filtering information to align with stage2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the structure of filtering information, add new Filtering Types. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment with Stage2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 31.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.369 CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

31.3	Structure of Filtering Information

The Filtering Information is used to identify or filter multiple AIoT Devices, it includes a list of filtering elements as specified in clause 5.8 of 3GPP TS 23.369 [152], with a maximum length of 256 bits, the structure is shown in Figure 31.3.1.

Figure 31.3.1: Structure of Filtering Information

The Filtering Information includes at most one filtering element within the Filtering Information for the following component of AIoT Device Permanent Identifier (i.e. ID Type, PLMN ID, NID or Third Party Identifier). The indicated filtering element within a Filtering Information should be matched and shall be coded as the binary representation of their value, it is composed of the following parts:

1)	Count of Filtering Element shall have a fixed length of 4 bits to indicate the number of Filtering Element which included in a Filtering Information

2)	The Filtering Element is composed of the following parts, the structure is shown in Figure 31.3.2:

Figure 31.3.2: Structure of Filtering Element

1)	Filtering Type indicates the AIoT Device Permanent Identifier is used to match the bitstring, with the following mapping:

-	000	indicate the Filtering Type for the filtering element is ID Type;

-	001	indicate the Filtering Type for the filtering element is PLMN ID;

-	010	indicate the Filtering Type for the filtering element is NID;

-	011	indicate the Filtering Type for the filtering element is Third Party Identifier;

-	100	indicate the Filtering Type for the filtering element is Identification Information;

-	101	indicate the Filtering Type for the filtering element is stored type of temporary ID

-	110	indicate the Filtering Type for the filtering element is concealed type of temporary ID;

-	111	reserved.

When the Filtering Type is set to 101 or 110, the other Filtering Elements within Filtering Information shall not be present.

2)	ID Type Filter, this field is present only if the Filtering Type is set to 000. The structure is show in Figure 31.3.3.

Figure 31.3.3: Structure of ID Type Filter

-	Offset consisting of 3 bits. It represents the position of the ID Type's bitstring to be compared. Value 0 means the starting position is 1, and value 7 means the starting position is 8.

-	Length consisting of 3 bits. It represents the length of value of the bitstring to be compared.

-	Component Value consisting of 1 - 8 bits. It represents the component value of the bitstring to be compared.

NOTE:	It is suggested that a Filtering Information does not only include ID Type Filter Element as Filter Information.

3)	Filtering information for PLMN ID shall have a fixed length of 24 bits, this field is present only if the Filtering Type is set to 001.

4)	Filtering information for NID shall have a fixed length of 44 bits, this field is present only if the Filtering Type is set to 010.

5)	Filtering information for Third Party Identifier shall have a fixed length of 24 bits, this field is present only if the Filtering Type is set to 011.

6)	Identification Information Filter, there should be one Identification Information Filter within a Filtering Element, this field is present only if the Filtering Type is set to 100. The structure is show in Figure 31.3.4.

It is used to filter the target devices with bitstring of Identification Information equal to a specific value.

Figure 31.3.4: Structure of Identification Information Filter

-	Offset consisting of 7 bits. It represents the position of the Identification Information's bitstring to be compared. Value 0 means the starting position is 1, and value 127 means the starting position is 128.

-	Length consisting of 7 bits. It represents the length of value of the bitstring to be compared.

-	Component Value consisting of 1 - 128 bits. It represents the component value of the bitstring to be compared.

7)	Filtering information for stored type of temporary ID, this field is present only if the Filtering Type is set to 101.

8)	Filtering information for concealed type of temporary ID, this field is present only if the Filtering Type is set to 110.

Editor's Note: The length of stored type of temporary ID and concealed type of temporary ID is based on Stage2’s conclusion, is FFS.

* * * * End of Changes * * * *