3GPP TSG-CT WG4 Meeting #131	C4-254048

# Sophia Antipolis, France; 13th – 17th October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.369 | CR | 0002 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorial Correction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AmbientIoT-CT |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TS 29.369 has defined that the NEF could be a NF service consumer to retrieve AF Authorization Data. However, as defined in TS 23.369, the NEF does not support this feature, the NEF only needs to obtain the last known AIOTF (which is included in AIoT Device Profile Data) for ADM selection. 4.5.4 NEF In addition to the functions defined in TS 23.501 [3], the NEF performs the following functions: - Providing a service exposure API to AFs of 3rd party for AIoT services. - Interacting with AF of 3rd party and AIOTF. - Selection of AIOTF for AIoT services. - Authorization of the untrusted AF’s AIoT operation request. 5.3.2 ADM Discovery and Selection The ADM discovery and selection function is supported by the AIOTF to select an ADM instance to retrieve the device profile data or update the last known AIOTF for the AIoT device. The AIOTF may also discover and select an ADM to retrieve AF authorization data. Similarly, the NEF uses the ADM discovery and selection function to select an ADM to obtain the last known AIOTF for the AIoT device. It proposes to remove NEF as NF service consumer for AF Authorization Data Retrieval |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove NEF as NF service consumer for AF Authorization Data Retrieval |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect NF service consumer is used in AF Authorization Data Retrieval procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR does not introduce any changes to the OpenAPIs. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

5.2.2.2.3	AF Authorization Data Retrieval

Figure 5.2.2.2.3-1 shows a scenario where the NF service consumer (e.g. AIOTF) sends a request to the ADM to receive the AF Authorization Data (see 3GPP TS 23.369 [14]).

Figure 5.2.2.2.3-1: Requesting AF Authorization Data

1.	The NF service consumer (e.g. AIOTF) sends a GET request to the resource of the AF authorization data (…/af-authorization-data), to get the Authorization Data of the AFs. The request may contain the target AF ID if the authorization data for a specific AF is to be retrieved.

2a.	On success, the ADM responds with "200 OK" with the Authorization Data of the target AF(s).

2b.	If there is no valid AF Authorization Data available, HTTP status code "404 Not Found" shall be returned including additional error information in the response body (in the "ProblemDetails" element).

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the GET response body.

* * * * End of Changes * * * *