3GPP TSG-CT WG4 Meeting #131	C4-254062

# Sophia Antipolis, France; 13th – 17th October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.502 | CR | 0898 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of table note |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - There is a NOTE under Table 6.1.6.2.65-1: NOTE: Either ipv4Addr, or ipv6Addr, or ipv6Prefix shall be present. This is incorrect, because "either" is used only when referring to two alternatives. So in the above sentence, the wording leads to a confusion about which two items "either" is referring to. - In Table 6.1.6.2.82-1, "t false" should be corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the table note in Table 6.1.6.2.65-1. Correct editorial mistake in Table 6.1.6.2.82-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect text remains in the specification and the presence condition of IEs will remain unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.6.2.65, 6.1.6.2.82 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR does not impact any yaml file. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * *1st Change * * * *

##### 6.1.6.2.65	Type: IpAddress

Table 6.1.6.2.65-1: Definition of type IpAddress

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| ipv4Addr | Ipv4Addr | C | 0..1 | Indicate an IPv4 Address |
| ipv6Addr | Ipv6Addr | C | 0..1 | Indicate an IPv6 Address |
| ipv6Prefix | Ipv6Prefix | C | 0..1 | Indicate an IPv6 Prefix |
| NOTE: One and only one of ipv4Addr, or ipv6Addr, or ipv6Prefix shall be present. |  |  |  |  |

* * * 2nd Change * * * *

##### 6.1.6.2.82	Type: LocalOffloadingMgtInfoToIsmf

Table 6.1.6.2.82-1: Definition of type LocalOffloadingMgtInfoToIsmf

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| localOffloadingMgtAllowedInd | boolean | C | 0..1 | This IE shall be present in the Create or Update Response, if the localOffloadingMgtAllowedInd IE was present and set to true in the request. When present, this IE shall indicate whether the SMF allows the I-SMF to perform Local Offloading Management: - true: allowed - false: not allowed |
| dnsAddr | IpAddress | O | 0..1 | When present, this IE shall contain the DNS server address to be used for DNS requests related with traffic not to be subject to Local Offloading Management. |
| psaUpfAddr | IpAddress | O | 0..1 | When present, this IE shall contain the PSA UPF address information (e.g. PSA UPF IP address on N6) to be used by the I-SMF to configure the EASDF to build EDNS Client Subnet option for target FQDN of DNS queries which are not authorized for Local Offloading Management. |
| localOffloadingInfoList | array(LocalOffloadingManagementInfo) | O | 0..N | When present, this IE shall include Local Offloading Information list applicable to the PDU session and whose offload identifiers are not yet known by the I-SMF service instance or whose offload identifier's version has changed. (NOTE 2)(NOTE 3) |
| offloadIds | array(OffloadIdentifier) | C | 0..N | When present, this IE shall include a list of specific Offload Ids applicable for the PDU session and that are part of the StoredOffloadIds included in the corresponding request message. (NOTE 2)(NOTE 3) |
| trafficInfluInfo | TrafficInfluenceInfo | O | 0..1 | When present, it shall include AF traffic influence information that applies at the I-SMF to the PDU session. |
| NOTE 1: The SMF sends the EAS rediscovery indication and EAS information to be refreshed for EAS re-discovery received from the I-SMF to the UE in n1SmInfoToUe attribute. NOTE 2: When the SMF determines that the I-SMF service instance already knows the corresponding Local Offloading Info based on the storedOffloadIds provided by the I-SMF service instance in the corresponding request message and the associated localOffloadInfo has not changed, then in the corresponding response message, the SMF shall include the corresponding offload identifier in the offloadIds attribute; otherwise, the SMF shall include it in the localOffloadingInfoList attribute. The SMF shall always provision a complete list of localOffloadingInfo and/or offloadIds. NOTE 3: The I-SMF service instance shall always overwrite any localOffloadingInfoList and the offloadIds stored for the PDU session with the latest localOffloadingInfoList and/or offloadIds received. The SMF may request the I-SMF to remove all localOffloadingInfoList provisioned earlier for a PDU session with an I-SMF by provisioning an empty localOffloadingInfoList array. When the I-SMF receives a LocalOffloadingInfo containing an offloadId which is known by the I-SMF, it shall consider that the localOffloadingInfo for the offloadId has changed if the offloadId does not contain a version field or if the offloadId has an incremented version number, and if so, the I-SMF service instance shall enforce the changed localOffloadingInfo for the PDU session with an I-SMF and also for any other PDU sessions with an I-SMF with the same offloadId. |  |  |  |  |

* * * End of Changes * * * *