3GPP TSG-CT WG4 Meeting #131	C4-254063

# Sophia Antipolis, France; 13th – 17th October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.518 | CR | 1254 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of table note |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There is a Table Note under Table 6.2.6.2.2-1: NOTE 2: Either information about a single UE (i.e. SUPI, GPSI, PEI) or groupId, or anyUE set to "TRUE" shall be included. Also, this Table Note is under Table 6.2.6.2.24-1: NOTE: Either information about taList or taiRangeList or anyTa set to "TRUE" shall be included. These notes are confusing, because "either" is used only when referring to two alternatives. So in these notes the wording leads to a confusion about which two items "either" is referring to. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update NOTE 2 in Table 6.2.6.2.2-1. - Update NOTE in Table 6.2.6.2.24-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect text remains in the specification and the presence condition of IEs will remain unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.6.2.2, 6.2.6.2.24 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR does not impact any yaml file. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * *1st Change * * * *

##### 6.2.6.2.2	Type: AmfEventSubscription

Table 6.2.6.2.2-1: Definition of type AmfEventSubscription

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| eventList | array(AmfEvent) | M | 1..N | Describes the events to be subscribed in subscription request or the events successfully subscribed for this subscription in subscription response. |  |
| eventNotifyUri | Uri | M | 1 | Identifies the recipient of notifications sent by AMF for this subscription (NOTE 1) |  |
| notifyCorrelationId | string | M | 1 | Identifies the notification correlation ID. The AMF shall include this ID in the notifications. The value of this IE shall be unique per subscription for a given NF service consumer. |  |
| nfId | NfInstanceId | M | 1 | Indicates the instance identity of the network function creating the subscription. |  |
| subsChangeNotifyUri | Uri | C | 0..1 | This IE shall be present if the subscription is created by an NF service consumer on behalf of another NF (e.g UDM creating event subscription at AMF for event notifications towards NEF). When present, this IE Identifies the recipient of notifications sent by AMF, for the creation of a new subscription ID, that is considered as a change of subscription ID by the NF service consumer for event subscriptions related to single UE or as the creation of a new subscription Id for event subscriptions related to UE groups (e.g during mobility procedures involving AMF change). (NOTE 3). |  |
| subsChangeNotifyCorrelationId | string | C | 0..1 | This IE shall be present when an NF Service Consumer (e.g. UDM) is subscribing for events on behalf of another NF Service Consumer (e.g. NEF). When present, this IE shall contain the notification correlation ID. The AMF shall include it in the notifications for the creation of a new subcription ID that is considered as a change of subscription ID by the NF service consumer for event subscriptions related to single UE or as the creation of a new subscription Id for event subscriptions related to UE groups. The value of this IE shall be unique per subscription for a given NF service consumer that is sending this IE. (NOTE 3). |  |
| supi | Supi | C | 0..1 | Subscription Permanent Identifier (NOTE 2) (NOTE 4) |  |
| groupId | GroupId | C | 0..1 | Identifies a group of UEs. (NOTE 2) |  |
| excludeSupiList | array(Supi) | O | 1..N | This IE may be present for a group subscription. When present, this IE shall carry the SUPI of the group member UE(s) that are excluded from the group subscription. | DGEM |
| excludeGpsiList | array(Gpsi) | O | 1..N | This IE may be present for a group subscription. When present, this IE shall carry the GPSI of the group member UE(s) that are excluded from the group subscription. | DGEM |
| includeSupiList | array(Supi) | O | 1..N | This IE may be present for a group subscription. When present, this IE shall carry the SUPI of the group member UE(s) that are included for the group subscription. | DGEM |
| includeGpsiList | array(Gpsi) | O | 1..N | This IE may be present for a group subscription. When present, this IE shall carry the GPSI of the group member UE(s) that are included for the group subscription. | DGEM |
| gpsi | Gpsi | C | 0..1 | Generic Public Subscription Identifier (NOTE 2) (NOTE 4) |  |
| pei | Pei | C | 0..1 | Permanent Equipment Identifier (NOTE 2) (NOTE 4) |  |
| anyUE | boolean | C | 0..1 | This IE shall be present and set to true if the event subscription is applicable to any UE. Default value "FALSE" is used, if not present. The attribute shall be set to "TRUE", when the AMF event type is "SNSSAI_TA_MAPPING_REPORT". (NOTE 2) |  |
| options | AmfEventMode | O | 0..1 | This IE may be included if the NF service consumer wants to describe how the reports of the event have to be generated. The absence of this IE, when creating an AMF event subscription or when transferring the UE context to another AMF, shall be interpreted as a "ONE_TIME" AMF event trigger. |  |
| sourceNfType | NFType | C | 0..1 | This IE should be present for a subscription that is created by an "intermediate NF" (e.g. UDM) on behalf of a "source NF" (e.g. NEF). When present, it shall contain the NF type of the "source NF". |  |
| termNotifyInd | boolean | O | 0..1 | When present this IE shall indicate whether the notification of event subscription termination from the AMF is requested by the NF consumer. - true: Event subscription termination notification requested - false (default): Event subscription termination notification not requested | STEN |
| NOTE 1: When an NF Service Consumer subscribes on behalf of another NF, the Notification URI identifies a resource under the authority of the other NF. NOTE 2: One and only one of the followings shall be included: information about a single UE (i.e. SUPI, GPSI, PEI) or groupId, or anyUE set to "TRUE". NOTE 3: Same values of "subsChangeNotifyUri" and "subsChangeNotifyCorrelationId" shall be provided by an NF service consumer to all the serving AMF if the subscriptions apply to a group and triggered by one subscription from another NF. This allows the NF service consumer to associate the subscription Id creation notifications received from different serving AMFs to the same group Id subscription. NOTE 4: At most one of these identifier IEs may be present. |  |  |  |  |  |

* * * 2nd Change * * * *

##### 6.2.6.2.24	Type: TargetArea

Table 6.2.6.2.24-1: Definition of type TargetArea

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| taList | array(Tai) | C | 1..N | When present, this IE shall contain the list of TAIs. (NOTE) |
| taiRangeList | array(TaiRange) | C | 1..N | When present, this IE shall contain range(s) of TAIs. (NOTE) |
| anyTa | boolean | C | 0..1 | This IE shall be present and set to true if the event subscription is applicable to any TA. Default value "FALSE" is used, if not present (NOTE) |
| NOTE: One of the taList or taiRangeList or anyTa set to "TRUE" shall be included. |  |  |  |  |

* * * End of Changes * * * *