3GPP TSG-CT WG4 Meeting #131	C4-254072

# Sophia Antipolis, France; 13th – 17th October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.510 | CR | 1245 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of charging pattern indicator in ChfInfo |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | SBIProtoc19 |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This CR is the resubmission of the postponed CR in C4-253383. The Nnrf_NFDiscovery service invoked by CHF service consumer (e.g. SMF, AMF) for retrieving CHF instance(s) and CHF service(s) instance(s) satisfies a number of filter criteria. According to the "NFprofile" and "ChfInfo", the NF consumer is currently able to discover the CHF instances based on user information (e.g. supi, gpsi, preferred-locality/ext-preferred-locality), PLMN information (e.g. PlmnRange), service information (e.g. target-nf-service-set-id). Currently the discovery based on CHF Group ID is supported which is based on the UE / user subscription as also stated in SA2 LS C4-253030/S2-2504504: One or both of CHF Set ID and CHF Group ID could be present. SA2 would emphasize that CHF Set ID is used for binding aspects while CHF Group ID is used for scaling aspects (i.e., one or more CHF instances manage a specific set of SUPIs); hence, they are not mutually exclusive. However, there is another scenario that can benefit the operator, in which an NF (e.g. SMF and PCF, etc) is to discover a CHF, dedicated to pre- or post-paid, directly and not based on a SUPI or User / UE-related discovery. For this scenario, an additional label or indicator (i.e. Prepaid/Postpaid) to the CHF itself is required. Therefore, this CR proposes to add a charging pattern indicator in the "ChfInfo" to enable the NF consumers to perform CHF discovery and selection based on the operator’s charging properites. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the "chargingPatternInd" in the "ChfInfo". Update OpenAPI accordingly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The NRF does not support an NF (e.g. SMF and PCF, etc) to discover a CHF, dedicated to pre- or post-paid, directly and not based on a SUPI or User / UE-related discovery. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.6.2.32, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible feature to the following APIs: TS29510_Nnrf_NFDiscovery.yaml TS29510_Nnrf_NFManagement.yaml TS29520_Nnwdaf_AnalyticsInfo.yaml TS29520_Nnwdaf_DataManagement.yaml TS29520_Nnwdaf_RoamingData.yaml TS29574_Ndccf_DataManagement.yaml TS29575_Nadrf_DataManagement.yaml TS29576_Nmfaf_3caDataManagement.yaml TS29576_Nmfaf_ContextManagement.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 6.1.6.2.32	Type: ChfInfo

Table 6.1.6.2.32-1: Definition of type ChfInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| supiRangeList | array(SupiRange) | O | 1..N | List of ranges of SUPIs that can be served by the CHF instance. (NOTE 1) |
| gpsiRangeList | array(IdentityRange) | O | 1..N | List of ranges of GPSI that can be served by the CHF instance. (NOTE 1) |
| plmnRangeList | array(PlmnRange) | O | 1..N | List of ranges of PLMNs (including the PLMN IDs of the CHF instance) that can be served by the CHF instance. If not provided, the CHF can serve any PLMN. |
| groupId | NfGroupId | O | 0..1 | Identity of the CHF group that is served by the CHF instance. If not provided, the CHF instance does not pertain to any CHF group. (NOTE 1) |
| primaryChfInstance | NfInstanceId | C | 0..1 | This IE shall be present if the CHF instance serves as a secondary CHF instance of another primary CHF instance. When present, it shall be set to the NF Instance Id of the primary CHF instance. This IE shall be absent if the secondaryChfInstance is present. (NOTE 2, NOTE 3) |
| secondaryChfInstance | NfInstanceId | C | 0..1 | This IE shall be present if the CHF instance serves as a primary CHF instance of another secondary CHF instance. When present, it shall be set to the NF Instance Id of the secondary CHF instance. This IE shall be absent if the primaryChfInstance is present. (NOTE 2, NOTE 3) |
| chargingPatternInd | boolean | O | 0..1 | Indicates the charging pattern of the subscribers, i.e. prepaid or postpaid, supported by the CHF. true: the CHF instance is allocated to support prepaidsubscribers false: the CHF instance is allocated to support post-paid subscribers |
| NOTE 1: If none of these parameters are provided, the CHF can serve any SUPI or GPSI managed by the PLMN of the CHF instance. If "supiRangeList" and "gpsiRangeList" attributes are absent, and "groupId" is present, the SUPIs / GPSIs served by this CHF instance is determined by the NRF (see 3GPP TS 23.501 [2], clause 6.2.6.2). NOTE 2: The NF Service Consumer of the CHF may use these attributes as primary/secondary redundancy mechanism, or alternatively, it may also rely on the availability of an NF Set (or NF Service Set) of CHF Instances (or CHF Service Instances) for the same purpose. NOTE 3: If the CHF does not provide NF set ID or NF Service Set ID in NFProfile, it shall provide one of these attributes. These attributes may be present if the CHF registers an NF set ID or NF service set ID. |  |  |  |  |

* * * Next Change * * * *

# A.2	Nnrf_NFManagement API

openapi: 3.0.0

info:

version: '1.4.0-alpha.4'

title: 'NRF NFManagement Service'

description: |

NRF NFManagement Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.510 V19.3.0; 5G System; Network Function Repository Services; Stage 3

url: 'https://www.3gpp.org/ftp/Specs/archive/29_series/29.510/'

[non-changing parts skipped for clarity]

ChfInfo:

description: Information of a CHF NF Instance

type: object

not:

required: [ primaryChfInstance, secondaryChfInstance ]

properties:

supiRangeList:

type: array

items:

$ref: '#/components/schemas/SupiRange'

minItems: 1

gpsiRangeList:

type: array

items:

$ref: '#/components/schemas/IdentityRange'

minItems: 1

plmnRangeList:

type: array

items:

$ref: '#/components/schemas/PlmnRange'

minItems: 1

groupId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfGroupId'

primaryChfInstance:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

secondaryChfInstance:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

chargingPatternInd:

type: boolean

[non-changing parts skipped for clarity]

* * * End of Changes * * * *