3GPP TSG-CT WG4 Meeting #131	C4-254080

# Sophia Antipolis, France; 13th – 17th October 2025

Source:	ZTE

Title:	6G Study - Enhancements to User Plane Management

Agenda item:	20.2.1

Document for:	DISCUSSION

# 1. Revisit of 5G User Plane Management

SA1 has started the FS_6G_REQ study item to identify use cases and service/operational requirements for 6G system, and SA2 has initiated the FS_6G_ARC study item and addressed the evolvements and enhancements to user plane architecture.

WT#1: Define the overall 6G architecture as collection of capabilities and high level functionalities considering the following sub work tasks and other work tasks to support 6G access network:

1.2.	Study whether and how to support and/or enhance the following aspects in 6G: the SBA framework, network slicing, network sharing, user plane architecture, QoS framework, policy framework, network exposure framework, architecture for specific scenarios e.g. fixed wireless access, localized service access.

Gaps and issues have been identified in existing user plane management framework, especially in the support of cloud/distributed deployment scenarios, and in the support of new services.

From CT4 point of view, it should investigate the limitation and gaps of existing CP/UP function and user plane protocols (i.e., PFCP, GTP-U), and study possible improvements, such as the following:

- Investigate limitations of PFCP/GTP-U protocols and possibly seek improvements

In cloud-native deployment, as the PFCP/GTP-U is not natively supported by cloud infrastructure (e.g. Kubernets, Docker, etc.), it increase the configuration complexity of PFCP/GTP-U (e.g. configuring IP addresses used by N4, and sometime requiring different internal network/interfaces).

Furthermore, when security is required for PFCP/GTP-U interfaces (e.g. N9 roaming), normally it requires the implementation of IPsec or TLS.

CT4 might need to be involved into the investigation on how to reduce the complexity of PFCP/GTP-U configuration and how to improve the PFCP/GTP-U signalling efficiency.

- Support CP/UP full mesh deployment

In the earlier stage of CP/UP deployment, one CP controls mostly only a few UPs. However, in the cloud/distributed deployment, operators have a strong requirement to deploy CP/UP in a full mesh manner.

Figure-1. CP/UP full mesh deployment

The current mechanism of CP/UP management specified in TS29.244 does not well support CP/UP full mesh deployment. Complexities and issues have been identified in a full mesh deployment:

- It requires a simple and efficient way to allow one CP to easily know the configuration details of each UP. In a full mesh deployment, it requires distributing the detailed configuration of one UP to each CP that might connect to this UP;

- It requires an efficient way for UP selection, taking into account of the status (e.g. load info) of each UP. However, in a full mesh deployment, it is difficult for one CP to maintain the status of all connected or possibly to-be-connected UPs;

- It requires an efficient way to handle the CP/UP failure. In a full mesh deployment, in case of UP failure, due to PDU sessions served by one UP could be controlled by multiple CPs, it causes complexity to transfer the impacted PDU sessions to other UP. And a similar situation happens to the CP failure;

- It requires a simple and efficient way for UE IP address allocation. However, in a full mesh deployment, it becomes complex to perform UE IP address allocation, especially when one UE IP pool is shared among multiple CPs/UPs;

- It requires mitigating excessive PFCP signalling exchange (e.g. PFCP heartbeat message) between CP and UP in a full mesh deployment;

- It requires eliminating restrictions in current PFCP protocol, e.g. the CP has to know the network instance of the UP when requesting the UP to allocate the UE IP address.

- Support header enrichment for secure HTTP

In current TS29.244 it only supports header enrichment for unsecure HTTP, while no support for secure HTTP (e.g. TCP/TLS/HTTP2, QUIC/HTTP3).

As it becomes a trend that secure HTTP is widely used for more and more applications, it prevents the operator from attaching certain service user information (e.g. user identification, user location, etc.) to the secure HTTP content, thus those service applications (e.g. controlled or trusted by the operator) requiring such information cannot work well.

To provide a better user experience, CT4 should study how to support the header enrichment for secure HTTP.

- Support multiple VN groups sharing the same configuration of DNN/S-NSSAI

Existing VN group management requires 1:1 mapping between the VN group and DNN/S-NSSAI configuration, while it forces operators to configure different DNN/S-NSSAI for each VN group of a large number of enterprise networks.

To make it easier for operators to promote their enterprise network services, it is necessary to remove the strict restriction between the VN group and DNN/S-NSSAI configuration.

In this case, improvements to PFCP procedures are required. CT4 should study how to decouple the VN group management from the strict mapping to DNN/S-NSSAI configuration.

- Support differentiated treatment for IP packets carrying signalling of operator services (i.e. NAS over IP)

In 5G, LCS-UPP is introduced to route the LCS signalling via user plane while NAS signalling path only acts as bootstrap channel. In 6G, the study of NAS modularization may result in those upper layer services (hosted by operators) originally running on top of NAS being decoupled from current NAS framework and become running on top of user plane.

These IP packets carrying service layer signalling (i.e. originally carried in NAS message) should be detected by the UPF and performed with differentiated treatment (e.g. priority to packet forwarding, exempt from charging, etc.).

However, in current UP management, such differentiated treatment has not been supported. CT4 should study how to handle the IP packet carrying service layer signalling originally carried in NAS message.

- Support UP processing chain to complete a complex task

As nowadays data/media services (e.g. generative AI media service) become more and more complex, operators may tend to deploy some data/media processing capability in the UPF. In such a case, one simple UPF may not support all the required data/media processing capability and may invoke another UPF to perform part of the specific task.

Figure-2. UP processing chain

To support the above UP deployment, existing user plane management needs enhancements to manage the UP processing chain. CT4 should study how to support this special UP deployment scenario.

- Support in-band application information awareness and corresponding treatment (based on SA2 study)

In 5G, the XRM study introduces enhancements to UPF functionality to detect in-band application information (e.g. PDU Set information) and thus perform corresponding treatment.

In 6G, more in-band application information is expected to be detected by the UP so that to trigger the CP to make corresponding rules to handle the detected packets.

CT4 could also study whether the detection of in-band application information other than PDU set information is needed and how to support it.

# 2. Proposed scope of the study

Given those gaps and issues identified in existing user plane management framework, it needs to be studied in CT4 e.g. for the following:

- Investigate the protocol limitations of PFCP/GTP-U protocols

CT4 might be involved in investigating the protocol limitations of PFCP/GTP-U (e.g. deployment complexity in the cloud-native deployment, lack of security e.g. for N9 roaming), and possibly seek improvements to the PFCP/GTP-U protocols.

If this task is performed, coordination with other working groups (e.g. SA3) may be required.

- Investigate new service requirements (from SA2) to CP/UP function

CT4 should also investigate new service requirements to CP/UP function and study potential solutions, based on SA2 requirements on the user plane architecture:

- possibly support new requirement on packet treatment and QoS enforcement specified by stage 2 (e.g. QUIC/MoQ traffic detection, detection and treatment of in-band application information, adaptive QoS enforcement, etc.).

- Improve existing CP/UP function and PFCP procedures

CT4 should investigate the gaps of existing CP/UP function and PFCP procedures, and study potential enhancements:

- study how to reduce the complexity and improve the efficiency when supporting CP/UP full mesh deployment;

- study how to support header enrichment for secure HTTP (e.g. TCP/TLS/HTTP3, QUC/HTTP3);

- study how to differentiate IP packets carrying service layer signalling of NAS over IP (e.g. LCS-UPP);

- study how to support multiple VN groups sharing the configuration of same DNN/S-NSSAI;

- study how to support the UP processing chain to perform a complex task in case that one UP function cannot fulfil the entire processing of the task;