3GPP TSG-CT WG4 Meeting #131	C4-254082

# Sophia Antipolis, France; 13th – 17th October 2025

Source:	ZTE

Title:	6G Study - SBA Evolvement and SBI Protocol Evolution

Agenda item:	20.2.1

Document for:	DISCUSSION

# 1. Revisit of 5G SBA design

SA1 has started the FS_6G_REQ study item to identify use cases and service/operational requirements for 6G system, and SA2 has initiated the FS_6G_ARC study item and addressed the evolvements and enhancements to service based architecture and interfaces.

WT#1: Define the overall 6G architecture as collection of capabilities and high level functionalities considering the following sub work tasks and other work tasks to support 6G access network:

1.2.	Study whether and how to support and/or enhance the following aspects in 6G: the SBA framework, network slicing, network sharing, user plane architecture, QoS framework, policy framework, network exposure framework, architecture for specific scenarios e.g. fixed wireless access, localized service access.

Several challenges and issues have been identified to the SBA framework in the existing 5G system:

- Challenge of TCP/TLS/HTTP2

TCP/TLS/HTTP2 as SBI protocol has achieved significant success in 5G networks, however it still faces several critical challenges such as incomplete resolution of HOL Blocking, high latency of Handshake and inefficient Header Compression.

Meanwhile, QUIC/HTTP3 has been widely used in IT industry. Compared with TCP/TLS/HTTP2, QUIC/HTTP3 provides more flexibility, efficiency and other advanced characteristics. Migrating SBI protocol to QUIC/HTTP3 can significantly improve the signalling efficiency and system robustness.

It should also be noted that due to QUIC encryption mechanism, it might cause difficulty to the operator's signalling tools to inspect the SBI signalling when QUIC/HTTP3 is utilized.

- Over-flexibility of callback URI provision for event notification

To receive event notifications, dynamic callback URI is widely provided during event subscription procedure across various SBI interfaces. On the other hands, static configuration of default notification URI for specific events might also be configured in the NRF.

This over-flexibility results in complexity, inefficiency or even problems:

(a) An subscribing NF needs to generate dynamic callback URI in each event subscription, or has to registers its default notification URI in NRF;

(b) When failure happens to the original NF, it is not possible to receive event notification at the alternative NF as the callback URI of alternative NF is not availablein failure;

(c) It is not possible to send simple event report or aggregated event reports at a single callback URI. To increase the efficiency of event reports, there are requirements to aggregate multiple event reports into one shot of delivery.

A possible way to improve this situation could be:

- not using callback URI for receiving event notification, instead defining particular service operation and dedicated resource structure to receive event notification;

- each event report should at least indicate the following items: event type, event subscription id, sender id.

One generic model could be designed for all kinds of NF to receive event notification: define a generic service operation and the resource structure for receiving event notification, and such generic service operation and resource structure is applicable to all kinds of NF, as shown in figure 1:

Figure-1. Generic resource for receiving notification

Alternatively, a particular service operation and the resource structure could be defined for a specific NF type, e.g. define an SMF service operation and corresponding resource structure to support PDU Session status notification, as shown in figure 2:

Figure-2. SMF specific resource for receiving notification

- Inefficiency of NF status detection and notification

The current mechanism for NF status detection and notification via NRF is observed with latency and inefficiency, especially in the case of NF failure.

Each NF itself is required to periodically refresh its NF status and report its load to the NRF. Meanwhile, the NRF needs to loop the detection of NF status at least for those NFs that have not refreshed their NF status to the NRF. To avoid too much load on the NRF, the periodicity of loop detection by an NRF cannot be set too short, which means the NF failure detection by the NRF is not timely updated. In addition, it also takes time for the NRF to schedule the notification to all interested NFs.

However, in most cases, one NF with an established signalling connection with its peer NF can easily detect the status of the peer side, more quickly than it gets the status notification of its peer NF from the NRF. If allowing one NF to report the NF failure of its peer NF to the NRF, it could fasten the NRF procedure to set the indicated NF to unavailable and quickly trigger notifications to other interested NFs.

An extremely simplified way could be, the NF failure detection by the NRF only affects the new NF discovery. In this case, the NRF would not trigger notification to other interested NFs, while relying on each NF to detect the availability of its peer NF.

- Complicated design in NRF

As per the initial principle of 5G design, using NRF as central repository of NF profiles is to provide a simple and universal way for NF discovery. However, the actual design of NRF creates excessive complexity which has deviated from the original principle.

A typical example is excessive number of NF discovery query parameter. In the latest release of TS29.510 (i.e. v19.4.0), 178 NF discovery query parameters are defined to support kinds of new services or functionalities in 5G. However, among those query parameters, over 50 of them are capability related.

In most cases discovering one particular NF only requires a few key discovery factors. In 4G, discovering an NE only requires a few service parameters as input to the DNS server.

Regarding kinds of capabilities that might be used for NF discovery, in TS29.510, there are diversified ways/styles to indicate certain capability registered in NF profile and might be used for NF discovery:

- SupportedFeatures defined in NF profile;

- SupportedFeatures defined in NF service profile;

- In style of XxxCapability: e.g. NsacfCapablity, V2xCapability, ProSeCapability, A2xCapability, smfOnboardingCapability, smfUPRPCapability, ScpCapability, etc;

- In style of YyyEvent or ZzzId: e.g. EventId, NwdafEvent, EventType, etc;

- In style of UuuInd or VvvInd or WwwSupport: e.g. lcHSupportInd, olcHSupportInd, anyUeInd, vsmfSupportInd, ismfSupportInd, iwkEpsInd, sxaInd, ueIpAddrInd, redundantGtpu, ipups, dataForwarding, geranUtranInd, v2xSupportInd, proseSupportInd, a2xSupportInd, rangingSlPosSupportInd, urspEpsSupport, pruSupportInd, aimlPosInd, uasNfFunctionalityInd, multiMemAfSessQosInd, memberUESelAssistInd, etc.

It creates unnecessary complexity to register kinds of capabilities in various methods, and also introduces complexity to NF discovery.

It should be considered to redesign a unified capability registration mechanism with high efficiency for NF discovery in the 6G study.

One possible simplification could be just utilizing the "SupportedFeatures" mechanism at different granularities, without defining any specific data type for a capability. For example, 3 different levels of supported features could be defined: (a) common supported features for all NF types, (b) supported features for each NF type, (c) supported features for each NF service type. A general mechanism can be designed to the NRF to support dynamic registration of supported features, while the NRF does not need to understand the exact meaning of each supported feature. Following this alteration, all the capabilities related to NF discovery query parameters can be simplified.

- Complexity of SCP to support various communication models

As per the design of 5G SBA framework, SCP needs to support various communication models, i.e. model A, B, C and D.

However, the indirect communication models, especially model D, introduce too much complexity to the SCP. In order to perform delegated NF discovery, the SCP has to support more and more customized HTTP headers to carry new NF discovery parameters. This mechanism does not simplify the NF logic, but rather bring additional complexity to the SCP.

A possible way to reduce the complexity is to remove the delegated discovery from the SCP. Instead, the SCP may act as a proxy to forward the NF discovery of the NF consumer to an appropriate NRF.

# 2. Proposed scope of the study

Several challenges and issues have been identified to SBA framework in the existing 5G system:

- Evolution to QUIC/HTTP3

CT4 should investigate the evolution of SBI protocol from TCP/TLS/HTTP2 to QUIC/HTTP3, possibly including:

- discovery of the supported SBI protocol (HTTP3 or HTTP2) of target NF, especially for the inter-PLMN NRF/SEPP discovery;

- interworking between TCP/TLS/HTTP2 SBA network and QUIC/HTTP3 network, especially the interworking between HTTP2 NRF and HTTP3 NRF, and HTTP2 SEPP and HTTP3 SEPP;

- study how to support the intermediary network node (e.g. operator signalling tools) to inspect the SBI signalling as per operational demand.

- Enhance the event subscription/notification mechanism

CT4 should investigate the drawbacks of existing event subscription/notification mechanism, and consider possible enhancements, possible including:

- study how to simplify the callback mechanism for event notification, and possibly provide a unified service operation for receiving event notification;

- study how to support event report aggregation by using the unified service operation for receiving event notification.

- Enhance the NF status detection mechanism

CT4 should investigate the drawbacks of existing NF status detection and notification via the NRF, and consider possible enhancements, possible including:

- study how to simplify the NF status (especially NF failure) detection and notification mechanism;

- Simplify the design of NRF

CT4 should study the issue of the current mechanism of NRF for NF profile management and NF discovery, and consider possible enhancements, possible including:

- identify the issue (e.g. inefficiency/complexity/mal-functionality, etc) of NF profile structure;

- study how to reduce the complexity and improve the efficiency of NF profile management and NF discovery.

- Simplify the design of SCP

CT4 should study the issue of current SCP for supporting various communication models, and consider possible enhancements, possible including:

- study how to reduce the complexity of SCP for supporting communication modes (especially communication model D).