3GPP TSG-CT WG4 Meeting #131	C4-254083

# Sophia Antipolis, France; 13th – 17th October 2025

Source:	ZTE

Title:	6G Study - Improvements to NF profile management and NF discovery

Agenda item:	20.2.1

Document for:	DISCUSSION

# 1.	General Review of NE/NF discovery in 4G and 5G

In 4G, DNS procedure defined in TS 29.303 is utilized for Network Entity (e.g. MME, SGW and PGW) selection. To support the NE selection requirement in 4G, 3GPP extended the DNS mechanism defined by IETF (e.g. IETF RFC 1034, IETF RFC 1035, etc.) with necessary enhancements (e.g. adopting SRV format record, attaching network capability to NAPTR service field, applying service parameters to DNS query, etc.). With such enhancements, the NE selection mechanism in 4G is still largely relied on static configurations of DNS records, without quick awareness of dynamic service availability.

In 5G, the SBA framework is designed to provide the infrastructure of 5G core system, and the NRF plays a key role as the central repository for NF registration and discovery. Each NF registers or updates its own NF profile in the NRF (or delegate this procedure to the OAM) so that other NF can request the NRF to filter out candidate NFs based on the stored NF profiles in the NRF. Meanwhile, an event subscription and notification mechanism is designed to allow all other interested NFs to be notified of the NF profile change of one particular NF. Such NF discovery and profile synchronization mechanism provides an almost real-time and dynamic reflection of the availability of an NF and its services. Meanwhile, allowing hierarchical NRF provides a scalable management of large amount of NF profiles in the whole PLMN.

However, the over-design to the NRF has been observed. To support the emergence of various 5G services, more and more NFs are introduced as well as more and more features and capabilities of these NFs in later release. Such excessively increased NF and related features/capabilities largely complicates the NRF, making the NF profile management more complex and largely decreasing the efficiency of NF discovery. One typical example is the rapid increase of the NF discovery query parameters.

At the starting point of 6G study, CT4 should investigate the current design of NF profile management and NF discovery via NRF, identify the issues and study possible solutions for improvements.

# 2	Analysis of NRF for NF profile management and NF discovery

In 5G, the NRF acts as a central repository to store the NF profile of each NF. The NF profile contains very detailed profile items where only some of them are related to NF discovery while other majority are NF discovery unrelated. The NF profile management and NF discovery via NRF requires to exchange the entire NF profile between the NRF and interested NF.

1) NF profile management:

In 5G, to support NF profile management of the entire NF profile, it requires:

- the NRF to be equipped with more storage capacity to maintain a large number of NF profiles with considerable number of profile items;

- the registering NF to send to the NRF its entire profile whose size may exceed an expected message size;

- the NRF to constantly update its service logic and database structure in case of introduction of new profile items not NF discovery related.

In contrast, in 4G, there is no such central repository to store the very detailed profile of each NE. The DNS only provides NE selection, configured with only small piece of information of each NE (e.g. IP addresses, a minority of supported interfaces and capabilities of the NE). After successfully retrieving a list of candidate NEs (e.g. SGW/PGW), the requesting NE (e.g. MME) can fetch more detailed information of the target NE (e.g. SGW/PGW features) through direct signaling between the two parties.

2) NF discovery:

In 5G, to support NF discover of the entire NF profile, it requires:

- the NRF to support more and more NF discovery query parameters which introduces more complexity to the NRF handling;

- the NRF to fetch and send a list of candidate NFs and the total size of these NF profiles may easily exceed an expected message size, on answering the NF discovery request;

The Shared Data mechanism is introduced in release 18 to reduce the size of NF profiles returned in the NF discovery. However, to manage and fetch the shared data, it still requires considerable size of signaling exchange between the NRF and the requesting NF.

Comparing with only a few service parameters supported in DNS based NE selection in 4G, the NF discovery in 5G is now facing more and more NF discovery query parameters. In the latest release of TS29.510 (i.e. v19.4.0), 178 NF discovery query parameters are defined to support kinds of new services or functionalities in 5G. However, among those query parameters, over 50 of them are capability related. Multiple query parameters may require the NRF to perform more than one traversal for all NF profiles. It will bring the discovery delay and also increase the load of NRF. Thus the potential improvement of NF discovery should reduce of the number of query parameters.

Possible improvements:

1) One possible way to improve the current situation could be falling back the NF discovery to the 4G similar mechanism.

The NRF only stores partial NF profile items which are necessary for NF discovery (i.e. discovery part of the NF profile), and the rest NF profile items (i.e. detailed part of the NF profile) can be stored in each NF itself. A requesting NF (e.g. AMF#1) performs SMF discovery from the NRF only using limited query parameters, and retrieves the NF profiles of SMF#2/SMF#3 only with the discovery part of the NF profile. Then the AMF#1 picks up SMF#2 and retrieves the detailed part of SMF#2, via signaling interaction between AMF#1 and SMF#2. Following this alteration, it will largely decrease the complexity of NRF for NF profile management and NF discovery.

2) Another possible improvement is to find a better method to organize the profile items and query parameters related to kinds of capabilities.

Diversified methods for kinds of features/capabilities can be unified with different levels of supported features: (a) common supported features for all NF types, (b) supported features for each NF type, (c) supported features for each NF service type. A general mechanism can be designed to the NRF to support dynamic registration of supported features, while the NRF does not need to understand the exact meaning of each supported feature. Following this alteration, all the capabilities related to NF discovery query parameters can be simplified.

Possible working tasks:

Improving the efficiency of the NRF is one of the key consideration in 6G study. Based on the analysis above, the following working tasks should be considered for improving the NF profile management and NF discovery:

WT#1:	Study how to simplify the NF profile management and improve the efficiency.

- WT#1.1: Investigate the necessity of storing the entire NF profile in the NRF.

- WT#1.2: Study the solution to simplify the NF profile management and reduce the signalling load exchanged between the requesting NF and the NRF.

WT#2:	Study how to simplify the NF discovery and improve the efficiency.

- WT#2.1: Investigate the necessity of defining so many NF discovery query parameters, and study the solution to reduce or simplify the unnecessary NF discovery query parameters.

# 3	Analysis of SCP for delegated NF discovery

As per the design of 5G SBA framework, SCP is introduced in Rel-16 for larger scale network, and two indirect communication models, i.e. model C and model D are defined as specified in Annex E of TS 23.501.

With the emergency of various 5G services, more and more NFs are introduced. Even for one particular type of NF, more and more new features are introduced in different releases. The SCP has to upgrade its logic to support more and more customized HTTP headers to carry new NF discovery parameters, so that to perform delegated NF discovery in communication model D.

Although the communication model D can seemly reduce the configuration to each NF with the knowledge of the NRF, it introduces unnecessary complexity to the SCP to perform delegated NF discovery. It also creates a bottleneck at the SCP and may increase the network failure risk due to the SCP overload or failure.

Thus at the beginning of 6G study, it is proposed to consider:

WT#3:	Investigate the communication models of SCP for supporting delegated NF discovery and study the solution to simplify the SCP procedure related to delegated NF discovery.

# 4. Proposal

It is proposed to start the study the improvement of NF profile management and NF discovery at the beginning of 6G study. The working tasks listed in clause 3 will base on the study of enhancements to SBA framework. However, it has no direct dependency on the SBI protocol evolvement.