3GPP TSG-CT WG4 Meeting #131	C4-254085

# Sophia Antipolis, France; 13th – 17th October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.502 | CR | 0901 | rev | - | Current version: | 18.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Offload Identifier Provision by V-SMF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGE_Ph2 |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | If the V-SMF has alread received VPLMN Speciefic Offloading Information and Offload Identifier for a HR-PDU session, it should indicate this to H-SMF when the V-SMF sends Nsmf_PDUSession_Create request for another HR-PDU session. (see S2-2507389/7380 CR#5571/5572). Currently, the PduSessionCreateData includes one "hrsboInfo" attribute (of HrsboInfoFromVplmn type) which further includes "storedOffloadIds" and serveral other attributes representing the V-SMF requesting for HR SBO authorization. However, once the V-SMF determines to reuse the stored offloading policy representing by the Offloading Identifiers, it is not necessary to further including the attributes for V-SMF requesting for HR SBO authorization. Thus, clairification to the attributes presented in "hrsboInfo" for PDU session creation is required. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that if the V-SMF includes the "storedOffloadIds" in the "hrsboInfo" in PduSessionCreateData, other attributes in "hrsboInfo" representing the V-SMF requesting for HR SBO authorization should not be included. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear whether the V-SMF still includes attributes for HR SBO authorization if it indicates the stored offloading identifiers in PduSessionCreateData in an new PDU session establishment. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.6.2.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR does not introduce any change to the OpenAPI files. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * *Start of Change * * * *

##### 6.1.6.2.9	Type: PduSessionCreateData

Table 6.1.6.2.9-1: Definition of type PduSessionCreateData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| supi | Supi | C | 0..1 | This IE shall be present, except if the UE is emergency registered and UICCless. When present, it shall contain the subscriber permanent identify. |  |
| unauthenticatedSupi | boolean | C | 0..1 | This IE shall be present if the SUPI is present in the message but is not authenticated and is for an emergency registered UE. When present, it shall be set as follows: - true: unauthenticated SUPI; - false (default): authenticated SUPI. |  |
| pei | Pei | C | 0..1 | This IE shall be present if the UE is emergency registered and it is either UIClless or the SUPI is not authenticated. For all other cases, this IE shall be present if it is available. When present, it shall contain the permanent equipment identifier. |  |
| pduSessionId | PduSessionId | C | 0..1 | This IE shall contain the PDU Session ID, except during an EPS to 5GS Idle mode mobility or handover using the N26 interface. |  |
| dnn | Dnn | M | 1 | This IE shall contain the requested DNN. The DNN shall be the full DNN (i.e. with both the Network Identifier and Operator Identifier) for a HR PDU session, and it should be the full DNN in LBO and non-roaming scenarios. If the Operator Identifier is absent, the serving core network operator shall be assumed. |  |
| selectedDnn | Dnn | C | 0..1 | This IE shall be present, if another DNN other than the UE requested DNN is selected for this PDU session. When present, it shall contain the selected DNN. The DNN shall be the full DNN (i.e. with both the Network Identifier and Operator Identifier) for a HR PDU session, and it should be the full DNN in LBO and non-roaming scenarios. If the Operator Identifier is absent, the serving core network operator shall be assumed. |  |
| sNssai | Snssai | C | 0..1 | This IE shall be present, except during an EPS to 5GS idle mode mobility or handover using the N26 interface. When present, it shall contain: - the HPLMN S-NSSAI for a HR PDU session, which is mapped from the requested S-NSSAI by the VPLMN; - the HPLMN S-NSSAI for a non-roaming PDU session with an I-SMF, if the Serving PLMN ID is not the same as the PLMN ID of the SUPI; or - the requested S-NSSAI in the serving PLMN for a PDU session with an I-SMF for other scenarios. |  |
| altSnssai | Snssai | C | 0..1 | This IE shall be present during the PDU session establishment procedure if the NF service consumer supports the network slice replacement, and the network slice indicated in the sNssai IE is requested to be replaced. In this case, the NF service consumer shall send the S-NSSAI and the alternative S-NSSAI. See clause 5.15.19 of 3GPP TS 23.501 [2]. | NSRP |
| hplmnSnssai | Snssai | C | 0..1 | This IE shall be present for an LBO PDU session with an I-SMF, except during an EPS to 5GS idle mode mobility or handover using the N26 interface. When present, it shall contain the HPLMN S-NSSAI of the LBO PDU session, which is mapped from the requested S-NSSAI by the VPLMN. | DTSSA |
| vsmfId | NfInstanceId | C | 0..1 | This IE shall be present for a HR PDU session. When present, it shall contain the identifier of the V-SMF. |  |
| ismfId | NfInstanceId | C | 0..1 | This IE shall be present for a PDU session with an I-SMF. When present, it shall contain the identifier of the I-SMF. | DTSSA |
| servingNetwork | PlmnIdNid | M | 1 | This IE shall contain the serving core network operator PLMN ID and, for an SNPN, the NID that together with the PLMN ID identifies the SNPN. |  |
| requestType | RequestType | C | 0..1 | This IE shall be present if the Request type IE is received from the UE for a single access PDU session and if the request refers to an existing PDU session or an existing emergency PDU session. The requestType IE shall not be included for a MA-PDU session establishment request. It may be present otherwise. When present, it shall indicate whether the request refers to a new PDU session or emergency PDU session, or to an existing PDU session or emergency PDU session. For request sent from AMF, this IE shall be set based on the requestType received. |  |
| epsBearerId | array(EpsBearerId) | C | 1..N | This IE shall be present during an EPS to 5GS Idle mode mobility or handover preparation using the N26 interface. When present, it shall contain the list of EPS bearer Id(s) received from the MME. |  |
| pgwS8cFteid | Bytes | C | 0..1 | This IE shall be present during an EPS to 5GS Idle mode mobility or handover preparation using the N26 interface. When present, it shall contain Base64-encoded characters, encoding the PGW S8 F-TEID for Control Plane as specified in Figure 8.22-1 of 3GPP TS 29.274 [16] (starting from octet 1), received from the MME. |  |
| vsmfPduSessionUri | Uri | C | 0..1 | This IE shall be present for a HR PDU session. When present, it shall include the callback URI representing the PDU session in the V-SMF. |  |
| ismfPduSessionUri | Uri | C | 0..1 | This IE shall be present for a PDU session with an I-SMF. When present, it shall include the callback URI representing the PDU session in the I-SMF. | DTSSA |
| vcnTunnelInfo | TunnelInfo | C | 0..1 | This IE shall be present for a HR PDU session, except for: - an EPS to 5GS handover using N26 interface; and - when Control Plane CIoT 5GS Optimisation is enabled and data delivery via NEF is selected for this PDU session. When present, this IE shall contain the DL N9 tunnel CN information of the visited CN side, i.e. V-UPF. |  |
| icnTunnelInfo | TunnelInfo | C | 0..1 | This IE shall be present for a PDU session involving an I-SMF, except for: - an EPS to 5GS handover using N26 interface with I-SMF insertion; and - when Control Plane CIoT 5GS Optimisation is enabled and data delivery via NEF is selected for this PDU session. When present, this IE shall contain the DL N9 tunnel CN information of the I-UPF controlled by the I-SMF. | DTSSA |
| n9ForwardingTunnelInfo | TunnelInfo | C | 0..1 | This IE shall be present during Service Request procedures with I-SMF insertion, if buffered DL data is available at the I-UPF that is controlled by the SMF (see clause 4.23.4 in 3GPP TS 23.502 [3]). When present, this IE shall contain the N9 tunnel information of the I-UPF controlled by the I-SMF. | DTSSA |
| additionalCnTunnelInfo | TunnelInfo | C | 0..1 | This IE shall be present if a MA PDU session is requested or if the PDU session is allowed to be upgraded to a MA PDU session, and the UE is registered over both 3GPP access and Non-3GPP access. When present, it shall contain additional DL N9 tunnel CN information of the UPF controlled by the V-SMF or I-SMF. | MAPDU |
| anType | AccessType | M | 1 | This IE shall indicate the Access Network Type to which the PDU session is to be associated. |  |
| additionalAnType | AccessType | C | 0..1 | This IE shall indicate the additional Access Network Type to which the PDU session is to be associated. This IE shall be present if a MA-PDU session is requested and the UE is registered over both 3GPP access and Non-3GPP access. | MAPDU |
| ratType | RatType | C | 0..1 | This IE shall be present and indicate the RAT Type used by the UE, if available. |  |
| ueLocation | UserLocation | C | 0..1 | This IE shall contain the UE location information (see clause 5.2.3.4), if it is available. (NOTE 1) |  |
| ueTimeZone | TimeZone | C | 0..1 | This IE shall contain the UE Time Zone, if it is available. |  |
| addUeLocation | UserLocation | O | 0..1 | Additional UE location. This IE may be present, if anType indicates a non-3GPP access and a valid 3GPP access user location information is available. When present, it shall contain: - the last known 3GPP access user location (see clause 5.2.3.4); and - the timestamp, if available, indicating the UTC time when the addUeLocation information was acquired. (NOTE 1) |  |
| gpsi | Gpsi | C | 0..1 | This IE shall be present if it is available. When present, it shall contain the user's GPSI. |  |
| n1SmInfoFromUe | RefToBinaryData | C | 0..1 | This IE shall be present if the V-SMF or I-SMF has received known N1 SM information from the UE that does not need to be interpreted by the V-SMF or I-SMF. When present, this IE shall reference the n1SmInfoFromUe binary data (see clause 6.1.6.4.4). |  |
| unknownN1SmInfo | RefToBinaryData | C | 0..1 | This IE shall be present if the V-SMF or I-SMF has received unknown N1 SM information from the UE. When present, this IE shall reference the unknownN1SmInfo binary data (see clause 6.1.6.4.4). |  |
| supportedFeatures | SupportedFeatures | C | 0..1 | This IE shall be present if at least one feature defined in clause 6.1.8 is supported. |  |
| hPcfId | NfInstanceId | O | 0..1 | This IE may be used by V-SMF to indicate the home PCF selected by the AMF for the UE to the H-SMF, for a HR PDU session. When present, this IE shall contain the identifier of the H-PCF selected by the AMF for the UE (for UE Policy Control). |  |
| pcfId | NfInstanceId | O | 0..1 | This IE may be used by I-SMF to indicate the (V-)PCF selected by the AMF for the UE to the SMF, for a PDU session with an I-SMF. When present, this IE shall contain the identifier of the PCF (for Access and Mobility Policy Control and/or UE Policy Control) in non-roaming scenarios, or the V-PCF (for Access and Mobility Policy Control) in LBO roaming scenarios. | DTSSA |
| pcfGroupId | NfGroupId | O | 0..1 | This IE may be present in non-roaming and HR roaming scenarios. When present, this IE shall contain the identity of the (home) PCF group serving the UE for Access and Mobility Policy and/or UE Policy. |  |
| pcfSetId | NfSetId | O | 0..1 | When present, it shall contain the NF Set ID of the H-PCF indicated by the hPcfId IE or the (V-)PCF indicated by the pcfId IE. |  |
| hoPreparationIndication | boolean | C | 0..1 | This IE shall be present during an EPS to 5GS handover preparation using the N26 interface or during N2 handover preparation with I-SMF insertion. When present, it shall be set as follows: - true: an EPS to 5GS handover preparation or N2 handover preparation with I-SMF is in progress; the PGW-C/SMF shall not switch the DL user plane of the PDU session yet. - false: there is no on-going EPS to 5GS handover preparation or N2 handover preparation with I-SMF in progress. If a handover preparation was in progress, the handover has been completed. The PGW-C/SMF shall switch the DL user plane of the PDU session using the N9 tunnel information that has been received in the vcnTunnelInfo or icnTunnelInfo. It shall be set to "true" during an EPS to 5GS handover preparation using the N26 interface or during N2 handover preparation with I-SMF insertion. |  |
| selMode | DnnSelectionMode | C | 0..1 | This IE shall be present if it is available. When present, it shall be set to: - "VERIFIED", if the requested DNN provided by UE or the selected DNN provided by the network corresponds to an explicitly subscribed DNN; or - "UE_DNN_NOT_VERIFIED", if the requested DNN provided by UE corresponds to the usage of a wildcard subscription; or - "NW_DNN_NOT_VERIFIED", if the selected DNN provided by the network corresponds to the usage of a wildcard subscription. If both the requested DNN (i.e. dnn IE) and selected DNN (i.e. selected Dnn IE) are present, the selMode shall be related to the selected DNN. |  |
| alwaysOnRequested | boolean | C | 0..1 | This IE shall be present and set to true if the UE requests to setup an always-on PDU session and this is allowed by local policy in the V-SMF or I-SMF. When present, it shall be set as follows: - true: request for an always-on PDU session - false (default): not a request for an always-on PDU session |  |
| udmGroupId | NfGroupId | O | 0..1 | When present, it shall indicate the identity of the UDM group serving the UE. |  |
| routingIndicator | string | O | 0..1 | When present, it shall indicate the Routing Indicator of the UE. |  |
| hNwPubKeyId | integer | O | 0..1 | When present, it shall indicate the Home Network Public Key Identifier of the UE. (NOTE 2) |  |
| epsInterworkingInd | EpsInterworkingIndication | O | 0..1 | This IE may be present if the indication has been received from AMF and is allowed to be forwarded to H-SMF by operator configuration. When present, this IE shall indicate whether the PDU session may possibly be moved to EPS or EPC/ePDG and whether N26 interface to be used during EPS interworking procedures. |  |
| vSmfServiceInstanceId | string | O | 0..1 | When present, this IE shall contain the serviceInstanceId of the V-SMF service instance serving the PDU session. This IE may be used by the H-SMF to identify PDU sessions affected by a failure or restart of the V-SMF service (see clauses 6.2 and 6.3 of 3GPP TS 23.527 [24]). |  |
| iSmfServiceInstanceId | string | O | 0..1 | When present, this IE shall contain the serviceInstanceId of I-SMF service instance serving the PDU session. This IE may be used by the SMF to identify PDU sessions affected by a failure or restart of the I-SMF service (see clauses 6.2 and 6.3 of 3GPP TS 23.527 [24]). | DTSSA |
| recoveryTime | DateTime | O | 0..1 | Timestamp (in UTC) when the V-SMF or I-SMF service instance serving the PDU session was (re)started (see clause 6.3 of 3GPP TS 23.527 [24]). |  |
| roamingChargingProfile | RoamingChargingProfile | O | 0..1 | Roaming Charging Profile applicable in the VPLMN (see clauses 5.1.9.1, 5.2.1.7 and 5.2.2.12.2 of 3GPP TS 32.255 [25]). |  |
| chargingId | string | O | 0..1 | Charging ID (see clauses 5.1.9.1 of 3GPP TS 32.255 [25]). The string shall encode the Charging ID (32-bit unsigned integer value, with maximum value "4294967295") in decimal representation. Pattern: '^(0¦([1-9]{1}[0-9]{0,9}))$' (NOTE 4) |  |
| smfChargingId | SmfChargingId | C | 0..1 | String based Charging ID (see 3GPP TS 32.255 [25]). This IE shall be present when the chargingId IE is present and the "SCID" feature is supported by both SMFs. If the V-SMF cannot determine, during a HR PDU session establishment, whether the H-SMF supports the SCID feature (e.g. if the AMF did not indicate the features supported by the anchor SMF), the V-SMF shall include this IE and determine, upon receiving the response from the H-SMF, whether the H-SMF supports the SCID feature and then act accordingly (e.g. only use the Charging ID if the H-SMF does not support the String based Charging ID). When present, this IE shall encode the String based Charging ID of the PDU session and the base Charging ID segment shall be idential to the value carried in the chargingId IE. | SCID |
| oldPduSessionId | PduSessionId | C | 0..1 | This IE shall be present if this information is received from the UE and the same SMF is selected for SSC mode 3. When present, it shall contain the old PDU Session ID received from the UE. See clauses 4.3.2.2.1 and 4.3.5.2 of 3GPP TS 23.502 [3]. |  |
| epsBearerCtxStatus | EpsBearerContextStatus | C | 0..1 | This IE shall be present during an EPS to 5GS idle mode mobility using the N26 interface, if received in the Create SM Context request. When present, it shall be set to the value received in the Create SM Context request. |  |
| amfNfId | NfInstanceId | C | 0..1 | This IE shall be present if it is received in the Create SM Context request, unless the PDU session is related to regulatory prioritized service. When present, it shall contain the identifier of the serving AMF. |  |
| guami | Guami | C | 0..1 | This IE shall be present if the amfNfId is present. When present, it shall contain the serving AMF's GUAMI. |  |
| maxIntegrityProtectedDataRateUl | MaxIntegrityProtectedDataRate | C | 0..1 | This IE shall be present if it is available. When present, it shall indicate the maximum integrity protected data rate supported by the UE for uplink. |  |
| maxIntegrityProtectedDataRateDl | MaxIntegrityProtectedDataRate | C | 0..1 | This IE shall be present if it is available. When present, it shall indicate the maximum integrity protected data rate supported by the UE for downlink. |  |
| cpCiotEnabled | boolean | C | 0..1 | This IE shall be present with the value "True" if the "5gCiotCpEnabled" attribute is received with "True" value in SM Context Create request, indicating the Control Plane CIoT 5GS Optimisation is enabled for the PDU session (see 3GPP TS 23.502 [3], clause 4.3.2.2.2). When present, it shall be set as follows: - True: Control Plane CIoT 5GS Optimisation is enabled. - False (default): Control Plane CIoT 5GS Optimisation is not enabled. | CIOT |
| cpOnlyInd | boolean | C | 0..1 | This IE shall be present with the value "True", if the PDU session shall only use Control Plane CIoT 5GS Optimisation (see clause 5.31.4.1 of 3GPP TS 23.501 [2]). When present, it shall be set as follows: - True: the PDU session shall only use Control Plane CIoT 5GS Optimisation - False (default): the PDU session is not constrained to only use Control Plane CIoT 5GS Optimisation. | CIOT |
| invokeNef | boolean | C | 0..1 | This IE shall be present with value "True", if Control Plane CIoT 5GS Optimisation is enabled and data delivery via NEF is selected for the PDU session. When present, it shall be set as follows: - True: Data delivery via NEF is selected. - False (default): Data delivery via NEF is not selected. | CIOT |
| maRequestInd | boolean | C | 0..1 | This IE shall be present if a MA-PDU session is requested to be established by the UE. When present, it shall be set as follows: - True: a MA-PDU session is requested - False (default): a MA-PDU session is not requested | MAPDU |
| maNwUpgradeInd | boolean | C | 0..1 | This IE shall only be present if the PDU session is allowed to be upgraded to MA PDU session (see clause 4.22.3 of 3GPP TS 23.502 [3]). When present, it shall be set as follows: - True: the PDU session is allowed to be upgraded to MA PDU session - False (default): the PDU session is not allowed to be upgraded to MA PDU session When maRequestInd is present and set to "true", this IE shall not be present. | MAPDU |
| dnaiList | array(Dnai) | C | 1..N | This IE shall be present over N16a if an I-SMF is inserted into a PDU session during the following procedures: PDU session establishment, Registration, Service Request, Xn based handover, Inter NG-RAN node N2 based handover (see clause 4.23 of 3GPP TS 23.502 [3]). When present, it shall include the list of DNAIs supported by the I-SMF. | DTSSA |
| presenceInLadn | PresenceState | C | 0..1 | This IE shall be present during Xn based handover with I-SMF insertion, if the DNN corresponds to a LADN. When present, it shall be set to "IN" or "OUT" to indicate that the UE is in or out of the LADN service area. | DTSSA |
| secondaryRatUsageInfo | array(SecondaryRatUsageInfo) | O | 1..N | This IE may be present to report usage data for a secondary RAT for QoS flows and/or the whole PDU session. | DTSSA |
| smallDataRateStatus | SmallDataRateStatus | C | 0..1 | This IE shall be present if the small data rate control status is received from AMF, see clause 5.31.14.3 of 3GPP TS 23.501 [2] and clause 4.3.2.2.2 of 3GPP TS 23.502 [3]. | CIOT |
| apnRateStatus | ApnRateStatus | C | 0..1 | This IE shall be present, if the APN rate control status (APN rates are shared by all PDN connections of the UE to this APN) is received from the AMF, see clause 4.7.7.3 in 3GPP TS 23.401 [33] and clause 4.11.5.3 in 3GPP TS 23.502 [3]. | CIOT |
| dlServingPlmnRateCtl | integer | C | 0..1 | This IE shall be present if Serving PLMN Rate Control for downlink data packets is enabled in the PLMN and Control Plane CIoT 5GS Optimisation is enabled for the PDU session. When present, this IE shall contain the maximum allowed number of Downlink NAS Data PDUs per deci hour of the serving PLMN, as specified in clause 5.31.14.2 of 3GPP TS 23.501 [2]. Minimum: 10 | CIOT |
| upSecurityInfo | UpSecurityInfo | C | 0..1 | This IE shall be present if received from NG-RAN during Xn handover procedure with I-SMF Insertion (see clause 5.2.2.7.5). When present, this IE shall contain the User Plane Security Information associated to the PDU session. See clause 9.3.1.60 of 3GPP TS 38.413 [9]. | DTSSA |
| vplmnQos | VplmnQos | C | 0..1 | This IE shall be present for a HR PDU session, if the V-SMF supports the VQOS feature and if VPLMN QoS constraints are required for the PDU session. When present, this IE shall contain the QoS constraints from the VPLMN. | VQOS |
| oldSmContextRef | Uri | C | 0..1 | This IE shall be present, if it is received in the Create SM Context request. When present, this IE shall contain the identifier of the SM Context resource in the old SMF. | EnEDGE |
| redundantPduSessionInfo | RedundantPduSessionInformation | C | 0..1 | This IE shall be present for a PDU session with an I-SMF, if an RSN and/or PDU Session Pair ID was received from the UE. | DCE2ER |
| oldPduSessionRef | Uri | C | 0..1 | This IE shall be present, if it is received in the Create SM Context request. When present, this IE shall contain the URI of the PDU session resource in the old SMF. The URI shall be an absolute URI, including apiRoot (see clause 6.1.3.6.2). | EnEDGE |
| smPolicyNotifyInd | boolean | C | 0..1 | This IE shall be included by I-SMF to SMF, if received from AMF. When present, this IE shall indicate whether the SM Policy Association Establishment and Termination events shall be reported for the PDU session by the PCF for the SM Policy to the PCF for the UE: - true: SM Policy Association Establishment and Termination events shall be reported - false (default): SM Policy Association Establishment and Termination events is not required (NOTE 3) | SPAE |
| pcfUeCallbackInfo | PcfUeCallbackInfo | C | 0..1 | This IE shall be present when the smPolicyNotifyInd IE is present with value true. When present, this IE shall contain the callback information of the PCF for the UE to receive SM Policy Association Establishment and Termination events notification from the PCF for the SM Policy. (NOTE 3) | SPAE |
| satelliteBackhaulCat | SatelliteBackhaulCategory | O | 0..1 | This IE may be present if the V-SMF/I-SMF supports the 5GSAT feature and the satelliteBackhaulCat IE has been received from the AMF. When present, this IE shall indicate the value received from the AMF. | 5GSAT |
| upipSupported | boolean | C | 0..1 | This IE shall be present during the PDU session establishment procedure if the UE supports User Plane Integrity Protection with EPS and if the AMF supports the related functionality. It may be present otherwise. When present, this IE shall be set as follows: - true: User Plane Integrity Protection with EPS is supported; - false (default): User Plane Integrity Protection with EPS is not supported. | UPIPE |
| upCnxState | UpCnxState | C | 0..1 | This IE shall be present to indicate that the User Plane resource for the PDU session is going to be established by the I-SMF/V-SMF, during a service request procedure with I-SMF/V-SMF insertion (see clause 4.23.4.3 of 3GPP TS 23.502 [3]). When present, this IE shall be set as specified in clause 5.2.2.7.6. |  |
| disasterRoamingInd | boolean | O | 0..1 | This IE may be set during the PDU session establishment or a V-SMF insertion procedure when the V-SMF is indicated by the AMF that the UE is registered for Disaster Roaming service. When present, this IE shall be set as follows: - true: the UE is registered for Disaster Roaming service - false (default): the UE is not registered for Disaster Roaming service |  |
| hrsboInfo | HrsboInfoFromVplmn | C | 0..1 | This IE shall be present in HR roaming scenarios if the V-SMF requests HR SBO authorization. When present, this IE shall Include the information for HR-SBO. (NOTE X) | HR-SBO |
| ecsAddrConfigInfos | array(EcsAddrConfigInfo) | C | 1..N | This IE shall be included by V-SMF to SMF, if received from NEF. When present, this IE shall contain the ECS Address Configuration Information Parameters. See 3GPP TS 23.548 [39]. | HR-SBO |
| pduSetSupportInd | boolean | C | 0..1 | This IE shall be included by I-SMF to SMF for a PDU session with an I-SMF if the NG-RAN has indicated it supports the PDU Set based handling during a Xn based inter NG-RAN handover with the I-SMF insertion. When present, this IE shall indicate whether the PDU Set based handling is supported by the NG-RAN: - true: the PDU Set based handling is supported - false(default): the PDU Set based handling is not supported | PDUSH |
| ecnMarkingCongestionInfoStatus | array(EcnMarkingCongestionInfoStatus) | C | 1..N | This IE shall be present when the V/I-SMF receives the ECN Marking or Congestion Monitoring Reporting Status during an I/V-SMF insertion procedure, e.g. during an N2 based handover or an Xn based handover. When present, this IE shall contain a list of QoS flows with status for QoS monitoring for congestion information or for ECN marking for L4S. | EMECI |
| NOTE 1: In shared networks, the PLMN ID that is communicated in this IE shall be that of the selected Core Network Operator. NOTE 2: If present, this attribute shall be used together with routingIndicator. This attribute is only used by the HPLMN in roaming scenarios. NOTE 3: If the smPolicyNotifyInd IE is received with the value "true", the SMF shall forward the callback information of the PCF for the UE to the PCF for SM Policy during SM Policy Association Establishment. See clause 4.3.2.2.1 of 3GPP TS 23.502 [3]. NOTE 4: Usage of Charging ID with Uint32 value for roaming scenarios may lead to Charging ID collision between SMFs. NOTE X: When requesting new PDU session establishment, if the V-SMF has got VPLMN specific offloading information associated with offload identifier, the V-SMF may include the storedOffloadIds attribute in the hrsboInfo attribute while omitting other attributes in the hrsboInfo related to V-SMF requesting HR SBO authorization. |  |  |  |  |  |

* * * End of Changes * * * *

* * * For Information * * * *

##### 6.1.6.2.72	Type: HrsboInfoFromVplmn

Table 6.1.6.2.72-1: Definition of type HrsboInfoFromVplmn

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| hrsboAuthReqInd | boolean | C | 0..1 | This IE shall be present, for a HR PDU session, if the V-SMF requests HR SBO authorization: - in a Create Request; and - in an Update Request sent during a V-SMF insertion (i.e. H-PLMN to V-PLMN mobility) and during an Inter-PLMN V-SMF change (i.e. mobility between different V-PLMNs). When present, this IE shall be set to true to indicate that authorization of Session Breakout for HR Session in VPLMN is requested. |
| vEasdfAddr | IpAddress | C | 0..1 | This IE shall be present during a request for HR-SBO authorization, when using the EAS Discovery procedure with V-EASF for HR-SBO (see clause 6.7.2.3 of 3GPP TS 23.548 [39]) and if available, or if the V-EASDF IP address previously signaled to the H-SMF needs to be changed e.g. as a result of a V-SMF change. When present, this IE shall contain the IP Address of the (new) V-EASDF. |
| localDnsAddr | IpAddress | O | 0..1 | This IE may be present during the EAS Discovery Procedure with Local DNS for HR-SBO (see clause 6.7.2.4 of 3GPP TS 23.548 [39]) after UL-CL/BP insertion. When present, it shall be set to the address of the Local DNS Server/Resolver. |
| easRediscoveryInd | boolean | C | 0..1 | This IE shall be present, for a HR PDU session, if the UE indicates the support of EAS rediscovery and the V-SMF requests EAS rediscovery in an Update Request sent during a V-SMF insertion (i.e. H-PLMN to V-PLMN mobility) and during an Inter-PLMN V-SMF change (i.e. mobility between different V-PLMNs). When present, this IE shall be set to true to indicate that EAS rediscovery is requested. |
| easInfoToRefresh | EasInfoToRefresh | C | 0..1 | This IE shall be present if the easRediscoveryInd is set to true and the V-SMF receives the information in SM context from source V-SMF or H-SMF. When present, it shall contain the EAS information to be refreshed for EAS re-discovery. |
| storedOffloadIds | array(OffloadIdentifier) | C | 1...N | The IE shall be present when the V-SMF service instance has stored a list of offload identifiers which were received from any H-SMF from the Home PLMN during previous signalling procedures for this HR-SBO PDU session and also for other HR-SBO PDU sessions. When present, this IE shall contain the list of offload identifiers of the HPLMN that are known by the V-SMF service instance. (NOTE 1) |
| easIpReplSupportInd | boolean | O | 0..1 | When present, this IE shall indicate the V-SMF capability to support EAS IP Replacement at the VPLMN for HR-SBO traffic. true: supported false (default): not supported |
| vEasdfSecurityInfo | array(DnsServerSecurityInformation) | O | 1..N | When present, this IE shall contain one or more DNS server security information with length of two octets container(s), set with the V-EASDF security information. (NOTE 2) |
| localDnsSecurityInfo | array(DnsServerSecurityInformation) | O | 1..N | When present, this IE shall contain one or more DNS server security information with length of two octets container(s), set with the Local DNS Server/Resolver security information. (NOTE 2) |
| NOTE 1: The H-SMF assumes that the V-SMF does not know any Offload Identifiers of the HPLMN if this IE is absent. NOTE 2: The V-SMF shall ensure that the V-EASDF security information or Local DNS Server/Resolver security information that it provides to the H-SMF is consistent with the DNS security protocol(s) supported by the UE that is received in the PCO IE from the UE. |  |  |  |  |

********TEXT SKIPPED********

* * * End of Information * * * *