3GPP TSG-CT WG4 Meeting #131	C4-254094

# Sophia Antipolis, France; 13th – 17th October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.176 | CR | 0043 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | ReplaceHttpUrl is only applicable to BDC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The ReplaceHttpUrl is only applicable to BDC establishment. However, the existing description might cause misleading of including it in the case of ADC establishment. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that the ReplaceHttpUrl shall be included only for BDC establishment. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | ReplaceHttpUrls might be included in the case of ADC establishment, while it is meaningless. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.6.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR does not introduce any change to OpenAPI files. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 6.1.6.2.5	Type: DcMedia

Table 6.1.6.2.5-1: Definition of type DcMedia

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| mediaProxyConfig | MediaProxy | M | 1 | Represents the media proxy configuration on the MF. The value shall be set to "HTTP_PROXY" if the mediaId represents the bootstrap data channel. The value shall be set to "DC_APPLICATION_PROXY" if the mediaId represents the network initiated P2P application data channel. |  |
| replaceHttpUrl | map(ReplaceHttpUrl) | C | 0..1 | A map (list of key-value pairs where streamId is used as key of the map) of ReplacementHttpUrl. Represents URL for the specific IMS subscriber when requesting the application list (e.g. graphical user interface) via the MDC1 interface. It shall be included if and only if the mediaId represents bootstrap data channel and streamId is 0 and 100. The streamId attribute within the ReplaceHttpUrl data type serves as the key of the map. |  |
| mdc1Info | Mdc1Info | C | 0..1 | Represents the information of MDC1 interface. It shall be contained when the mediaId represents a bootstrap data channel. |  |
| mdc2Into | Mdc2Info | C | 0..1 | Represents the information of MDC2 interface. It shall be contained when the mediaId represents a P2A/A2P application data channel. |  |
| streams | map(DcStream) | M | 1..N | A map (list of key-value pairs where streamId is used as key of the map) of DcStream. Represents the data channel mapping and configuration information when originating/terminating data channel media flows on the Mb interface. |  |
| maxMessageSize | MaxMessageSize | O | 0..1 | Represents the maximum size of to be expected. |  |
| localDcEndpoint | DcEndpoint | C | 0..1 | Represents the DC endpoint for the Data Channel. It will be allocated by MF. It shall be contained in the response when creating a new media stream, i.e. Nmf_MRM_Create operation or Nmf_MRM_Update operation with adding a new termination or adding a media in an existing termination. (NOTE) |  |
| remoteDcEndpoint | DcEndpoint | C | 0..1 | Represents the DC Endpoint for the Data Channel on the Mb interface. It shall be contained in the request of creating a new media stream, i.e. Nmf_MRM_Create operation or Nmf_MRM_Update operation with adding a new termination or adding a media in an existing termination. It can be NULL when originating a data channel media. (NOTE) |  |
| interworkingInfo | InterworkingInstruction | O | 0..1 | Represents instructions on how to interwork to other media types. |  |
| NOTE: The IE cannot be changed once the media has been established. |  |  |  |  |  |

* * * End of Change * * * *