3GPP TSG-CT WG4 Meeting #131	C4-254095

# Sophia Antipolis, France; 13th – 17th October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.244 | CR | 0999 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Report of Available Bitrate |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2 |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The SMF may indicate in the "Requested QoS Monitoring" IE to request the UPF to report downlink/uplink available bitrate. However, the UPF report of detected downlink/uplink available bitrate are missing from the "QoS Monitoring Measurement" IE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add downlink/uplink available bitrate report into the "QoS Monitoring Measurement" IE, so that the UPF can report the detected available bitrate. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UPF receives instruction from the UPF to detect downlink/uplink avaialble bitrate, while is not possible to report the detected downlink/uplink available bitrate. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.171 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 8.2.171	QoS Monitoring Measurement

The QoS Monitoring Measurement IE contains the QoS measurements from the UP function. It shall be encoded as shown in Figure 8.2.171-1.

|  |  | Bits |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Octets | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | 1 to 2 | Type = 248 (decimal) |  |  |  |  |  |  |  |  |
|  | 3 to 4 | Length = n |  |  |  |  |  |  |  |  |
|  | 5 | ULDR | DLDR | ULCI | DLCI | PLMF | RPPD | ULPD | DLPD |  |
|  | 6 | Spare |  |  |  |  |  | ULAB | DLAB |  |
|  | m to (m+3) | Downlink packet delay |  |  |  |  |  |  |  |  |
|  | p to (p+3) | Uplink packet delay |  |  |  |  |  |  |  |  |
|  | q to (q+3) | Round trip packet delay |  |  |  |  |  |  |  |  |
|  | r to (r+1) | Downlink Congestion information |  |  |  |  |  |  |  |  |
|  | s to (s+1) | Uplink Congestion information |  |  |  |  |  |  |  |  |
|  | t to (t+4) | Average downlink data rate |  |  |  |  |  |  |  |  |
|  | u to (u+4) | Average uplink data rate |  |  |  |  |  |  |  |  |
|  | v to (v+4) | Downlink available bitrate |  |  |  |  |  |  |  |  |
|  | w to (w+4) | Uplink available bitrate |  |  |  |  |  |  |  |  |
|  | x to (n+4) | These octet(s) is/are present only if explicitly specified |  |  |  |  |  |  |  |  |

Figure 8.2.171-1: QoS Monitoring Measurement

The following flags are coded within Octet 5:

-	Bit 1 – DLPD (Downlink Packet Delay): If this bit is set to "1", then the Downlink packet delay field shall be present, otherwise the Downlink packet delay field shall not be present.

-	Bit 2 – ULPD (Uplink Packet Delay): If this bit is set to "1", then the Uplink packet delay field shall be present, otherwise the Uplink packet delay field shall not be present.

-	Bit 3 – RPPD (Round Trip Packet Delay): If this bit is set to "1", then the Round trip packet delay field shall be present, otherwise the Round trip packet delay field shall not be present.

-	Bit 4 – PLMF (Packet Delay Measurement Failure): If this bit is set to "1", this indicates no timestamp is received in uplink packet for a delay exceeding the Measurement Period.

-	Bit 5 – DLCI (Downlink Congestion Info): If this bit is set to "1", then the Downlink Congestion information field shall be present, otherwise the Downlink Congestion information field shall not be present.

-	Bit 6 – ULCI (Uplink Congestion Info): If this bit is set to "1", then the Uplink Congestion information field shall be present, otherwise the Uplink Congestion information field shall not be present.

-	Bit 7 – DLDR (Downlink Data Rate): If this bit is set to "1", then the Average downlink data rate field shall be present, otherwise it shall not be present.

-	Bit 8 – ULDR (Uplink Data Rate): If this bit is set to "1", then the Average uplink data rate field shall be present, otherwise it shall not be present.

The following flags are coded within Octet 6:

-	Bit 1 – DLAB (Downlink Available Bitrate): If this bit is set to "1", then the downlink available bitrate shall be present, otherwise it shall not be present.

-	Bit 2 – ULAB (Uplink Available Bitrate): If this bit is set to "1", then the uplink available bitrate shall be present, otherwise it shall not be present.

-	Bits 3 to 8, Spare, for future use, set to zero.

At least one bit shall be set to "1". Several bits may be set to "1".

When the PLMF flag is set to "1", the RP, UL and DL bits shall not be set to "1", i.e. the DL/UL/Round Trip packet delay fields shall not be present when reporting a packet delay measurement failure.

The Downlink packet delay, Uplink packet delay and Round trip packet delay fields shall be encoded as an Unsigned32 binary integer value. They shall contain the downlink, uplink or round trip packet delay in milliseconds respectively.

The Downlink Congestion information field and the Uplink Congestion information field shall be encoded as an Unsigned16 binary integer value, in the (decimal) values range 0 to 10000 (included), representing the percentage of congestion level in the downlink and in the uplink directions respectively, up to two decimal points, for the QoS flow. As an example, the (decimal) value 9574 corresponds to a percentage of 95.74%.

The Average downlink data rate and the Average uplink data rate fields shall be encoded as kilobits per second (1 kbps = 1000 bps) in binary value (rounded upwards).

3GPP TS 23.502 [83].

* * * Next Change * * * *

* * * End of Change * * * *