3GPP TSG-CT WG4 Meeting #131	C4-254111

# Sophia Antipolis, France; 13th – 17th October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.010 | CR | 0142 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Cause mapping in Inter-RAT Handover |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-09-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Table 6.2.2 requires the S4-SGSN to map (Handover Required) S1AP cause values other than those listed in the table to the (Relocation Request) RANAP cause "Resource Optimized Relocation" but there is no such cause defined in the RANAP specification. Likewise, Table 7.2.3 requires the AMF to map (Handover Required) NGAP cause values other than those listed in the table to the (Handover Request) S1AP cause "Resource Optimized Relocation" but there is no such cause defined in the S1AP specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The two tables are corrected to map (Handover Required) "Any other cause" to the (Relocation Request) RANAP cause "Resource Optimisation Relocation" and to the (Handover Request) S1AP cause "Resource Optimization Handover". |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 29.010 requirements cannot be implemented. Inconsistent RAN3 and CT4 specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2, 7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * *1st Change * * * *

## 6.2	Interworking between RANAP protocol messages and S1AP protocol messages

This clause defines a mapping of RANAP and S1AP cause values used in connection with inter RAT Handover to avoid different mappings in different implementations.

Inter RAT Handover from UTRAN to LTE:

The table 6.2.1 defines a cause value mapping performed by the S4-SGSN when the S4-SGSN receives the Forward Relocation Response message from the MME. This mapping is only needed if the inter RAT Handover failed in LTE.

Table 6.2.1: Cause value mapping from S1AP Cause to RANAP Cause for failure case

| 36.413 [21] |  | 25.413 [7] |  |
| --- | --- | --- | --- |
| HANDOVER FAILURE (S1AP Cause) |  | RELOCATION PREPARATION FAILURE (RANAP Cause) |  |
| Group | Value | Group | Value |
| Radio Network Layer Cause | No Radio Resources Available in Target Cell | Radio Network Layer Cause | No Radio Resources Available in Target Cell |
| Radio Network Layer Cause | Encryption and/or integrity protection algorithms not supported | Radio Network Layer Cause | Requested Ciphering and/or Integrity Protection algorithms not supported |
| Miscellaneous Cause | O&M Intervention | Miscellaneous Cause | O&M Intervention |
| Any other value |  | Radio Network Layer Cause | Relocation Failure in Target CN/RNC or Target System |

Inter RAT Handover from LTE to UTRAN:

Table 6.2.2 defines a cause value mapping performed by the S4-SGSN when the S4-SGSN receives the Forward Relocation Request message from the MME.

Table 6.2.2: Cause value mapping from S1AP Cause to RANAP Cause

| 36.413 [21] |  | 25.413 [7] |  |
| --- | --- | --- | --- |
| HANDOVER REQUIRED (S1AP Cause) |  | RELOCATION REQUEST (RANAP Cause) |  |
| Radio Network Layer Cause | Handover Desirable for Radio Reasons | Radio Network Layer Cause | Relocation Desirable for Radio Reasons |
| Radio Network Layer Cause | Time Critical Handover | Radio Network Layer Cause | Time critical Relocation |
| Radio Network Layer Cause | Reduce Load in Serving Cell | Radio Network Layer Cause | Reduce Load in Serving Cell |
| Radio Network Layer Cause | CS Fallback Triggered | Radio Network Layer Cause | CS Fallback triggered |
| Any other value |  | Radio Network Layer Cause | Resource Optimisation Relocation |

* * * 2nd Change * * *

## 7.2	Interworking between S1AP protocol messages and NGAP protocol messages

This clause defines a mapping of S1AP and NGAP cause values used in connection with inter RAT Handover to avoid different mappings in different implementations.

Inter RAT Handover from LTE to NG-RAN:

Table 7.2.1 defines a cause value mapping performed by the AMF when the AMF receives the Forward Relocation Request message from the MME.

Table 7.2.1: Cause value mapping from S1AP Cause to NGAP Cause

| 36.413 [21] |  | 38.413 [26] |  |
| --- | --- | --- | --- |
| HANDOVER REQUIRED (S1AP Cause) |  | HANDOVER REQUEST (NGAP Cause) |  |
| Group | Value | Group | Value |
| Radio Network Layer Cause | Time critical Handover | Radio Network Layer Cause | Time critical handover |
| Radio Network Layer Cause | Resource Optimisation Handover | Radio Network Layer Cause | Resource optimisation handover |
| Radio Network Layer Cause | Reduce Load in Serving Cell | Radio Network Layer Cause | Reduce load in serving cell |
| Any other value |  | Radio Network Layer Cause | Handover Desirable for Radio Reasons |

For inter RAT handover from LTE served by MME to NG-RAN, the AMF maps the S1AP cause in the GTPv2 Forward Relocation Request message to an NGAP cause as per Table 7.2.1. The AMF sends this NGAP cause in the NGAP Handover Request message to the NG-RAN.

The table 7.2.2 defines a cause value mapping performed by the AMF when the AMF receives the NGAP Handover Failure message from the NG-RAN. This mapping is only needed if the inter RAT Handover failed in NG-RAN.

Table 7.2.2: Cause value mapping from NGAP Cause to S1AP Cause for failure case

| 38.413 [26] |  | 36.413 [21] |  |
| --- | --- | --- | --- |
| HANDOVER FAILURE (NGAP Cause) |  | HANDOVER PREPARATION FAILURE (S1AP Cause) |  |
| Group | Value | Group | Value |
| Radio Network Layer Cause | No Radio Resources Available in Target Cell | Radio Network Layer Cause | No Radio Resources Available in Target Cell |
| Radio Network Layer Cause | Encryption and/or integrity protection algorithms not supported | Radio Network Layer Cause | Encryption and/or integrity protection algorithms not supported |
| Radio Network Layer Cause | Insufficient UE Capabilities | Radio Network Layer Cause | Insufficient UE Capabilities |
| Miscellaneous Cause | O&M Intervention | Miscellaneous Cause | O&M Intervention |
| Any other value |  | Radio Network Layer Cause | Handover Failure In Target EPC/eNB Or Target System |

For inter RAT handover from LTE served by MME to NG-RAN, if the handover fails in the NG-RAN access, the AMF maps the NGAP cause in the NGAP Handover Failure message to a S1AP cause by using the mapping in Table 7.2.2. The AMF uses the S1AP cause in the GTPv2 Forward Relocation Response message to the MME.

Inter RAT Handover from NG-RAN to LTE:

The table 7.2.3 defines a cause value mapping performed by the AMF when the AMF receives the HANDOVER REQUIRED from the NG-RAN.

Table 7.2.3: Cause value mapping from NGAP Cause to S1AP Cause

| 38.413 [26] |  | 36.413 [21] |  |
| --- | --- | --- | --- |
| HANDOVER REQUIRED (NGAP Cause) |  | HANDOVER REQUEST (S1AP Cause) |  |
| Radio Network Layer Cause | Handover Desirable for Radio Reasons | Radio Network Layer Cause | Handover Desirable for Radio Reasons |
| Radio Network Layer Cause | Time Critical Handover | Radio Network Layer Cause | Time critical Handover |
| Radio Network Layer Cause | Reduce Load in Serving Cell | Radio Network Layer Cause | Reduce Load in Serving Cell |
| Any other value |  | Radio Network Layer Cause | Resource Optimisation Handover |

For inter RAT handover from NG-RAN to LTE served by MME, the AMF maps the NGAP cause in NGAP Handover Required message to a S1AP cause by using the mapping in Table 7.2.3. The AMF uses this S1AP cause in the GTPv2 Forward Relocation Request message to the MME.

Table 7.2.4 defines a cause value mapping performed by the AMF when the AMF receives the Forward Relocation Response message from the MME. This mapping is only needed if the inter RAT Handover failed in LTE.

Table 7.2.4: Cause value mapping from S1AP Cause to NGAP Cause for failure case

| 36.413 [21] |  | 38.413 [26] |  |
| --- | --- | --- | --- |
| HANDOVER FAILURE (S1AP Cause) |  | HANDOVER PREPARATION FAILURE (NGAP Cause) |  |
| Group | Value | Group | Value |
| Radio Network Layer Cause | No Radio Resources Available in Target Cell | Radio Network Layer Cause | No Radio Resources Available in Target Cell |
| Radio Network Layer Cause | Encryption and/or integrity protection algorithms not supported | Radio Network Layer Cause | Encryption and/or integrity protection algorithms not supported |
| Radio Network Layer Cause | No Radio Resources Available in Target Cell | Radio Network Layer Cause | No Radio Resources Available in Target Cell |
| Radio Network Layer Cause | Insufficient UE Capabilities | Radio Network Layer Cause | Insufficient UE Capabilities |
| Miscellaneous Cause | O&M Intervention | Miscellaneous Cause | O&M Intervention |
| Any other value |  | Radio Network Layer Cause | Handover failure in target 5GC/ NG-RAN node or target system |

For inter RAT handover from NG-RAN to LTE served by MME, if the handover fails in the LTE access, the AMF maps the S1AP cause received in the GTPv2 Forward Relocation Response message to an NGAP cause as per Table 7.2.4. The AMF sends this NGAP cause in the NGAP Handover Preparation Failure message to the NG-RAN.

* * * End of Changes * * *