3GPP TSG-CT WG4 Meeting #131	C4-254121

# Sophia Antipolis, France; 13th – 17th October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.564 | CR | 0150 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to UPF measurements reporting per RAT Type |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | PAIDC_UPF |  |  |  |  |  | Date: |  |  | 2025-09-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The Nupf_EE service has been enhanced in Rel-19 to support reporting of the RAT type in event reports (including monitoring, and starting new measurements upon, changes of RAT type) and to skip measurement reports for UEs not camping on the RAT Types requested in the UPF event subscription. A new feature should be defined for these functional enhancements for the following reasons: - Using feature control consistently provides clarity in the specification about what was added and when. - Support of these enhancements should be optional (like new features added to specifications). - Without feature control, the NF service consumer (NFc) cannot know whether the UPF supports the enhancements, which may cause suboptimal behavior, e.g.: - NFc unnecessarily including the list of RAT types for event(s) of a UPF event exposure subscription, for a UPF not supporting the enhancements. - NFc unnecessarily requesting to modify the UPF event exposure subscription to modify the list of requested RAT types, for a UPF not supporting the enhancements. - Without feature control, the UPF may include the RAT type in measurement reports, including possibly multiple NotificationItems for the same event but measured over different RAT types during a same reporting period, towards a legacy NF service consumer which does not support so. Besides, a few editorials are proposed (e.g. simplifying and improving the text, using normative language instead of descriptive text, typos). |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new feature is defined for the functional enhancements defined for RAT Type reporting and filtering. Editorial corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification lacks clarity on which functional enhancements are added and when. Sub-optimal behavior/signaling due to NF service consumer being unaware of whether the enhancements are supported by the UPF. Possible interoperabilty issues. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.2.2, 6.1.6.2.3, 6.1.6.2.13, 6.1.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR does not cause any OpenAPI change. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * 1st Change * * *

##### 5.2.2.2.2	Creation of a subscription

An NF Service Consumer shall invoke the Subscribe service operation towards the UPF to create a subscription to monitor at least one UPF event. The NF Service Consumer may subscribe to multiple events in a subscription. A subscription may be associated with one UE's PDU session or with any UE.

The NF Service Consumer shall request to create a new subscription by using the HTTP method POST with the URI of the subscriptions collection, see clause 6.1.3.2.

Figure 5.2.2.2.2-1 Subscription creation

1.	The NF Service Consumer shall send a POST request to create a subscription resource in the UPF. The content of the POST request shall contain a representation of the individual subscription resource to be created.

The NF Service Consumer shall include the following information in the HTTP message body:

-	NF ID, indicating the identity of the network function instance creating the subscription;

-	Target of Event Reporting, indicating the target(s) to be monitored, i.e.

-	a specific PDU Session of a UE identified with a (5GC assigned) UE IP address for an IP PDU session type;

-	a specific PDU Session of a UE identified with a SUPI (and S-NSSAI/DNN) for a non-IP PDU session type if the SMF allows the UPF to receive the SUPI associated with a N4 session based on the local configuration; or

-	any UE (identified by the "anyUE" flag);

-	List of UPF events requested to be subscribed;

-	Type of measurement, for UPF events supporting multiple types of measurement, e.g. for a subscription to the UserDataUsageMeasures event;

-	Event Reporting Mode, indicating how the events shall be reported (One-time Report, Periodic Report or Continuous Report);

-	UPF event consumer notification URI, indicating the address where to send the event notifications generated by the subscription;

-	Public IPv4 or IPv6 address of the remote end, for a subscription to "UE_NAT_MAPPING_INFO" event; and

-	the bundledEventNotifyUri set to the URI to be used by the UPF for sending notification requests including bundled event reports of multiple UPF event subscriptions (see clause 5.2.2.3.4), if the bundlingAllowed IE is set to true and the NF service consumer provides a distinct eventNotifyUri per UPF event subscription.

The NF Service Consumer may include the following information in the HTTP message body:

-	The S-NSSAI and/or the DNN of PDU sessions to which the subscription applies;

-	either one or more Application ID(s) or traffic filters identifying the traffic to be monitored by the subscription;

-	Granularity of Measurement, indicating that the granularity of the required measurements is per PDU Session, per data flow or per application;

-	Reporting period, defining the period for periodic reporting;

-	Skip Reporting Instruction, instructing the UPF to skip sending event reports when certain conditions are met as described in clause 5.2.2.3.3;

-	Maximum number of reports, defining the maximum number of reports after which the event subscription ceases to exist;

-	Expiry time, suggested by the NF Service Consumer representing the time up to which the subscription is desired to be kept active and the time after which the subscribed event(s) shall stop generating reports;

-	Reporting suggestion information, i.e. Report urgency indicating whether the event report can be delayed (i.e. it is delay-tolerant) and if so, the Reporting time information defining the last valid reporting time for the UPF to report the detected event;

-	Deactivate notification flag, indicating that the notification of the available events shall be muted until the event consumer NF (e.g. NWDAF or DCCF) provides the retrieval notification flag to retrieve the stored events;

-	Immediate Report Flag per event, indicating an immediate report to be generated with the current event status;

-	Notification Correlation ID, indicating the correlation identity to be signalled in the event notifications generated by the subscription;

-	Sampling ratio, defining the random subset of PDU sessions among target PDU sessions, in which case the UPF shall only report the event(s) related to the selected subset of PDU sessions;

-	partitioningCriteria, defining the criteria for partitioning PDU sessions before applying the sampling ratio;

-	Muting Exception Instructions, which specify instructions to apply to the subscription and the stored events when an exception occurs at the UPF while the event is muted (e.g., the buffer of stored event reports is full, or the number of stored event reports exceeds a certain number), if the EEMM feature is supported (see clause 6.1.8);

-	Port number of the remote end, for a subscription to "UE_NAT_MAPPING_INFO" event;

-	IP domain, for a subscription to "UE_NAT_MAPPING_INFO" event;

-	the subTerminationReportInd IE set to true to indicate a request to send a notification when the subscription is terminated;

-	the remainingDataReports IE per subscribed event requesting to discard or to send the remaining data for the related event to the NF service consumer, if the subscription is terminated at the UPF (e.g. when the PFCP session of the PDU session that is the target of the subscription is released) and the UPF has not yet reported the collected data for the related event to the consumer;

-	the bundlingAllowed IE set to true to allow the UPF to bundle event reports generated for this subscription with event reports generated for other subscriptions in a same Nupf_EventExposure_Notify request message (see clause 5.2.2.3.4), if the UPF supports the BERMS feature (see clause 6.1.8); and/or

-	the bundleId IE containing a Bundle Identifier identifying the subscriptions whose event reports may be bundled together (see clause 5.2.2.3.4), if the bundlingAllowed IE is set to true and if the NF service consumer only allows to bundle event reports of specific subscriptions, e.g. if the NWDAF wishes to only bundle event reports generated for a same analytic service request.

-	the inclRatType IE per subscribed event requesting the UPF to include the RAT type in the corresponding event reports (i.e. in NotificationItem), if the UPF supports the RATTE feature (see clause 6.1.8).

-	the RatTypeList IE per subscribed event, indicating the list of RAT types for which the NF service consumer requires to receive event reports, if the UPF supports the RATTE feature (see clause 6.1.8). 

If the RatTypeList IE is received and the UPF supports the RATTE feature, the UPF shall send event reports for the PDU session(s) of UE(s) camping on these RAT types and the UPF may skip sending measurement reports for the PDU session(s) of UE(s) not camping on these RAT types.

NOTE:	If the UPF supports the RATTE feature and if the UPF does not skip sending measurement reports for UEs not camping on the RAT types requested in the subscription, the NF service consumer can use the RatType IE included in the event reports to filter measurements reported for different RAT types than those requested in the subscription.

2a.	On success (i.e. if the request is accepted), the UPF shall include a HTTP Location header to provide the location of the newly created resource (subscription) together with the status code 201 in the response message indicating that the requested resource is created.

If the NF Service Consumer has included more than one events in the event subscription and some of the events cannot be subscribed, the UPF shall accept the request and provide the successfully subscribed event(s) in the CreatedEventSubscription.

If the NF Service Consumer has included the Immediate Report Flag with the value true in the event subscription, and if the current status of the events subscribed are available, the UPF shall include the current status of the events subscribed in the response. Otherwise, the UPF shall generate reports for the events and notify the NF service consumer using the Nupf_EventExposure_Notify service operation. If the events with the Immediate Report Flag set to true are subscribed via an SMF, the notification shall be sent to the actual NF service consumer directly, i.e. the current status of the events subscribed shall not be included in the subscription creation response.

If the NF Service Consumer has set the event reporting option to ONE_TIME and if the UPF has included the current status of the events subscribed in the response, then the UPF shall not do any subsequent event notification for the corresponding events.

The response, based on operator policy and taking into account the expiry time included in the request, may contain the expiry time, as determined by the UPF, after which the subscription becomes invalid. Once the subscription expires, if the NF Service Consumer wants to keep receiving notifications, it shall create a new subscription in the UPF. The UPF shall not provide the same expiry time for many subscriptions in order to avoid all of them expiring and recreating the subscription at the same time. If the expiry time is not included in the response, the NF Service Consumer shall consider the subscription to be valid without an expiry time.

If the sampling ratio ("sampRatio") attribute is included in the subscription without a partitioningCriteria, the UPF shall select a random subset of PDU sessions among target PDU sessions according to the sampling ratio and only report the event(s) related to the selected subset of PDU sessions. If the partitioningCriteria attribute is also included along with sampling ratio, the UPF shall apply the sampling ratio on the group of PDU sessions determined according to the partitioning criteria.

If the "notifFlag" attribute is included and set to "DEACTIVATE" in the request by e.g. the NWDAF or DCCF, the UPF shall mute the event notification and store the available events. Additionally, if the UPF supports the EEMM feature (see clause 6.1.8) and if the NF service consumer includes event muting instructions in the request, the UPF should evaluate the received event muting instructions against to local actions (if configured) and, if the subscription creation request is accepted, the UPF may indicate the following information to the NF service consumer in the response:

-	the maximum number of notifications that the UPF expects to be able to store for the subscription;

-	an estimate of the duration for which notifications can be buffered.

2b.	On failure or redirection, one of the HTTP status code listed in Table 6.1.3.2.3.1-3 shall be returned. For a 4xx/5xx response, the message body shall contain a ProblemDetails structure with the "cause" attribute set to one of the application error listed in Table 6.1.3.2.3.1-3.

If the UPF supports the EEMM features (see clause 6.1.8), the NF service consumer sets the "notifFlag" attribute to "DEACTIVATE" and event muting instructions in the request, but the UPF cannot accept the received instructions, the UPF may reject the request with a 403 Forbidden response and the application error "MUTING_EXC_INSTR_NOT_ACCEPTED".

For a subscription request targeting a PDU session, if the UPF cannot find a unique PDU session due to no DNN and/or S-NSSAI being received in the request, the UPF shall reject the request with a 403 Forbidden response and the application error "REJECTION_DUE_TO_NO_DNN_SNSSAI" (see clause 4.4.1.2 of 3GPP TS 23.502 [3]).

* * * Next Change * * *

##### 6.1.6.2.3	Type: NotificationItem

Table 6.1.6.2.3-1: Definition of type NotificationItem

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| eventType | EventType | M | 1 | The event type of the event for which the notification is generated. |  |
| ueIpv4Addr | Ipv4Addr | C | 0..1 | IPv4 address of the UE (NOTE 1, NOTE 3, NOTE 5) |  |
| ueIpv6Prefix | Ipv6Prefix | C | 0..1 | IPv6 address prefix of the UE (NOTE 1, NOTE 3, NOTE 5) |  |
| ueMacAddr | MacAddr48 | O | 0..1 | MAC address of the UE. (NOTE 2, NOTE 3) |  |
| dnn | Dnn | O | 0..1 | When present, this attribute indicates the DNN of the PDU session for which the notification is generated. |  |
| snssai | Snssai | O | 0..1 | When present, this attribute indicates the S-NSSAI of the PDU session for which the notification is generated. |  |
| gpsi | Gpsi | O | 0..1 | When present, this attribute indicates the GPSI of the UE for which the notification is generated. |  |
| supi | Supi | O | 0..1 | Subscription Permanent Identifier |  |
| timeStamp | DateTime | M | 1 | The value represents the UTC time when the information in this report was generated. |  |
| startTime | DateTime | O | 0..1 | When present, this attribute shall provide the timestamp when the information measured for generating this report was started. |  |
| ratType | RatType | C | 0..1 | This IE shall be present if the NF service consumer requested the UPF to include the RAT Type in the event reports and if the RAT type is available in the UPF for the PFCP session. When present, it shall contain the RAT Type on which the UE is camping when the measurement is performed. | RATTE |
| qosMonitoringMeasurement | QosMonitoringMeasurement | C | 0..1 | This attribute shall be present if eventType is set to "QOS_MONITORING". |  |
| userDataUsageMeasurements | array(UserDataUsageMeasurements) | C | 1..N | This IE shall be present if eventType is set to "USER_DATA_USAGE_MEASURES" or "USER_DATA_USAGE_TRENDS". |  |
| tscMngtInfo | TscManagementInfo | C | 0..1 | This attribute shall be present if eventType is set to "TSC_MNGT_INFO". |  |
| UeNatMappingInfo | UeNatMappingInfo | C | 0..1 | This IE shall be present if eventType is set to "UE_NAT_MAPPING_INFO" (NOTE 4) |  |
| handlingOfPayloadHeaderInfo | array(HandlingOfPayloadHeader) | C | 1..N | This attribute shall be present if eventType is set to "HANDLING_OF_PAYLOAD_HEADERS_INFO". When present, it shall contain a list of applied header handling actions. |  |
| terminationCause | TerminationCause | O | 0..1 | This IE may be present if the eventType is set to "SUBSCRIPTION_TERMINATION". When present, this IE shall indicate the reason for the subscription termination. |  |
| skippedReportInfo | array(SkipReportingCondition) | O | 1..N | This IE may be present if prior to the event report, there were some event reports for this event were skipped due to one or more of the following reasons: - "SKIP_NULL_REPORT" - "SKIP_OUTSIDE_VALIDITY_TIME" - "SKIP_BELOW_THRESHOLD_REPORT". | SEER |
| NOTE 1: At least one of ueIpv4Addr and ueIpv6Prefix shall be present if the subscription applies to an IP PDU session. NOTE 2: An NF service consumer subscribing to receive QoS Monitoring Measurement report for an ethernet PDU session shall accept the NotificationItem having neither ueIpv4Addr nor ueIpv6Prefix. NOTE 3: At least one of ueIpv4Addr, ueIpv6Prefix and ueMacAddr shall be present. NOTE 4: The UPF may report a list of NatMapping for the same private UE IP address and the same remote IP address, for different destination port numbers, if no remote port number was received in the subscription. NOTE 5: For a UPF deployed with NAT functionality, this IE shall contain the UE (private) IP address allocated by the 5GC. |  |  |  |  |  |

* * * Next Change * * *

##### 6.1.6.2.13	Type: UpfEvent

Table 6.1.6.2.13-1: Definition of type UpfEvent

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| type | EventType | M | 1 | Describes the UPF event type to be reported |  |
| immediateFlag | boolean | O | 0..1 | Indicates if an immediate event report containing the currently available value / status of the event is requested. The report contains the value / status of the event currently available at the UPF at the time of the subscription. This IE shall be present and set to true if the type IE is set to "UE_NAT_MAPPING_INFO". It may be present otherwise. The default value is false. |  |
| measurementTypes | array(MeasurementType) | C | 1..N | This IE shall be present if the type IE is set to "USER_DATA_USAGE_MEASURES". When present, this IE shall indicate the types of requested measurements. |  |
| appIds | array(ApplicationId) | O | 1..N | Contains the application identifiers. (NOTE 1, NOTE 2) |  |
| trafficFilters | array(FlowInformation) | O | 1..N | Identifies IP or Ethernet packet filters. (NOTE 1, NOTE 2) |  |
| granularityOfMeasurement | GranularityOfMeasurement | O | 0..1 | Indicates the granularity of measurement. (NOTE 2) |  |
| reportingSuggestionInfo | ReportingSuggestionInformation | C | 0..1 | The IE should be present if the event notification can be delayed, i.e. it is delay tolerant. |  |
| remoteIpv4Addr | Ipv4Addr | C | 0..1 | Either this or the remoteIpv6Addr IE shall be present if the type IE is set to "UE_NAT_MAPPING_INFO". When present, this IE shall indicate the remote end IPv4 address for which the NAT mapping is requested. |  |
| remoteIpv6Addr | Ipv6Addr | C | 0..1 | Either this or the remoteIpv4Addr IE shall be present if the type IE is set to "UE_NAT_MAPPING_INFO". When present, this IE shall indicate the remote end IPv6 address for which the NAT mapping is requested. |  |
| remotePortNumber | Uint16 | O | 0..1 | This IE may be present if the type IE is set to "UE_NAT_MAPPING_INFO". When present, this IE shall identify the Port number of the remote end for which the NAT mapping is requested. |  |
| ipDomain | String | C | 0..1 | This IE shall be present if the type IE is set to "UE_NAT_MAPPING_INFO" and the DNN is configured with overlapping UE private address range(s). When present, this IE shall indicate the IP domain of the UE IP address. |  |
| remainingDataReports | RemainingDataReports | O | 0..1 | This IE may be present when subscribing to an event. When present, this IE shall indicate how to handle the data collected for the related event by the source UPF and not yet reported to the NF service consumer, when the subscription is terminated (e.g. when the corresponding N4 session is released). |  |
| skipReportInstruction | SkipReportingInstruction | O | 0..1 | This IE may be present when subscribing to an event. When present, it shall include the conditions under which the UPF shall skip sending event reports, e.g. when event reports would only contain null measurements, or measurements below a specific threshold, or outside the reporting period(s). | SEER |
| inclRatType | boolean | O | 0..1 | This IE may be present and set to true if the NF service consumer requests the UPF to include the RAT type in the corresponding event reports (i.e. in NotificationItem). true: the RAT type on which the UE is camping when the measurement is performed shall be included in the NotificationItem and multiple NotificationItems corresponding to the measurement reports for the same event but measured over different RAT types during a same reporting period may be included in the report. Presence of this IE with false value shall be prohibited. | RATTE |
| ratTypeList | array(RatType) | O | 1..N | This IE may be present if the inclRatType is set to true. When present, it shall indicate the list of RAT types for which the NF service consumer requires to receive the event reports. If this IE is present, the UPF shall send event reports for the PDU session(s) of UE(s) camping on these RAT types and the UPF may skip sending measurement reports for the PDU session(s) of UE(s) not camping on these RAT types. | RATTE |
| NOTE 1: Either the appIds IE or the trafficFilters IE may be present, not both. NOTE 2: If the appIds or trafficFilters is provided, the granularityOfMeasurement shall not be set to "PER_SESSION". If neither appIds nor trafficFilters is provided, the granularityOfMeasurement may be set to "PER_SESSION", "PER_APPLICATION" or "PER_FLOW" to request the UPF to provide measurements with the corresponding granularity. |  |  |  |  |  |

* * * Next Change * * *

### 6.1.8	Feature negotiation

The optional features listed in table 6.2.8-1 are defined for the Nupf_EventExposure API. They shall be negotiated using the extensibility mechanism defined in clause 6.6 of 3GPP TS 29.500 [4].

Table 6.1.8-1: Supported Features

| Feature number | Feature Name | M/O | Description |
| --- | --- | --- | --- |
| 1 | EEMM | O | Event Exposure Muting Mechanism A UPF supporting this feature shall support the handling of event muting exception instructions as specified in clause 6.2.7.2 of 3GPP TS 23.288 [17]. |
| 2 | SEER | O | Skip Event Exposure Reporting An UPF supporting this feature shall support to skip sending event reports as described in clause 5.2.2.3.3. |
| 3 | BERMS | O | Bundling Event Reports of Multiple Subscriptions A NF Service Consumer and a UPF supporting this feature shall support UPF notifying bundled event reports of multiple subscriptions (see clause 5.2.2.3.4). |
| X | RATTE | O | RAT Type information Exposure An NF service consumer and a UPF supporting this feature shall support measurements reporting per RAT type as defined in clauses 5.2.2.2.2, 6.1.6.2.3 and 6.1.6.2.13. |

* * * End of Changes * * *